| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June –1 July 2020 | Document: JVET-S0003-v1 |
| --- | --- |

| Title: | AHG report: Test model software development (AHG3) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Frank Bossen Xiang Li Karsten Suehring | Tel: Email: | frank@bossentech.com xlxiangli@tencent.com karsten.suehring@hhi.fraunhofer.de |
| Source: | AhG3 on Test model software development |  |  |

_____________________________

# Abstract

This report summarizes the activities of the AhG3 on Test model software development that has taken place between the 18th and 19th JVET meetings.

# Introduction

The mandates given to the AHG are:

- Coordinate development of test model (VTM) software and associated configuration files.

- Produce documentation of software usage for distribution with the software.

- Discuss and make recommendations on the software development process.

- Propose improvements to the guideline document for developments of the test model software.

- Perform tests of VTM behaviour relative to HEVC and previous VTM using the VTM common test conditions.

- Coordinate with AHG on Draft text and test model algorithm description editing (AHG2) to identify any mismatches between software and text, and make further updates and cleanups to the software as appropriate.

- Coordinate with AHG6 for integration with 360lib software.

The software development continued on the GitLab server. VTM version 8.2 was tagged on Apr. 27, VTM 9.0 on May 26. VTM 9.1 was tagged on Jun. 15. VTM 9.2 is expected during the 19th JVET meeting.

# VTM software development

Development was continued on the GitLab server, which allows participants to register accounts and use a distributed development workflow based on git.

The server is located at:

https://vcgit.hhi.fraunhofer.de

The registration and development workflow is documented at:

https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/wikis/VVC-Software-Development-Workflow

The VTM software can be found at

https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/

VTM 8.2 was tagged on Apr. 27, 2020.

Changes relative to VTM 8.1 include:

- removal of previous cycle macros

After one release candidate, VTM 9.0 was tagged on May 26, 2020.

Changes related to VTM 8.2 include:

- JVET-R0185-SecondRebased: Updating signalling related to OLS DPB parameters in VPS

- JVET-R0058: the combination of RPR, subpictures, and scalability

- Retrained CABAC initial values

- JVET-R0332: Grouping syntax elements in SPS based on slice type

- JVET-Q0397: Scalable nesting SEI for subpictures

- JVET-R0078: cleanup for chroma tc and beta DBF offset in PPS, PH and SH

- JVET-Q0404: add a flag to subpicture info SEI to indicate constant bitrate

- JVET-R0194: Constraint for referencing parameter set from different layer

- JVET-R0099: Proposal 2 and 3, DPB and HRD parameter signaling for OLS

- JVET-R0103 Proposal 1: Decoding unit signaling change

- JVET-R0110: Fix interface of JVET-R0110 of mixed lossy lossless coding

- JVET-R0162 proposal1: change the signaling of wraparound offset to picture width minus wraparound offset

- JVET-R0107: Proposal 2 - VPS signaling change and updated inference rule

- JVET-R0112: Picture header semantics cleanup for gdr_or_irap_flag

- JVET-R0188: signalling of slice width and height

- JVET-R0101: SEI alternative picture timing information inference rules

- JVET-R0110: Mixed lossy/lossless coding

- JVET-R0203: Constraints on allowed NAL unit types for picture with mixed NAL unit types

- JVET-Q0488: Constraint on number of identical SEI messages per picture.

- JVET-R0108 Proposal 1: DCI signaling changes

- JVET-R0214: MMVD syntax modifications

- JVET-Q0764: Wrap around with RPR

- JVET-R0275: sps_ptl_dbp_hrd constraint

- JVET-R0413: HRD timing signalling update

- JVET-Q0394: Apply timing SEI to multiple OLSs

- JVET-R0267: RPL constraint for IDR picture

- JVET-R0191: Aspect 3 – Constraints for value range and referencing of PTL, DPB params, and HRD params in VPS

- JVET-R0286: general constraints flag cleanup

- JVET-R0055: Infer chroma scaling matrix to all 16 in 4:0:0

- JVET-R0166: Scaling list for Chroma 444

- JVET-R0380: Disabling scaling matrix for YCC or RGB

- JVET-R0276: reference picture constraint for reordered sub-pictures

- JVET-R0165: Optional entry point offsets

- JVET-R0201: Prefix and suffix APS cleanups

- JVET-R0091: constraint on slice orders

- JVET-R0277 items 1 and 2: Modified condition for the number of active entries override flag, inference for slice_collocated_from_l0_flag

- Fix for re-opened ticket #1005: special case for single_slice_per_subpic_flag

- JVET-R0161 proposal 2: skip PTL index signaling when PTL structure number is equal to OLS number

- JVET-R0097: SPS clean-up of maximum transform size signaling

- JVET-R0278: ph_inter_slice_allowed_flag constraint

- JVET-R0483 Combination 4: Conditional signalling of slice_ts_residual_coding_disabled_flag

- JVET-R0437: fix the DBF bS derivation for palette mode

- JVET-R0130: Bugfix for tC derivation used in chroma deblocking filter.

- JVET-R0130 on cleanup of tC derivation

- JVET-R0324: Modify PH syntax condition on ph_collocated_from_l0_flag, mvd_l1_zero_flag, ph_disable_bdof_flag, and ph_disable_dmvr_flag

- JVET-R0094: DPB output temporal sublayers offsets

- JVET-R0090: VUI syntax

- JVET-R0266: code PPS ID with u(6) and VB position with ue(v)

- JVET-R0225: Use two separate flags for ALF chroma PH/SH control

- JVET-R0113: Picture Parameter Set Cleanup

- JVET-R0100 Proposal 1: Signalling and inference rule for duiSubLayerDelaysPresentFlag

- JVET-R0334: Disable chroma palette for local dual tree

- JVET-R0045: cleaup for signalling minQP for transform skip

- JVET-R0205: inter_layer_ref_pics_present_flag

- JVET-R0247: Skip pps_loop_filter_across_slices_enabled_flag when the picture contains one slice

- JVET-R0186 aspect 1: signal pps_num_subpics_minus1 depending on pps_no_pic_partition_flag

- JVET-R0327: One-pass CCALF

- JVET-R0114: Allow negative scaling window offsets

- JVET-R0098: Only signall LMCS and explicit scaling list enable flags in SH when PH is not in SH

- JVET_R0330: Remove redundant clipping in chroma residual scaling factor derivation

- JVET-R0059 Aspect 2: condition the signalling of ltrp_in_header_flag[ listIdx ][ rplsIdx ]

- JVET-R0202: Add PH in SH constraints

- JVET-R0347: Set upper limit of minQtSize and maxTtSize to 64, set upper limit...

- JVET-R0083: Fix addressing ticket #859 for Q0089 related DQ and TS interaction issue.

- JVET-R0350: MIP for chroma in case of 4:4:4 format and single tree

- JVET-R0371: set the range of max number of subblock based merge candidate to 0 to 5 – sps_sbtmvp

- JVET-R0200: Move SH flags for LMCS and scaling lists

- JVET-R0164: Use a mean scaled version of SATD in encoder decisions

- JVET-R0388: cleanups on deblocking signalling

- JVET-R0233 method 2: skip CCALF between luma and chroma ALF VBs for 4:2:2 and 4:4:4 sequences.

- JVET-R0131: Revert JVET-Q0471

- JVET-R0071 item 1, R0156 item 4, R0284 item 1

- JVET-R0210 section 3.3: conditional signalling of number of tiles in slice

- JVET-R0071 Items 2-4: Inference of subpic_treated_as_pic_flag, loop_filter_across_subpic_enabled_pic_flag and single_slice_per_subpic_flag

- JVET-R0156 proposal 3, JVET-R0170, JVET-R0222 proposal 2

- JVET-R0271: Slice level DQ and SDH granularity for mixed lossy and lossless coding.

- JVET-R0143: Disable TSRC for lossless coding via configuration parameter

- Fix sps_virtual_boundaries_present_flag compliance check accordingly to spec

- JVET-R0232 section 3.2: CCALF APS constraint

- JVET-R0208: Rounding offset fix for ALF virtual boundary processing

- JVET-R0468: Use single tree in I slice for RGB configuration setting

- Software manual updates

- various bug fixes and cleanups

VTM 9.1 was tagged on Jun. 15, 2020.

Changes relative to VTM 9.0 include:

- JVET-R0041: Picture types related constraints

- JVET-R0064: Move scaling_matrix_for_lfnst_disabled_flag from the scaling_list_data( ) syntax to the SPS

- various bug fixes

VTM 9.2 is expected to be tagged during the 18th JVET meeting. Changes include so far:

- JVET-R0042: SubPicture types related constraints

- JVET-Q0398: Sublayer Dependency (max_tid_il_ref_pics)

- JVET-R0066: Signal ols_dpb_chroma_format[ i ] and ols_dpb_bitdepth_minus8[ i ] in the VPS,...

- JVET-P0359: Implementation of the parameter sets inclusion indication SEI message

- JVET-R0062: Changes on uniform tile and rectangular slice partitioning

- JVET-R0068: Aspect 1 - slice_type constraint and Aspect 6 - signalling of conformance window parameters

- JVET-R0067: Update the derivation of PictureOutputFlag

- JVET-R0101 Proposal 2: Bug fix for signalling some syntax elements length

- JVET-R0041: Picture types related constraints

- JVET-R0065: Add a flag aud_irap_or_gdr_au_flag to the AUD, and check the completeness of the first AU of a CVS

- JVET-R0122: Conditional signaling of PicOutputFlag depending on non_ref_pic_flag

- JVET-R0351: High bit depth coding

- JVET-R0118: Constraint flag on PH in SH

- JVET-R0184: Wraparound for Subpictures

- JVET-R0080: Change the syntax condition for pps_tile_idx_delta_present_flag

- JVET-N0278: HLS for immersive media delivery and access requirementvarious bug fixes

- various bug fixes and cleanups

# CTC Performance

The following tables show VTM 9.0 performance over HM 16.20:

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over HM 16.20 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -29,10% | -32,37% | -34,36% | 1630% | 190% |
| Class A2 | -29,29% | -23,97% | -21,08% | 2626% | 199% |
| Class B | -21,74% | -26,92% | -30,75% | 2946% | 198% |
| Class C | -22,54% | -18,92% | -22,71% | 4046% | 202% |
| Class E | -25,78% | -26,10% | -24,52% | 2381% | 187% |
| Overall | -25,08% | -25,42% | -26,91% | 2712% | 196% |
| Class D | -18,44% | -13,25% | -13,25% | 4528% | 204% |
| Class F | -39,39% | -39,90% | -42,49% | 5346% | 192% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over HM 16.20 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -38,74% | -37,19% | -44,34% | 884% | 186% |
| Class A2 | -43,13% | -39,74% | -38,35% | 999% | 199% |
| Class B | -34,74% | -46,77% | -44,61% | 935% | 189% |
| Class C | -29,90% | -30,58% | -32,56% | 1212% | 199% |
| Class E |  |  |  |  |  |
| Overall | -35,93% | -39,13% | -40,09% | 1004% | 193% |
| Class D | -27,64% | -26,48% | -26,11% | 1326% | 194% |
| Class F | -41,55% | -44,78% | -46,09% | 689% | 163% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over HM 16.20 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -30,78% | -37,52% | -35,44% | 836% | 171% |
| Class C | -29,11% | -22,39% | -22,49% | 1006% | 183% |
| Class E | -33,10% | -39,76% | -34,51% | 420% | 147% |
| Overall | -30,80% | -33,03% | -30,89% | 749% | 168% |
| Class D | -25,98% | -16,62% | -16,00% | 1024% | 182% |
| Class F | -42,77% | -44,36% | -44,85% | 558% | 145% |
|  |  |  | Low delay P Main10 |  |  |
|  |  |  | Over HM 16.20 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -35,12% | -40,02% | -37,82% | 751% | 174% |
| Class C | -30,84% | -22,48% | -22,76% | 898% | 191% |
| Class E | -35,79% | -43,00% | -37,53% | 402% | 155% |
| Overall | -33,86% | -34,92% | -32,73% | 682% | 174% |
| Class D | -27,43% | -15,80% | -15,13% | 916% | 184% |
| Class F | -42,33% | -43,52% | -43,97% | 581% | 151% |

The following tables show VTM 9.0 performance compared to VTM 8.1:

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-8.1 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1,56% | 9,81% | 7,68% | 99% | 103% |
| Class A2 | -2,29% | 7,40% | 7,12% | 100% | 102% |
| Class B | -0,89% | 8,39% | 9,46% | 99% | 100% |
| Class C | -0,97% | 7,90% | 7,87% | 99% | 99% |
| Class E | -0,78% | 7,21% | 8,15% | 100% | 101% |
| Overall | -1,23% | 8,16% | 8,20% | 99% | 101% |
| Class D | -0,89% | 7,83% | 7,93% | 95% | 100% |
| Class F | -1,05% | 5,97% | 5,79% | 99% | 102% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-8.1 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -2,37% | 13,13% | 11,16% | 100% | 100% |
| Class A2 | -2,90% | 13,87% | 12,49% | 100% | 100% |
| Class B | -1,12% | 15,00% | 15,23% | 98% | 105% |
| Class C | -1,14% | 13,16% | 12,82% | 99% | 104% |
| Class E |  |  |  |  |  |
| Overall | -1,73% | 13,91% | 13,22% | 99% | 103% |
| Class D | -0,94% | 13,25% | 12,87% | 98% | 98% |
| Class F | -1,51% | 8,04% | 8,09% | 98% | 99% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-8.1 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -1,04% | 17,89% | 18,49% | 99% | 101% |
| Class C | -1,37% | 15,24% | 15,35% | 100% | 108% |
| Class E | -0,65% | 16,77% | 16,67% | 102% | 108% |
| Overall | -1,05% | 16,73% | 16,99% | 100% | 105% |
| Class D | -0,91% | 16,28% | 17,43% | 99% | 98% |
| Class F | -1,53% | 9,67% | 8,71% | 100% | 102% |
|  |  |  | Low delay P Main10 |  |  |
|  |  |  | Over VTM-8.1 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -1,02% | 18,65% | 19,14% | 100% | 98% |
| Class C | -1,47% | 15,41% | 15,15% | 100% | 108% |
| Class E | -0,65% | 16,34% | 18,45% | 103% | 109% |
| Overall | -1,08% | 16,99% | 17,64% | 101% | 104% |
| Class D | -0,88% | 17,77% | 18,18% | 98% | 97% |
| Class F | -1,80% | 9,84% | 10,08% | 100% | 103% |

Full results are attached to this AHG report as Excel files.

# Issues encountered during the software implementation process

Several issues were encountered during software development:

- It was not always clear from the list of actions taken from the “R” meeting whether software changes are required for a given action. This made tracking of progress more difficult. It is suggested that, in the future, authors of documents listed for action inform software coordinators of cases where no software changes are required.

- Implementations should be tested on a wider range of configurations, e.g. the example configurations provided with the software. VTM 9.1 could not make significant progress with new implementations because multi-layer decoding was broken in 9.0, when bitstream constraint checks were only tested on single-layer bitstreams.

# Status of implementation of proposals of previous JVET meetings

The following list contains all adoptions of the Q meeting that were not marked as merged or specification only change in the software coordinator tracking sheet:

- JVET-Q0112

- JVET-Q0119

- JVET-Q0764

- JVET-Q0402

- JVET-Q0406

- JVET-Q0443

# Status of proposals of the 18th JVET meeting (Online)

The following list contains all adoptions of the R meeting that were not marked as merged or specification only change in the software coordinator tracking sheet:

- JVET-R0073

- JVET-R0070

- JVET-R0082

- JVET-R0244

- JVET-R0245

- JVET-R0178

- JVET-R0179

- JVET-R0227

- JVET-R0341

- JVET-R0266 proposal 2

- JVET-R0266 proposal 5, JVET-R0178

- JVET-R0433

- JVET-R0189, R0202

- JVET-R0251

- JVET-R0203

- JVET-R0270

- JVET-R0059

- JVET-R0059, JVET-R0323

- JVET-R0297

- JVET-R0264

- JVET-R0295

- JVET-R0249

- JVET-R0068

- JVET-R0092, R0294

- JVET-R0093, R0294

- JVET-R0242

- JVET-R0221

- JVET-R0247

- JVET-R0046

- JVET-R0274

- JVET-R0065

- JVET-R0191

- JVET-R0193

- JVET-R0107 Proposal 3

- JVET-R0296 aspect2

- JVET-R0158

- JVET-R0222 aspect 1

# Bug tracking

The bug tracker for VTM and specification text is located at:

https://jvet.hhi.fraunhofer.de/trac/vvc

The bug tracker uses the same accounts as the HM software bug tracker. Users may need to log in again due to the different sub-domain. For spam fighting reasons account registration is only possible at the HM software bug tracker at

https://hevc.hhi.fraunhofer.de/trac/hevc

Please file all issues related to the VVC reference software into the bug tracker. Try to provide all the details, which are necessary to reproduce the issue. Patches for solving issues and improving the software are always appreciated.

# Recommendations

The AHG recommends to:

- Continue to develop the VTM reference software

- Improve documentation, especially the software manual

- Resolve any normative issues resulting from the large number of integrations in the most recent development cycle

- Encourage people to test VTM software more extensively outside of common test conditions.

- Encourage people to report all (potential) bugs that they are finding.

- Encourage people to submit bit-streams/test cases that trigger bugs in VTM.

- Encourage people to submit non-normative changes that reduce encoder run time without significantly sacrificing compression performance

- Make sure that contributions considered for adoption in the future are subject to adequate text and software review by the JVET at large

- Design and add configuration files to the VTM software for testing of HLS features