| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June –1 July 2020 | Document: JVET-S0007-v1 |
| --- | --- |

| Title: | JVET AHG report: Coding of HDR/WCG Material (AHG7) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | A. Segall E. Francois W. Husak S. Iwamura D. Rusanovskyy | Email: | asegall@sharplbas.com Edouard.Francois@InterDigital.com WJH@dolby.com iwamura.s-gc@nhk.or.jp> dmytror@qti.qualcomm.com |
| Source: | AHG7 |  |  |

_____________________________

# Abstract

This document summarizes the activity of AHG7: Coding of HDR/WCG Material between the 18th meeting by teleconference (14 – 24 April 2020)and the 19th meeting by teleconference (22 June – 1 July 2020).

# Mandates

The AHG was established with the following mandates:

- Study and evaluate available HDR/WCG test content.

- Study objective metrics for quality assessment of HDR/WCG material, including investigation of the correlation between subjective and objective results.

- Compare the performance of the VTM and HM for HDR/WCG content.

- Generate CTC anchors for the VTM according to JVET-P2011 within two weeks of availability of SDR CTC anchors.

- Prepare for expert viewing of HDR content at the next JVET meeting if feasible.

- Coordinate implementation of HDR anchor aspects in the test model software with AHG3.

- Coordinate with AHG4 in preparation for verification testing for HDR video content.

- Study additional aspects of coding HDR/WCG content.

# Activities

The primary activity of the AhG was related to the mandates of (i) study and evaluate available HDR/WCG test content, (ii) comparing the performance of the VTM for HDR/WCG content, (iii) preparing for expert viewing of HDR content at the next JVET meeting, and (iv) coordinating with AHG4 in preparation for verification testing for HDR video content. This work is described in the following subsection.

## Anchor Generation

The AhG generated CTC anchors for the VTM according to JVET-P2011.  A summary of the performance is provided below, and more detailed information may be found in the included XLS data.

### VTM 8.0 versus VTM 7.0

|  | All Intra |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -0.12% | -0.13% | -0.09% | -0.04% | -0.09% | -0.09% | 0.01% | -0.14% | 99% | 87% |
| Class H2 |  |  |  |  |  | -0.09% | -0.09% | -0.17% | 99% | 85% |
| Overall | -0.12% | -0.13% | -0.09% | -0.04% | -0.09% | -0.09% | -0.02% | -0.15% | 99% | 87% |
|  | Random Access |  |  |  |  |  |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -0.22% | -0.04% | -0.06% | -0.04% | -0.74% | -0.06% | -0.04% | -0.60% | 96% | 99% |
| Class H2 |  |  |  |  |  | -0.10% | -0.04% | -0.20% | 97% | 94% |
| Overall | -0.22% | -0.04% | -0.06% | -0.04% | -0.74% | -0.07% | -0.04% | -0.45% | 96% | 97% |

### VTM 7.0 versus HM 16.18

|  | All Intra |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over HM-16.18 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -41.23% | -26.73% | -26.20% | -56.66% | -52.00% | -23.52% | -52.55% | -45.17% |  |  |
| Class H2 |  |  |  |  |  | -21.23% | -47.48% | -49.00% |  |  |
| Overall | -41.23% | -26.73% | -26.20% | -56.66% | -52.00% | -22.69% | -50.70% | -46.56% |  |  |
|  | Random Access |  |  |  |  |  |  |  |  |  |
|  | Over HM-16.18 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -31.57% | -31.46% | -31.17% | -46.14% | -38.90% | -28.14% | -41.07% | -31.22% |  |  |
| Class H2 |  |  |  |  |  | -28.61% | -56.29% | -58.70% |  |  |
| Overall | -31.57% | -31.46% | -31.17% | -46.14% | -38.90% | -28.31% | -46.60% | -41.21% |  |  |

## Tool-On / Tool-Off Comparisons

In addition to evaluating the performance of VTM 9.0, the AhG also studied the performance of individual coding tools in the context of HDR content.  This was accomplished by conducting a Tool-On/Tool-Off test according to the methodology established in AhG13.

Results are summarized in the tables below.  Additionally, more detailed results are provided in the included XLS data.

The AhG would like to thank the following companies for contributing to the Tool-On tests: Alibaba, Dolby, InterDigital, LG, MediaTek, NHK, and Sharp.

### Class H1 (PQ)

Simulation Results for AI (Class H1)

|  |  |  |  |  | AI |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abbreviation | DE100 | PSNR-L | BDR-wY | BDR-wU | BDR-wV | Tester EncTime | Tester DecTime | XChecker EncTime | XChecker DecTime |
| CST | 15.99% | 0.98% | 0.88% | 14.34% | 18.76% | 151% | 99% | 149% | 102% |
| DQ | 0.11% | 1.35% | 1.45% | 0.17% | 0.25% | 94% | 108% | 97% | 104% |
| CCLM | 18.99% | 2.33% | 2.15% | 46.64% | 52.53% | 101% | 99% | 101% | 100% |
| MTS | 1.16% | 1.31% | 1.30% | 0.72% | 0.96% | 86% | 109% | 85% | 100% |
| ALF | 10.13% | 2.75% | 2.18% | 18.37% | 37.13% | 96% | 88% | 96% | 89% |
| MRLP | 0.33% | 0.34% | 0.30% | 0.12% | 0.06% | 98% | 99% | 98% | 100% |
| IBC | -0.27% | -0.32% | -0.32% | -0.15% | -0.03% | 145% | 96% | 183% | 101% |
| ISP | 0.13% | 0.60% | 0.70% | -0.39% | -0.19% | 90% | 100% | 85% | 99% |
| LMCS | 1.07% | 0.85% | 4.27% | 1.63% | 4.54% | 100% | 101% | 101% | 110% |
| BDPCM | 0.00% | 0.01% | 0.00% | -0.13% | -0.05% | 101% | 100% | 108% | 101% |
| MIP | 0.52% | 0.73% | 0.58% | 0.21% | 0.09% | 94% | 98% | 89% | 101% |
| LFNST | 0.95% | 0.91% | 0.81% | 1.27% | 2.23% | 107% | 96% | 105% | 100% |
| JCCR | 0.19% | 0.54% | 0.55% | 0.30% | -1.87% | 99% | 98% | 97% | 101% |
| SAO | 0.94% | 0.07% | 0.00% | 1.12% | 2.35% | 100% | 95% | 99% | 103% |
| CCALF | 7.38% | -0.16% | -0.17% | 13.26% | 33.77% | 98% | 99% | 100% | 98% |

Simulation Results for RA (Class H1)

|  |  |  |  |  | VTM RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abbreviation | DE100 | PSNR-L | BDR-wY | BDR-wU | BDR-wV | Tester EncTime | Tester DecTime | XChecker EncTime | XChecker DecTime |
| CST | 13.74% | 0.54% | 0.45% | 10.53% | 15.03% | 103% | 99% | 106% | 101% |
| DQ | -0.34% | 1.51% | 1.44% | -1.07% | -0.48% | 100% | 103% | 102% | 102% |
| CCLM | 15.43% | 1.10% | 0.99% | 36.08% | 39.94% | 100% | 98% | 102% | 101% |
| MTS | 0.93% | 1.07% | 1.02% | 0.86% | 1.52% | 96% | 105% | 97% | 101% |
| ALF | 11.66% | 1.98% | 1.66% | 17.73% | 46.72% | 95% | 86% | 96% | 91% |
| AFF | 0.93% | 1.12% | 1.11% | 0.63% | 0.58% | 81% | 96% | 83% | 100% |
| SbTMVP | 0.31% | 0.34% | 0.37% | 0.23% | 0.37% | 102% | 98% | 102% | 99% |
| AMVR | 0.97% | 0.72% | 0.74% | 1.38% | 1.70% | 89% | 101% | 88% | 101% |
| GPM | 0.55% | 0.39% | 0.44% | 1.00% | 1.04% | 97% | 99% | 99% | 101% |
| BDOF | 0.77% | 0.93% | 1.00% | 0.45% | 0.25% | 99% | 95% | 100% | 100% |
| CIIP | 0.05% | 0.12% | 0.15% | -0.10% | -0.02% | 99% | 97% | 99% | 101% |
| MMVD | 0.31% | 0.28% | 0.30% | 0.26% | 0.31% | 92% | 101% | 93% | 102% |
| BCW | 0.70% | 0.26% | 0.22% | 0.42% | 0.65% | 97% | 100% | 96% | 103% |
| MRLP | 0.17% | 0.20% | 0.14% | -0.25% | 0.56% | 99% | 98% | 102% | 101% |
| IBC | 0.29% | -0.09% | -0.01% | 0.16% | 0.90% | 103% | 99% | 111% | 101% |
| ISP | 0.32% | 0.43% | 0.45% | -0.02% | 0.71% | 97% | 102% | 99% | 100% |
| DMVR | 1.26% | 1.11% | 0.96% | 1.32% | 1.42% | 102% | 97% | 103% | 99% |
| SBT | 0.15% | 0.15% | 0.37% | 0.06% | 0.08% | 96% | 98% | 98% | 100% |
| LMCS | -0.85% | 0.12% | 4.41% | 0.86% | 4.29% | 100% | 96% | 101% | 100% |
| SMVD | 0.16% | 0.13% | 0.15% | 0.14% | 0.24% | 98% | 99% | 99% | 101% |
| BDPCM | 0.15% | 0.01% | 0.00% | 0.07% | 0.65% | 102% | 100% | 104% | 102% |
| MIP | 0.58% | 0.51% | 0.34% | 0.12% | 0.96% | 98% | 101% | 99% | 101% |
| LFNST | 0.68% | 0.57% | 0.44% | 0.85% | 2.40% | 96% | 102% | 99% | 100% |
| JCCR | -0.14% | 0.36% | 0.35% | -0.60% | -0.97% | 99% | 100% | 101% | 100% |
| SAO | 1.00% | -0.04% | -0.11% | 1.23% | 2.32% | 100% | 97% | 101% | 101% |
| PROF | 0.07% | 0.08% | 0.10% | 0.03% | -0.15% | 95% | 102% | 101% | 100% |
| CCALF | 9.23% | -0.54% | -0.55% | 14.75% | 42.44% | 99% | 99% | 103% | 100% |

Figure 1. wPSNR-Y vs encoding runtime ratio of VTM with VTM tool tests (Class H1)

Figure 2. wPSNR-Y vs decoding runtime ratio of VTM with VTM tool tests (Class H1)

Figure 3. wPSNR-Y vs weighted runtime ratio (a = 6) of VTM with VTM tool tests (Class H1)

### Class H2 (HLG)

Simulation Results for AI (Class H2)

|  |  |  | AI |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Abbreviation | BDR-wY | BDR-wU | BDR-wV | Tester EncTime | Tester DecTime | XChecker EncTime | XChecker DecTime |
| CST | 0.61% | 13.02% | 19.42% | 163% | 107% | 159% | 104% |
| DQ | 1.69% | 0.67% | 0.82% | 99% | 104% | 99% | 106% |
| CCLM | 1.80% | 26.99% | 17.03% | 103% | 101% | 101% | 100% |
| MTS | 1.87% | 2.62% | 2.22% | 87% | 98% | 86% | 99% |
| ALF | 2.70% | 18.55% | 20.85% | 97% | 89% | 95% | 89% |
| MRLP | 0.04% | -0.04% | 0.12% | 99% | 101% | 98% | 101% |
| IBC | -0.11% | -0.03% | -0.01% | 208% | 101% | 184% | 101% |
| ISP | 0.31% | -0.65% | -0.09% | 85% | 99% | 85% | 100% |
| LMCS | 0.06% | -0.99% | -0.73% | 100% | 101% | 100% | 101% |
| MIP | 0.72% | 0.75% | 0.54% | 90% | 101% | 90% | 100% |
| LFNST | 0.53% | 1.38% | 1.63% | 110% | 100% | 109% | 101% |
| JCCR | 0.28% | 0.84% | 5.83% | 99% | 100% | 98% | 102% |
| SAO | 0.05% | 0.27% | 0.65% | 101% | 97% | 100% | 98% |
| CCALF | -0.12% | 16.13% | 16.74% | 103% | 100% | 100% | 99% |

Simulation Results for RA (Class H2)

|  |  |  | VTM RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Abbreviation | BDR-wY | BDR-wU | BDR-wV | Tester EncTime | Tester DecTime | XChecker EncTime | XChecker DecTime |
| CST | 0.24% | 7.24% | 13.57% | 104% | 103% | 105% | 104% |
| DQ | 1.73% | 0.97% | 0.85% | 103% | 104% | 105% | 104% |
| CCLM | 0.85% | 24.19% | 18.37% | 101% | 102% | 104% | 105% |
| MTS | 1.15% | 1.43% | 1.18% | 96% | 101% | 97% | 99% |
| ALF | 3.37% | 26.70% | 28.86% | 97% | 91% | 97% | 95% |
| AFF | 0.76% | 0.50% | 0.60% | 80% | 99% | 83% | 101% |
| SbTMVP | 0.37% | 0.20% | 0.14% | 102% | 102% | 103% | 101% |
| AMVR | 0.72% | 0.91% | 1.25% | 85% | 100% | 88% | 104% |
| GPM | 0.64% | 0.76% | 0.93% | 97% | 100% | 101% | 104% |
| BDOF | 0.60% | 0.24% | 0.22% | 96% | 98% | 100% | 101% |
| CIIP | 0.19% | -0.31% | -0.34% | 97% | 99% | 102% | 104% |
| MMVD | 0.23% | 0.22% | 0.24% | 92% | 103% | 90% | 100% |
| BCW | 0.20% | 0.16% | 0.24% | 95% | 103% | 93% | 101% |
| MRLP | 0.05% | -0.08% | 0.18% | 101% | 102% | 102% | 101% |
| IBC | 0.10% | -0.01% | -0.07% | 111% | 102% | 109% | 102% |
| ISP | 0.23% | 0.18% | 0.29% | 97% | 101% | 99% | 102% |
| DMVR | 0.87% | 1.07% | 1.10% | 101% | 99% | 100% | 98% |
| SBT | 0.31% | -0.17% | -0.21% | 97% | 101% | 96% | 99% |
| LMCS | 0.98% | 0.60% | 0.66% | 99% | 97% | 103% | 102% |
| SMVD | 0.20% | 0.06% | 0.13% | 98% | 102% | 96% | 99% |
| BDPCM | 0.03% | -0.13% | 0.01% | 103% | 101% | 101% | 100% |
| MIP | 0.48% | 0.49% | 0.06% | 98% | 102% | 96% | 99% |
| LFNST | 0.48% | 0.85% | 1.34% | 96% | 99% | 99% | 101% |
| JCCR | 0.21% | 0.35% | 6.84% | 99% | 99% | 101% | 101% |
| SAO | 0.07% | 0.34% | 1.67% | 101% | 101% | 100% | 97% |
| PROF | 0.13% | 0.02% | -0.08% | 100% | 100% | 102% | 100% |
| CCALF | -0.13% | 22.73% | 24.55% | 100% | 96% | 106% | 104% |

Figure 4. PSNR-Y vs encoding runtime ratio of VTM with VTM tool tests (Class H2)

Figure 5. PSNR-Y vs decoding runtime ratio of VTM with VTM tool tests (Class H2)

Figure 6. PSNR-Y vs weighted runtime ratio (a = 6) of VTM with VTM tool tests (Class H2)

## Evaluating Additional HDR/WCG Content

During the AhG period, 12 new HDR-HLG sequences were provided for study.  These sequences are further described in JVET-S0218.  Compression results were generated for the sequences, and visual evaluation was performed.  As a result, three of the sequences were identified as potential candidates for HDR verification testing.

Figure 7. Coding performance of the new “Amsterdam” sequences when using HEVC.

## Preparing for Expert Viewing

In advance of the 19th meeting, a set of procedures were developed to allow for remote, expert viewing during the 19th meeting.  These procedures are defined below and consist of two major areas of work.  The first is defining what tone mapping should be used for viewing.  And, the second is defining how viewing should be performed when using consumer equipment.

### Tone Mapping

The HDR-PQ content that is available to JVET has largely been master for a 4,000 nit display.  This is representative of a portion of content that is available in the eco-system today, but it has historically created issues for performing expert viewing in JVET as (i) there are no consumer displays available with a brightness of 4,000 nits, and (ii) the only non-consumer display available to JVET that supports 4,000 nits is limited in resolution to HD.  During the Call for Evidence and Call for Proposals, this was addressed by using the available non-consumer display and down-sampling the HDR-PQ sequences to HD resolution.  Additionally, for less critical viewing, a 1,000 nit display has been used.  And, the content was scaled (approximately in linear light) to map the entire dynamic range into the capability of the 1,000 nit display.

During the interim period, the tone mapping process was re-studying in the context of candidate sequences for the verification tests.  In these sequences, it was determined applying the scaling described above was less than ideal, and that applying tone mapping in only the brighter regions of the sequences would be more suitable for the content.  As a result of this work, it was decided to not use the previous tone mapping methods for verification testing, but instead to allow the brighter regions of the image to clip.  It is noted that this process may be further refined to include some form of roll-off, but this is only mentioned as future work – and not expected to impact the verification testing.

### Consumer Playback

To facilitate remote expert viewing, a number of procedures were developed to assist in experts being able to reproduce evaluation environments.  This procedures are listed below, and the result was that two participants were able to re-create a similar viewing environment remote from each other successfully.

Compressed Playback

To simplify the management of meta-data and allow experts to participate in viewing remotely, it was recommened that compressed versions of test content could be created with the following commands:

PQ content:

ffmpeg -pix_fmt yuv420p10le -r 60 -s 3840x2160 -i <filename.yuv> -c:v libx265 -crf <crf_value> -x265-params colorprim=bt2020:transfer=smpte2084:colormatrix=bt2020nc <filename.mkv>

HLG content:

ffmpeg -pix_fmt yuv420p10le -r 60 -s 3840x2160 -i <filename.yuv> -c:v libx265 -crf <crf_value> -x265-params colorprim=bt2020:transfer=arib-std-b67:colormatrix=bt2020nc <filename.mkv>

Here, the crf_value is used to control the bit-rate.  For those using the internal decoder of a monitor (e.g., via USB playback), a crf_value of 15 is typically required.  For those using a PC based playback system, the crf_value will be determined by the playback capabilities of the PC hardware.  Typically, a crf_value of less than 10 will result in visually transparent playback.

PC Platform

During the interim period, it was recommended to use a a consumer LG OLED display when possible to evaluate the HDR content.  To drive the LG display in HDR mode, a set of procedures was developed to facilitate reproduceable results.  They are:

- Enable the 4:2:2 12-bit mode in the Nvidia driver

- Enable HDR support in Windows 10  (For example, using the steps at: https://support.microsoft.com/en-us/help/4040263/windows-10-hdr-advanced-color-settings)

- Download a recent version of Video LAN (3.x)

- Select the “HDR Standard” mode (or similar) on the LG display and disable automatic tone mapping

After performing Step 1 and 2, the monitor should provide visual feedback that it is operating in HDR mode.

## Coordinating with AHG4

The AHG had significant coordination with AHG4, including sharing results on the new HDR content and expert viewing procedures.

# Contributions

There are three contribution related to HDR video coding.

| JVET-S0151 | Status Report and Proposed Agenda for the AHG4 Meeting on HDR Verification Test Preparation | A. Segall (Sharp) |
| --- | --- | --- |
| JVET-S0153 | Agenda and Report of the AHG4 Meeting on HDR Verification Test on 2020-05-29 | A. Segall, M. Wien, V. Baroncini, T. Suzuki |
| JVET-S0218 | 4K HLG test sequences for HDR verification test | S. Iwamura, S. Nemoto, A. Ichigaya (NHK) |

# Recommendations

The AHG recommends the following:

- Review all input contributions

- Establish a BoG during the 19th meeting to progress the work on HDR verification test preparations