| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0014 |
| --- | --- |

| Title: | JVET AHG report: Lossless and near-lossless coding (AHG14) |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | AHG Report |  |  |
| Author(s) or Contact(s): | T. Nguyen and T.-C. Ma (co-chairs), M. Ikeda, H. Jang, X. Zhao (vice-chairs) | Email: | tung.nguyen@hhi.fraunhofer.de tsung-chuanma@kwai.com masaru.ikeda@sony.com hm.jang@lge.com xinzzhao@tencent.com |
| Source: | AHG14 on lossless and near-lossless coding tools |  |  |

_____________________________

# Abstract

This document reports the activity of AHG 14 on lossless and near-lossless coding tools between the 18th JVET meeting via Teleconference and the 18th Meeting via Teleconference.

# Introduction

The JVET established the AHG 14 at the Brussels meeting with the following mandates:

- Produce, study and develop improvements of the JVET-Q2014 testing condition description.

- Study coding tools for lossless and near-lossless coding, including transform skip, BDPCM, and other potential technologies.

- Consider the interaction between coding tools and other processing such as loop filtering and LMCS for lossless and near-lossless coding.

- Consider throughput bottlenecks for lossless and near-lossless coding at high resolutions and frame rates.

# Activities

Discussions related to AHG14 used the JVET email reflector (jvet@lists.rwth-aachen.de), and the AHG chairs sent a kick-off message on 20th May 2020. No further emails have been exchanged related to the AHG.

The results for VTM-9.0 relative to VTM-8.0 for the 420 test set are as follows.

|  | All Intra |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM-8 | VTM9 |  | VTM8 | VTM9 |  |
| Class A1 | 2.4 | 2.4 | 0.05% | 2.4 | 2.4 | 0.05% |
| Class A2 | 1.8 | 1.8 | 0.03% | 1.9 | 1.9 | 0.02% |
| Class B | 2.3 | 2.3 | 0.03% | 2.5 | 2.5 | 0.03% |
| Class C | 2.1 | 2.1 | 0.04% | 2.6 | 2.6 | 0.05% |
| Class D | 2.1 | 2.1 | 0.05% | 2.9 | 2.9 | 0.02% |
| Class E | 3.1 | 3.1 | 0.01% |  |  |  |
| Class F | 5.8 | 5.8 | 0.03% | 35.4 | 35.4 | 0.04% |
| TGM | 12.4 | 12.4 | 0.05% | 109.0 | 108.9 | 0.05% |
| Overall | 2.3 | 2.3 | 0.03% | 2.4 | 2.4 | 0.04% |
| Enc Time[%] | 101% |  |  | 95% |  |  |
| Dec Time[%] | 103% |  |  | 102% |  |  |

The results for HEVC RExt relative to HEVC Main/Main10 are as follows using HM-16.20.

|  | All Intra |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | HM-16.20 | HM-16.20 Rext |  | HM-16.20 | HM-16.20 Rext |  |
| Class A1 | 2,2 | 2,3 | -4,50% | 2,3 | 2,4 | -3,88% |
| Class A2 | 1,7 | 1,8 | -5,88% | 1,8 | 1,9 | -4,52% |
| Class B | 2,2 | 2,3 | -5,06% | 2,3 | 2,4 | -2,59% |
| Class C | 1,9 | 2,0 | -5,42% | 2,5 | 2,5 | -2,22% |
| Class D | 1,9 | 2,1 | -7,85% | 2,8 | 2,9 | -2,56% |
| Class E | 2,7 | 3,0 | -8,22% |  |  |  |
| Class F | 4,5 | 5,2 | -12,17% | 26,6 | 30,6 | -8,54% |
| TGM | 6,1 | 8,1 | -22,91% | 74,4 | 99,5 | -20,65% |
| Overall | 2,1 | 2,3 | -5,71% | 2,3 | 2,3 | -3,14% |
| Enc Time[%] | 95% |  |  | 105% |  |  |
| Dec Time[%] | 93% |  |  | 90% |  |  |

The results for VTM-9.0 relative to HEVC Main/Main10 are as follows.

|  | All Intra |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | HM-16.20 | VTM-9.0 |  | HM-16.20 | VTM-9.0 |  |
| Class A1 | 2.2 | 2.4 | -6.83% | 2.3 | 2.4 | -5.82% |
| Class A2 | 1.7 | 1.8 | -5.09% | 1.8 | 1.9 | -3.52% |
| Class B | 2.2 | 2.3 | -6.57% | 2.3 | 2.5 | -4.52% |
| Class C | 1.9 | 2.1 | -7.49% | 2.5 | 2.6 | -4.63% |
| Class D | 1.9 | 2.1 | -9.66% | 2.8 | 2.9 | -5.04% |
| Class E | 2.7 | 3.1 | -11.12% |  |  |  |
| Class F | 4.5 | 5.8 | -19.77% | 26.6 | 35.4 | -15.48% |
| TGM | 6.1 | 12.4 | -46.88% | 74.4 | 108.9 | -32.08% |
| Overall | 2.1 | 2.3 | -7.33% | 2.3 | 2.4 | -4.61% |
| Enc Time[%] | 2790% |  |  | 1270% |  |  |
| Dec Time[%] | 156% |  |  | 135% |  |  |

The results for VTM-9.0 relative to HEVC RExt are as follows.

|  | All Intra |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | HM-16.20 RExt | VTM-9.0 |  | HM-16.20 RExt | VTM-9.0 |  |
| Class A1 | 2.3 | 2.4 | -2.44% | 2.4 | 2.4 | -2.02% |
| Class A2 | 1.8 | 1.8 | 0.85% | 1.9 | 1.9 | 1.08% |
| Class B | 2.3 | 2.3 | -1.58% | 2.4 | 2.5 | -1.97% |
| Class C | 2.0 | 2.1 | -2.18% | 2.5 | 2.6 | -2.46% |
| Class D | 2.1 | 2.1 | -1.98% | 2.9 | 2.9 | -2.54% |
| Class E | 3.0 | 3.1 | -3.16% |  |  |  |
| Class F | 5.2 | 5.8 | -8.94% | 30.6 | 35.4 | -7.85% |
| TGM | 8.1 | 12.4 | -31.61% | 99.5 | 108.9 | -13.59% |
| Overall | 2.3 | 2.3 | -1.72% | 2.3 | 2.4 | -1.50% |
| Enc Time[%] | 2966% |  |  | 1225% |  |  |
| Dec Time[%] | 168% |  |  | 145% |  |  |

# Recommendations

The AHG recommends:

- Close the AHG.