| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June –1 July 2020 | Document: JVET-S0041-v2 |
| --- | --- |

| Title: | Status Report on SDR Verification Test Preparation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Mathias Wien Vittorio Baroncini | Tel: Email: | wien@lfb.rwth-aachen.de baroncini@gmx.com |
| Source: | Verification Test Coordinators |  |  |

_____________________________

# Introduction

The draft plan for VVC verification testing (Draft 2) [JVET-R2009] includes a set of SDR test sequences under consideration for use in the formal verification tests of VVC. According to the timeline of this plan, a preselection and preparation of test sequences and corresponding rate points for viewing has been carried out. Out of the 13 test sequences listed in JVET-R2009, 8 test sequences were selected. The selection process and the experts viewing session subsequently conducted are described in this document.

# Selection of test sequences

The test sequences in Table 1 of JVET-R2009 were assessed by the test coordinators in terms of rate-distortion behaviour and in terms of visual impairment over the provided QP range (QP=24…50 for HM, QP=26:4:50 for VTM). Based on this assessment, the test sequences and corresponding QP settings were selected as candidates for evaluation in subjective tests.

Table 1 – Selected SDR UHD test sequences and rate points

| No. | Test sequence | HM QPs | VTM QPs |
| --- | --- | --- | --- |
| 01 | Marathon_3840x2160_30fps_8bit_420pf | 46, 40, 36, 32 | 46, 42, 38, 34 |
| 02 | MountainBay2_3840x2160_30fps_420_10bit | 41, 37, 33, 29 | 42, 38, 34, 30 |
| 03 | Netflix_DrivingPOV_4096x2160_60fps_10bit_420 | 40, 36, 32, 28 | 42, 38, 34, 30 |
| 04 | Runners_3840x2160_30fps_8bit_420pf | 45, 41, 37, 33 | 46, 42, 38, 34 |
| 05 | TallBuildings_3840x2160_30fps_8bit_420pf | 45, 41, 37, 33 | 46, 42, 38, 34 |
| 09 | NeptuneFountain3_3840x2160_60fps_420_10bit | 40, 36, 33, 28 | 42, 38, 34, 30 |
| 12 | SubwayTree_3840x2160_60fps_420_10bit | 45, 41, 37, 33 | 46, 42, 38, 34 |
| 13 | TiergartenParkway_3840x2160_60fps_420_10bit | 45, 41, 37, 33 | 46, 42, 38, 34 |

Rate distortion plots for the listed QP selection are provided in Annex A to this document.

## Preparation of remote viewing assessment

### Experts viewing setup and protocol

It was decided to assess the impairment induced by HM and VTM encoding using the Experts Viewing Protocol of [ITU-R BT.2095]. Participants were required to have a suitable computer which is capable of smoothly playing mp4-files in UHD resolution. The participants further were required to perform the viewing on a monitor (or TV set) of 55” size or larger. UHD 55” OLED TV sets were recommended.

The test design includes the presentation of basic test cells of the following form

BTC:

- 2 sec	Text “Source” on mid-grey background

- 10 sec	Playback of uncompressed source file

- 2 sec	Text “A” on mid-grey background

- 10 sec	Playback of processed video sequence A (HM)

- 2 sec	Text “B” on mid-grey background

- 10 sec	Playback of processed video sequence B (VTM)

- 5 sec	Text “Vote <id>” on mid-grey background

Thereby, each BTC has a duration of 41 sec. The test design included two sessions consisting of 20 randomized and numbered BTCs each, with the BTC identifier number being displayed with the “Vote” text.

### Transcoding of YUV files

In order to facilitate remote experts viewing remotely in a coordinated session with participants not necessarily attending from their professional environment, playout on conventional state-of-the-art PCs was required. For this purpose, it was decided to transcode all selected bitstreams to HEVC at high bitrate and wrap those in mp4-containers using ffmpeg. For the original sequences, the following command line was used:

ffmpeg -f rawvideo -vcodec rawvideo -s <pix>x<lin> -r <fps> -pix_fmt yuv420p10le -i <original yuv file> -c:v libx265 -x265-params vbv-bufsize=150000:vbv-maxrate=150000:bitrate=150000 <output>.mp4

For the encoded bistreams, the reconstructed YUV files were encoded with the following command line:

ffmpeg -f rawvideo -vcodec rawvideo -s <pix>x<lin> -r <fps> -pix_fmt yuv420p10le -i <decoded yuv file> -c:v libx265 -x265-params vbv-bufsize=100000:vbv-maxrate=100000:bitrate=100000 <output>.mp4

It is noted that ffmpeg version 3.2 was used for the encoding task which encoded the videos with HEVC Main profile.

### Preparation of playout

Due to general availability, the VLC media player was selected as the tool for playout. Participants were requested to have the most recent version of this software installed. The recommended configuration of VLC was as follows:

- Go to Preferences, select "Show settings: All" at the bottom right of the window

- Select Video

  - Activate Fullscreen

  - Deactivate "Show media title on video"

- Select Playlist

  - Deactivate any looping options

- It was recommended to turn off other programs and any file synchronizations since they can infer with the playout. Participants should also make sure that they turned off any picture enhancements of the TV. The mp4-files have been tested on decent PCs and Laptops and should play in a stable manner.

# Remote experts viewing session

The session was conducted on Wednesday 2020-05-13 starting at 18:00 CEST. The test was remotely conducted using a Zoom session for coordination and discussion. Five experts participated in this session. One more expert conducted the EVP viewing sessions on the following day under the guidance of the test coordinators.

The test data for this session was made available on the JVET ftp site using the regular jvet login in the following directory:

/ahg/verificationtests/JVET-R_VT/mp4-sdr

The data was made available for download to the participants one day before the session, together with a template for voting sheets and an Excel file to enter their voting data. The total size of the dataset on the ftp site was about 9.9GB. The dataset included a playlist for a single BTC and a training playlist containing three BTC representing very high, very low, and mid quality, respectively.

The session included three parts: the first two viewing sessions were designed to scan the selected rate points for the test sequences listed in Table 1. The third viewing session included an anonymized A-B comparison of VTM bitstreams with ALF switched on and off. For the third viewing session, the VTM rate points for the test sequences Marathon and SubwayTree were employed.

## EVP test

Before the start of the session, the test coordinators informed participants on the procedure of the test, the number of test cells, and the voting procedure. The participants were asked to play the training playlist in order to accommodate to the viewing protocol and the expectable quality range. Stable playout by the VLC player was confirmed. It was confirmed that all participants were equipped with the provided voting sheets.

After the introduction, the two viewing sessions were conducted. Participants were asked to fill an EVP scoring sheet of each of these sub-sessions using the 11-grade impairment scale of BT.2095 shown in Fig. 1 below.

Fig. 1: Numerical impairment scale of ITU-T BT.2095

After completion of each session, the participants were asked to transfer their scores to a provided Excel-sheet. They were requested to send it together with a photo of their filled voting sheets to the test coordinators after the end of the session. The layout of the voting sheets is shown in Fig. 2 below.

Fig. 1: Numerical impairment scale of ITU-T BT.2095

## A-B comparison session for ALF

In order to determined the impact of ALF on the perceived quality of the decoded video bitstreams, an A-B comparison was conducted with VTM bitstreams having ALF switched on and off. The test sequences Marathon and SubwayTree were used for this assessment. The mp4-files for this viewing session were anonymized with a randomized assignment of ALF-on and ALF-off to the letters A and B. For each ratepoint, a separate VLC playlist file was provided to allow for repetitive presentation of the BTC under consideration. In the session the experts were requested to play the BTCs in a randomized order. For each BTC, experts were allowed to watch the BTC as often as needed. After completion of the viewing of the BTC, they were requested to vote on the comparison by simultaneously using the “thumbs-up” and “thumbs-down” buttons provided by the Zoom video conferencing software. Experts were allowed to vote “equal” by using the Zoom “clap-hands” symbol.

# Results

## EVP test

A number of 6 experts participated in this session.

Here below the graph showing the comparison among HM and VVC with ALFon for the test sequences listed in table 1.

The above graphs report only the MOS values computed on 6 viewers; therefore, no CI is reported being the sample widely below the minimum value of 15 acceptable to make any analysis of variance.

## ALF A-B comparison

A number of five experts participated in this session. In this A/B comparison, the participants were requested to rate "A better than B" or "B better than A" for the randomized and anonymized BTCs. The results are reported in Table 2 below. BTC1-4 correspond to the test sequence Marathon at QP=34, 38, 42, 46, respectively. BTC1-4 correspond to the test sequence SubwayTree at QP=34, 38, 42, 46, respectively. With a distance of two votes between A and B, a slight preference toward the higher vote is assumed. It is noted that the number of test participants is small. The results therefore should be seen as an indication but not as a proof.

Table 2 – Selected SDR UHD test sequences and rate points

|  | A better | Equal | B better | Result |
| --- | --- | --- | --- | --- |
| BTC6 | 3 | 1 | 1 | ALF off preferred |
| BTC2 | 0 | 3 | 2 | ALF on preferred |
| BTC5 | 1 | 4 | 0 | None |
| BTC3 | 2 | 0 | 3 | None |
| BTC7(*) | 4 | 0 | 0 | ALF on preferred |
| BTC1 | 2 | 1 | 2 | None |
| BTC4 | 4 | 1 | 0 | ALF on preferred |
| BTC8 | 2 | 2 | 1 | None |

(*) One expert did not provide a vote for this BTC.

# Conclusions

The results of the EVP viewing session suggests the suitability of the test sequences Marathon, MountainBay2, Netflix_DrivingPOV, Runners, NeptuneFountain3, and TiergartenParkway for consideration for a dry run of the formal subjective assessment in the context of the VVC verification tests. The test sequences TallBuildings and SubwayTree are recommended to serve as spare test sequences.

Note that the results are considered to be preliminary as some experts did not score all cases due to the lack of ability to express a proper judgement. It is further noted that not all subjects used the same player, which may have had an impact on the scoring. It is recommended to enforce alignment of the playout tools in future experiments.

Based on the observations in the tests the following modifications of the rate point configurations might be considered:

- Modification of lowest QP for HM and VTM for MountainBay2 and NeptunFountain3. The same may apply for TiergartenPartway.

- Modification of the second-lowest QP for HM and VTM for DrivingPOV. Further, the low two and high two QPs respectively have too close quality for the VTM

- For Marathon, the lowest two QP points of VTM have too similar quality

Based on the results reported in Section 4.2, it is suggested to keep ALF on in further subjective assessments.

It is suggested to consistently use 10 bit test sequences. For Runners and TallBuildings, it is suggested to consider available 10 bit versions of these test sequences as proposed in [JVET-B0046]. Further, in order to harmonize the size of the sequences, it is suggested to crop the test sequence Netflix_DrivingPOV from 4096x2160 to 3840x2160 using a center crop with offset of 128.

# ANNEX A – Rate-distortion plots

## Marathon

## MountainBay2

## Netflix_DrivingPOV

## Runners

## TallBuildings

## NeptuneFountain3

## SubwayTree

## TiergartenParkway