| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0044 |
| --- | --- |

| Title: | AHG9: Cleanup of high-level signalling of BDOF, DMVR and PROF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Martin Pettersson Rickard Sjöberg Mitra Damghanian Jack Enhorn Zhi Zhang Jacob Ström Ruoyang Yu Du Liu | Email: | martin.m.pettersson@ericsson.com rickard.sjoberg@ericsson.com mitra.damghanian@ericsson.com jack.enhorn@ericsson.com zhi.a.zhang@ericsson.com jacob.strom@ericsson.com ruoyang.yu@ericsson.com du.liu@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution proposes to remove the syntax elements sps_bdof_control_present_in_ph_flag, sps_dmvr_control_present_in_ph_flag and sps_prof_control_present_in_ph_flag from the VVC specification and instead condition the presence of ph_bdof_disabled_flag, ph_dmvr_disabled_flag and ph_prof_disabled_flag on the existing syntax elements sps_bdof_enabled_flag, sps_dmvr_enabled_flag and sps_affine_prof_enabled_flag, respectively.

It is asserted that the three control present flags together at most saves three bits per picture, which is asserted to not motivate the extra complexity of the signalling mechanism. The authors estimate that the proposed cleanup removes 6 syntax table rows and 23 lines of semantics text.

# Introduction

The current version of VVC contains the syntax elements sps_bdof_control_present_in_ph_flag, sps_dmvr_control_present_in_ph_flag and sps_prof_control_present_in_ph_flag that specifies the presence of ph_bdof_disabled_flag, ph_dmvr_disabled_flag and ph_prof_disabled_flag, respectively.

To our understanding the three control present flags are only used for bit cost reduction and together they may at most save three bits per picture.

At the 18th JVET meeting similar presence flags were proposed in JVET-R0050 and JVET-R0258 to indicate the presence of control syntax elements in the PH for dependent quantization, sign data hiding, temporal MVP, and fpel MMVD, but these flags were not adopted. The following can be seen from the notes of JVET-R0258.

“It was expressed that in case of multiple slices, VTM would probably have less header bits in total than HM. The PH was introduced exactly for the benefit of saving header bits in the multiple slice case. In case of single slice in a picture, saving header bits is not so important.”

Since the bit savings are very small and to be consistent with the previous decision to not use control presence flags for other tools, we propose to remove the syntax elements sps_bdof_control_present_in_ph_flag, sps_dmvr_control_present_in_ph_flag and sps_prof_control_present_in_ph_flag from the VVC specification.

# Proposal

Proposed changes on top of JVET-R2001-v10 are shown below with removed text in red strike through and added text in yellow:

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_bdof_enabled_flag | u(1) |
| if( sps_bdof_enabled_flag ) |  |
| sps_bdof_control_present_in_ph_flag | u(1) |
| sps_smvd_enabled_flag | u(1) |
| sps_dmvr_enabled_flag | u(1) |
| if( sps_dmvr_enabled_flag) |  |
| sps_dmvr_control_present_in_ph_flag | u(1) |
| sps_mmvd_enabled_flag | u(1) |
| if( sps_mmvd_enabled_flag ) |  |
| sps_mmvd_fullpel_only_flag | u(1) |
| sps_six_minus_max_num_merge_cand | ue(v) |
| sps_sbt_enabled_flag | u(1) |
| sps_affine_enabled_flag | u(1) |
| if( sps_affine_enabled_flag ) { |  |
| sps_five_minus_max_num_subblock_merge_cand | ue(v) |
| sps_affine_type_flag | u(1) |
| if( sps_amvr_enabled_flag ) |  |
| sps_affine_amvr_enabled_flag | u(1) |
| sps_affine_prof_enabled_flag | u(1) |
| if( sps_affine_prof_enabled_flag ) |  |
| sps_prof_control_present_in_ph_flag | u(1) |
| } |  |
| … |  |
| } |  |

sps_bdof_enabled_flag equal to 0 specifies that the bi-directional optical flow inter prediction is disabled and not used in decoding of pictures in the CLVS. sps_bdof_enabled_flag equal to 1 specifies that the bi-directional optical flow inter prediction is enabled and may be used in decoding of pictures in the CLVS.

sps_bdof_control_present_in_ph_flag equal to 1 specifies that ph_bdof_disabled_flag is present in PHs referring to the SPS. sps_bdof_control_present_in_ph_flag equal to 0 specifies that ph_bdof_disabled_flag is not present in PHs referring to the SPS. When sps_bdof_control_present_in_ph_flag is not present, the value of sps_bdof_control_present_in_ph_flag is inferred to be equal to 0.

…

sps_dmvr_enabled_flag equal to 1 specifies that decoder motion vector refinement based inter bi-prediction is enabled and may be used in decoding of pictures in the CLVS. sps_dmvr_enabled_flag equal to 0 specifies that decoder motion vector refinement based inter bi-prediction is disabled and not used in decoding of pictures in the CLVS.

sps_dmvr_control_present_in_ph_flag equal to 1 specifies that ph_dmvr_disabled_flag is present in PHs referring to the SPS. sps_dmvr_control_present_in_ph_flag equal to 0 specifies that ph_dmvr_disabled_flag is not present in PHs referring to the SPS. When sps_dmvr_control_present_in_ph_flag is not present, the value of sps_dmvr_control_present_in_ph_flag is inferred to be equal to 0.

…

sps_affine_prof_enabled_flag equal to 0 specifies that the affine motion compensation refined with optical flow is disabled and not used in decoding of pictures in the CLVS. sps_affine_prof_enabled_flag equal to 1 specifies that the affine motion compensation refined with optical flow is enabled and may be used in decoding of pictures in the CLVS. When not present, the value of sps_affine_prof_enabled_flag is inferred to be equal to 0.

sps_prof_control_present_in_ph_flag equal to 1 specifies that ph_prof_disabled_flag is present in PHs referring to the SPS. sps_prof_control_present_in_ph_flag equal to 0 specifies that ph_prof_disabled_flag is not present in PHs referring to the SPS. When sps_prof_control_present_in_ph_flag is not present, the value of sps_prof_control_present_in_ph_flag is inferred to be equal to 0.

| picture_header_structure( ) { | Descriptor |
| --- | --- |
| … |  |
| if( ph_inter_slice_allowed_flag ) { |  |
| … |  |
| if( !pps_rpl_info_in_ph_flag ¦ ¦ num_ref_entries[ 1 ][ RplsIdx[ 1 ] ] > 0 ) { |  |
| ph_mvd_l1_zero_flag | u(1) |
| if( sps_bdof_control_present_in_phenabled_flag ) |  |
| ph_bdof_disabled_flag | u(1) |
| if( sps_dmvr_control_present_in_phenabled_flag ) |  |
| ph_dmvr_disabled_flag | u(1) |
| } |  |
| if( sps_affine_prof_control_present_in_phenabled_flag ) |  |
| ph_prof_disabled_flag | u(1) |
| if( ( pps_weighted_pred_flag ¦ ¦ pps_weighted_bipred_flag ) && pps_wp_info_in_ph_flag ) |  |
| pred_weight_table( ) |  |
| } |  |
| … |  |
| } |  |

ph_bdof_disabled_flag equal to 1 specifies that bi-directional optical flow inter prediction based inter bi-prediction is disabled and not used in the slices associated with the PH. ph_bdof_disabled_flag equal to 0 specifies that bi-directional optical flow inter prediction based inter bi-prediction is enabled and may be used in the slices associated with the PH.

When not present, the value of ph_bdof_disabled_flag is inferred as follows:

- If sps_bdof_control_present_in_ph_flag is equal to 0, the value of ph_bdof_disabled_flag is inferred to be equal to 1 − sps_bdof_enabled_flag.

- Otherwise (sps_bdof_control_present_in_ph_flag is equal to 1), the value of ph_bdof_disabled_flag is inferred to be equal to 1.

ph_dmvr_disabled_flag equal to 1 specifies that decoder motion vector refinement based inter bi-prediction is disabled and not used in the slices associated with the PH. ph_dmvr_disabled_flag equal to 0 specifies that decoder motion vector refinement based inter bi-prediction is enabled and may be used in the slices associated with the PH.

When not present, the value of ph_dmvr_disabled_flag is inferred as follows:

- If sps_dmvr_control_present_in_ph_flag is equal to 0, the value of ph_dmvr_disabled_flag is inferred to be equal to 1 − sps_dmvr_enabled_flag.

- Otherwise (sps_dmvr_control_present_in_ph_flag is equal to 1), the value of ph_dmvr_disabled_flag is inferred to be equal to 1.

ph_prof_disabled_flag equal to 1 specifies that prediction refinement with optical flow is disabled and not used in the slices associated with the PH. ph_prof_disabled_flag equal to 0 specifies that prediction refinement with optical flow is enabled and may be used in the slices associated with the PH.

When ph_prof_disabled_flag is not present, the following applies:

- If sps_affine_prof_enabled_flag is equal to 1, the value of ph_prof_disabled_flag is inferred to be equal to 0.

- Otherwise (sps_affine_prof_enabled_flag is equal to 0), the value of ph_prof_disabled_flag is inferred to be equal to 1.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).