| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0049-v1 |
| --- | --- |

| Title: | AHG9/AHG8/AHG12: On parameter sets and picture header |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang1, Li Zhang1 Zhipin Deng2, Kai Zhang1 18910 University Center Lane San Diego, CA 92122, USA 2Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China | Tel: Email: | yekui.wang@bytedance.com lizhang.idm@bytedance.com zhipin.deng@bytedance.com zhangkai.video@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution proposes the following changes related to parameter sets and picture header:

- When sps_video_parameter_set_id is equal to 0, the value of vps_max_sublayers_minus1 is inferred to be equal to sps_max_sublayers_minus1 (instead of 6).

- When sps_inter_layer_ref_pics_present_flag is equal to 1, sps_idr_rpl_present_flag is skipped and inferred to be equal to 1, and constrain the value of sps_idr_rpl_present_flag to be equal to 1 when vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 0.

- Allow the value of pps_no_pic_partition_flag to be different for different PPSs that are referred to by coded pictures within a CLVS.

- Constrain the value of pps_alf_info_in_ph_flag to be equal to 0 when the PH is in the SH.

- When pps_rect_slice_flag is equal to 1 and pps_num_slices_in_pic_minus1 is equal to 0, constrain the values of the 6 PPS flags pps_alf_info_in_ph_flag, pps_rpl_info_in_ph_flag, pps_dbf_info_in_ph_flag, pps_sao_info_in_ph_flag, pps_wp_info_in_ph_flag, and pps_qp_delta_info_in_ph_flag to be all equal to 0.

- Split the u(5)-coded aps_adaptation_parameter_set_id into u(2)-coded aps_reserved_zero_2bits followed by u(3)-coded aps_adaptation_parameter_set_id.

- If aps_chroma_present_flag is equal to 0, alf_luma_filter_signal_flag is skipped and inferred to be equal to 1. Otherwise, when alf_chroma_filter_signal_flag and alf_cc_cb_filter_signal_flag are both equal to 0, alf_cc_cr_filter_signal_flag is skipped and inferred to be equal to 1.

- Constrain the value of ph_inter_slice_allowed_flag to be equal to 1 when vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 0.

- Allow the aps_chroma_present_flag of the scaling list APS NAL unit referenced by the PH to be equal to 0 when ChromaArrayType is greater than 0.

# On vps_max_sublayers_minus1 and sps_max_sublayers_minus1

The value of vps_max_sublayers_minus1 is used for specifying the value range of opTid in clause C.1 step 1 as well as the value range of bp_max_sublayers_minus1, even when there no VPS is present (i.e., sps_video_parameter_set_id is equal to 0). Currently, the value of vps_max_sublayers_minus1 is inferred to be equal to 6 when sps_video_parameter_set_id is equal to 0. However, it would make more sense to infer it to be equal to sps_max_sublayers_minus1 in that case due for the two places that use the value of vps_max_sublayers_minus1.

Therefore, it is proposed that, when sps_video_parameter_set_id is equal to 0, the value of vps_max_sublayers_minus1 is inferred to be equal to sps_max_sublayers_minus1 (instead of 6).

# On sps_idr_rpl_present_flag

The flag sps_idr_rpl_present_flag was designed for single-layer bitstreams for merging of IDR and non-IDR pictures without the need of changing the SHs.

In multi-layer contexts, when sps_inter_layer_ref_pics_present_flag is equal to 1, the layer is not an independent layer. In this case, it does not make sense to have sps_idr_rpl_present_flag equal to 0 as that would disable IDR slices (i.e., slices with nal_unit_type equal to IDR_N_LP or IDR_W_RADL) in enhancement layer (EL) pictures to contain RPLs and consequently disable IDR slices in EL pictures to be coded using inter-layer prediction (ILP), while ILP of IRAP pictures often contributes most to the coding gain provided by scalability.

Similarly, when vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 0, it also does not make sense to have sps_idr_rpl_present_flag equal to 0.

It is therefore proposed that, when sps_inter_layer_ref_pics_present_flag is equal to 1, sps_idr_rpl_present_flag is skipped and inferred to be equal to 1, and when vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 0, the value of sps_idr_rpl_present_flag shall be equal to 1.

The proposed text changes are as follows:

#### 7.3.2.3	Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ... |  |
| if( sps_video_parameter_set_id > 0 ) |  |
| sps_inter_layer_ref_pics_present_flag | u(1) |
| if( !sps_inter_layer_ref_pics_present_flag ) |  |
| sps_idr_rpl_present_flag | u(1) |
| ... |  |
| } |  |

#### 7.4.3.3	Sequence parameter set RBSP semantics

...

sps_inter_layer_ref_pics_present_flag equal to 0 specifies that no ILRP is used for inter prediction of any coded picture in the CLVS. sps_inter_layer_ref_pics_present_flag equal to 1 specifies that ILRPs may be used for inter prediction of one or more coded pictures in the CLVS. When sps_video_parameter_set_id is equal to 0, the value of sps_inter_layer_ref_pics_present_flag is inferred to be equal to 0. When vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 1, the value of sps_inter_layer_ref_pics_present_flag shall be equal to 0.

sps_idr_rpl_present_flag equal to 1 specifies that reference picture list syntax elements may be present in slice headers of IDR pictures. sps_idr_rpl_present_flag equal to 0 specifies that reference picture list syntax elements are not present in slice headers of IDR pictures. When not present, the value of sps_idr_rpl_present_flag is inferred to be equal to 1. When vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 0, the value of sps_idr_rpl_present_flag shall be equal to 1.

...

# On pps_no_pic_partition_flag within a CLVS

In the latest VVC text, it is required that the value of pps_no_pic_partition_flag shall be the same for all PPSs that are referred to by coded pictures within a CLVS. When pps_no_pic_partition_flag is equal to 1, each picture referring to the PPS has only one of each of tile, slice, and subpicture. However, the encoder should be allowed to have only one tile or slice for some pictures in a CLVS and multiple tiles or slices in other pictures in the same CLVS. Therefore, it would make more sense to remove this constraint.

It is therefore proposed to allow the value of pps_no_pic_partition_flag to be different for different PPSs that are referred to by coded pictures within a CLVS, by removing the constraint highlighted below from the semantics of pps_no_pic_partition_flag:

pps_no_pic_partition_flag equal to 1 specifies that no picture partitioning is applied to each picture referring to the PPS. pps_no_pic_partition_flag equal to 0 specifies that each picture referring to the PPS may be partitioned into more than one tile or slice.

It is a requirement of bitstream conformance that the value of pps_no_pic_partition_flag shall be the same for all PPSs that are referred to by coded pictures within a CLVS.

When sps_num_subpics_minus1 is greater than 0 or pps_mixed_nalu_types_in_pic_flag is equal to 1, the value of pps_no_pic_partition_flag shall be equal to 0.

# On pps_alf_info_in_ph_flag when the PH is in the SH

There are 6 PPS flags, pps_alf_info_in_ph_flag, pps_rpl_info_in_ph_flag, pps_dbf_info_in_ph_flag, pps_sao_info_in_ph_flag, pps_wp_info_in_ph_flag, and pps_qp_delta_info_in_ph_flag, that determine whether the corresponding parameters are signalled in the PH or the SH.

Currently, when pps_no_pic_partition_flag is equal to 1, these flags are not present and are all inferred to be equal to 0. While when the PH is in the SH and when these flags are present, 5 of the 6 flags, except the pps_alf_info_in_ph_flag, are constrained to be equal to 0, aligned with the inference when not present.

The reason why the same constraint was not specified for pps_alf_info_in_ph_flag was because a consideration of the place of the ALF related syntax elements in the single VCL NAL unit in each picture when the PH is in the SH. However, since the value of pps_alf_info_in_ph_flag is inferred to be equal to 0 anyway when pps_no_pic_partition_flag is equal to 1, in which case it is also possible that the PH is in the SH, decoders and systems would anyway need to cope with the case of pps_alf_info_in_ph_flag equal to 0 with the PH being in the SH, speciying the same constraint for pps_alf_info_in_ph_flag would not bring any addition burden, and would make the design more consistent.

On the other hand, speciying the same constraint for pps_alf_info_in_ph_flag would enable the use of ALF for the use case that applies independent parallel encoding of different subpictures that were split before encoding into individual VVC bitstreams each containing one subpicture, and later merging into one VVC bitstream wherein each picture contains multiple subpictures. In that use case, each of such individual bitstreams contains one slice/subpicture per picture, but the PH is not in the SH such that when merging the PH and SHs for each picture do not need to be rewritten.

Therefore, it is proposed to constrain the value of pps_alf_info_in_ph_flag to be equal to 0 when the PH is in the SH, same as the other 5 PPS flags that determine whether the corresponding parameters are signalled in the PH or the SH.

# On "the six PPS flags" when pps_rect_slice_flag is equal to 1 and pps_num_slices_in_pic_minus1 is equal to 0

When pps_rect_slice_flag is equal to 1 and pps_num_slices_in_pic_minus1 is equal to 0, there is only one slice in each picture referring to the PPS. In this case, when any of the 6 PPS flags pps_alf_info_in_ph_flag, pps_rpl_info_in_ph_flag, pps_dbf_info_in_ph_flag, pps_sao_info_in_ph_flag, pps_wp_info_in_ph_flag, and pps_qp_delta_info_in_ph_flag is present, it would make sense to require its value to be equal to 0, for the following benefits:

- To be better aligned with the inference of the value to 0 when pps_no_pic_partition_flag is equal to 1.

- For more optimal syntax in the case of pps_rpl_info_in_ph_flag or pps_wp_info_in_ph_flag.

- For the case of pps_alf_info_in_ph_flag, to enable the use of ALF for the same use case that applies independent parallel encoding of different subpictures as above.

Therefore, it is proposed that, when pps_rect_slice_flag is equal to 1 and pps_num_slices_in_pic_minus1 is equal to 0, the values of the 6 PPS flags pps_alf_info_in_ph_flag, pps_rpl_info_in_ph_flag, pps_dbf_info_in_ph_flag, pps_sao_info_in_ph_flag, pps_wp_info_in_ph_flag, and pps_qp_delta_info_in_ph_flag shall all be equal to 0.

# On APS ID

Currently, the APS ID (i.e., the syntax element adaptation_parameter_set_id) is coded as u(5), while when aps_params_type is equal to ALF_APS or SCALING_APS, the value of adaptation_parameter_set_id shall be in the range of 0 to 7, inclusive, and when aps_params_type is equal to LMCS_APS, the value of adaptation_parameter_set_id shall be in the range of 0 to 3, inclusive. In other words, using 3 bits instead of 5 bits for the APS ID would be sufficient.

It is therefore proposed to split the u(5)-coded aps_adaptation_parameter_set_id into u(2)-coded aps_reserved_zero_2bits followed by u(3)-coded aps_adaptation_parameter_set_id with the following semantics:

aps_reserved_zero_2bits shall be equal to 0 in bitstreams conforming to this version of this Specification. The values greater than 0 for aps_reserved_zero_2bits are reserved for future use by ITU-T | ISO/IEC.

aps_adaptation_parameter_set_id provides an identifier for the APS for reference by other syntax elements.

When aps_params_type is equal to ALF_APS or SCALING_APS, the value of aps_adaptation_parameter_set_id shall be in the range of 0 to 7, inclusive.

When aps_params_type is equal to LMCS_APS, the value of aps_adaptation_parameter_set_id shall be in the range of 0 to 3, inclusive.

Let apsLayerId be the value of the nuh_layer_id of a particular APS NAL unit, and vclLayerId be the value of the nuh_layer_id of a particular VCL NAL unit. The particular VCL NAL unit shall not refer to the particular APS NAL unit unless apsLayerId is less than or equal to vclLayerId and all OLSs specified by the VPS that contain the layer with nuh_layer_id equal to vclLayerId also contain the layer with nuh_layer_id equal to apslayerId.

This way, in the future, if some more types of APSs need to be specified and need more than 3 bits for the APS ID, the two reserved bits can be utilized in a manner that is backward compatible.

# On alf_luma_filter_signal_flag and alf_cc_cr_filter_signal_flag

The alf_data( ) syntax starts with the following:

| alf_data( ) { | Descriptor |
| --- | --- |
| alf_luma_filter_signal_flag | u(1) |
| if( aps_chroma_present_flag ) { |  |
| alf_chroma_filter_signal_flag | u(1) |
| alf_cc_cb_filter_signal_flag | u(1) |
| alf_cc_cr_filter_signal_flag | u(1) |
| } |  |
| ... |  |

If aps_chroma_present_flag is equal to 0, the value of the luma flag, alf_luma_filter_signal_flag, has to be equal to 1 and therefore does not need to be signalled.

Otherwise (aps_chroma_present_flag is equal to 1), to make sense, at least one of the three chroma flags needs to be equal to 1. Therefore, when the first two chroma flags are both equal to 0, the third chroma flag has to be equal to 1 and does not need to be signalled.

Therefore, it is proposed that, if aps_chroma_present_flag is equal to 0, alf_luma_filter_signal_flag is skipped and inferred to be equal to 1. Otherwise, when alf_chroma_filter_signal_flag and alf_cc_cb_filter_signal_flag are both equal to 0, alf_cc_cr_filter_signal_flag is skipped and inferred to be equal to 1.

The proposed text changes are as follows:

| alf_data( ) { | Descriptor |
| --- | --- |
| if( !aps_chroma_present_flag ) |  |
| alf_luma_filter_signal_flag | u(1) |
| if( aps_chroma_present_flag ) { |  |
| alf_chroma_filter_signal_flag | u(1) |
| alf_cc_cb_filter_signal_flag | u(1) |
| if( alf_chroma_filter_signal_flag ¦ ¦ alf_cc_cb_filter_signal_flag ) |  |
| alf_cc_cr_filter_signal_flag | u(1) |
| } |  |
| ... |  |

...

alf_luma_filter_signal_flag equal to 1 specifies that a luma filter set is signalled. alf_luma_filter_signal_flag equal to 0 specifies that a luma filter set is not signalled. When not present, the value of alf_luma_filter_signal_flag is inferred to be equal to 1.

alf_chroma_filter_signal_flag equal to 1 specifies that a chroma filter is signalled. alf_chroma_filter_signal_flag equal to 0 specifies that a chroma filter is not signalled. When not present, the value of alf_chroma_filter_signal_flag is inferred to be equal to 0.

The variable NumAlfFilters specifying the number of different adaptive loop filters is set equal to 25.

alf_cc_cb_filter_signal_flag equal to 1 specifies that cross-component filters for the Cb colour component are signalled. alf_cc_cb_filter_signal_flag equal to 0 specifies that cross-component filters for Cb colour component are not signalled. When not present, the value of alf_cc_cb_filter_signal_flag is inferred to be equal to 0.

alf_cc_cr_filter_signal_flag equal to 1 specifies that cross-component filters for the Cr colour component are signalled. alf_cc_cr_filter_signal_flag equal to 0 specifies that cross-component filters for the Cr colour component are not signalled. When not present, the value of alf_cc_cr_filter_signal_flag is inferred to be equal to 0 aps_chroma_present_flag.

At least one of the values of alf_luma_filter_signal_flag, alf_chroma_filter_signal_flag, alf_cc_cb_filter_signal_flag, and alf_cc_cr_filter_signal_flag shall be equal to 1.

...

# On ph_inter_slice_allowed_flag

When vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 0 for a picture, the layer containing the picture is not an independent layer and has at least one reference layer. For such a picture, naturally inter-layer prediction should be allowed, even when it is an IRAP picture. Therefore, it makes sense to require the value of the PH syntax element ph_inter_slice_allowed_flag to be equal to 1 when vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 0.

Therefore, it is proposed to constrain the value of ph_inter_slice_allowed_flag to be equal to 1 when vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 0.

# On aps_chroma_present_flag in scaling list APSs

Currently, when ChromaArrayType is greater than 0, the value of aps_chroma_present_flag is allowed to be equal to 0 for both ALF APSs and LMCS APSs, but not for scaling list APSs. Before the previous JVET meeting, during which the solution of using the default scaling list was adopted, the decoding process would be undefined if this were allowed. However, now that the solution of using the default scaling list is in place, we don't see a reason why to still keep this inconsistency and think that it would make more sense to make it consistent in this regard, at the same time it would provide more flexibility for encoders.

Therefore, it is proposed to allow the aps_chroma_present_flag of the scaling list APS NAL unit referenced by the PH to be equal to 0 when ChromaArrayType is greater than 0, by making the following text changes:

ph_scaling_list_aps_id specifies the aps_adaptation_parameter_set_id of the scaling list APS.

When ph_scaling_list_aps_id is present, the following applies:

- The TemporalId of the APS NAL unit having aps_params_type equal to SCALING_APS and aps_adaptation_parameter_set_id equal to ph_scaling_list_aps_id shall be less than or equal to the TemporalId of the picture associated with PH.

- When ChromaArrayType is equal to 0, tThe value of aps_chroma_present_flag of the APS NAL unit having aps_params_type equal to SCALING_APS and aps_adaptation_parameter_set_id equal to ph_scaling_list_aps_id shall be equal to 0 ChromaArrayType  = =  0 ? 0 : 1.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).