| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0053-v1 |
| --- | --- |

| Title: | AHG9: Inference of chroma deblocking offset values in the PH and SH |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhi Zhang Martin Pettersson Mitra Damghanian Jack Enhorn Kenneth Andersson Jacob Ström Rickard Sjöberg Ruoyang Yu Du Liu | Email: | zhi.a.zhang@ericsson.com martin.m.pettersson@ericsson.com mitra.damghanian@ericsson.com jack.enhorn@ericsson.com kenneth.r.andersson@ericsson.com jacob.strom@ericsson.com rickard.sjoberg@ericsson.com ruoyang.yu@ericsson.com du.liu@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution claims that chroma deblocking (DB) offset values (β and tC) that are signaled in the PPS or in the PH can never be used in the deblocking process due to the inference rule in the current VVC draft specification. This document contains two proposals:

Proposal 1 suggests that when the chroma DB offsets are not present in the PH and SH, the inference values are determined based on the value of pps_chroma_tool_offsets_present_flag. It is asserted that proposal 1 follows the intention of the decision in the meeting notes from the 18-th JVET meeting. The proposed changes are as follows (exact text is to be found in the contribution):

- When pps_chroma_tool_offsets_present_flag is equal to 1, the values of chroma DB offset syntax elements in the PH and SH, when not present, are inferred to be equal to the chroma DB offsets in the PPS and PH respectively.

- Otherwise, (pps_chroma_tool_offsets_present_flag is equal to 0), the values of the chroma DB offset syntax elements are inferred to be equal to the luma DB offsets in the same parameter set or header (same infererence rule as the current VVC).

As an alternative solution, the authors considered to infer the chroma DB offsets in the PH and SH to be equal to the values of the chroma DB offsets in the PPS and PH respectively when they are not present, but this solution is not recommended.

Proposal 2 changes the semantics of pps_chroma_tool_offsets_present_flag such that when the flag is equal to 1, the chroma tool offsets related syntax elements are present in the PPS RBSP syntax structure and the chroma deblocking tc and β offset syntax elements may be present in the PHs or the SHs of pictures referring to the PPS. The current draft uses “are” instead of “may be” above, and it is asserted by the proponents that there is a case when the offset syntax elements are not present in the PHs or the SHs of pictures referring to the PPS even though pps_chroma_tool_offsets_present_flag is equal to 1.

# Introduction

At the 18th JVET meeting, it was decided to condition the presence of the chroma deblocking (DB) β and tC offsets syntax elements in the PPS, PH and SH based on pps_chroma_tool_offsets_present_flag. In the meeting notes, the decision specified the inferred values of the chroma DB offsets as follows: “If the flag is zero, the chroma offsets (if needed) are inferred from the luma offsets.”. However, the meeting notes did not specify how to infer the chroma DB offset values when “the flag is one”. According to the current VVC draft specification, when the chroma DB offsets are not present in the PPS, PH and SH, the values of the chroma β and tC offsets are inferred to be equal to the luma β and tC offsets in the same syntax table. This is illustrated by Figure 1.

Figure 1 – Inference in the current VVC draft specification

These current inference rules for the chroma DB offsets in the PH and SH have a problem: When the chroma DB offsets are signaled in the PPS or PH, the signaled values are never used in the deblocking process because the deblocking process uses the SH chroma DB offsets values, and they are either signaled in the SH or are inferred to be equal to the SH luma DB offset values.

We believe that a desirable use case is to only signal luma and chroma deblocking offsets in the PPS, and use those values in the deblocking filtering process, but this is not supported in the current draft.

# Proposals

## Proposal 1

We propose to infer the values of the chroma DB offsets in the PH and SH based on the value of pps_chroma_tool_offsets_present_flag when the chroma DB offsets are not present in the PH or SH as follows:.

- When pps_chroma_tool_offsets_present_flag is equal to 1,

  - the values of the chroma DB offsets in the PH are inferred to be equal to the chroma DB offsets in the PPS.

  - the values of the chroma DB offsets in the SH are inferred to be equal to the chroma DB offsets in the PH.

- Otherwise, (pps_chroma_tool_offsets_present_flag is equal to 0), the values of the chroma DB offsets are inferred to be equal to the luma DB offsets in the same parameter set or header (same infererence rule as the current VVC).

Proposal 1 is asserted to follow the intention of the meeting notes from the 18-th JVET meeting. The proposal is illustrated by Figure 2.

Figure 2 – Inference as proposed in proposal 1

Proposed changes on top of the VVC draft (JVET-R2001-vA) are shown below with added text in yellow and removed text in red strike-through.

### Proposed PH changes

ph_cb_beta_offset_div2 and ph_cb_tc_offset_div2 specify the deblocking parameter offsets for β and tC (divided by 2) that are applied to the Cb component for the slices associated with the PH. The values of ph_cb_beta_offset_div2 and ph_cb_tc_offset_div2 shall both be in the range of −12 to 12, inclusive. When not present, the values of ph_cb_beta_offset_div2 and ph_cb_tc_offset_div2 are inferred as follows:to be equal to ph_luma_beta_offset_div2 and ph_luma_tc_offset_div2, respectively.

- When pps_chroma_tool_offsets_present_flag is equal to 1,  the values of ph_cb_beta_offset_div2 and ph_cb_tc_offset_div2 are inferred to be equal to pps_cb_beta_offset_div2 and pps_cb_tc_offset_div2, respectively.

- Otherwise, (pps_chroma_tool_offsets_present_flag is equal to 0), the values of ph_cb_beta_offset_div2 and ph_cb_tc_offset_div2 are inferred to be equal to ph_luma_beta_offset_div2 and ph_luma_tc_offset_div2, respectively.

ph_cr_beta_offset_div2 and ph_cr_tc_offset_div2 specify the deblocking parameter offsets for β and tC (divided by 2) that are applied to the Cr component for the slices associated with the PH. The values of ph_cr_beta_offset_div2 and ph_cr_tc_offset_div2 shall both be in the range of −12 to 12, inclusive. When not present, the values of ph_cr_beta_offset_div2 and ph_cr_tc_offset_div2 are inferred as follows:to be equal to ph_luma_beta_offset_div2 and ph_luma_tc_offset_div2, respectively.

- When pps_chroma_tool_offsets_present_flag is equal to 1,  the values of ph_cr_beta_offset_div2 and ph_cr_tc_offset_div2 are inferred to be equal to pps_cr_beta_offset_div2 and pps_cr_tc_offset_div2, respectively.

- Otherwise, (pps_chroma_tool_offsets_present_flag is equal to 0), the values of ph_cr_beta_offset_div2 and ph_cr_tc_offset_div2 are inferred to be equal to ph_luma_beta_offset_div2 and ph_luma_tc_offset_div2, respectively.

### Proposed SH changes

sh_cb_beta_offset_div2 and sh_cb_tc_offset_div2 specify the deblocking parameter offsets for β and tC (divided by 2) that are applied to the Cb component for the current slice. The values of sh_cb_beta_offset_div2 and sh_cb_tc_offset_div2 shall both be in the range of −12 to 12, inclusive. When not present, the values of sh_cb_beta_offset_div2 and sh_cb_tc_offset_div2 are inferred as follows:to be equal to sh_luma_beta_offset_div2 and sh_luma_tc_offset_div2, respectively.

- When pps_chroma_tool_offsets_present_flag is equal to 1,  the values of sh_cb_beta_offset_div2 and sh_cb_tc_offset_div2 are inferred to be equal to ph_cb_beta_offset_div2 and ph_cb_tc_offset_div2, respectively.

- Otherwise, (pps_chroma_tool_offsets_present_flag is equal to 0), the values of sh_cb_beta_offset_div2 and sh_cb_tc_offset_div2 are inferred to be equal to sh_luma_beta_offset_div2 and sh_luma_tc_offset_div2, respectively.

sh_cr_beta_offset_div2 and sh_cr_tc_offset_div2 specify the deblocking parameter offsets for β and tC (divided by 2) that are applied to the Cr component for the current slice. The values of sh_cr_beta_offset_div2 and sh_cr_tc_offset_div2 shall both be in the range of −12 to 12, inclusive. When not present, the values of sh_cr_beta_offset_div2 and sh_cr_tc_offset_div2 are inferred as follows:to be equal to sh_luma_beta_offset_div2 and sh_luma_tc_offset_div2, respectively.

- When pps_chroma_tool_offsets_present_flag is equal to 1,  the values of sh_cr_beta_offset_div2 and sh_cr_tc_offset_div2 are inferred to be equal to ph_cr_beta_offset_div2 and ph_cr_tc_offset_div2, respectively.

- Otherwise, (pps_chroma_tool_offsets_present_flag is equal to 0), the values of sh_cr_beta_offset_div2 and sh_cr_tc_offset_div2 are inferred to be equal to sh_luma_beta_offset_div2 and sh_luma_tc_offset_div2, respectively.

### An alternative solution

As an alternative solution, we considered to infer the chroma DB offsets in the PH and SH to be equal to the chroma DB offsets in the PPS and PH respectively when they are not present. This is illustrated by Figure 3.

Figure 3 – Inference of the alternative solution

Proposed PH changes for the alternative solution:

ph_cb_beta_offset_div2 and ph_cb_tc_offset_div2 specify the deblocking parameter offsets for β and tC (divided by 2) that are applied to the Cb component for the slices associated with the PH. The values of ph_cb_beta_offset_div2 and ph_cb_tc_offset_div2 shall both be in the range of −12 to 12, inclusive. When not present, the values of ph_cb_beta_offset_div2 and ph_cb_tc_offset_div2 are inferred to be equal to pps_cb_beta_offset_div2 and pps_cb_tc_offset_div2, respectively.ph_luma_beta_offset_div2 and ph_luma_tc_offset_div2, respectively.

ph_cr_beta_offset_div2 and ph_cr_tc_offset_div2 specify the deblocking parameter offsets for β and tC (divided by 2) that are applied to the Cr component for the slices associated with the PH. The values of ph_cr_beta_offset_div2 and ph_cr_tc_offset_div2 shall both be in the range of −12 to 12, inclusive. When not present, the values of ph_cr_beta_offset_div2 and ph_cr_tc_offset_div2 are inferred to be equal to pps_cr_beta_offset_div2 and pps_cr_tc_offset_div2, respectively.ph_luma_beta_offset_div2 and ph_luma_tc_offset_div2, respectively.

Proposed SH changes for the alternative solution

sh_cb_beta_offset_div2 and sh_cb_tc_offset_div2 specify the deblocking parameter offsets for β and tC (divided by 2) that are applied to the Cb component for the current slice. The values of sh_cb_beta_offset_div2 and sh_cb_tc_offset_div2 shall both be in the range of −12 to 12, inclusive. When not present, the values of sh_cb_beta_offset_div2 and sh_cb_tc_offset_div2 are inferred to be equal to ph_cb_beta_offset_div2 and ph_cb_tc_offset_div2, respectively.sh_luma_beta_offset_div2 and sh_luma_tc_offset_div2, respectively.

sh_cr_beta_offset_div2 and sh_cr_tc_offset_div2 specify the deblocking parameter offsets for β and tC (divided by 2) that are applied to the Cr component for the current slice. The values of sh_cr_beta_offset_div2 and sh_cr_tc_offset_div2 shall both be in the range of −12 to 12, inclusive. When not present, the values of sh_cr_beta_offset_div2 and sh_cr_tc_offset_div2 are inferred to be equal to ph_cr_beta_offset_div2 and ph_cr_tc_offset_div2, respectively.sh_luma_beta_offset_div2 and sh_luma_tc_offset_div2, respectively.

Although the amount of specification text changes for this alternative solution is shorter than for proposal 1, we recommend proposal 1 for the following reasons:

- When pps_chroma_tool_offsets_present_flag is equal to 0 and luma β and tC offsets are sent in either the PH or the SH, it is more appropriate to use the PH or SH luma β and tC offset values for chroma rather than the PPS luma β and tC offset values.

- To stay with the decision made at the previous meeting.

## Proposal 2

We also propose to change the semantics of pps_chroma_tool_offsets_present_flag such that when the flag is equal to 1, the chroma tool offsets related syntax elements are present in the PPS RBSP syntax structure and the chroma deblocking tc and β offset syntax elements may be present in the PHs or the SHs of pictures referring to the PPS.

Even if pps_chroma_tool_offsets_present_flag is equal to 1, the chroma deblocking tc and β offset syntax elements are not present in the PHs or the SHs if any of the following conditions are true:

- pps_deblocking_filter_override_enabled_flag is equal to 0,

- ph_deblocking_filter_override_flag or sh_deblocking_filter_override_flag is equal to 0,

- ph_deblocking_filter_disabled_flag or sh_deblocking_filter_disabled_flag is equal to 1

### Proposed PPS changes

pps_chroma_tool_offsets_present_flag equal to 1 specifies that chroma tool offsets related syntax elements are present in the PPS RBSP syntax structure and the chroma deblocking tc and β offset syntax elements may be are present in the PHs or the SHs of pictures referring to the PPS. pps_chroma_tool_offsets_present_flag equal to 0 specifies that chroma tool offsets related syntax elements are not present in in the PPS RBSP syntax structure and the chroma deblocking tc and β offset syntax elements are not present in the PHs or the SHs of pictures referring to the PPS. When ChromaArrayType is equal to 0, the value of pps_chroma_tool_offsets_present_flag shall be equal to 0.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).