| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0064-v1 |
| --- | --- |

| Title: | AHG9: On Buffering Period Message Signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sachin Deshpande Jonatan Samuelsson Andrew Segall Philip Cowan | Tel: Email: | +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

Following is proposed as a cleanup:

It is proposed to conditionally signal bp_sublayer_dpb_output_offsets_present_flag, only when bp_max_sublayers_minus1 is greater than 0 and infer it otherwise.

# Introduction

VVC Draft text [1] includes a design of HRD including a buffering period SEI message.

It is asserted that under certain conditions a Buffering Period SEI message syntax element (bp_sublayer_dpb_output_offsets_present_flag) has a known value and thus does not need to be signalled and instead can be inferred.

Thus following is proposed as a cleanup:

It is proposed to conditionally signal bp_sublayer_dpb_output_offsets_present_flag, only when bp_max_sublayers_minus1 is greater than 0 and infer it otherwise.

# Proposals

It is proposed to conditionally signal bp_sublayer_dpb_output_offsets_present_flag, only when bp_max_sublayers_minus1 is greater than 0. It is asserted that bp_sublayer_dpb_output_offsets_present_flag controls the presence of bp_dpb_output_tid_offset[ i ] for i in the range of 0 to bp_max_sublayers_minus1−1, inclusive. As a result when bp_max_sublayers_minus1 is equal to 0, none of bp_dpb_output_tid_offset[ i ]  syntax elements are actually signalled and thus there is no need to waste signalling and parsing bp_sublayer_dpb_output_offsets_present_flag in this case and its value can be inferred.

The proposed changes are shown highlighted compared with VVC Draft Text (JVET-R2001-v9).

D.3. Buffering period SEI message

D.3.1 Buffering period SEI message syntax

| buffering_period( payloadSize ) { | Descriptor |
| --- | --- |
| bp_nal_hrd_params_present_flag | u(1) |
| bp_vcl_hrd_params_present_flag | u(1) |
| … |  |
| bp_max_sublayers_minus1 | u(3) |
| bp_cpb_cnt_minus1 | ue(v) |
| … |  |
| if(bp_max_sublayers_minus1 > 0) |  |
| bp_sublayer_dpb_output_offsets_present_flag | u(1) |
| if( bp_sublayer_dpb_output_offsets_present_flag ) |  |
| for( i = 0; i < bp_max_sublayers_minus1; i++ ) |  |
| bp_dpb_output_tid_offset[ i ] | ue(v) |
| if( bp_alt_cpb_params_present_flag ) |  |
| bp_use_alt_cpb_params_flag | u(1) |
| } |  |

bp_sublayer_dpb_output_offsets_present_flag equal to 1 specifies that DPB output time offsets are present for temporal sublayer representation(s) in the range of 0 to bp_max_sublayers_minus1 − 1, inclusive. bp_sublayer_dpb_output_offsets_present_flag equal to 0 specified that no such DPB output time offsets are present. When not present bp_sublayer_dpb_output_offsets_present_flag is inferred to be equal to 0.

Alternatively, a conformance constraint is proposed on bp_sublayer_dpb_output_offsets_present_flag based on the value of bp_max_sublayers_minus1.

It is a requirement of bitstream conformance that when bp_max_sublayers_minus1 is equal to 0, bp_sublayer_dpb_output_offsets_present_flag shall be equal to 0.

# Conclusion

This document proposes modifications related to the buffering period message signaling. It is proposed to adopt the proposal in the next version of VVC draft text.

# Patent rights declaration(s)

Sharp Labs of America, Inc., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] JVET-R2001, Versatile Video Coding (Draft 9), April 2020