| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0067-v1 |
| --- | --- |

| Title: | AHG9: General constraint information on features using APS |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shih-Ta Hsiang, Olena Chubach, Lulin Chen, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {shih-ta.hsiang, olena.chubach, lulin.chen, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution proposes the following modifications to the general constraint information on features using APS:

- Mandate the value of no_lmcs_constraint_flag to be equal to 1 when no_aps_constraint_flag is equal to 1.

- Add a new general constraint flag no_explicit_scaling_list_constraint_flag constraining sps_explicit_scaling_list_enabled_flag to be equal to 0.

# Proposals

## Proposal 1: Semantic modifications for no_lmcs_constraint_flag

In VVC Draft 9 [1], when the value of the syntax element no_aps_constraint_flag is equal to 1, it is a requirement of the bitstream conformance that the value of the syntax element sps_lmcs_enabled_flag shall be equal to 0 and the constraint of not enabling LMCS shall be always satisfied. This contribution proposes to mandate that the value of the constraint flag no_lmcs_constraint_flag shall be equal to 1 when no_aps_constraint_flag is equal to 1. The proposed modifications to VVC Draft 9 with modified parts highlighted in turquoise are provided as follows:

      - 7.4.4.2 General constraint information semantics

no_lmcs_constraint_flag equal to 1 specifies that sps_lmcs_enabled_flag shall be equal to 0. no_lmcs_constraint_flag equal to 0 does not impose such a constraint. When no_aps_constraint_flag is equal to 1, the value of no_lmcs_constraint_flag shall be equal to 1.

## Proposal 2: New constraint flag no_explicit_scaling_list_constraint_flag

This contribution further proposes to add a new constraint flag no_explicit_scaling_list_constraint_flag for constraining the syntax element sps_explicit_scaling_list_enabled_flag to be equal to 0. The proposed modifications to VVC Draft 9 with modified parts highlighted in turquoise are provided as follows:

      - 7.3.3.2 General constraint information syntax

| general_constraint_info( ) { | Descriptor |
| --- | --- |
| … |  |
| no_lmcs_constraint_flag | u(1) |
| no_explicit_scaling_list_constraint_flag |  |
| … |  |
| no_aps_constraint_flag | u(1) |
| … |  |
| } |  |

      - 7.4.4.2 General constraint information semantics

no_explicit_scaling_list_constraint_flag equal to 1 specifies that sps_explicit_scaling_list_enabled_flag shall be equal to 0. no_explicit_scaling_list_constraint_flag equal to 0 does not impose such a constraint. When no_aps_constraint_flag is equal to 1, the value of no_explicit_scaling_list _constraint_flag shall be equal to 1.

# References

- B. Bross, J. Chen, and S. Liu “Versatile Video Coding (Draft 9),” Document of Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-R2001, 18th Meeting: by teleconference, 15–24 April 2020.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).