| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0069-v1 |
| --- | --- |

| Title: | AHG9: General constraint information on features related to transform skip mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shih-Ta Hsiang, Olena Chubach, Lulin Chen, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {shih-ta.hsiang, olena.chubach, lulin.chen, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

- Abstract

This contribution proposes following modifications to the general constraint information of features related to transform skip mode:

- Mandate the value of no_bdpcm_constraint_flag to be equal to 1 when no_transform_skip_constraint_flag is equal to 1.

- Fix the spec bug by having no_tsrc_constraint_flag constrain sh_ts_residual_coding_disabled_flag to be equal to 1 (#1071). Modify the constraint of no_tsrc_constraint_flag such that sps_transform_skip_enabled_flag equal to 0 satisfies the constraint imposed by no_tsrc_constraint_flag equal to 1. Mandate the value of no_tsrc_constraint_flag to be equal to 1 when no_transform_skip_constraint_flag is equal to 1.

Introduction

This contribution proposes modifications to the general constraint information of features related to transform skip mode. The high-level syntax tables and semantics related to using transform skip mode according to VVC Draft 9 [1] are provided as follows:

#### 7.3.2.3 Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_seq_parameter_set_id | u(4) |
| … |  |
| sps_transform_skip_enabled_flag | u(1) |
| if( sps_transform_skip_enabled_flag ) { |  |
| log2_transform_skip_max_size_minus2 | ue(v) |
| sps_bdpcm_enabled_flag | u(1) |
| } |  |
| … |  |
| } |  |

      - 7.3.3.2 General constraint information syntax

| general_constraint_info( ) { | Descriptor |
| --- | --- |
| … |  |
| no_transform_skip_constraint_flag | u(1) |
| no_bdpcm_constraint_flag |  |
| … |  |
| no_sign_data_hiding_constraint_flag | u(1) |
| no_tsrc_constraint_flag | u(1) |
| … |  |
| } |  |

      - 7.3.7.1 General slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_transform_skip_enabled_flag && !sh_dep_quant_enabled_flag && !sh_sign_data_hiding_enabled_flag ) |  |
| sh_ts_residual_coding_disabled_flag | u(1) |
| … |  |
| } |  |

      - 7.4.3.3. Sequence parameter set RBSP semantics

sps_transform_skip_enabled_flag equal to 1 specifies that transform_skip_flag may be present in the transform unit syntax. sps_transform_skip_enabled_flag equal to 0 specifies that transform_skip_flag is not present in the transform unit syntax.

log2_transform_skip_max_size_minus2 specifies the maximum block size used for transform skip, and shall be in the range of 0 to 3, inclusive.

The variable MaxTsSize is set equal to 1  <<  ( log2_transform_skip_max_size_minus2 + 2 ).

sps_bdpcm_enabled_flag equal to 1 specifies that intra_bdpcm_luma_flag and intra_bdpcm_chroma_flag may be present in the coding unit syntax for intra coding units. sps_bdpcm_enabled_flag equal to 0 specifies that intra_bdpcm_luma_flag and intra_bdpcm_chroma_flag are not present in the coding unit syntax for intra coding units. When not present, the value of sps_bdpcm_enabled_flag is inferred to be equal to 0.

      - 7.4.4.2 General constraint information semantics

no_transform_skip_constraint_flag equal to 1 specifies that sps_transform_skip_enabled_flag shall be equal to 0. no_transform_skip_constraint_flag equal to 0 does not impose such a constraint.

no_bdpcm_constraint_flag equal to 1 specifies that sps_bdpcm_enabled_flag shall be equal to 0. no_bdpcm_constraint_flag equal to 0 does not impose such a constraint.

no_tsrc_constraint_flag equal to 1 specifies that sh_ts_residual_coding_disabled_flag shall be equal to 0. no_tsrc_constraint_flag equal to 0 does not impose such a constraint.

      - 7.4.8.1 General slice header semantics

sh_ts_residual_coding_disabled_flag equal to 1 specifies that the syntax structure residual_coding( ) is used to parse the residual samples of a transform skip block for the current slice. sh_ts_residual_coding_disabled_flag equal to 0 specifies that the syntax structure residual_ts_coding( ) is used to parse the residual samples of a transform skip block for the current slice. When sh_ts_residual_coding_disabled_flag is not present, it is infered to be equal to 0.

Proposals

Proposal 1: no_bdpcm_constraint_flag imposed by no_transform_skip_constraint_flag

In VVC Draft 9, the syntax element no_transform_skip_constraint_flag equal to 1 specifies that the value of the syntax element sps_transform_skip_enabled_flag shall be equal to 0. When sps_transform_skip_enabled_flag is equal to 0, the syntax element sps_bdpcm_enabled_flag is not present and is inferred to be equal to 0. Therefore, when the constraint of no_transform_skip_constraint_flag equal to 1 is imposed, the constraint of not enabling BDPCM is also imposed.

This contribution proposes to mandate that the value of the constraint flag no_bdpcm_constraint_flag shall be equal to 1 when no_transform_skip_constraint_flag is equal to 1. The proposed modifications to VVC Draft 9 with modified parts highlighted in turquoise are provided as follows:

      - 7.4.4.2 General constraint information semantics

no_bdpcm_constraint_flag equal to 1 specifies that sps_bdpcm_enabled_flag shall be equal to 0. no_bdpcm_constraint_flag equal to 0 does not impose such a constraint. When no_transform_skip_constraint_flag is equal to 1, the value of no_bdpcm_constraint_flag shall be equal to 1.

Proposal 2: Modified constraint for no_tsrc_constraint_flag

In VVC Draft 9, the syntax element no_tsrc_constraint_flag equal to 1 specifies that the syntax element sh_ts_residual_coding_disabled_flag shall be equal to “0”. It does not agree with the decision recorded on the meeting notes “no_tsrc_constraint_flag constraining slice_ts_residual_coding_disabled_flag to be equal to 1”. This contribution proposes to fix this spec bug by having no_tsrc_constraint_flag constrain sh_ts_residual_coding_disabled_flag to be equal to “1”. A bug ticket on this issue has been submitted (#1071).

With the bug fix, no_tsrc_constraint_flag equal to 1 specifies that sh_ts_residual_coding_disabled_flag shall be equal to 1. In order to satisfy such a constraint, sh_ts_residual_coding_disabled_flag shall be explicitly signalled in the bitstream with the value equal to 1 for each slice header. Moreover, sps_transform_skip_enabled_flag shall be equal to 1 in the SPS and both sh_dep_quant_enabled_flag and sh_sign_data_hiding_enabled_flag shall be equal to 0 for each slice header (to allow signalling sh_ts_residual_coding_disabled_flag explicitly in the bitstream).

This contribution proposes to modify the constraint imposed by no_tsrc_constraint_flag. It is asserted that the design intent of no_tsrc_constraint_flag is for imposing the constraint of not using the transform skip residual coding process in the bitstream. Such a constraint can be satisfied when transform skip mode is not enabled. The three options of proposed semantic modifications to VVC Draft 9 with modified parts highlighted in turquoise are provided as follows:

- Option 1

      - 7.4.4.2 General constraint information semantics

no_tsrc_constraint_flag equal to 1 specifies that sh_ts_residual_coding_disabled_flag shall be equal to 0 1 or sps_transform_skip_enabled_flag shall be equal to 0. no_tsrc_constraint_flag equal to 0 does not impose such a constraint. When no_transform_skip_constraint_flag is equal to 1, the value of no_tsrc_constraint_flag shall be equal to 1.

- Option 2

      - 7.4.4.2 General constraint information semantics

no_tsrc_constraint_flag equal to 1 specifies that sh_ts_residual_coding_disabled_flag shall be equal to 0 1 when sps_transform_skip_enabled_flag is equal to 1. no_tsrc_constraint_flag equal to 0 does not impose such a constraint. When no_transform_skip_constraint_flag is equal to 1, the value of no_tsrc_constraint_flag shall be equal to 1.

- Option 3

      - 7.4.4.2 General constraint information semantics

no_tsrc_constraint_flag equal to 1 specifies that sh_ts_residual_coding_disabled_flag shall be equal to 0 1. no_tsrc_constraint_flag equal to 0 does not impose such a constraint. When no_transform_skip_constraint_flag is equal to 1, the value of no_tsrc_constraint_flag shall be equal to 1.

      - 7.4.8.1 General slice header semantics

sh_ts_residual_coding_disabled_flag equal to 1 specifies that the syntax structure residual_coding( ) is used to parse the residual samples of a transform skip block for the current slice. slice_ts_residual_coding_disabled_flag equal to 0 specifies that the syntax structure residual_ts_coding( ) is used to parse the residual samples of a transform skip block for the current slice. When slice_ts_residual_coding_disabled_flag is not present, it is infered to be equal to 0.the following applies:

–      If sps_transform_skip_enabled_flag is equal to 0, the value of sh_ts_residual_coding_disabled_flag is inferred to be equal to 1.

–      Otherwise, the value of sh_ts_residual_coding_disabled_flag is inferred to be equal to 0.

References

- B. Bross, J. Chen, and S. Liu “Versatile Video Coding (Draft 9),” Document of Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-R2001, 18th Meeting: by teleconference, 15–24 April 2020.

Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).