| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0070-v1 |
| --- | --- |

| Title: | AHG9: On flag ph_gdr_or_irap_pic_flag |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Lulin Chen, Chih-Wei Hsu, Shih-Ta Hsiang, Olena Chubach, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {lulin.chen, cw.hsu, shih-ta.hsiang, olena.chubach, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

- Abstract

This contribution proposes to improve the signalling of an IRAP picture and a GDR picture in the PH.

Proposal 1:

- Rename ph_gdr_or_irap_pic_flag to ph_irap_pic_flag.

- Make ph_irap_pic_flag and ph_gdr_pic_flag the two way flags for an IRAP picture and a GDR picture, respectively.

- Signal the flag ph_no_output_of_prior_pics_flag on the condition of ph_irap_pic_flag or ph_gdr_pic_flag.

- Signal the flag ph_inter_slice_allowed_flag only for picture with no inter-layer prediction and non-irap picture.

Proposal 2:

- Proposal 1 with moving down the location of syntax elements ph_inter_slice_allowed_flag and ph_intra_slice_allowed_flag.

- Signal the flag ph_inter_slice_allowed_flag only for picture with no inter-layer prediction and non-irap picture and non-gdr picture when ph_recovery_poc_cnt is equal to 0.

Proposals

In the current VVC Draft 9 [1], the joint flag ph_gdr_or_irap_pic_flag is a two way flag for signalling a GDR picture, while it is a one way flag for signalling an IRAP picture. During the last JVET meeting, it was commented that the current notion (ph_gdr_or_irap_pic_flag) is to allow a system to check just one bit to determine whether random access is possible or not. When ph_gdr_or_irap_pic_flag is equal to 1, the flag ph_gdr_pic_flag equal to 0 is used to further signal an IRAP picture. Further, it was mentioned that perhaps instead of using ph_gdr_or_irap_pic_flag, it may be simpler to just have two flags: ph_irap_pic_flag and ph_gdr_pic_flag. Also, it was suggested that the presence of one could be conditioned on the other. This topic was proposed to be studied offline.

When ph_gdr_or_irap_pic_flag is equal to 0, the current picture is not a GDR picture while it may or may not be an IRAP picture. In this case, the flag ph_no_output_of_prior_pics_flag is skipped but no inference rule is specified if it is an IRAP picture.

This contribution proposes to improve the signalling of an IRAP picture and a GDR picture in the PH.

Proposal 1:

- Rename ph_gdr_or_irap_pic_flag to ph_irap_pic_flag.

- Make ph_irap_pic_flag and ph_gdr_pic_flag the two way flags for an IRAP picture and a GDR picture, respectively.

- Signal the flag ph_no_output_of_prior_pics_flag on the condition of ph_irap_pic_flag or ph_gdr_pic_flag.

- Signal the flag ph_inter_slice_allowed_flag only for picture with no inter-layer prediction and non-irap picture.

Proposal 2:

- Proposal 1 with moving down the location of syntax elements ph_inter_slice_allowed_flag and ph_intra_slice_allowed_flag.

- Signal the flag ph_inter_slice_allowed_flag only for picture with no inter-layer prediction and non-irap picture and non-gdr picture when ph_recovery_poc_cnt is equal to 0.

The relevant and modified text parts are highlighted in cyan and green, respectively.

Proposal 1

| picture_header_structure( ) { | Descriptor |
| --- | --- |
| ph_gdr_or_irap_pic_flag ph_irap_pic_flag | u(1) |
| if( ph_gdr_or_irap_pic_flag !ph_irap_pic_flag ) |  |
| ph_gdr_pic_flag | u(1) |
| if( !sps_inter_layer_ref_pics_present_flag && !ph_irap_pic_flag) |  |
| ph_inter_slice_allowed_flag | u(1) |
| if( ph_inter_slice_allowed_flag ) |  |
| ph_intra_slice_allowed_flag | u(1) |
| ph_non_ref_pic_flag | u(1) |
| ph_pic_parameter_set_id | ue(v) |
| ph_pic_order_cnt_lsb | u(v) |
| if( ph_gdr_or_irap_pic_flag ph_irap_pic_flag ¦ ¦ ph_gdr_pic_flag ) |  |
| ph_no_output_of_prior_pics_flag | u(1) |
| if( ph_gdr_pic_flag ) |  |
| ph_recovery_poc_cnt | ue(v) |
| for( i = 0; i < NumExtraPhBits; i++ ) |  |
| ph_extra_bit[ i ] | u(1) |
| … |  |
| } |  |

ph_gdr_or_irap_pic_flag equal to 1 specifies that the current picture is a GDR or IRAP picture. ph_gdr_or_irap_pic_flag equal to 0 specifies that the current picture is not a GDR picture and may or may not be an IRAP picture.

ph_irap_pic_flag equal to 1 specifies that the current picture is an IRAP picture. ph_irap_pic_flag equal to 0 specifies that the current picture is not an IRAP picture.

ph_gdr_pic_flag equal to 1 specifies that the picture associated with the PH is a GDR picture. ph_gdr_pic_flag equal to 0 specifies that the picture associated with the PH is not a GDR picture. When not present, the value of ph_gdr_pic_flag is inferred to be equal to 0. When sps_gdr_enabled_flag is equal to 0, the value of ph_gdr_pic_flag shall be equal to 0.

ph_inter_slice_allowed_flag equal to 0 specifies that all coded slices of the picture have sh_slice_type equal to 2. ph_inter_slice_allowed_flag equal to 1 specifies that there may or may not be one or more coded slices in the picture that have sh_slice_type equal to 0 or 1. When not present, the value of ph_inter_slice_allowed_flag is inferred to be equal to 0.

When when ph_gdr_or_irap_pic_flag is equal to 1 and ph_gdr_pic_flag is equal to 0 (i.e., the picture is an IRAP picture), and vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 1, the value of ph_inter_slice_allowed_flag shall be equal to 0.

Proposal 2

| picture_header_structure( ) { | Descriptor |
| --- | --- |
| ph_gdr_or_irap_pic_flag ph_irap_pic_flag | u(1) |
| if(ph_gdr_or_irap_pic_flag !ph_irap_pic_flag ) |  |
| ph_gdr_pic_flag | u(1) |
| ph_inter_slice_allowed_flag | u(1) |
| if( ph_inter_slice_allowed_flag ) |  |
| ph_intra_slice_allowed_flag | u(1) |
| ph_non_ref_pic_flag | u(1) |
| ph_pic_parameter_set_id | ue(v) |
| ph_pic_order_cnt_lsb | u(v) |
| if(ph_gdr_or_irap_pic_flag ph_irap_pic_flag ¦ ¦ ph_gdr_pic_flag ) |  |
| ph_no_output_of_prior_pics_flag | u(1) |
| if( ph_gdr_pic_flag ) |  |
| ph_recovery_poc_cnt | ue(v) |
| if( !sps_inter_layer_ref_pics_present_flag && !ph_irap_pic_flag && !(ph_gdr_pic_flag && ph_recovery_poc_cnt == 0) ) |  |
| ph_inter_slice_allowed_flag | u(1) |
| if( ph_inter_slice_allowed_flag ) |  |
| ph_intra_slice_allowed_flag | u(1) |
| for( i = 0; i < NumExtraPhBits; i++ ) |  |
| ph_extra_bit[ i ] | u(1) |
| … |  |
| } |  |

ph_gdr_or_irap_pic_flag equal to 1 specifies that the current picture is a GDR or IRAP picture. ph_gdr_or_irap_pic_flag equal to 0 specifies that the current picture is not a GDR picture and may or may not be an IRAP picture.

ph_irap_pic_flag equal to 1 specifies that the current picture is an IRAP picture. ph_irap_pic_flag equal to 0 specifies that the current is not an IRAP picture.

ph_gdr_pic_flag equal to 1 specifies the picture associated with the PH is a GDR picture. ph_gdr_pic_flag equal to 0 specifies that the picture associated with the PH is not a GDR picture. When not present, the value of ph_gdr_pic_flag is inferred to be equal to 0. When sps_gdr_enabled_flag is equal to 0, the value of ph_gdr_pic_flag shall be equal to 0.

ph_inter_slice_allowed_flag equal to 0 specifies that all coded slices of the picture have sh_slice_type equal to 2. ph_inter_slice_allowed_flag equal to 1 specifies that there may or may not be one or more coded slices in the picture that have sh_slice_type equal to 0 or 1. When not present, the value of ph_inter_slice_allowed_flag is inferred to be equal to 0.

When when ph_gdr_or_irap_pic_flag is equal to 1 and ph_gdr_pic_flag is equal to 0 (i.e., the picture is an IRAP picture), and vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 1, the value of ph_inter_slice_allowed_flag shall be equal to 0.

References

- B. Bross, J. Chen, and S. Liu “Versatile Video Coding (Draft 9),” Document of Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-R2001-vA, 18th Meeting: by teleconference, 15–24 April 2020.

Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).