| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0075 |
| --- | --- |

| Title: | AHG9: On constraints and inference values when VPS is not present |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Seethal Paluri Seung Hwan Kim | Email: | dr.hendry@lge.com seethal.paluri@lge.com seunghwan3.kim@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This contribution asserted that the following aspects in the current VVC spec text are not clear when VPS is not present for a CVS (i.e., the value of sps_video_parameter_set_id is equal to 0):

- It is currently constrained that, when VPS is not present in a CVS, the CVS shall contain only one layer (i.e., all VCL NAL unit in the CVS shall have the same value of nuh_layer_id). It is asserted the constraint is not complete. It does not provide constraint on non-VCL NAL units such as parameter sets referred to by the VCL NAL units in the CVS.

- When VPS is not present in a CVS, OLS information is not established. Consequently, some variables needed such as TotalNumOlss and NumLayersInOls[ ] are not available as they are only derived when VPS is present. However, these variables are referred to in decoding process.

Based on the above assertions, the following changes are proposed:

- When VPS is not present in the CVS, the CVS shall contain only one layer (i.e., all VCL NAL unit in the CVS and the parameter sets they referred to shall have the same value of nuh_layer_id).

- When VPS is not present in the CVS, the value of TotalNumOlss and NumLayersInOls[ 0 ] are inferred to be equal to 1.

# Problem Statement

According to the current VVC spec text, the presence of VPS in the CVS is optional when it contains only one layer. When VPS is not present (i.e., the value of sps_video_parameter_set_id is equal to 0), inference for values of some variables or syntax elements and also constraints that are needed for decoding process have been established. However, it is asserted more inferences and constraints are needed.

The first aspect that is needed is regarding the number of layers and parameter set referencing. According to spec text, when sps_video_parameter_set_id is equal to 0, the CVS shall contain only one layer (i.e., all VCL NAL unit in the CVS shall have the same value of nuh_layer_id). This is asserted incomplete since it should also be constrained that the VCL NAL units refers to parameter sets in the same layer).

It may be argued that the current constraint about parameter set reference and sharing may have prevented the case such that VCL NAL unit in a single layer CVS refers to parameter set from other layer. However, in the authors’ understanding, it is not covered. The snippet text below show the constraint about parameter set refereence and sharing.

Let spsLayerId be the value of the nuh_layer_id of a particular SPS NAL unit, and vclLayerId be the value of the nuh_layer_id of a particular VCL NAL unit. The particular VCL NAL unit shall not refer to the particular SPS NAL unit unless spsLayerId is less than or equal to vclLayerId and all OLSs specified by the VPS that contain the layer with nuh_layer_id equal to vclLayerId also contain the layer with nuh_layer_id equal to spslayerId.

Note that the contraint applies only when VPS is present since the condition is based on all OLSs specified by the VPS. When VPS is not present, there is nothing to check on.

The second aspect that is needed is regarding inference value for variables / syntax element related to output layer set (OLS). Some variables related to OLS are not specified when VPS is not present but they are referred to. For example the variable NumLayersInOls[ ] is referred to in Annex A and Annex C to determine which set of PTL, DPB, and HRD parameters to be used, whether from VPS or SPS, but its value is not set when VPS is not present. Likewise, the variable TotalNumOlss is also not initialized in such case.

# Proposal

To overcome the problems described above, the following changes are proposed:

- When VPS is not present in the CVS, the CVS shall contain only one layer (i.e., all VCL NAL unit in the CVS and the parameter sets they referred to shall have the same value of nuh_layer_id).

- When VPS is not present in the CVS, the value of TotalNumOlss and NumLayersInOls[ 0 ] are inferred to be equal to 1.

The spec text changes for implementing the above proposed changes is as follows:

sps_video_parameter_set_id, when greater than 0, specifies the value of vps_video_parameter_set_id for the VPS referred to by the SPS.

When sps_video_parameter_set_id is equal to 0, the following applies:

–	The SPS does not refer to a VPS, and no VPS is referred to when decoding each CLVS referring to the SPS.

–	The value of vps_max_layers_minus1 is inferred to be equal to 0.

–	The value of vps_max_sublayers_minus1 is inferred to be equal to 6.

–	The CVS shall contain only one layer (i.e., all VCL NAL unit in the CVS and parameter sets they referred to shall have the same value of nuh_layer_id).

–	The value of TotalNumOlss is inferred to be equal to 1.

–	The value of NumLayersInOls[ 0 ] is inferred to be equal to 1.

–	The value of GeneralLayerIdx[ nuh_layer_id ] is inferred to be equal to 0.

–	The value of vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is inferred to be equal to 1.

When vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 1, the SPS referred to by a CLVS with a particular nuh_layer_id value nuhLayerId shall have nuh_layer_id equal to nuhLayerId.

The value of sps_video_parameter_set_id shall be the same in all SPSs that are referred to by CLVSs in a CVS.

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).