| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0077 |
| --- | --- |

| Title: | AHG9: On derivation of PictureOutputFlag for RASL |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Hyeongmun Jang Junghak Nam Seethal Paluri Seung-hwan Kim Jaehyun Lim | Email: | dr.hendry@lge.com hm.jang@lge.com junghak.nam@lge.com seethal.paluri@lge.com seunghwan3.kim@lge.com jaehyun.lim@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

# Abstract

It is asserted that in the current specification text, the derivation of PictureOutputFlag for RASL picture is unclear. Because a picture containing with mix of RASL_NUT and RADL_NUT NAL units is considered as RASL picture in the decoding process as well, when receiving the first slice of a picture with nal_unit_type that is RASL_NUT or RADL_NUT, decoder cannot determine whether the picture a RASL picture or not. Consequently, decoder cannot set the value of PictureOutputFlag properly since the rest of slices of the picture, which the decoder has not received yet, can be slice with TRAIL_NUT or STSA_NUT.

To solve the above asserted problem, in this contribution, the following is proposed:

- Add a constrained that when a picture contains a mix of RASL_NUT with NAL unit type(s) other than RADL_NUT or a mixed of RADL_NUT with other NAL unit type(s) other than RASL_NUT, then the following applies:

  - If interlace coding is not used (i.e., sps_field_seq_flag is equal to 0), there shall be at least one non-leading picture between the picture and its associated IRAP picture.

  - Otherwise, if interlace coding is used (i.e., sps_field_seq_flag is equal to 1), there shall be at least two non-leading pictures between the picture and its associated IRAP picture.

- Add a NOTE to clarify the derivation of PictureOutputFlag that when decoder received the first slice of the current picture and the NAL unit type of the slice is either RASL_NUT or RADL_NUT and the value of pps_mixed_nalu_types_in_pic_flag is equal to 1, if decoder has not received any non-leading picture from the last IRAP picture, then decoder can determine that the current picture is a RASL picture.

# Problem Statement

In the current VVC spec text, the derivation process for PictureOutputFlag is not clear particularly for RASL picture. When decoder receives the first slice of the current picture and the NAL unit type of the slice is either RASL_NUT or RADL_NUT and the value of pps_mixed_nalu_types_in_pic_flag is equal to 1, decoder cannot determine if the picture is actually a RASL picture or not since the NAL unit type of the rest slice(s) of the picture can also be either TRAIL_NUT or STSA_NUT.

In our observation, when a RASL subpicture is merged with a RADL subpicture, the merged picture would still be associated with the CRA with the same POC value as in their original bitstream. Further, we have a constraint such that leading pictures shall precede, in both decoding and output order, trailing pictures that are associated with the same IRAP picture. By adapting the constraint considering the picture with mixed NAL unit types, we can solve the problem described above.

# Proposal and spec text changes

We propose the following:

- Add a constrained that when a picture contains a mix of RASL_NUT with NAL unit type(s) other than RADL_NUT or a mixed of RADL_NUT with other NAL unit type(s) other than RASL_NUT, then the following applies:

  - If interlace coding is not used (i.e., sps_field_seq_flag is equal to 0), there shall be at least one non-leading picture between the picture and its associated IRAP picture.

  - Otherwise, if interlace coding is used (i.e., sps_field_seq_flag is equal to 1), there shall be at least two non-leading pictures between the picture and its associated IRAP picture.

- Add a NOTE to clarify the derivation of PictureOutputFlag that when decoder received the first slice of the current picture and the NAL unit type of the slice is either RASL_NUT or RADL_NUT and the value of pps_mixed_nalu_types_in_pic_flag is equal to 1, if decoder has not received any non-leading picture from the last IRAP picture, then decoder can determine that the current picture is a RASL picture.

The spec text changes for implementing the above proposed changes is as follows:

#### 7.4.2.2 NAL unit header semantics

...

It is a requirement of bitstream conformance that the following constraints apply:

–	When a picture is a leading picture of an IRAP picture, it shall be a RADL or RASL picture.

–	When a subpicture is a leading subpicture of an IRAP subpicture, it shall be a RADL or RASL subpicture.

–	When a picture is not a leading picture of an IRAP picture, it shall not be a RADL or RASL picture.

–	When a subpicture is not a leading subpicture of an IRAP subpicture, it shall not be a RADL or RASL subpicture.

–	....

–	If sps_field_seq_flag is equal to 0 and the current picture, with nuh_layer_id equal to a particular value layerId, is a leading picture associated with an IRAP picture, it shall precede, in decoding order, all non-leading pictures that are associated with the same IRAP picture. Otherwise, let picA and picB be the first and the last leading pictures, in decoding order, associated with an IRAP picture, respectively, there shall be at most one non-leading picture with nuh_layer_id equal to layerId preceding picA in decoding order, and there shall be no non-leading picture with nuh_layer_id equal to layerId between picA and picB in decoding order.

–	If sps_field_seq_flag is equal to 0 and the current subpicture, with nuh_layer_id equal to a particular value layerId and subpicture index equal to a particular value subpicIdx, is a leading subpicture associated with an IRAP subpicture, it shall precede, in decoding order, all non-leading subpictures that are associated with the same IRAP subpicture. Otherwise, let subpicA and subpicB be the first and the last leading subpictures, in decoding order, associated with an IRAP subpicture, respectively, there shall be at most one non-leading subpicture with nuh_layer_id equal to layerId and subpicture index equal to subpicIdx preceding subpicA in decoding order, and there shall be no non-leading picture with nuh_layer_id equal to layerId and subpicture index equal to subpicIdx between picA and picB in decoding order.

–	When current picture, with nuh_layer_id equal to a particular value layerId, is a picture with pps_mixed_nalu_types_in_pic_flag equal to 1 containing at least one slice with NAL unit type other than RASL_NUT or RADL_NUT, the following applies:

–	If sps_field_seq_flag is equal to 0, there shall be at least one picture that is not a leading picture (i.e., not a RASL picture nor a RADL picture) that precede the current picture and follow the associated IRAP picture, in decoding order.

–	Otherwise, there shall be at least two pictures that are not a leading picture (i.e., not a RASL picture nor a RADL picture) that precede the current picture and follow the associated IRAP picture, in decoding order.

8.1.2	Decoding process for a coded picture

–	The variable PictureOutputFlag of the current picture is derived as follows:

–	If sps_video_parameter_set_id is greater than 0 and the current layer is not an output layer (i.e., nuh_layer_id is not equal to OutputLayerIdInOls[ TargetOlsIdx ][ i ] for any value of i in the range of 0 to NumOutputLayersInOls[ TargetOlsIdx ] − 1, inclusive), or one of the following conditions is true, PictureOutputFlag is set equal to 0:

–	The current picture is a RASL picture and NoOutputBeforeRecoveryFlag of the associated IRAP picture is equal to 1.

NOTE – In an implementation, the decoder could determine that the current picture with pps_mixed_nalu_types_in_pic_flag equal to 1 is RASL picture if one of the following is satisfied

–	sps_field_seq_flag is equal to 0, NAL unit type of the first slice of the picture is RASL_NUT or RADL_NUT, and there is no non-leading picture since the last IRAP picture.

–	sps_field_seq_flag is equal to 1, NAL unit type of the first slice of the picture is RASL_NUT or RADL_NUT, and there is exactly one non-leading picture since the last IRAP picture.

–	The current picture is a GDR picture with NoOutputBeforeRecoveryFlag equal to 1 or is a recovering picture of a GDR picture with NoOutputBeforeRecoveryFlag equal to 1.

–	Otherwise, PictureOutputFlag is set equal to ph_pic_output_flag.

NOTE – In an implementation, the decoder could output a picture not belonging to an output layer. For example, when there is only one output layer while in an AU the picture of the output layer is not available, e.g., due to a loss or layer down-switching, the decoder could set PictureOutputFlag set equal to 1 for the picture that has the highest value of nuh_layer_id among all pictures of the AU available to the decoder and having ph_pic_output_flag equal to 1, and set PictureOutputFlag equal to 0 for all other pictures of the AU available to the decoder.

# Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).