| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0078 |
| --- | --- |

| Title: | AHG9: On derivation of NoOutputOfPriorPicsFlag |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Seethal Paluri Seung Hwan Kim | Email: | dr.hendry@lge.com seethal.paluri@lge.com seunghwan3.kim@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

It is asserted that in the current VVC specification text, the derivation of NoOutputOfPriorPicsFlag has problem because it is invoked for each picture in the coded video sequence start access unit (CVSS AU) that is not AU 0. This would create a problem when the AU has more than one picture because starting from the second picture, the process would remove the previous picture from the DPB before the previous picture being outputted. This is asserted to be not the correct decoder behavior.

In this contribution, it is proposed to invoke the derivation process for NoOutputOfPriorPicsFlag only once per AU, that is, before the decoding process of the first picture in the CVSS AU but after parsing the slice header of the first slice of the current picture. When NoOutputOfPriorPicsFlag is equal to 1, the process of removing pictures stored in the DPB without outputting them is invoked only once per access unit.

# Problem statement

In the current VVC specification draft text, the invocation of output process and removal of pictures from the DPB is done once per picture. While invoking those process once per picture is correct, it is asserted that the process involving the derivation of NoOutputOfPriorPicsFlag and the removal of pictures from the DPB based on the value of NoOutputOfPriorPicsFlag should not be done for every picture in the CVSS AU that is not AU 0. When the CVSS AU that is not AU 0, such process is fine only for the first picture; starting from the second picture, the process would remove the previous picture from the DPB before the previous picture being output.

# Proposal changes and spec text

The proposed changes are described as follows with the derivation for the variable NoOutputOfPriorPicsFlag is modified as follows:

- Invoke the derivation process for the value of the NoOutputOfPriorPicsFlag only once per AU and it should be invoked before the decoding process of the first picture in the CVSS AU but after parsing the slice header of the first slice of the current picture.

- Additionally, when NoOutputOfPriorPicsFlag is equal to 1, the process of the removing pictures stored in the DPB without outputting them is invoked only once per access unit.

- When NoOutputOfPriorPicsFlag is equal to 1, the process of the removing pictures stored in the DPB without outputting them is invoked before the decoding process of the first picture in the coded video sequence start (CVSS) AU but after parsing the slice header of the first slice of the current picture.

### Removal of pictures from the DPB before decoding of the current picture

The removal of pictures from the DPB before decoding of the current picture (but after parsing the slice header of the first slice of the current picture) happens instantaneously at the CPB removal time of the first DU of AU n (containing the current picture) and proceeds as follows:

–	The decoding process for reference picture list construction as specified in clause 8.3.2 is invoked and the decoding process for reference picture marking as specified in clause 8.3.3 is invoked.

–	When the current picture is the first picture and the current AU (i.e., the AU that contains the current picture) is a CVSS AU that is not AU 0, the following ordered steps are applied:

1.	The variable NoOutputOfPriorPicsFlag is derived for the decoder under test as follows:

–	If the value of PicWidthMaxInSamplesY, PicHeightMaxInSamplesY, MaxChromaFormat, MaxBitDepthMinus8, or max_dec_pic_buffering_minus1[ Htid ] derived for the current AU is different from the value of PicWidthMaxInSamplesY, PicHeightMaxInSamplesY, MaxChromaFormat, MaxBitDepthMinus8, or max_dec_pic_buffering_minus1[ Htid ], respectively, derived for the preceding AU in decoding order, NoOutputOfPriorPicsFlag may (but should not) be set equal to 1 by the decoder under test, regardless of the value of ph_no_output_of_prior_pics_flag of the current AU.

NOTE – Although setting NoOutputOfPriorPicsFlag equal to ph_no_output_of_prior_pics_flag of the current AU is preferred under these conditions, the decoder under test is allowed to set NoOutputOfPriorPicsFlag equal to 1 in this case.

–	Otherwise, NoOutputOfPriorPicsFlag is set equal to ph_no_output_of_prior_pics_flag of the current AU.

2.	The value of NoOutputOfPriorPicsFlag derived for the decoder under test is applied for the HRD, such that when the value of NoOutputOfPriorPicsFlag is equal to 1, all picture storage buffers in the DPB are emptied without output of the pictures they contain, and the DPB fullness is set equal to 0.

–	When the current AU is not a CVSS AU or the current AU is a CVSS AU that is not AU 0 but the current picture is not the first picture in the current AU and when both of the following conditions are true for any pictures k in the DPB, all such pictures k in the DPB are removed from the DPB:

–	picture k is marked as "unused for reference".

–	picture k has PictureOutputFlag equal to 0 or its DPB output time is less than or equal to the CPB removal time of the first DU (denoted as DU m) of the current picture n; i.e., DpbOutputTime[ k ] is less than or equal to DuCpbRemovalTime[ m ].

–	For each picture that is removed from the DPB, the DPB fullness is decremented by one.

...

### Output and removal of pictures from the DPB

The output and removal of pictures from the DPB before the decoding of the current picture (but after parsing the slice header of the first slice of the current picture) happens instantaneously when the first DU of the AU containing the current picture is removed from the CPB and proceeds as follows:

–	The decoding process for reference picture list construction as specified in clause 8.3.2 and decoding process for reference picture marking as specified in clause 8.3.3 are invoked.

–	If the current picture is the first picture and the current AU  is a CVSS AU that is not AU 0, the following ordered steps are applied:

1.	The variable NoOutputOfPriorPicsFlag is derived for the decoder under test as follows:

–	If the value of PicWidthMaxInSamplesY, PicHeightMaxInSamplesY, MaxChromaFormat, MaxBitDepthMinus8, or max_dec_pic_buffering_minus1[ Htid ] derived for the current AU is different from the value of PicWidthMaxInSamplesY, PicHeightMaxInSamplesY, MaxChromaFormat, MaxBitDepthMinus8, or max_dec_pic_buffering_minus1[ Htid ], respectively, derived for the preceding AU in decoding order, NoOutputOfPriorPicsFlag may (but should not) be set equal to 1 by the decoder under test, regardless of the value of ph_no_output_of_prior_pics_flag of the current AU.

NOTE – Although setting NoOutputOfPriorPicsFlag equal to ph_no_output_of_prior_pics_flag of the current AU is preferred under these conditions, the decoder under test is allowed to set NoOutputOfPriorPicsFlag equal to 1 in this case.

–	Otherwise, NoOutputOfPriorPicsFlag is set equal to ph_no_output_of_prior_pics_flag of the current AU.

2.	The value of NoOutputOfPriorPicsFlag derived for the decoder under test is applied for the HRD as follows:

–	If NoOutputOfPriorPicsFlag is equal to 1, all picture storage buffers in the DPB are emptied without output of the pictures they contain and the DPB fullness is set equal to 0.

–	Otherwise (NoOutputOfPriorPicsFlag is equal to 0), all picture storage buffers containing a picture that is marked as "not needed for output" and "unused for reference" are emptied (without output) and all non-empty picture storage buffers in the DPB are emptied by repeatedly invoking the "bumping" process specified in clause C.5.2.4 and the DPB fullness is set equal to 0.

–	Otherwise (the current AU is not a CVSS AU or the current AU is a CVSS AU that is not AU 0 but the current picture is not the first picture in the current AU), all picture storage buffers containing a picture which are marked as "not needed for output" and "unused for reference" are emptied (without output). For each picture storage buffer that is emptied, the DPB fullness is decremented by one. When one or more of the following conditions are true, the "bumping" process specified in clause C.5.2.4 is invoked repeatedly while further decrementing the DPB fullness by one for each additional picture storage buffer that is emptied, until none of the following conditions are true:

- The number of pictures in the DPB that are marked as "needed for output" is greater than max_num_reorder_pics[ Htid ].

- max_latency_increase_plus1[ Htid ] is not equal to 0 and there is at least one picture in the DPB that is marked as "needed for output" for which the associated variable PicLatencyCount is greater than or equal to MaxLatencyPictures[ Htid ].

- The number of pictures in the DPB is greater than or equal to max_dec_pic_buffering_minus1[ Htid ] + 1.

...

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).