| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0079 |
| --- | --- |

| Title: | AHG9: On signalling of sps_max_sublayers_minus1 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry | Email: | dr.hendry@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This contribution asserted that the signalling of sps_max_sublayers_minus1 is redundant when the value of sps_ptl_dpb_hrd_params_present_flag is equal to 0. Therefore, the following changes are proposed:

- Move the signalling of sps_max_sublayers_minus1 and sps_reserved_zero_4bits immediately before the signalling of profile_tier_level( 1, sps_max_sublayers_minus1 )

- Condition the presence of the two syntax elements (i.e., sps_max_sublayers_minus1 and sps_reserved_zero_4bits) such that they are present only when sps_ptl_dpb_hrd_params_present_flag is equal to 1.

The proposed changes preserve the fact that PTL signalling, when it present, starts from byte-aligned position.

# Introduction

The signalling of maximum temporal sublayer is currently needed only for signalling of PTL, DPB parameters, and HRD parameters. These information (i.e., PTL, DPB parameters, and HRD parameters) are designed to be associated with output layer set, instead of layers. Furthermore, according to the current signalling design, PTL, DPB parameters, and HRD parameters associated with OLS with multi layers are those that are signalled in the VPS; whereas for OLS with single layer are those that are signalled in the SPS referred to by the VCL NAL units of the layer. Based on this understanding, the signalling of maximum temporal sublayer information in the SPS (i.e., sps_max_sublayers_minus1) is only needed when PTL, DPB parameters, and HRD parameters are present in the SPS (i.e., when the value of sps_ptl_dpb_hrd_params_present_flag is equal to 1). Therefore, sps_max_sublayers_minus1 should be signalled only when sps_ptl_dpb_hrd_params_present_flag is equal to 1.

One concern that may arise when changing the signalling of syntax elements that are earlier than the signalling of PTL information in the SPS is that we may lose the byte-aligned position for the start of the signalling of PTL information. However, the changes (i.e., move the signalling of sps_max_sublayers_minus1 to just before the signalling of PTL information and condition its presence on when sps_ptl_dpb_hrd_params_present_flag is equal to 1) still preserve the byte-align position for the start of PTL signalling.

Furthermore, the syntax element sps_reserved_zero_4bits is needed only for ensuring byte-alignment position for signalling PTL information. This syntax element too is only needed when sps_ptl_dpb_hrd_params_present_flag is equal to 1.

# Proposal changes and spec text

The proposed clean-ups to address the aspects described above are as follows:

- Move the signalling of sps_max_sublayers_minus1 and sps_reserved_zero_4bits immediately before the signalling of profile_tier_level( 1, sps_max_sublayers_minus1 )

- Condition the presence of the two syntax elements (i.e., sps_max_sublayers_minus1 and sps_reserved_zero_4bits) such that they are present only when sps_ptl_dpb_hrd_params_present_flag is equal to 1.

The spec text changes for implementing the above proposed changes is as follows (yellow-highlighted text are added text and italic font text are removed text):

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_seq_parameter_set_id | u(4) |
| sps_video_parameter_set_id | u(4) |
| sps_max_sublayers_minus1 | u(3) |
| sps_reserved_zero_4bits | u(4) |
| sps_ptl_dpb_hrd_params_present_flag | u(1) |
| if( sps_ptl_dpb_hrd_params_present_flag ) { |  |
| sps_max_sublayers_minus1 | u(3) |
| sps_reserved_zero_4bits | u(4) |
| profile_tier_level( 1, sps_max_sublayers_minus1 ) |  |
| } |  |
| ... |  |
| } |  |

...

sps_max_sublayers_minus1 plus 1 specifies the maximum number of temporal sublayers that may be present in each CLVS referring to the SPS. The value of sps_max_sublayers_minus1 shall be in the range of 0 to vps_max_sublayers_minus1, inclusive.

sps_reserved_zero_4bits shall be equal to 0 in bitstreams conforming to this version of this Specification. Other values for sps_reserved_zero_4bits are reserved for future use by ITU-T | ISO/IEC.

sps_ptl_dpb_hrd_params_present_flag equal to 1 specifies that a profile_tier_level( ) syntax structure and a dpb_parameters( ) syntax structure are present in the SPS, and a general_hrd_parameters( ) syntax structure and an ols_hrd_parameters( ) syntax structure may also be present in the SPS. sps_ptl_dpb_hrd_params_present_flag equal to 0 specifies that none of these four syntax structures is present in the SPS.

When sps_video_parameter_set_id is greater than 0 and there is an OLS that contains only one layer with nuh_layer_id equal to the nuh_layer_id of the SPS, or when sps_video_parameter_set_id is equal to 0, the value of sps_ptl_dpb_hrd_params_present_flag shall be equal to 1.

sps_max_sublayers_minus1 plus 1 specifies the maximum number of temporal sublayers that may be present in each CLVS referring to the SPS. The value of sps_max_sublayers_minus1 shall be in the range of 0 to vps_max_sublayers_minus1, inclusive.

sps_reserved_zero_4bits shall be equal to 0 in bitstreams conforming to this version of this Specification. Other values for sps_reserved_zero_4bits are reserved for future use by ITU-T | ISO/IEC.

...

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).