| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0087 |
| --- | --- |

| Title: | AHG9: On target OLS and sublayers for decoding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Technologies, Tampere, Finland | Email: | firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution proposes signalling of the OLS represented by the CVS and the highest temporal sublayer present in the CVS. The contribution consists of the following pieces:

- The following syntax elements are proposed to be added into the AUD NAL unit, when aud_irap_or_gdr_au_flag is equal to 1:

  - Two gating flags, aud_htid_info_present_flag and aud_ols_info_present_flag

  - aud_cvs_htid_plus1, which is present when aud_htid_info_present_flag is equal to 1

- aud_cvs_htid_plus1 equal to 0 specifies that all the pictures in the CVS starting with the AUD NAL unit are IRAP pictures or GDR pictures with ph_recovery_poc_cnt equal to 0.

- aud_cvs_htid_plus1 greater than 0 specifies that all the pictures in the CVS starting with the AUD NAL unit have TemporalId less than aud_cvs_htid_plus1.

  - aud_cvs_ols_idx, which is present when aud_ols_info_present_flag is equal to 1

- aud_cvs_ols_idx specifies that the CVS starting with the AUD NAL unit does not contain any other layers than those included in the OLS with OLS index equal to aud_cvs_ols_idx.

- When aud_cvs_htid_plus1 is present, it is used to derive the value of Htid. Otherwise, Htid is set by external means (like in VVC Draft 9).

- When aud_cvs_ols_idx is present, it is used to derive the value of TargetOlsIdx. Otherwise, TargetOlsIdx is set by external means (like in VVC Draft 9).

- The sub-bitstream extraction process is modified so that the highest temporal sublayer and the target OLS given as inputs to the process are used in modifying the AUD NAL units in the output sub-bitstream.

# Problem Statement

TargetOlsIdx and Htid are provided by external means to the decoding process as specified in clause 8.1.1:

The variables TargetOlsIdx, which identifies the OLS index of the target OLS to be decoded, and the variable Htid, which identifies the highest temporal sublayer to be decoded, are set by some external means not specified in this Specification. The bitstream BitstreamToDecode does not contain any other layers than those included in the target OLS and does not include any NAL unit with TemporalId greater than Htid.

It is asserted that arranging external means for setting TargetOlsIdx and Htid correctly might not be available for example in the following cases:

- Decoder APIs might not provide the means to pass TargetOlsIdx and HTid from the application to the decoder.

- A network entity, such as a media mixer, may prune layers or sublayers from the bitstream before forwarding the pruned bitstream to a receiver. The network entity might not have an out-of-band interface with the receiving decoder to indicate the TargetOlsIdx or HTid of the forwarded bitstream.

It is asserted that using the correct values of TargetOlsIdx and Htid is essential in the decoder due to the following reasons:

- The decoding process uses TargetOlsIdx to derive the value of PictureOutputFlag.

- TargetOlsIdx is needed to select the correct DPB parameters, specifically the width, height, chroma format, and bit depth of the DPB picture storage buffers.

- TargetOlsIdx and HTid enable selecting the correct HRD parameters, such as the initial buffering delay or the DPB reordering latency, for the decoder operation.

It is asserted that the knowledge of the TargetOlsIdx and Htid represented by a bitstream is essential for entities other than the decoder, e.g. for the following reasons:

- When a decoder is capable of decoding some but not all OLSs in the VPS, the knowledge of the TargetOlsIdx represented by the bitstream would enable an application to conclude whether sub-bitstream extraction is required before passing the bitstream to the decoder.

- The sub-bitstream extraction process of clause C.6 is based on TargetOlsIdx and removes those pictures from non-output layers that are not needed for decoding the pictures at output layers. A subsequent sub-bitstream extraction might no longer be possible (since some layers are no longer present in the bitstream) or might not result into a desirable bitstream (since many temporal sublayers were removed in the previous sub-bitstream extraction). Thus, the knowledge of the TargetOlsIdx and Htid represented by a bitstream would enable concluding whether sub-bitstream extraction with certain TargetOlsIdx and Htid as inputs is possible and reasonable.

# Summary of the proposal

This contribution proposes signalling of the OLS represented by the CVS and the highest temporal sublayer present in the CVS. The contribution consists of the following pieces:

- The following syntax elements are proposed to be added into the AUD NAL unit, when aud_irap_or_gdr_au_flag is equal to 1:

  - Two gating flags, aud_htid_info_present_flag and aud_ols_info_present_flag

  - aud_cvs_htid_plus1, which is present when aud_htid_info_present_flag is equal to 1

- aud_cvs_htid_plus1 equal to 0 specifies that all the pictures in the CVS starting with the AUD NAL unit are IRAP pictures or GDR pictures with ph_recovery_poc_cnt equal to 0.

- aud_cvs_htid_plus1 greater than 0 specifies that all the pictures in the CVS starting with the AUD NAL unit have TemporalId less than aud_cvs_htid_plus1.

  - aud_cvs_ols_idx, which is present when aud_ols_info_present_flag is equal to 1

- aud_cvs_ols_idx specifies that the CVS starting with the AUD NAL unit does not contain any other layers than those included in the OLS with OLS index equal to aud_cvs_ols_idx.

- When aud_cvs_htid_plus1 is present, it is used to derive the value of Htid. Otherwise, Htid is set by external means (like in VVC Draft 9).

- When aud_cvs_ols_idx is present, it is used to derive the value of TargetOlsIdx. Otherwise, TargetOlsIdx is set by external means (like in VVC Draft 9).

- The sub-bitstream extraction process is modified so that the highest temporal sublayer and the target OLS given as inputs to the process are used in modifying the AUD NAL units in the output sub-bitstream.

# Proposed specification text

## Syntax

Add the syntax table lines indicated in red font color:

| access_unit_delimiter_rbsp( ) { | Descriptor |
| --- | --- |
| aud_irap_or_gdr_au_flag | u(1) |
| aud_pic_type | u(3) |
| if( aud_irap_or_gdr_au_flag ) { |  |
| aud_htid_info_present_flag | u(1) |
| aud_ols_info_present_flag | u(1) |
| if( aud_htid_info_present_flag ) |  |
| aud_cvs_htid_plus1 | u(3) |
| if( aud_ols_info_present_flag ) |  |
| aud_cvs_ols_idx | ue(v) |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

## Semantics

Add the following semantics for AUD NAL unit:

aud_htid_info_present_flag equal to 0 specifies that aud_cvs_htid_plus1 is not present in the AUD NAL unit. aud_htid_info_present_flag equal to 1 specifies that aud_cvs_htid_plus1 is present in the AUD NAL unit.

aud_ols_info_present_flag equal to 0 specifies that aud_cvs_ols_idx is not present in the AUD NAL unit. aud_ols_info_present_flag equal to 1 specifies that aud_cvs_ols_idx is present in the AUD NAL unit.

aud_cvs_htid_plus1 equal to 0 specifies that all the pictures in the CVS starting with the AUD NAL unit are IRAP pictures or GDR pictures with ph_recovery_poc_cnt equal to 0. aud_cvs_htid_plus1 greater than 0 specifies that all the pictures in the CVS starting with the AUD NAL unit have TemporalId less than aud_cvs_htid_plus1.

aud_cvs_ols_idx specifies that the CVS starting with the AUD NAL unit does not contain any other layers than those included in the OLS with OLS index equal to aud_cvs_ols_idx.

## Decoding process

Replace the following paragraph in clause 8.1.1:

The variables TargetOlsIdx, which identifies the OLS index of the target OLS to be decoded, and the variable Htid, which identifies the highest temporal sublayer to be decoded, are set by some external means not specified in this Specification. The bitstream BitstreamToDecode does not contain any other layers than those included in the target OLS and does not include any NAL unit with TemporalId greater than Htid.

with:

For each CVS in the bitstream BitstreamToDecode, the following applies:

–	The variable Htid, which identifies the highest temporal sublayer to be decoded, is specified as follows:

–	If aud_cvs_htid_plus1 is present in an AUD NAL unit, Htid is set equal to ( ( aud_cvs_htid_plus1 > 0 ) ? aud_cvs_htid_plus1 − 1 : 0 ) for the CVS starting with the AUD NAL unit.

–	Otherwise, Htid is set by some external means not specified in this Specification.

–	The variable TargetOlsIdx, which identifies the OLS index of the target OLS to be decoded, is specified as follows:

–	If aud_cvs_ols_idx is present in an AUD NAL unit, TargetOlsIdx is set equal to aud_cvs_ols_idx for the CVS starting with the AUD NAL unit..

–	Otherwise, TargetOlsIdx is set by some external means not specified in this Specification.

–	The CVS does not contain any other layers than those included in the target OLS and does not include any NAL unit with TemporalId greater than Htid.

## Sub-bitstream extraction

Add the following into the sub-bitstream extraction process of clause C.6:

–	Set the syntax element values of each AUD NAL unit with aud_irap_or_gdr_au_flag equal to 1 as follows:

–	aud_htid_info_present_flag is set equal to 1.

–	aud_ols_info_present_flag is set equal to 1.

–	aud_cvs_htid_plus1 is set equal to tIdTarget + 1.

–	aud_cvs_ols_idx is set equal to targetOlsIdx.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).