| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June –1 July 2020 | Document: JVET-S0100 |
| --- | --- |

| Title: | AHG9: On OLS and sublayers |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yago Sanchez Robert Skupin Karsten Suehring Thomas Schierl | Tel: Email: | {firstname.lastname}@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This contribution discusses three asserted issues related to OLS and sublayers:

- The maximum sublayer within an OLS is not considered in the extraction process.

- When vps_max_tid_il_ref_pics_plus1[ i ][ j ] equal to 0, a value equal to 0 is derived for an indirect layer k for NumSubLayersInLayerInOLS[ i ][ k ] but might not contain an IRAP or GDR picture with ph_recovery_poc_cnt equal to 0 aligned with the direct reference layers of the output layers in the OLS.

- The following syntax elements that apply to an OLS vps_ptl_max_temporal_id[ i ], vps_dpb_max_temporal_id[ i ], vps_hrd_max_tid[ i ] might have different values and are not constrained in any manner.

# Introduction

This contribution discusses three asserted issues related to OLS and sublayers:

- The maximum sublayer within an OLS is not considered in the extraction process.

  - It is proposed that NumSubLayersInLayerInOLS[ i ][ k ] is set equal to vps_ptl_max_temporal_id[ i ] + 1 for output layers instead of vps_max_sublayers_minus1 + 1.

- When vps_max_tid_il_ref_pics_plus1[ i ][ j ] is equal to 0, a value equal to 0 is derived for an indirect layer k for NumSubLayersInLayerInOLS[ i ][ k ] but might not contain an IRAP or GDR picture with ph_recovery_poc_cnt equal to 0 aligned with the direct reference layers of the output layers in the OLS.

  - A bitstream constraint is proposed to enforce IRAP or GDR alignment in the described case.

- The following syntax elements that apply to an OLS vps_ptl_max_temporal_id[ i ], vps_dpb_max_temporal_id[ i ], vps_hrd_max_tid[ i ] might have different values and are not constrained in any manner.

  - A bitstream constraint is proposed to make sure that vps_dpb_max_temporal_id[ i ], vps_hrd_max_tid[ i ] are greater than or equal to vps_ptl_max_temporal_id[ i ].

# Proposal

Item 1) maximum sublayer in an OLS

Figure 1: A two-layer bitstream wherein each layer has a different picture rate with two OLS.

Figure 1 shows an example with two OLS. OLS1 consist of layer 0 (L0) and OLS2 consist of both layers (L0+L1) but with only the first sublayer (TL0).

The current specification derives the variable NumSubLayersInLayerInOLS[ i ][ k ] when vps_max_tid_il_ref_pics_plus1[ i ][ j ] is not present or when a layer is an output layer in an OLS to be equal to vps_max_sublayers_minus1 + 1. Therefore, it is asserted that there is a bug in the specification that allows to extract sublayers higher with TemporalId higher than the vps_ptl_max_temporal_id[ 0 ] + 1 for an OLS.

It is proposed to change this value to vps_ptl_max_temporal_id[ i ] + 1 as an indication of the maximum sublayer included in the OLS. The changes are shown in the following:

NumOutputLayersInOls[ 0 ] = 1
OutputLayerIdInOls[ 0 ][ 0 ] = vps_layer_id[ 0 ]
NumSubLayersInLayerInOLS[ 0 ][ 0 ] = vps_max_sublayers_minus1 vps_ptl_max_temporal_id[ 0 ] + 1
LayerUsedAsOutputLayerFlag[ 0 ] = 1
for( i = 1, i  <=  vps_max_layers_minus1; i++ ) {
	if( vps_each_layer_is_an_ols_flag  | |  vps_ols_mode_idc < 2 )
		LayerUsedAsOutputLayerFlag[ i ] = 1
	else /*( !vps_each_layer_is_an_ols_flag  &&  vps_ols_mode_idc  = =  2 ) */
		LayerUsedAsOutputLayerFlag[ i ] = 0
}
for( i = 1; i < TotalNumOlss; i++ )
	if( vps_each_layer_is_an_ols_flag  | |  vps_ols_mode_idc  = =  0 ) {
		NumOutputLayersInOls[ i ] = 1
		OutputLayerIdInOls[ i ][ 0 ] = vps_layer_id[ i ]
		if( vps_each_layer_is_an_ols_flag )
			NumSubLayersInLayerInOLS[ i ][ 0 ] = vps_max_sublayers_minus1 vps_ptl_max_temporal_id[ i ] + 1
		else {
			NumSubLayersInLayerInOLS[ i ][ i ] = vps_max_sublayers_minus1 vps_ptl_max_temporal_id[ i ] + 1
			for( k = i − 1, k  >=  0; k− − ) {
				NumSubLayersInLayerInOLS[ i ][ k ] = 0
				for( m = k + 1; m  <=  i; m++ ) {
					maxSublayerNeeded = min( NumSubLayersInLayerInOLS[ i ][ m ],
							vps_max_tid_il_ref_pics_plus1[ m ][ k ] )
					if( vps_direct_ref_layer_flag[ m ][ k ]  &&
							NumSubLayersInLayerInOLS[ i ][ k ] < maxSublayerNeeded )
						NumSubLayersInLayerInOLS[ i ][ k ] = maxSublayerNeeded
				}
			}
		}
	} else if( vps_ols_mode_idc  = =  1 ) {
		NumOutputLayersInOls[ i ] = i + 1
		for( j = 0; j < NumOutputLayersInOls[ i ]; j++ ) {
			OutputLayerIdInOls[ i ][ j ] = vps_layer_id[ j ]
			NumSubLayersInLayerInOLS[ i ][ j ] = vps_max_sublayers_minus1 vps_ptl_max_temporal_id[ i ] + 1
		}
	} else if( vps_ols_mode_idc  = =  2 ) {
		for( j = 0; j  <=  vps_max_layers_minus1; j++ ) {
			layerIncludedInOlsFlag[ i ][ j ] = 0
			NumSubLayersInLayerInOLS[ i ][ j ] = 0
		}
		highestIncludedLayer = 0
		numLayerInOls = 0
		for( k = 0, j = 0; k  <=  vps_max_layers_minus1; k++ )	(40)
			if( vps_ols_output_layer_flag[ i ][ k ] ) {
				layerIncludedInOlsFlag[ i ][ k ] = 1
				highestIncludedLayer = k
				numLayerInOls++
				LayerUsedAsOutputLayerFlag[ k ] = 1
				OutputLayerIdx[ i ][ j ] = k
				OutputLayerIdInOls[ i ][ j++ ] = vps_layer_id[ k ]
				NumSubLayersInLayerInOLS[ i ][ k ] = vps_max_sublayers_minus1 vps_ptl_max_temporal_id[ i ] + 1
			}
		NumOutputLayersInOls[ i ] = j
		for( j = 0; j < NumOutputLayersInOls[ i ]; j++ ) {
			idx = OutputLayerIdx[ i ][ j ]
			for( k = 0; k < NumRefLayers[ idx ]; k++ ) {
				if (!layerIncludedInOlsFlag[ i ][ RefLayerIdx[ idx ][ k ] ] )
					numLayerInOls++
				layerIncludedInOlsFlag[ i ][ RefLayerIdx[ idx ][ k ] ] = 1
			}
		}
		for( k = highestIncludedLayer − 1; k  >=  0; k− − )
			if( layerIncludedInOlsFlag[ i ][ k ]  &&  !vps_ols_output_layer_flag[ i ][ k ] )
				for( m = k + 1; m  <=  highestIncludedLayer; m++ ) {
					maxSublayerNeeded = min( NumSubLayersInLayerInOLS[ i ][ m ],
							vps_max_tid_il_ref_pics_plus1[ m ][ k ] )
					if( vps_direct_ref_layer_flag[ m ][ k ]  &&  layerIncludedInOlsFlag[ i ][ m ]  &&
							NumSubLayersInLayerInOLS[ i ][ k ] < maxSublayerNeeded )
						NumSubLayersInLayerInOLS[ i ][ k ] = maxSublayerNeeded
				}
	}

Item 2) vps_max_tid_il_ref_pics_plus1[ i ][ j ] equal to 0 and indirect reference layers

The current specification has an asserted bug explained in the following. The derivation of NumSubLayersInLayerInOLS[ i ][ k ] for indirect reference layers k of an OLS i is set equal to 0, when all direct reference layers l of all output layers j in the OLS have vps_max_tid_il_ref_pics_plus1[ j ][ l ] equal to 0. However, in such a case two reference layers with NumSubLayersInLayerInOLS[ i ][ k ] equal to 0 may have non aligned IRAPs as described in the following example.

Let’s assume an OLS with three layers L0, L1 and L2, where L2 is the only output layer and L1 is a direct reference layer of L2 and L0 is an indirect reference layer as illustrated in the following Figure 2. Let’s assume that vps_max_tid_il_ref_pics_plus1[ 2 ][ 1 ] is 0 and vps_max_tid_il_ref_pics_plus1[ 1 ][ 0 ] is 1 (which is visible as an arrow from IDR in L1 to TRAIL in L0). In such a case, as discussed above, NumSubLayersInLayerInOLS[ i ][ 0 ], as well as NumSubLayersInLayerInOLS[ i ][ 1 ] are both derived as 0 but a non-IRAP NAL unit (4th Trail picture in L0) is needed for decoding.

Figure 2: Three-layer OLS with unaligned IRAPs in lower layers.

The Figure 2 shows in the example that the derived NumSubLayersInLayerInOLS[ i ][ 0 ] would be lead to a wrongfully extracted bitstream as the TRAIL_NUT in L0 would be dropped although it is required as reference of the co-located IDRs in higher layers.

Therefore, a bitstream constraint is proposed that disallows such indirect reference layers when the value of vps_max_tid_il_ref_pics_plus1[ i ][ j ] is 0 for all direct reference layers.

For any two layers k and j within an OLS i, with k > j, if both NumSubLayersInLayerInOLS[ i ][ j ] and NumSubLayersInLayerInOLS[ i ][ k ] equal to 0, j is a reference layer of k (direct or indirect) and there is a picture in layer k that is an IRAP or GDR picture with ph_recovery_poc_cnt equal to 0 in an AU, the picture in layer j shall be an IRAP or GDR picture with ph_recovery_poc_cnt equal to 0 respectively.

Item 3) vps_ptl_max_temporal_id[ i ], vps_ols_dpb_params_idx[ i ] and vps_hrd_max_tid[ i ]

The current specification contains the three syntax elements to control the presence of PTL, DPB and HRD parameters for sublayers in the corresponding structures in the VPS. These PTL, DPB and HRD parameters are mapped to the each OLS via ptlIdx=vps_ols_ptl_idx[ i ], dpbIdx=vps_ols_dpb_params_idx[ i ], hrdIdx=vps_ols_hrd_idx[ i ].

However, there is no constraint that the value of vps_ptl_max_temporal_id[ ptlIdx ], vps_ols_dpb_params_idx[ dpbIdx ] and vps_hrd_max_tid[ hrdIdx ] shall be the same or fulfill any condition.

For instance, let’s assume that vps_hrd_max_tid[ hrdIdx ] is smaller than vps_ptl_max_temporal_id[ ptlIdx ]. In such a case, only HRD parameters up to vps_hrd_max_tid[ hrdIdx ] are derived from the VPS, and HRD parameters for TemporalIds from vps_hrd_max_tid[ hrdIdx ] +1 to vps_ptl_max_temporal_id[ ptlIdx ] are neither present in the VPS nor inferred to any value.

To solve this issue, there are three meaningful options on how to prevent the asserted bug. The first two solve the issue by constraining the values of vps_ptl_max_temporal_id[ i ], vps_ols_dpb_params_idx[ i ] and vps_hrd_max_tid[ i ]. The third solution consists of inferring the non-present parameters:

- All three shall have the same value.

Might be too restrictive and harm parameter sharing across OLSs

- vps_ols_dpb_params_idx[ dpbIdx ] and vps_hrd_max_tid[ hrdIdx ] shall be greater than or equal to vps_ptl_max_temporal_id[ ptlIdx ].

Less restrictive and allows more parameter sharing than option 1) and DPB and HRD parameters for sublayers greater than vps_ptl_max_temporal_id[ ptlIdx ] are irrelevant for the OLS and therefore not used.

- vps_ols_dpb_params_idx[ dpbIdx ] and vps_hrd_max_tid[ hrdIdx ] are not constraint compared to vps_ptl_max_temporal_id[ ptlIdx ].

Allows more parameter sharing than option 1) and 2) but requires inference of DPB and HRD parameter for sublayers from vps_ols_dpb_params_idx[ dpbIdx ] and vps_hrd_max_tid[ hrdIdx ] up to vps_ptl_max_temporal_id[ ptlIdx ] to the last present DPB and HRD parameters (for highest sublayer - vps_ols_dpb_params_idx[ dpbIdx ] and vps_hrd_max_tid[ hrdIdx ]) respectively, which is not currently the case in the specification.

For asserted simplicity, the authors prefer option 2) as follows.

The value of vps_ols_dpb_params_idx[ vps_ols_dpb_params_idx[ i ] ] and vps_hrd_max_tid[ vps_ols_hrd_idx[ i ] ] shall be greater than or equal to vps_ptl_max_temporal_id[ vps_ols_ptl_idx[ i ] ]

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form)