| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June –1 July 2020 | Document: JVET-S0103 |
| --- | --- |

| Title: | AHG9: On OLS identification |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Robert Skupin Yago Sanchez Karsten Suehring Thomas Schierl | Tel: Email: | {firstname.lastname}@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

The contribution asserts that it is an issue that the TargetOlsIdx in some cases cannot be detected from the BitstreamToDecode itself and proposes two aspects to overcome the asserted issue.

Aspect 1)	Add a rule to select among OLSs matching BitstreamToDecode.

Aspect 2)	Add a constraint flag so that each layer set shall match to one OLS

# Introduction

A decoder receiving a bitstream to decode is given additional information via its API regarding what operation point it should target to decode from the BitstreamToDecode. For instance, in the current VVC draft specification, two variables TargetOlsIdx and Htid are set via external means as follows.

The variables TargetOlsIdx, which identifies the OLS index of the target OLS to be decoded, and the variable Htid, which identifies the highest temporal sublayer to be decoded, are set by some external means not specified in this Specification. The bitstream BitstreamToDecode does not contain any other layers than those included in the target OLS and does not include any NAL unit with TemporalId greater than Htid.

The specification is silent about the case that these variables are not set, because the expectation is that a decoder simply decodes the whole BitstreamToDecode it is given, instead of a sub-bitstream. For instance, in terms of temporal sublayers, Htid can be derived from the temporal sublayers of the bitstream itself.

However, the authors assert there to be an issue with respect to TargetOlsIdx as follows. When a decoder is given a BitstreamToDecode that contains more than one layer and the parameter sets define more than one OLS that contains all layers in the bitstream (e.g. variants with different output layers), there is no simple determination from the bitstream itself of which output layer set the decoder is supposed to decode.

The VPS-defined OLSs to select from could pose varying DPB requirements depending on the output layer setting. It is therefore vital to allow a decoder to select an OLS unambiguously even in absence of outside signals via its API. In other words, a fallback method as in other cases of external means absence seems necessary for a decoder to be sure about the operation point from the BitstreamToDecode itself.

# Proposal

Two aspects are proposed to overcome the mentioned issue:

Aspect 1)	Add a rule to select among OLSs matching BitstreamToDecode.

Aspect 2)	Add a constraint flag so that each layer set shall match to one OLS

Aspect 1) add a rule for selecting among the OLSs that correspond to the layers in BitstreamToDecode in absence of the TargetOlsIdx external means indication. Thereby, a decoder can unambiguously select among OLSs in absence of information via its API.

The variables TargetOlsIdx, which identifies the OLS index of the target OLS to be decoded, and the variable Htid, which identifies the highest temporal sublayer to be decoded, are set by some external means not specified in this Specification. The bitstream BitstreamToDecode does not contain any other layers than those included in the target OLS and does not include any NAL unit with TemporalId greater than Htid. When TargetOlsIdx is not provided via external means, it is set equal to the OLS with the lowest index that contains all layers of AU 0 of BitstreamToDecode and the highest number of output layers.

Aspect 2) add a constraint flag to the general constraint information that guarantees that no two OLSs in the VPS contain the same layer set. Thereby, it is indicated that a bitstream is constraint in such a way that a decoder does not need an API to determine TargetOlsIdx and a system can accommodate decoders without API by limiting the available OLSs to be unambiguous.

| general_constraint_info( ) { | Descriptor |
| --- | --- |
| […] | u(1) |
| general_single_ols_per_layer_set_constraint_flag | u(1) |
| […] | u(1) |
| while( !byte_aligned( ) ) |  |
| gci_alignment_zero_bit | f(1) |
| gci_num_reserved_bytes | u(8) |
| for( i = 0; i < gci_num_reserved_bytes; i++ ) |  |
| gci_reserved_byte[ i ] | u(8) |
| } |  |

general_single_ols_per_layer_set_constraint_flag equal 1 specifies that no two OLSs as defined in the VPS contain the same set of layers. general_single_ols_per_layer_set_constraint_flag equal 0 specifies that two OLSs as defined in the VPS may or may not contain the same set of layers.

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form)