| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0106 |
| --- | --- |

| Title: | AhG9: Some cleanups on general constraint flags. |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Guillaume Laroche Nael Ouedraogo Patrice Onno Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33 299876800 guillaume.laroche@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

This contribution proposes the following modifications of the general constraint information semantics:

- When intra_only_constraint_flag equal to 1, no_sbt_constraint_flag shall be equal to 1.

- When intra_only_constraint_flag equal to 1, no_trail_constraint_flag, no_stsa_constraint_flag, no_rasl_constraint_flag, no_radl_constraint_flag shall be equal to 1.

- When intra_only_constraint_flag equal to 1, no_cra_constraint_flag and no_gdr_constraint_flag shall be equal to 1 and no_idr_constraint_flag shall be equal to 0.

- When intra_only_constraint_flag equal to 1, all_layers_independent_constraint_flag shall be equal to 1.

- When one_subpic_per_pic_constraint_flag equal to 1, no_mixed_nalu_types_in_pic_constraint_flag shall be equal to 1.

- When one_slice_per_pic_constraint_flag is equal to 1, the value of pps_rect_slice_flag shall be equal to 1.

# Proposal 1: When intra_only_constraint_flag=1, no_sbt_constraint_flag=1

During the previous meeting, several all inter only tool constraint flags were disabled when intra_only_constraint_flag is set equal to 1. But one is missing the no_sbt_constraint_flag. So we propose to change the semantics of no_sbt_constraint_flag as the following:

no_sbt_constraint_flag equal to 1 specifies that sps_sbt_enabled_flag shall be equal to 0. no_sbt_constraint_flag equal to 0 does not impose such a constraint. When intra_only_constraint_flag is equal to 1, the value of no_sbt_constraint_flag shall be equal to 1.

# Proposal 2: When intra_only_constraint_flag=1, no_trail_constraint_flag=1, no_stsa_constraint_flag=1 no_rasl_constraint_flag=1, no_radl_constraint_flag=1

The Nal unit types TRAIL_NUT, STSA_NUT, RASL_NUT, RADL_NUT are dedicated to pictures dedicated to inter prediction. As other inter coding tools, these types of Nal units seem not needed for intra only configurations. So we propose the following modifications:

no_trail_constraint_flag equal to 1 specifies that there shall be no NAL unit with nuh_unit_type equal to TRAIL_NUT present in OlsInScope. no_trail_constraint_flag equal to 0 does not impose such a constraint. When intra_only_constraint_flag is equal to 1, the value of no_trail_constraint_flag shall be equal to 1.

no_stsa_constraint_flag equal to 1 specifies that there shall be no NAL unit with nuh_unit_type equal to STSA_NUT present in OlsInScope. no_stsa_constraint_flag equal to 0 does not impose such a constraint. When intra_only_constraint_flag is equal to 1, the value of no_stsa_constraint_flag shall be equal to 1.

no_rasl_constraint_flag equal to 1 specifies that there shall be no NAL unit with nuh_unit_type equal to RASL_NUT present in OlsInScope. no_rasl_constraint_flag equal to 0 does not impose such a constraint. When intra_only_constraint_flag is equal to 1, the value of no_rasl_constraint_flag shall be equal to 1.

no_radl_constraint_flag equal to 1 specifies that there shall be no NAL unit with nuh_unit_type equal to RADL_NUT present in OlsInScope. no_radl_constraint_flag equal to 0 does not impose such a constraint. When intra_only_constraint_flag is equal to 1, the value of no_radl_constraint_flag shall be equal to 1.

# Proposal 3: When intra_only_constraint_flag=1, no_cra_constraint_flag=1 and no_gdr_constraint_flag=1 and no_idr_constraint_flag=0

In addition to proposal 3, we also propose to disable CRA_NUT and GDR_NUT for intra only configuration, as these types are not useful for such configuration. And in that case, the IDR shall be enabled.

no_cra_constraint_flag equal to 1 specifies that there shall be no NAL unit with nuh_unit_type equal to CRA_NUT present in OlsInScope. no_cra_constraint_flag equal to 0 does not impose such a constraint. When intra_only_constraint_flag is equal to 1, the value of no_cra_constraint_flag shall be equal to 1.

no_gdr_constraint_flag equal to 1 specifies that sps_gdr_enabled_flag shall be equal to 0. no_gdr_constraint_flag equal to 0 does not impose such a constraint. When intra_only_constraint_flag is equal to 1, the value of no_cra_constraint_flag shall be equal to 1.

no_idr_constraint_flag equal to 1 specifies that there shall be no NAL unit with nuh_unit_type equal to IDR_W_RADL or IDR_N_LP present in OlsInScope. no_idr_constraint_flag equal to 0 does not impose such a constraint. When intra_only_constraint_flag is equal to 1, the value of no_idr_constraint_flag shall be equal to 0.

# Proposal 4: When intra_only_constraint_flag = 1, all_layers_independent_constraint_flag = 1

When there is only intra slices, there is no dependency between layers. We propose the following modification to reflect it:

all_layers_independent_constraint_flag equal to 1 specifies that vps_all_independent_layers_flag shall be equal to 1. all_layers_independent_constraint_flag equal to 0 does not impose such a constraint. When intra_only_constraint_flag is equal to 1, the value of all_layers_independent_constraint_flag shall be equal to 1.

# Proposal 5: When one_subpic_per_pic_constraint_flag=1, no_mixed_nalu_types_in_pic_constraint_flag = 1

We propose to avoid mixed nal unit types when there is only one subpicture as the mixed Nal unit types are allowed only for more than 1 subpicture.

no_mixed_nalu_types_in_pic_constraint_flag equal to 1 specifies that it is a requirement of bitstream conformance that pps_mixed_nalu_types_in_pic_flag shall be equal to 0. no_mixed_nalu_types_in_pic_constraint_flag equal to 0 does not impose such a constraint. When one_subpic_per_pic_constraint_flag is equal to 1, the value of no_mixed_nalu_types_in_pic_constraint_flag shall be equal to 1.

# Proposal 6: When one_slice_per_pic_constraint_flag=1, pps_rect_slice_flag = 1

In the current specification, when the picture header is transmitted in the slice header, the pps_rect_slice_flag is set equal to 1. And when there is one tile per picture the pps_rect_slice_flag is set equal to 1. Moreover when one_slice_per_pic_constraint_flag is set equal to 1, there is only one slice per picture, so there is no difference in the picture splitting for raster scan slice (pps_rect_slice_flag=0) and rectangular slice (pps_rect_slice_flag=1). We propose to add the following constraint to one_slice_per_pic_constraint_flag:

one_slice_per_pic_constraint_flag equal to 1 specifies that each picture shall contain only one slice and that pps_rect_slice_flag shall be equal to 1. one_slice_per_pic_constraint_flag equal to 0 does not impose such a constraint. When pic_header_in_slice_header_constraint_flag is equal to 1, the value of one_slice_per_pic_constraint_flag shall be equal to 1.

# Conclusion

In this contribution, we propose some editorial modifications of the general constraint information semantics to impose correct configuration settings especially when there is one slice per picture or one subpicture or only intra slices.

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).