| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET- S0108 |
| --- | --- |

| Title: | AHG8/AHG9: Refinement of proposed positioning information SEI message of output independent layers |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Emmanuel Thomas Anna van Buerenplein 1 2595 DA Den Haag The Netherlands | Email: | emmanuel.thomas@tno.nl |
| Source: | TNO |  |  |

_____________________________

# Abstract

This contribution is following up on JVET-R0307 submitted at JVET #18. It is asserted that the contribution aims at supporting use cases where a multi-layer bitstream with independent layers can be used. Multi-party video conferencing is considered to be a scenario where multiple participant views can be encoded as independent layers of a single multi-layer bitstream.

This contribution proposed the definition of a SEI message to signal the positioning information of each output layer for a given output layer set. Based on the discussion related to JVET-R0307 at JVET #18, the following changes are introduced in this contribution:

- All the layers in an OLS are connected to the same 4-connected graph, i.e. no orphan layer.

- An informative algorithm describes how to calculate the position of each cropped decoded picture in the final output picture.

- The process creating the final output picture clarifies that cropped decoded pictures are used and not decoded pictures.

It is reminded that JVET-R0308 provided the software implementation in VTM corresponding to the presented syntax and operations in JVET-R0307.

# Introduction

This contribution is following up on JVET-R0307 submitted at JVET #18. The purpose of the contribution is to offer a better support of use cases where multi-layer bitstream with independent layers is used. As an example, multi-party video conferencing is a suitable candidate scenario where multiple participant views can be decoded as part as independent layers of a single multi-layer bitstream.

To this end, this contribution proposed the definition of a new SEI message to signal the positioning information of each output layer for a given output layer set in a final output picture.

Based on the discussion related to JVET-R0307 at JVET #18, the following changes are introduced in this contribution:

- All the layers in an OLS are connected to the same 4-connected graph, i.e. no orphan layer.

- An informative algorithm describes how to calculate the position of each cropped decoded picture in the final output picture.

- The process creating the final output picture clarifies that cropped decoded pictures are used and not decoded pictures.

As a reminder, JVET-R0308 provided the software implementation in VTM corresponding to the presented syntax in JVET-R0307 that is:

- Implementation of a multi-layer decoding support

- Composition into a single picture of independent output layers of the same output layer set

Note that as discussed at JVET #18, example bitstreams with different arrangements will be made available for the upcoming JVET meeting in addition to the already provided software.

# Proposed syntax

Table D.1 – Persistence scope of SEI messages (informative)

| SEI message | Persistence scope |
| --- | --- |
| Buffering period | The remainder of the bitstream |
| Picture timing | The AU containing the SEI message |
| DU information | The AU containing the SEI message |
| Scalable nesting | Depending on the scalable-nested SEI messages. Each scalable-nested SEI message has the same persistence scope as if the SEI message was not scalable-nested |
| Subpicture level information | The CLVS containing the SEI message |
| Independent layer info | The CLVS containing the SEI message |

(New text below)

### D.x Independent layer info SEI message syntax

### D.x.1 Independent layer info SEI message syntax

| independent_layer_info( payloadSize ) { | Descriptor |
| --- | --- |
| boundary_identifier_north_present_flag | u(1) |
| if(boundary_identifier_north_present_flag) |  |
| boundary_identifier_north | u(16) |
| boundary_identifier_east_present_flag | u(1) |
| if(boundary_identifier_east_present_flag) |  |
| boundary_identifier_east | u(16) |
| boundary_identifier_south_present_flag | u(1) |
| if(boundary_identifier_south_present_flag) |  |
| boundary_identifier_south | u(16) |
| boundary_identifier_west_present_flag | u(1) |
| if(boundary_identifier_west_present_flag) |  |
| boundary_identifier_west | u(16) |
| } |  |

boundary_identifier_north_present_flag, boundary_identifier_east_present_flag, boundary_identifier_south_present_flag and boundary_identifier_west_present_flag equal to 1 specify that the SEI message contains a boundary identifier, respectively, for the north, east, south and west boundary.

boundary_identifier_north, boundary_identifier_east, boundary_identifier_south and boundary_identifier_west specifiy the boundary identifier, respecitvely, at the north, east, south and west boundary of the decoded picture of the associated layer; the associated layer being the layer whose nuh_layer_id is equal to the nuh_layer_id of the SEI message. If not present, the boundary identifiers respecitvely at the north, east, south and west boundary of the decoded picture of the associated layer are not defined.

Figure 3 - Representation of where the bououndaries are on the layers

For two layers, the i-th and j-th layers, when the pair of the boundary_identifier_north value of the i-th layer and the boundary_identifier_south value of the j-th layer are equal then the decoded picture of the i-th layer and the decoded picture of the j-th layer are adjacent in the composed output picture and they share a common boundary at the boundary north/south. For i-th and j-th layers, when the pair of the boundary_identifier_east value of the i-th layer and the boundary_identifier_west value of the j-th layer are equal then the decoded picture of the i-th layer and the decoded picture of the j-th layer are adjacent in the composed output picture and they share a common boundary at the boundary east/west.

Two decoded pictures adjacent by the north/south boundary are aligned on their west boundary in the composed picture. Two decoded pictures adjacent by the east/west boundary are aligned on their north boundary in the composed picture.

All the Independent layer info SEI message present in layers of an OLS shall collectively describe a 4-connected graph and each layer of the OLS shall be connected to the graph.

### D.x.2 Process for generating the aggregated output picture

The process for generating the final output picture is informative. The following section provides the expected operations performed for generating the final output picture based on the decoded pictures of each layer from a selected OLS:

- For each Access Unit:

  - If VPS present, parse VPS and store the list of layers in the bitstream

  - For each present PPS, determine the size in luma samples of the corresponding layer

  - For each present Independent layer info SEI, parse the payload and store the boundary identifiers for the corresponding layer

  - If any of VPS, PPS or Independent layer info SEI is present in the current Access Unit, execute calculate_layer_layout_in_final_output_picture()

  - Initialize a picture buffer of size FinalWidth of width and FinalHeight of height for the final output picture

  - For each Picture Unit

    - Decode the coded picture

    - If the coded picture belongs to a layer in the selected OLS

      - Apply conformance window cropping on the decoded picture

      - Retrieve the horizontal, XPos, and vertical, YPos, positions of the layer of the current Picture Unit in the final output picture in luma sample

      - Copy cropped picture in output picture buffer at position (XPos, YPos).

### D.x.2.1 Process for calculating the layer layout in the final output picture

Input to this process are:

- the list of layer identifers included in the VPS, LayerIdList

- the list of cropped picture size of each layer, LayerPicSizeList

- the boundary identifier for each layer.

Outputs of this process are:

- the horizontal and vertical position in luma sample of each layer in the final output picture, [{layer_id, XPos, YPos}][NbLayer]

- the size in luma samples of the final output picture, {FinalWidth, FinalHeight}

The steps of the process are:

- Initialise neighboringLayers as a map containing {north, east, south, west} for each layer_id in LayerIdList where north, east, south and west being are set to value null.

- For layer_id in LayerIdList

  - Find the North neighboring layer, if found store this layer identifier as neighboringLayers[layer_id].north

  - Find the East neighboring layer, if found store this layer identifier as neighboringLayers[layer_id].east

  - Find the South neighboring layer, if found store this layer identifier as neighboringLayers[layer_id].south

  - Find the West neighboring layer, if found store this layer identifier, neighboringLayers[layer_id].west

- Initialize a 2-dimensional grid, layerGrid

- Set curCellX, curCellY to ‘0’

- For layer_id in LayerIdList

  - Set layerGrid(CurCellX, CurCellY) to layer_id

  - If neighboringLayers[layer_id].North exists

    - Set layerGrid(CurCellX, CurCellY-1).layer_id to neighboringLayers[layer_id].north

    - Set layerGrid(CurCellX, CurCellY-1).size to LayerPicSizeList[layer_id]

  - If neighboringLayers[layer_id].East exists

    - Set layerGrid(CurCellX-1, CurCellY).layer_id to neighboringLayers[layer_id].east

    - Set layerGrid(CurCellX-1, CurCellY).size to LayerPicSizeList[layer_id]

  - If neighboringLayers[layer_id].South exists

    - Set layerGrid(CurCellX, CurCellY+1).layer_id to neighboringLayers[layer_id].south

    - Set layerGrid(CurCellX, CurCellY+1).size to LayerPicSizeList[layer_id]

  - If neighboringLayers[layer_id].West exists

    - Set layerGrid(CurCellX+1, CurCellY).layer_id to neighboringLayers[layer_id].west

    - Set layerGrid(CurCellX+1, CurCellY).size to LayerPicSizeList[layer_id]

- Set FinalWidth to the maximum value among the sums, for each row of LayerGrid, of the cropped picture width

- Set FinalHeight to the maximum value among the sums, for each column of LayerGrid, of the cropped picture height

- For layer_id in LayerIdList

  - Set XPos to the sum of the widths of each layer in the same row of LayerGrid, left of the current layer

  - Set YPos to the sum of the heights of each layer in the same column of LayerGrid, above the current layer

# Conclusion

It is proposed to adopt the proposed syntax for signalling the aggregation of cropped decoded pictures of layers of an OLS into a final output picture.

# References

- JVET-R0307, “AHG8/AHG9: Positioning information SEI message of output independent layers”, Emmanuel Thomas (TNO)

- JVET-R0308, “AHG8: Implementation of multi-layer decoding and output independent layer composition in VTM”, Emmanuel Thomas, Karim El Assal, Alexandre Gabriel (TNO)

# Patent rights declaration(s)

TNO may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).