| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0110v2 |
| --- | --- |

| Title: | AHG9: On HLS editorial cleanups |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yong He, Vadim Seregin, Muhammed Coban, Marta Karczewicz | Email: | yonghe@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution proposes the following HLS editorial cleanups:

- Remove "RASL pictures are not used as reference pictures for the decoding process of non-RASL pictures" from RASL picture definition note

- Remove "non-IDR" from reference picture list definition note

- Code ph_pic_parameter_set_id with u(6)

# Proposals

## RASL picture definition

RASL picture is defined in VVC D9 as follows:

random access skipped leading (RASL) picture: A coded picture for which there is at least one VCL NAL unit with nal_unit_type equal to RASL_NUT and other VCL NAL units all have nal_unit_type equal to RASL_NUT or RADL_NUT. NOTE – All RASL pictures are leading pictures of an associated CRA picture. When the associated CRA picture has NoOutputBeforeRecoveryFlag equal to 1, the RASL picture is not output and may not be correctly decodable, as the RASL picture may contain references to pictures that are not present in the bitstream. RASL pictures are not used as reference pictures for the decoding process of non-RASL pictures. When sps_field_seq_flag is equal to 0, all RASL pictures, when present, precede, in decoding order, all non-leading pictures of the same associated CRA picture.

Since a RASL picture with mixed_nalu_types_in_pic_flag equal to 1 may contain RADL subpictures, and the RADL subpictures may be used as reference picture for a RADL subpictures in a RADL picture, it is proposed to remove "RASL pictures are not used as reference pictures for the decoding process of non-RASL pictures" as follows:

random access skipped leading (RASL) picture: A coded picture for which there is at least one VCL NAL unit with nal_unit_type equal to RASL_NUT and other VCL NAL units all have nal_unit_type equal to RASL_NUT or RADL_NUT. NOTE – All RASL pictures are leading pictures of an associated CRA picture. When the associated CRA picture has NoOutputBeforeRecoveryFlag equal to 1, the RASL picture is not output and may not be correctly decodable, as the RASL picture may contain references to pictures that are not present in the bitstream. RASL pictures are not used as reference pictures for the decoding process of non-RASL pictures. When sps_field_seq_flag is equal to 0, all RASL pictures, when present, precede, in decoding order, all non-leading pictures of the same associated CRA picture.

The note may also be updated as follows:

NOTE – All RASL pictures are leading pictures of an associated CRA picture. When the associated CRA picture has NoOutputBeforeRecoveryFlag equal to 1, the RASL picture is not output and may not be correctly decodable, as the RASL picture may contain references to pictures that are not present in the bitstream. RASL pictures with mixed_nalu_types_in_pic_flag equal to 0 are not used as reference pictures for the decoding process of non-RASL pictures. When sps_field_seq_flag is equal to 0, all RASL pictures, when present, precede, in decoding order, all non-leading pictures of the same associated CRA picture.

## Reference picture list definition

Reference picture list is defined in VVC D9 as follows:

reference picture list: A list of reference pictures that is used for inter prediction of a P or B slice. NOTE – Two reference picture lists, reference picture list 0 and reference picture list 1, are generated for each slice of a non-IDR picture. The set of unique pictures referred to by all entries in the two reference picture lists associated with a picture consists of all reference pictures that may be used for inter prediction of the associated picture or any picture following the associated picture in decoding order. For the decoding process of a P slice, only reference picture list 0 is used for inter prediction. For the decoding process of a B slice, both reference picture list 0 and reference picture list 1 are used for inter prediction. For decoding the slice data of an I slice, no reference picture list is used for for inter prediction.

Since an IDR picture with pps_rpl_info_in_ph_flag  equal to 1 or sps_idr_rpl_present_flag equal to 1 carries reference picture list, it is proposed to remove "non-IDR" from the note as follows:

reference picture list: A list of reference pictures that is used for inter prediction of a P or B slice. NOTE – Two reference picture lists, reference picture list 0 and reference picture list 1, are generated for each slice of a non-IDR picture. The set of unique pictures referred to by all entries in the two reference picture lists associated with a picture consists of all reference pictures that may be used for inter prediction of the associated picture or any picture following the associated picture in decoding order. For the decoding process of a P slice, only reference picture list 0 is used for inter prediction. For the decoding process of a B slice, both reference picture list 0 and reference picture list 1 are used for inter prediction. For decoding the slice data of an I slice, no reference picture list is used for for inter prediction.

The note may also be updated as follows:

NOTE – Two reference picture lists, reference picture list 0 and reference picture list 1, are generated for each slice of a non-IDR picture or an IDR picture with pps_rpl_info_in_ph_flag equal to 1 or sps_idr_rpl_present_flag equal to 1. The set of unique pictures referred to by all entries in the two reference picture lists associated with a picture consists of all reference pictures that may be used for inter prediction of the associated picture or any picture following the associated picture in decoding order. For the decoding process of a P slice, only reference picture list 0 is used for inter prediction. For the decoding process of a B slice, both reference picture list 0 and reference picture list 1 are used for inter prediction. For decoding the slice data of an I slice, no reference picture list is used for for inter prediction.

## ph_pic_parameter_set_id

pps_pic_parameter_set_id is coded in u(6) in VVC D9, but ph_pic_parameter_set_id is still coded in ue(v). It is proposed to code ph_pic_parameter_set_id in u(6) to align with pps_pic_parameter_set_id.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).