| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0112 |
| --- | --- |

| Title: | AHG9: On GCI semantic constraints |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yong He, Nan Hu, Yao-Jen Chang, Vadim Seregin, Muhammed Coban, Marta Karczewicz | Email: | yonghe@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution proposes the semantic constraints on the existing GCI flags:

- all_layers_independent_constraint_flag

- no_ref_pic_resampling_constraint_flag

- no_res_change_in_clvs_constraint_flag

- one_subpic_per_pic_constraint_flag

- no_sbt_constraint_flag

- no_prof_constraint_flag

- no_bdpcm_constraint_flag

- no_chroma_qp_offset_constraint_flag

- no_mixed_nalu_types_in_pic_constraint_flag

- no_trail_constraint_flag

- no_stsa_constraint_flag

- no_rasl_constraint_flag

- no_radl_constraint_flag

The contribution also proposes to constrain the following syntax elements with the GCI flags:

- sps_num_subpics_minus1

- pps_no_pic_partition_flag

- ph_inter_slice_allowed_flag

- pps_output_flag_present_flag

- ph_non_ref_pic_flag

- ph_pic_output_flag

# Proposals

## GCI semantic constraints

General constraint flag is used to indicate particular coding feature or tool is restricted in a bitstream, and the value of one particular constraint flag may be determined by the value of other constraint flags. The following semantic constraints apply to general constraint information syntax elements.

all_layers_independent_constraint_flag equal to 1 specifies that vps_all_independent_layers_flag shall be equal to 1. all_layers_independent_constraint_flag equal to 0 does not impose such a constraint. When intra_only_constraint_flag is 1, the value of all_layers_independent_constraint_flag shall be equal to 1.

no_ref_pic_resampling_constraint_flag equal to 1 specifies that sps_ref_pic_resampling_enabled_flag shall be equal to 0. no_ref_pic_resampling_constraint_flag equal to 0 does not impose such a constraint. When intra_only_constraint_flag is equal to 1, the value of no_ref_pic_resampling_constraint_flag shall be equal to 1.

no_res_change_in_clvs_constraint_flag equal to 1 specifies that sps_res_change_in_clvs_allowed_flag shall be equal to 0. no_res_change_in_clvs_constraint_flag equal to 0 does not impose such a constraint. When no_ref_pic_resampling_constraint_flag is equal to 1, the value of no_res_change_in_clvs_constraint_flag shall be equal to 1.

one_subpic_per_pic_constraint_flag equal to 1 specifies that each picture shall contain only one subpicture sps_num_subpics_minues shall be equal to 0. one_subpic_per_pic_constraint_flag equal to 0 does not impose such a constraint. When one_slice_per_pic_constraint_flag is equal to 1, the value of one_subpic_per_pic_constraint_flag shall be equal to 1.

no_sbt_constraint_flag equal to 1 specifies that sps_sbt_enabled_flag shall be equal to 0. no_sbt_constraint_flag equal to 0 does not impose such a constraint. When intra_only_constraint_flag is equal to 1, the value of no_sbt_constraint_flag shall be equal to 1.

no_prof_constraint_flag equal to 1 specifies that sps_affine_prof_enabled_flag shall be equal to 0. no_prof_constraint_flag equal to 0 does not impose such a constraint. When intra_only_constraint_flag is equal to 1 no_affine_motion_constraint_flag is equal to1, the value of no_prof_constraint_flag shall be equal to 1.

no_bdpcm_constraint_flag equal to 1 specifies that sps_bdpcm_enabled_flag shall be equal to 0. no_bdpcm_constraint_flag equal to 0 does not impose such a constraint. When no_transform_skip_constraint_flag is equal to 1, the value of no_bdpcm_constraint_flag shall be equal to 1.

no_chroma_qp_offset_constraint_flag equal to 1 specifies that pps_cu_chroma_qp_offset_list_enabled_flag shall be equal to 0. no_chroma_qp_offset_constraint_flag equal to 0 does not impose such a constraint. When max_chroma_format_constraint_idc is equal to 0, the value of no_chroma_qp_offset_contraint_flag shall be equal to 1.

no_mixed_nalu_types_in_pic_constraint_flag equal to 1 specifies that it is a requirement of bitstream conformance that pps_mixed_nalu_types_in_pic_flag shall be equal to 0. no_mixed_nalu_types_in_pic_constraint_flag equal to 0 does not impose such a constraint. When intra_only_constraint_flag is equal to 1 or one_subpic_per_pic_constraint_flag is equal to 1, the value of no_mixed_nalu_types_in_pic_constraint_flag shall be equal to 1.

no_trail_constraint_flag equal to 1 specifies that there shall be no NAL unit with nuh_unit_type equal to TRAIL_NUT present in OlsInScope. no_trail_constraint_flag equal to 0 does not impose such a constraint. When intra_only_constraint_flag is equal to 1, the value of no_trail_constraint_flag shall be equal to 1.

no_stsa_constraint_flag equal to 1 specifies that there shall be no NAL unit with nuh_unit_type equal to STSA_NUT present in OlsInScope. no_stsa_constraint_flag equal to 0 does not impose such a constraint. When intra_only_constraint_flag is equal to 1, the value of no_stsa_constraint_flag shall be equal to 1.

no_rasl_constraint_flag equal to 1 specifies that there shall be no NAL unit with nuh_unit_type equal to RASL_NUT present in OlsInScope. no_rasl_constraint_flag equal to 0 does not impose such a constraint. When intra_only_constraint_flag is equal to 1 or no_cra_constraint_flag is equal to 1, the value of no_rasl_constraint_flag shall be equal to 1.

no_radl_constraint_flag equal to 1 specifies that there shall be no NAL unit with nuh_unit_type equal to RADL_NUT present in OlsInScope. no_radl_constraint_flag equal to 0 does not impose such a constraint. When intra_only_constraint_flag is equal to 1, the value of no_radl_constraint_flag shall be equal to 1.

no_idr_constraint_flag equal to 1 specifies that there shall be no NAL unit with nuh_unit_type equal to IDR_W_RADL or IDR_N_LP present in OlsInScope. no_idr_constraint_flag equal to 0 does not impose such a constraint.

no_cra_constraint_flag equal to 1 specifies that there shall be no NAL unit with nuh_unit_type equal to CRA_NUT present in OlsInScope. no_cra_constraint_flag equal to 0 does not impose such a constraint.

no_gdr_constraint_flag equal to 1 specifies that sps_gdr_enabled_flag shall be equal to 0. no_gdr_constraint_flag equal to 0 does not impose such a constraint.

The value of no_idr_constraint_flag, no_cra_constraint_flag and no_gdr_constraint_flag shall not be all equal to 1.

## Semantic constraints on SPS, PPS and PH syntax elements

The semantic constraints on SPS, PPS and PH syntax elements using GCI flags are proposed as follows:

### sps_num_subpics_minus1

The GCI flag, one_subpic_per_pic_constraint_flag, is to impose the constraint on the number of subpictures available in a picture. It is proposed to add the semantic constraint to the SPS syntax element, sps_num_subpics_minus1 as follows.

sps_num_subpics_minus1 plus 1 specifies the number of subpictures in each picture in the CLVS. The value of sps_num_subpics_minus1 shall be in the range of 0 to Ceil( sps_pic_width_max_in_luma_samples ÷ CtbSizeY ) * Ceil( sps_pic_height_max_in_luma_samples ÷ CtbSizeY ) − 1, inclusive. When not present, the value of sps_num_subpics_minus1 is inferred to be equal to 0. When one_subpic_per_pic_constraint_flag is equal to 1, the value of sps_num_subpics_minus1 shall be equal to 0.

### pps_no_pic_partition_flag

GCI flag one_tile_per_pic_constraint_flag equal to 1 specifies that each picture shall contain only one tile. GCI flag one_slice_per_pic_constraint_flag equal to 1 specifies that each picture shall contain only one slice.  When these two flags are equal to 1, the semantic constraint may be imposed to the syntax element, pps_no_pic_partition_flag as follows.

pps_no_pic_partition_flag equal to 1 specifies that no picture partitioning is applied to each picture referring to the PPS. pps_no_pic_partition_flag equal to 0 specifies that each picture referring to the PPS may be partitioned into more than one tile or slice. It is a requirement of bitstream conformance that the value of pps_no_pic_partition_flag shall be the same for all PPSs that are referred to by coded pictures within a CLVS. When sps_num_subpics_minus1 is greater than 0 or pps_mixed_nalu_types_in_pic_flag is equal to 1, the value of pps_no_pic_partition_flag shall be equal to 0. When one_tile_per_pic_constraint_flag and one_slice_per_pic_constraint_flag both are equal to 1, the value of pps_no_pic_partition_flag shall beequal to 1.

### ph_inter_slice_allowed_flag

When intra_only_constraint_flag is equal to 1, only intra coded pictures/slices are allowed in bitstream. The semantic constraint may be imposed to the syntax element ph_inter_slice_allowed_flag as follows:

ph_inter_slice_allowed_flag equal to 0 specifies that all coded slices of the picture have sh_slice_type equal to 2. ph_inter_slice_allowed_flag equal to 1 specifies that there may or may not be one or more coded slices in the picture that have sh_slice_type equal to 0 or 1. When when ph_gdr_or_irap_pic_flag is equal to 1 and ph_gdr_pic_flag is equal to 0 (i.e., the picture is an IRAP picture), and vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 1, the value of ph_inter_slice_allowed_flag shall be equal to 0. When intra_only_constraint_flag is equal to 1, the value of ph_inter_slice_allowed_flag shall be equal to 0.

### pps_output_flag_present_flag

When intra_only_constraint_flag is equal to 1, each picture should be output since none of the pictures is used as reference picture of any other pictures. In this case, the value of pps_output_flag_present_flag shall be equal to 0 and the ph_pic_output_flag is inferred to be equal to 1.  The semantic constraint on ph_output_flag_present_flag is proposed as follows:

pps_output_flag_present_flag equal to 1 specifies that the ph_pic_output_flag syntax element is present in PHs referring to the PPS. pps_output_flag_present_flag equal to 0 specifies that the ph_pic_output_flag syntax element is not present in PHs referring to the PPS. When intra_only_constraint_flag is equal to 1, the value of pps_output_flag_present_flag shall be equal to 0.

### ph_non_ref_pic_flag

When intra_only_constraint_flag is equal to 1, each picture is not used as reference picture for any other pictures. The semantic constraint on ph_non_ref_pic_flag is proposed as follows:

ph_non_ref_pic_flag equal to 1 specifies the picture associated with the PH is never used as a reference picture. ph_non_ref_pic_flag equal to 0 specifies the picture associated with the PH may or may not be used as a reference picture. When intra_only_constraint_flag is equal to 1, the value of ph_non_ref_pic_flag shall be equal to 1

### ph_pic_output_flag

When intra_only_constraint_flag is equal to 1, each picture is not used as reference picture for any other pictures, and each picture shall be output. The semantic constraint on ph_pic_output_flag is proposed as follows:

ph_pic_output_flag affects the decoded picture output and removal processes as specified in Annex C. When ph_pic_output_flag is not present, it is inferred to be equal to 1. When intra_only_constraint_flag is equal to 1, the value of ph_pic_output_flag shall be equal to 1 NOTE 5 – There is no picture in the bitsteam that has ph_non_ref_pic_flag equal to 1 and ph_pic_output_flag equal to 0.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).