| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0118 |
| --- | --- |

| Title: | AHG9: On HRD bitstream |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yong He, Vadim Seregin, Muhammed Coban, Marta Karczewicz | Email: | yonghe@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to include PH NAL unit into the bitstream subject to HRD conformance checking.

# Proposals

VVC D9 Annex C HRD specifies two types of bitstreams or bitstream subsets are subject to HRD conformance checking.

The first type, called a Type I bitstream, is a NAL unit stream containing only the VCL NAL units and NAL units with nal_unit_type equal to FD_NUT (filler data NAL units) for all AUs in the bitstream. The second type, called a Type II bitstream, contains, in addition to the VCL NAL units and filler data NAL units for all AUs in the bitstream, at least one of the following: – additional non-VCL NAL units other than filler data NAL units, – all leading_zero_8bits, zero_byte, start_code_prefix_one_3bytes and trailing_zero_8bits syntax elements that form a byte stream from the NAL unit stream (as specified in Annex B).

PH NAL unit is the new NAL unit type specified in VVC and equivalent to the original SH, it is proposed to include PH NAL unit in Type I bitstream as follows:

The first type, called a Type I bitstream, is a NAL unit stream containing only the VCL NAL units when a VCL NAL unit has sh_picture_header_in_slice_header_flag equal to 1, or the PH NAL units and the VCL NAL units following the PH NAL unit, and NAL units with nal_unit_type equal to FD_NUT (filler data NAL units) for all AUs in the bitstream. The second type, called a Type II bitstream, contains, in addition to the VCL NAL units, PH NAL units and filler data NAL units for all AUs in the bitstream, at least one of the following: – additional non-VCL NAL units other than PH and filler data NAL units, – all leading_zero_8bits, zero_byte, start_code_prefix_one_3bytes and trailing_zero_8bits syntax elements that form a byte stream from the NAL unit stream (as specified in Annex B). Figure C.1 shows the types of bitstream conformance points checked by the HRD.

Alternatively, PH NAL units may be included in Type II bitstreams together with other parameter sets such as DCI, VPS, SPS, PPS and APS. The HRD text can be be updated as follows:

All DCI NAL units, when available, all VPSs, SPSs, PPSs, PHs and APSs referred to in the VCL NAL units, and appropriate BP, PT, and DUI SEI messages shall be conveyed to the HRD, in a timely manner, either in the bitstream (by non-VCL NAL units), or by other means not specified in this Specification. In Annexes C and D, the specification for "presence" of non-VCL NAL units that contain DCI NAL units, VPSs, SPSs, PPSs, APSs, PHs, BP SEI messages, PT SEI messages, or DUI SEI messages, is also satisfied when those NAL units (or just some of them) are conveyed to decoders (or to the HRD) by other means not specified in this Specification. For the purpose of counting bits, only the appropriate bits that are actually present in the bitstream are counted.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).