| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0119 |
| --- | --- |

| Title: | AHG9: Expression of existing intent for VPS/SPS/PPS syntax elements |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Nan Hu, Vadim Seregin, Yong He, Marta Karczewicz 5775 Morehouse Dr, San Diego, CA, 92121, USA | Tel: Email: | +1(858)651-3483 nanh@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution proposes asserted fixes to the semantics of some VPS, SPS and PPS syntax elements to express the existing intent.

# Proposal

The modified semantics are listed and highlighted below.

VPS:

vps_max_tid_ref_present_flag[ i ] equal to 1 specifies that the syntax element vps_max_tid_il_ref_pics_plus1[ i ][ j ] is may be present. vps_max_tid_ref_present_flag[ i ] equal to 0 specifies that the syntax element vps_max_tid_il_ref_pics_plus1[ i ][ j ] is not present.

SPS:

sps_idr_rpl_present_flag equal to 1 specifies that reference picture list syntax elements are may be present in slice headers of IDR VCL NAL unit pictures. sps_idr_rpl_present_flag equal to 0 specifies that reference picture list syntax elements are not present in slice headers of IDR VCL NAL unit pictures .

sps_bdof_control_present_in_ph_flag equal to 1 specifies that ph_bdof_disabled_flag is may be present in PHs referring to the SPS. sps_bdof_control_present_in_ph_flag equal to 0 specifies that ph_bdof_disabled_flag is not present in PHs referring to the SPS. When sps_bdof_control_present_in_ph_flag is not present, the value of sps_bdof_control_present_in_ph_flag is inferred to be equal to 0.

sps_dmvr_control_present_in_ph_flag equal to 1 specifies that ph_dmvr_disabled_flag is may be present in PHs referring to the SPS. sps_dmvr_control_present_in_ph_flag equal to 0 specifies that ph_dmvr_disabled_flag is not present in PHs referring to the SPS. When sps_dmvr_control_present_in_ph_flag is not present, the value of sps_dmvr_control_present_in_ph_flag is inferred to be equal to 0.

sps_prof_control_present_in_ph_flag equal to 1 specifies that ph_prof_disabled_flag is may be present in PHs referring to the SPS. sps_prof_control_present_in_ph_flag equal to 0 specifies that ph_prof_disabled_flag is not present in PHs referring to the SPS. When sps_prof_control_present_in_ph_flag is not present, the value of sps_prof_control_present_in_ph_flag is inferred to be equal to 0.

PPS:

pps_sao_info_in_ph_flag equal to 1 specifies that SAO filter information is may be present in the PH syntax structure and not present in slice headers referring to the PPS that do not contain a PH syntax structure. pps_sao_info_in_ph_flag equal to 0 specifies that SAO filter information is not present in the PH syntax structure and may be present in slice headers referring to the PPS. When not present, the value of pps_sao_info_in_ph_flag is inferred to be equal to 0.

pps_alf_info_in_ph_flag equal to 1 specifies that ALF information is may be present in the PH syntax structure and not present in slice headers referring to the PPS that do not contain a PH syntax structure. pps_alf_info_in_ph_flag equal to 0 specifies that ALF information is not present in the PH syntax structure and may be present in slice headers referring to the PPS. When not present, the value of pps_alf_info_in_ph_flag is inferred to be equal to 0.

pps_output_flag_present_flag equal to 1 specifies that the ph_pic_output_flag syntax element is may be present in PHs referring to the PPS. pps_output_flag_present_flag equal to 0 specifies that the ph_pic_output_flag syntax element is not present in PHs referring to the PPS.

pps_cu_qp_delta_enabled_flag equal to 1 specifies that at least one of the ph_cu_qp_delta_subdiv_intra_slice and ph_cu_qp_delta_subdiv_inter_slice syntax elements are present in PHs referring to the PPS, and the cu_qp_delta_abs and cu_qp_delta_sign_flag syntax elements may be present in the transform unit syntax and the palette coding syntax. pps_cu_qp_delta_enabled_flag equal to 0 specifies that the ph_cu_qp_delta_subdiv_intra_slice and ph_cu_qp_delta_subdiv_inter_slice syntax elements are not present in PHs referring to the PPS, and the cu_qp_delta_abs and cu_qp_delta_sign_flag syntax elements are not present in the transform unit syntax or the palette coding syntax.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).