| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0122 |
| --- | --- |

| Title: | AHG9: On APS memory constraint |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Nan Hu, Vadim Seregin, Marta Karczewicz 5775 Morehouse Dr, San Diego, CA, 92121, USA | Tel: Email: | +1(858)651-3483 nanh@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

In VVC draft 9, in a PU, prefix and suffix APS NAL units with particular APS identifier and type can have different contents. However, it is reported that the memory to store APSs may exceed the max memory used to store the max number of APSs in a PU. To solve this issue, constraints on referenced and suffix APSs are proposed: in a PU, for an APS type, a signalled suffix APS shall not have the same identifier as a referenced APS. Second, it is proposed that any two suffix (prefix resp.) APS NAL units with particular APS identifier and type signalled in a PU cannot have the same APS identifier.

# Introduction

In VVC draft 9, prefix and suffix APS NAL units with particular APS identifier and type can have different contents in a PU. However, in a PU, if a signalled suffix APS NAL has the same APS identifier and type as a referenced APS, additional buffers are required to store the content in this suffix APS until decoding finishes.

For example, Figure 1 represents two PUs of a bitstream.

Figure 1. Two PUs of a bitstreams. In PU1 suffix APSs have the same ID as referenced APSs.

In this bitstream, for an APS type (for example ALF_APS), PU0 signalled suffix APS0 and APS1 which are referenced by PU1. PU1 signalled prefix APS2, APS3, APS4 and APS5, and all these four prefix APSs are referenced by PU1. Then, PU1 signalled six suffix APSs (suffix APS0 – APS5). If a decoder wants to decode PU1 after parsing suffix APS5, the contents in prior APS0 – APS5 must be stored to avoid being overwritten by the new suffix APS0 – APS5 in PU1. As a result, at least a total of 12 APSs must be stored in memory, which exceed the max number of 8 ALF_APSs.

On the second aspect, in VVC draft 9, for an APS type, there is no constraint on how many APSs could be signalled in a PU. It is possible that multiple APSs can have the same APS identifier. However, the last APS will overwrite the prior APSs with the same identifier. For example, in Figure 2, a PU has two prefix APSs and three suffix APSs. All these APSs have identifiers equal to 0. However, only the last APS0 will be stored in memory. The first four APSs are redundant.

Figure 2. Multiple signalled APSs have the same APS ID

# Proposals

## Constraints for APS IDs on referenced and signalled suffix APSs in a PU

This contribution proposes to add bitstream constraints to avoid additional memory required to store APS contents.

For each PU and each value of aps_params_type, suffix APS signalled after the PU shall not have the value of aps_adaptation_parameter_set_id equal to the aps_adaptation_parameter_set_id value of any referenced APS by the same PU.

## Constraints for signalled APSs in a PU

It is proposed that, for an APS type, any two signalled APS NAL units in a PU cannot have the same identifier.

All APS NAL units with a particular value of nal_unit_type, a particular value of aps_adaptation_parameter_set_id, and a particular value of aps_params_type within a PU shall have the same content. Any two APS NAL units with a particular value of aps_params_type within a PU shall not have the same value of aps_adaptation_parameter_set_id.

Alternatively, it is proposed that, for an APS type and an APS nal_unit_type (PREFIX_APS_NUT or SUFFIX_APS_NUT), any two signalled APS NAL units in a PU cannot have the same identifier.

All APS NAL units with a particular value of nal_unit_type, a particular value of aps_adaptation_parameter_set_id, and a particular value of aps_params_type within a PU shall have the same content. Any two APS NAL units with a particular value of aps_params_type and a particular value of APS nal_unit_type within a PU shall not have the same value of aps_adaptation_parameter_set_id.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).