| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0123 |
| --- | --- |

| Title: | AHG9: On reference picture list constraints |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Vadim Seregin, Yong He, Muhammed Coban, Adarsh K. Ramasubramonian, Marta Karczewicz | Tel: Email: | vseregin@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In VVC, IDR VCL NAL may have reference picture list signalled in a picture header or a slice header, i.e, when pps_rpl_info_in_ph_flag is equal to 1 or sps_idr_rpl_present_flag is equal to 1, however there is no constraint to have those syntax elements equal to 1 for inter-layer prediction.

This contribution proposes to invoke generating unavailable reference picture for IDR and two approaches are proposed to handle inter-layer prediction:

- Remove sps_idr_rpl_present_flag for IDR VCL NAL and always signal RPL for IDR, which is like I-slice case.

- Require either sps_idr_rpl_present_flag is equal to 1 or pps_rpl_info_in_ph_flag is equal to 1 for a dependent layer or require having slice type equal to I_SLICE, otherwise. It is also proposed to set reference picture list to be empty only when sps_idr_rpl_present_flag is equal 0 and pps_rpl_info_in_ph_flag equal to 0.

# Proposal

In the reference picture list derivation process, pictures not present in DPB are set equal to “no reference picture”. There are constraints imposed on reference pictures and to check such constraints for “no reference picture”, there is a process to generate unavailable reference pictures.

This process is not invoked for IDR pictures with sps_idr_rpl_present_flag equal to 1 or pps_rpl_info_in_ph_flag equal to 1, but IDR picture may have "no reference pictures”, for example for IDR in the original bitstream before merging in the mixed NAL use case.

It is proposed to invoke decoding process for generating unavailable reference pictures for IDR pictures in the first approach and for IDR pictures with sps_idr_rpl_present_flag equal to 1 or pps_rpl_info_in_ph_flag equal to 1.

## Approach 1. Remove sps_idr_rpl_present_flag

IDR picture is not that different from just I-slice, where RPL is always signalled for the latter of a non-IDR picture and number of reference picture entries can be signalled equal to 0, when RPL is not needed.

This contribution proses to remove sps_idr_rpl_present_flag and always signal RPL for IDR VCL NALs.

The specification changes are as follows.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_idr_rpl_present_flag | u(1) |
| … |  |

sps_idr_rpl_present_flag equal to 1 specifies that reference picture list syntax elements are present in slice headers of IDR pictures. sps_idr_rpl_present_flag equal to 0 specifies that reference picture list syntax elements are not present in slice headers of IDR pictures.

pps_mixed_nalu_types_in_pic_flag equal to 1 specifies that each picture referring to the PPS has more than one VCL NAL unit and the VCL NAL units do not have the same value of nal_unit_type. pps_mixed_nalu_types_in_pic_flag equal to 0 specifies that each picture referring to the PPS has one or more VCL NAL units and the VCL NAL units of each picture refering to the PPS have the same value of nal_unit_type.

When no_mixed_nalu_types_in_pic_constraint_flag is equal to 1, the value of pps_mixed_nalu_types_in_pic_flag shall be equal to 0.

NOTE 1– pps_mixed_nalu_types_in_pic_flag equal to 1 indicates that pictures referring to the PPS contain slices with different NAL unit types, e.g., coded pictures originating from a subpicture bitstream merging operation for which encoders have to ensure matching bitstream structure and further alignment of parameters of the original bitstreams. One example of such alignments is as follows: When the value of sps_idr_rpl_present_flag is equal to 0 and pps_mixed_nalu_types_in_pic_flag is equal to 1, a picture referring to the PPS cannot have slices with nal_unit_type equal to IDR_W_RADL or IDR_N_LP.

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( !pps_rpl_info_in_ph_flag && ( ( nal_unit_type != IDR_W_RADL && nal_unit_type != IDR_N_LP ) ¦ ¦ sps_idr_rpl_present_flag ) ) |  |
| ref_pic_lists( ) |  |

    - Decoding process for a coded picture

The decoding processes specified in this clause apply to each coded picture, referred to as the current picture and denoted by the variable CurrPic, in BitstreamToDecode.

Depending on the value of sps_chroma_format_idc, the number of sample arrays of the current picture is as follows:

–	If sps_chroma_format_idc is equal to 0, the current picture consists of 1 sample array SL.

–	Otherwise (sps_chroma_format_idc is not equal to 0), the current picture consists of 3 sample arrays SL, SCb, SCr.

The decoding process for the current picture takes as inputs the syntax elements and upper-case variables from clause 7. When interpreting the semantics of each syntax element in each NAL unit, and in the remaining parts of clause 8, the term "the bitstream" (or part thereof, e.g., a CVS of the bitstream) refers to BitstreamToDecode (or part thereof).

Depending on the value of sps_separate_colour_plane_flag, the decoding process is structured as follows:

–	If sps_separate_colour_plane_flag is equal to 0, the decoding process is invoked a single time with the current picture being the output.

–	Otherwise (sps_separate_colour_plane_flag is equal to 1), the decoding process is invoked three times. Inputs to the decoding process are all NAL units of the coded picture with identical value of sh_colour_plane_id. The decoding process of NAL units with a particular value of sh_colour_plane_id is specified as if only a CVS with monochrome colour format with that particular value of sh_colour_plane_id would be present in the bitstream. The output of each of the three decoding processes is assigned to one of the 3 sample arrays of the current picture, with the NAL units with sh_colour_plane_id equal to 0, 1 and 2 being assigned to SL, SCb and SCr, respectively.

NOTE – The variable ChromaArrayType is derived as equal to 0 when sps_separate_colour_plane_flag is equal to 1 and sps_chroma_format_idc is equal to 3. In the decoding process, the value of this variable is evaluated resulting in operations identical to that of monochrome pictures (when sps_chroma_format_idc is equal to 0).

The decoding process operates as follows for the current picture CurrPic:

- The decoding of NAL units is specified in clause 8.2.

- The processes in clause 8.3 specify the following decoding processes using syntax elements in the slice header layer and above:

–	Variables and functions relating to picture order count are derived as specified in clause 8.3.1. This needs to be invoked only for the first slice of a picture.

–	At the beginning of the decoding process for each slice of a picture, the decoding process for reference picture lists construction specified in clause 8.3.2 is invoked for derivation of reference picture list 0 (RefPicList[ 0 ]) and reference picture list 1 (RefPicList[ 1 ]).

–	The decoding process for reference picture marking in clause 8.3.3 is invoked, wherein reference pictures may be marked as "unused for reference" or "used for long-term reference". This needs to be invoked only for the first slice of a picture.

–	When the current picture is an IDR, CRA picture with NoOutputBeforeRecoveryFlag equal to 1 or GDR picture with NoOutputBeforeRecoveryFlag equal to 1, the decoding process for generating unavailable reference pictures specified in subclause 8.3.4 is invoked, which needs to be invoked only for the first slice of a picture.

–	The variable PictureOutputFlag of the current picture is derived as follows:

–	If sps_video_parameter_set_id is greater than 0 and the current layer is not an output layer (i.e., nuh_layer_id is not equal to OutputLayerIdInOls[ TargetOlsIdx ][ i ] for any value of i in the range of 0 to NumOutputLayersInOls[ TargetOlsIdx ] − 1, inclusive), or one of the following conditions is true, PictureOutputFlag is set equal to 0:

–	The current picture is a RASL picture and NoOutputBeforeRecoveryFlag of the associated IRAP picture is equal to 1.

–	The current picture is a GDR picture with NoOutputBeforeRecoveryFlag equal to 1 or is a recovering picture of a GDR picture with NoOutputBeforeRecoveryFlag equal to 1.

–	Otherwise, PictureOutputFlag is set equal to ph_pic_output_flag.

NOTE – In an implementation, the decoder could output a picture not belonging to an output layer. For example, when there is only one output layer while in an AU the picture of the output layer is not available, e.g., due to a loss or layer down-switching, the decoder could set PictureOutputFlag set equal to 1 for the picture that has the highest value of nuh_layer_id among all pictures of the AU available to the decoder and having ph_pic_output_flag equal to 1, and set PictureOutputFlag equal to 0 for all other pictures of the AU available to the decoder.

- The processes in clauses 0, 8.5, 8.6, 8.7, and 8.8 specify decoding processes using syntax elements in all syntax structure layers. It is a requirement of bitstream conformance that the coded slices of the picture shall contain slice data for every CTU of the picture, such that the division of the picture into slices, and the division of the slices into CTUs each forms a partitioning of the picture.

- After all slices of the current picture have been decoded, the current decoded picture is marked as "used for short-term reference", and each ILRP entry, when present, in RefPicList[ 0 ] or RefPicList[ 1 ] is marked as "used for short-term reference".

    - Decoding process for reference picture lists construction

This process is invoked at the beginning of the decoding process for each slice of a picture.

Reference pictures are addressed through reference indices. A reference index is an index into a reference picture list. When decoding an I slice, no reference picture list is used in decoding of the slice data. When decoding a P slice, only reference picture list 0 (i.e., RefPicList[ 0 ]), is used in decoding of the slice data. When decoding a B slice, both reference picture list 0 and reference picture list 1 (i.e., RefPicList[ 1 ]) are used in decoding of the slice data.

At the beginning of the decoding process for each slice of a picture, the reference picture lists RefPicList[ 0 ] and RefPicList[ 1 ] are derived. The reference picture lists are used in marking of reference pictures as specified in clause 8.3.3 or in decoding of the slice data.

NOTE 1 – For an I slice of a picture, RefPicList[ 0 ] and RefPicList[ 1 ] may be derived for bitstream conformance checking purpose, but their derivation is not necessary for decoding of the current picture or pictures following the current picture in decoding order. For a P slice of a picture, RefPicList[ 1 ] may be derived for bitstream conformance checking purpose, but its derivation is not necessary for decoding of the current picture or pictures following the current picture in decoding order.

If sps_idr_rpl_present_flag is equal to 0 and nal_unit_type is equal to IDR_W_RADL or IDR_N_LP, the reference picture lists RefPicList[ 0 ] and RefPicList[ 1 ] are both derived to be empty, i.e., to contain 0 entries, and the following applies for each i equal to 0 or 1:

- The value of RplsIdx[ i ] is inferred to be equal to sps_num_ref_pic_lists[ i ].

- The value of num_ref_entries[ i ][ RplsIdx[ i ] ] is inferred to be equal to 0.

- The value of NumRefIdxActive[ i ] is inferred to be equal to 0.

Otherwise, the reference picture lists RefPicList[ 0 ] and RefPicList[ 1 ], the reference picture scaling ratios RefPicScale[ i ][ j ][ 0 ] and RefPicScale[ i ][ j ][ 1 ], and the reference picture scaled flags RprConstraintsActive[ 0 ][ j ] and RprConstraintsActive[ 1 ][ j ] are derived as follows:

for( i = 0; i < 2; i++ ) {
	for( j = 0, k = 0, pocBase = PicOrderCntVal; j < num_ref_entries[ i ][ RplsIdx[ i ] ]; j++) {
		if( !inter_layer_ref_pic_flag[ i ][ RplsIdx[ i ] ][ j ] ) {
			if( st_ref_pic_flag[ i ][ RplsIdx[ i ] ][ j ] ) {
				RefPicPocList[ i ][ j ] = pocBase − DeltaPocValSt[ i ][ RplsIdx[ i ] ][ j ]
				if( there is a reference picture picA in the DPB with the same nuh_layer_id as the current picture
							and PicOrderCntVal equal to RefPicPocList[ i ][ j ] )
					RefPicList[ i ][ j ] = picA
				else
					RefPicList[ i ][ j ] = "no reference picture"	(205)
				pocBase = RefPicPocList[ i ][ j ]
			} else {
				if( !delta_poc_msb_cycle_present_flag[ i ][ k ] ) {
					if( there is a reference picA in the DPB with the same nuh_layer_id as the current picture and
									PicOrderCntVal & ( MaxPicOrderCntLsb − 1 ) equal to PocLsbLt[ i ][ k ] )
						RefPicList[ i ][ j ] = picA
					else
						RefPicList[ i ][ j ] = "no reference picture"
					RefPicLtPocList[ i ][ j ] = PocLsbLt[ i ][ k ]
				} else {
					if( there is a reference picA in the DPB with the same nuh_layer_id as the current picture and
								PicOrderCntVal equal to 	FullPocLt[ i ][ k ] )
						RefPicList[ i ][ j ] = picA
					else
						RefPicList[ i ][ j ] = "no reference picture"
					RefPicLtPocList[ i ][ j ] = FullPocLt[ i ][ k ]
				}
				k++
			}
		} else {
			layerIdx = DirectRefLayerIdx[ GeneralLayerIdx[ nuh_layer_id ] ][ ilrp_idx[ i ][ RplsIdx ][ j ] ]
			refPicLayerId = vps_layer_id[ layerIdx ]
			if( there is a reference picture picA in the DPB with nuh_layer_id equal to refPicLayerId and
						the same PicOrderCntVal as the current picture )
				RefPicList[ i ][ j ] = picA
			else
				RefPicList[ i ][ j ] = "no reference picture"
		} 
		fRefWidth is set equal to CurrPicScalWinWidthL of the reference picture RefPicList[ i ][ j ]
		fRefHeight is set equal to CurrPicScalWinHeightL of the reference picture RefPicList[ i ][ j ]

		refPicWidth, refPicHeight, refScalingWinLeftOffset, refScalingWinRightOffset, refScalingWinTopOffset,
			and refScalingWinBottomOffset, are set equal to the values of pps_pic_width_in_luma_samples,
			pps_pic_height_in_luma_samples, pps_scaling_win_left_offset, pps_scaling_win_right_offset,
			pps_scaling_win_top_offset, and pps_scaling_win_bottom_offset, respectively, of the reference picture
			RefPicList[ i ][ j ]

		RefPicScale[ i ][ j ][ 0 ] = ( ( fRefWidth  <<  14 ) + ( CurrPicScalWinWidthL  >>  1 ) ) /
				CurrPicScalWinWidthL
		RefPicScale[ i ][ j ][ 1 ] = ( ( fRefHeight  <<  14 ) + ( CurrPicScalWinHeightL  >>  1 ) ) /
				CurrPicScalWinHeightL
		RprConstraintsActive[ i ][ j ] = (	 pps_pic_width_in_luma_samples  !=  refPicWidth  | |
				pps_pic_height_in_luma_samples  !=  refPicHeight  | |
				pps_scaling_win_left_offset  !=  refScalingWinLeftOffset  | |
				pps_scaling_win_right_offset  !=  refScalingWinRightOffset  | |
				pps_scaling_win_top_offset  !=  refScalingWinTopOffset  | |
				pps_scaling_win_bottom_offset  !=  refScalingWinBottomOffset )
	}
}

For each i equal to 0 or 1, the first NumRefIdxActive[ i ] entries in RefPicList[ i ] are referred to as the active entries in RefPicList[ i ], and the other entries in RefPicList[ i ] are referred to as the inactive entries in RefPicList[ i ].

NOTE 2 – It is possible that a particular picture is referred to by both an entry in RefPicList[ 0 ] and an entry in RefPicList[ 1 ]. It is also possible that a particular picture is referred to by more than one entry in RefPicList[ 0 ] or by more than one entry in RefPicList[ 1 ].

NOTE 3 – The active entries in RefPicList[ 0 ] and the active entries in RefPicList[ 1 ] collectively refer to all reference pictures that may be used for inter prediction of the current picture and one or more pictures that follow the current picture in decoding order. The inactive entries in RefPicList[ 0 ] and the inactive entries in RefPicList[ 1 ] collectively refer to all reference pictures that are not used for inter prediction of the current picture but may be used in inter prediction for one or more pictures that follow the current picture in decoding order.

NOTE 4 – There may be one or more entries in RefPicList[ 0 ] or RefPicList[ 1 ] that are equal to "no reference picture" because the corresponding pictures are not present in the DPB. Each inactive entry in RefPicList[ 0 ] or RefPicList[ 1 ] that is equal to "no reference picture" should be ignored. An unintentional picture loss should be inferred for each active entry in RefPicList[ 0 ] or RefPicList[ 1 ] that is equal to "no reference picture".

It is a requirement of bitstream conformance that the following constraints apply:

- For each i equal to 0 or 1, num_ref_entries[ i ][ RplsIdx[ i ] ] shall not be less than NumRefIdxActive[ i ].

- The picture referred to by each active entry in RefPicList[ 0 ] or RefPicList[ 1 ] shall be present in the DPB and shall have TemporalId less than or equal to that of the current picture.

- The picture referred to by each entry in RefPicList[ 0 ] or RefPicList[ 1 ] shall not be the current picture and shall have ph_non_ref_pic_flag equal to 0.

- An STRP entry in RefPicList[ 0 ] or RefPicList[ 1 ] of a slice of a picture and an LTRP entry in RefPicList[ 0 ] or RefPicList[ 1 ] of the same slice or a different slice of the same picture shall not refer to the same picture.

- There shall be no LTRP entry in RefPicList[ 0 ] or RefPicList[ 1 ] for which the difference between the PicOrderCntVal of the current picture and the PicOrderCntVal of the picture referred to by the entry is greater than or equal to 224.

- Let setOfRefPics be the set of unique pictures referred to by all entries in RefPicList[ 0 ] that have the same nuh_layer_id as the current picture and all entries in RefPicList[ 1 ] that have the same nuh_layer_id as the current picture. The number of pictures in setOfRefPics shall be less than or equal to MaxDpbSize − 1, inclusive, where MaxDpbSize is as specified in clause A.4.2, and setOfRefPics shall be the same for all slices of a picture.

- When the current slice has nal_unit_type equal to STSA_NUT, there shall be no active entry in RefPicList[ 0 ] or RefPicList[ 1 ] that has TemporalId equal to that of the current picture and nuh_layer_id equal to that of the current picture.

- When the current picture is a picture that follows, in decoding order, an STSA picture that has TemporalId equal to that of the current picture and nuh_layer_id equal to that of the current picture, there shall be no picture that precedes the STSA picture in decoding order, has TemporalId equal to that of the current picture, and has nuh_layer_id equal to that of the current picture included as an active entry in RefPicList[ 0 ] or RefPicList[ 1 ].

- When the current subpicture, with TemporalId equal to a particular value tId, nuh_layer_id equal to a particular value layerId, and subpicture index equal to a particular value subpicIdx, is a subpicture that follows, in decoding order, an STSA subpicture with TemporalId equal to tId, nuh_layer_id equal to layerId, and subpicture index equal to subpicIdx, there shall be no picture with TemporalId equal to tId and nuh_layer_id equal to layerId that precedes the picture containing the STSA subpicture in decoding order included as an active entry in RefPicList[ 0 ] or RefPicList[ 1 ].

- When the current picture, with nuh_layer_id equal to a particular value layerId, is an IRAP picture, there shall be no picture referred to by an entry in RefPicList[ 0 ] or RefPicList[ 1 ] that precedes, in output order or decoding order, any preceding IRAP picture with nuh_layer_id equal to layerId in decoding order (when present).

- When the current subpicture, with nuh_layer_id equal to a particular value layerId and subpicture index equal to a particular value subpicIdx, is an IRAP subpicture, there shall be no picture referred to by an entry in RefPicList[ 0 ] or RefPicList[ 1 ] that precedes, in output order or decoding order, any preceding picture, in decoding order (when present), containing an IRAP subpicture with nuh_layer_id equal to layerId and subpicture index equal to subpicIdx.

- When the current picture is not a RASL picture associated with a CRA picture with NoOutputBeforeRecoveryFlag equal to 1, there shall be no picture referred to by an active entry in RefPicList[ 0 ] or RefPicList[ 1 ] that was generated by the decoding process for generating unavailable reference pictures for the CRA picture associated with the current picture.

- When the current subpicture is not a RASL subpicture associated with a CRA subpicture in a CRA picture with NoOutputBeforeRecoveryFlag equal to 1, there shall be no picture referred to by an active entry in RefPicList[ 0 ] or RefPicList[ 1 ] that was generated by the decoding process for generating unavailable reference pictures for the CRA picture containing the CRA subpicture associated with the current subpicture.

- When the current picture, with nuh_layer_id equal to a particular value layerId, is not any of the following, there shall be no picture referred to by an entry in RefPicList[ 0 ] or RefPicList[ 1 ] that was generated by the decoding process for generating unavailable reference pictures for the IRAP or GDR picture associated with the current picture:

  - An IDR picture

  - A CRA picture with NoOutputBeforeRecoveryFlag equal to 1

  - A picture, associated with a CRA picture with NoOutputBeforeRecoveryFlag equal to 1, that precedes, in decoding order, the leading pictures associated with the same CRA picture

  - A leading picture associated with a CRA picture with NoOutputBeforeRecoveryFlag equal to 1

  - A GDR picture with NoOutputBeforeRecoveryFlag equal to 1

  - A recovering picture of a GDR picture with NoOutputBeforeRecoveryFlag equal to 1 and nuh_layer_id equal to layerId

- When the current subpicture, with nuh_layer_id equal to a particular value layerId and subpicture index equal to a particular value subpicIdx, is not any of the following, there shall be no picture referred to by an entry in RefPicList[ 0 ] or RefPicList[ 1 ] that was generated by the decoding process for generating unavailable reference pictures for the IRAP or GDR picture containing the IRAP or GDR subpicture associated with the current subpicture:

  - An IDR subpicture

  - A CRA subpicture in a CRA picture with NoOutputBeforeRecoveryFlag equal to 1

  - A subpicture, associated with a CRA subpicture in a CRA picture with NoOutputBeforeRecoveryFlag equal to 1, that precedes, in decoding order, the leading pictures associated with the same CRA picture

  - A leading subpicture associated with a CRA subpicture in a CRA picture with NoOutputBeforeRecoveryFlag equal to 1

  - A GDR subpicture in a GDR picture with NoOutputBeforeRecoveryFlag equal to 1

  - A subpicure in a recovering picture of a GDR picture with NoOutputBeforeRecoveryFlag equal to 1 and nuh_layer_id equal to layerId

- When the current picture follows an IRAP picture having the same value of nuh_layer_id in both decoding order and output order, there shall be no picture referred to by an active entry in RefPicList[ 0 ] or RefPicList[ 1 ] that precedes that IRAP picture in output order or decoding order.

- When the current subpicture follows an IRAP subpicture having the same value of nuh_layer_id and the same value of subpicture index in both decoding and output order, there shall be no picture referred to by an active entry in RefPicList[ 0 ] or RefPicList[ 1 ] that precedes the picture containing that IRAP subpicture in output order or decoding order.

- When the current picture follows an IRAP picture having the same value of nuh_layer_id and the leading pictures, if any, associated with that IRAP picture in both decoding order and output order, there shall be no picture referred to by an entry in RefPicList[ 0 ] or RefPicList[ 1 ] that precedes that IRAP picture in output order or decoding order.

- When the current subpicture follows an IRAP subpicture having the same value of nuh_layer_id and the same value of subpicture index and the leading subpictures, if any, associated with that IRAP subpicture in both decoding and output order, there shall be no picture referred to by an entry in RefPicList[ 0 ] or RefPicList[ 1 ] that precedes the picture containing that IRAP subpicture in output order or decoding order.

- When the current picture is a RADL picture, there shall be no active entry in RefPicList[ 0 ] or RefPicList[ 1 ] that is any of the following:

  - A RASL picture

  - A picture that precedes the associated IRAP picture in decoding order

- When the current subpicture, with nuh_layer_id equal to a particular value layerId and subpicture index equal to a particular value subpicIdx, is a RADL subpicture, there shall be no active entry in RefPicList[ 0 ] or RefPicList[ 1 ] that is any of the following:

  - A picture with nuh_layer_id equal to layerId containing a RASL subpicture with subpicture index equal to subpicIdx

  - A picture that precedes the picture containing the associated IRAP subpicture in decoding order

- The following constraints apply for the picture referred to by each ILRP entry, when present, in RefPicList[ 0 ] or RefPicList[ 1 ] of a slice of the current picture:

  - The picture shall be in the same AU as the current picture.

  - The picture shall be present in the DPB.

  - The picture shall have nuh_layer_id refPicLayerId less than the nuh_layer_id of the current picture.

  - Either of the following constraints applies:

  - The picture shall be an IRAP picture.

  - The picture shall have TemporalId less than or equal to Max( 0, vps_max_tid_il_ref_pics_plus1[ currLayerIdx ][ refLayerIdx ] − 1 ), where currLayerIdx and refLayerIdx are equal to GeneralLayerIdx[ nuh_layer_id ] and GeneralLayerIdx[ refpicLayerId ], respectively.

- Each ILRP entry, when present, in RefPicList[ 0 ] or RefPicList[ 1 ] of a slice shall be an active entry.

- When vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 0 and sps_num_subpics_minus1 is greater than 0, either of the following two conditions (but not both) shall be true:

  - The picture referred to by each active entry in RefPicList[ 0 ] or RefPicList[ 1 ] has the same subpicture layout as the current picture (i.e., the SPSs referred to by that picture and the current picture have the same value of sps_num_subpics_minus1 and the same values of sps_subpic_ctu_top_left_x[ j ], sps_subpic_ctu_top_left_y[ j ], sps_subpic_width_minus1[ j ], and sps_subpic_height_minus1[ j ], respectively, for each value of j in the range of 0 to sps_num_subpics_minus1, inclusive).

  - The picture referred to by each active entry in RefPicList[ 0 ] or RefPicList[ 1 ] is an ILRP for which the value of sps_num_subpics_minus1 is equal to 0.

      - General decoding process for generating unavailable reference pictures

This process is invoked once per coded picture when the current picture is an IDR picture, a CRA picture with NoOutputBeforeRecoveryFlag equal to 1 or a GDR picture with NoOutputBeforeRecoveryFlag equal to 1.

When this process is invoked, the following applies:

- For each 	RefPicList[ i ][ j ], with i in the range of 0 to 1, inclusive, and j in the range of 0 to num_ref_entries[ i ][ RplsIdx[ i ] ] − 1, inclusive, that is equal to "no reference picture", a picture is generated as specified in subclause 8.3.4.2 and the following applies:

- The value of nuh_layer_id for the generated picture is set equal to nuh_layer_id of the current picture.

- If st_ref_pic_flag[ i ][ RplsIdx[ i ] ][ j ] is equal to 1 and inter_layer_ref_pic_flag[ i ][ RplsIdx[ i ] ][ j ] is equal to 0, the value of PicOrderCntVal for the generated picture is set equal to RefPicPocList[ i ][ j ] and the generated picture is marked as "used for short-term reference".

- Otherwise, when st_ref_pic_flag[ i ][ RplsIdx[ i ] ][ j ] is equal to 0 and inter_layer_ref_pic_flag[ i ][ RplsIdx[ i ] ][ j ] is equal to 0, the value of PicOrderCntVal for the generated picture is set equal to RefPicLtPocList[ i ][ j ], the value of ph_pic_order_cnt_lsb for the generated picture is inferred to be equal to ( RefPicLtPocList[ i ][ j ] & ( MaxPicOrderCntLsb – 1 ) ), and the generated picture is marked as "used for long-term reference".

- The value of PictureOutputFlag for the generated reference picture is set equal to 0.

- RefPicList[ i ][ j ] is set to be the generated reference picture.

## Approach 2. Impose constraints on sps_idr_rpl_present_flag and pps_rpl_info_in_ph_flag

## 2.1 Reference picture list signalling

Reference picture list may be signalled in a slice header for IDR VCL NAL when sps_idr_rpl_present_flag is equal to 1 as shown in the next syntax table.

| if( !pps_rpl_info_in_ph_flag && ( ( nal_unit_type != IDR_W_RADL && nal_unit_type != IDR_N_LP ) ¦ ¦ sps_idr_rpl_present_flag ) ) |  |
| --- | --- |
| ref_pic_lists( ) |  |

IDR in the dependent layer may have inter-layer reference picture (ILRP), however reference picture list (RPL) is not signaled when sps_idr_rpl_present_flag is equal to 0 and pps_rpl_info_in_ph_flag is equal to 0 but it is needed for such VCL NAL decoding.

a. It is proposed to signal RPL for IDR VCL NAL when the VCLNAL in the dependent layer.

| if( !pps_rpl_info_in_ph_flag && ( !vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] ¦ ¦ ( ( nal_unit_type != IDR_W_RADL && nal_unit_type != IDR_N_LP ) ¦ ¦ sps_idr_rpl_present_flag ) ) ) |  |
| --- | --- |
| ref_pic_lists( ) |  |

If sps_idr_rpl_present_flag is equal to 0, vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 1, and nal_unit_type is equal to IDR_W_RADL or IDR_N_LP, the reference picture lists RefPicList[ 0 ] and RefPicList[ 1 ] are both derived to be empty, i.e., to contain 0 entries, and the following applies for each i equal to 0 or 1:

- The value of RplsIdx[ i ] is inferred to be equal to sps_num_ref_pic_lists[ i ].

- The value of num_ref_entries[ i ][ RplsIdx[ i ] ] is inferred to be equal to 0.

- The value of NumRefIdxActive[ i ] is inferred to be equal to 0.

b. It is proposed to require sps_idr_rpl_present_flag or pps_rpl_info_in_ph_flag equal to 1 for IDR VCL NAL in the dependent layer.

If vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 0 and slice referred to SPS has nal_unit_type is equal to IDR_W_RADL or IDR_N_LP, the value of sps_idr_rpl_present_flag or pps_rpl_info_in_ph_flag shall be equal to 1.

c. It is proposed to require the slice type shall be equal to I_SLICE for IDR VCL NAL in the dependent layer.

When both of the following conditions are true, the value of sh_slice_type shall be equal to 2:

- The value of nal_unit_type is in the range of IDR_W_RADL to CRA_NUT, inclusive.

- The value of vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 1 or the current picture is the first picture in the current AU, or both values of sps_idr_rpl_present_flag and pps_rpl_info_in_ph_flag are equal to 0.

## 2.2 Empty reference picture list for IDR

There is a constraint in the decoding process of reference picture list construction to set empty reference picture list for IDR VCL NAL as follows.

If sps_idr_rpl_present_flag is equal to 0 and nal_unit_type is equal to IDR_W_RADL or IDR_N_LP, the reference picture lists RefPicList[ 0 ] and RefPicList[ 1 ] are both derived to be empty, i.e., to contain 0 entries, and the following applies for each i equal to 0 or 1:

- The value of RplsIdx[ i ] is inferred to be equal to sps_num_ref_pic_lists[ i ].

- The value of num_ref_entries[ i ][ RplsIdx[ i ] ] is inferred to be equal to 0.

- The value of NumRefIdxActive[ i ] is inferred to be equal to 0.

There is another constraint that setOfRefPics shall be the same for all slices of a picture as follows.

Let setOfRefPics be the set of unique pictures referred to by all entries in RefPicList[ 0 ] that have the same nuh_layer_id as the current picture and all entries in RefPicList[ 1 ] that have the same nuh_layer_id as the current picture. The number of pictures in setOfRefPics shall be less than or equal to MaxDpbSize − 1, inclusive, where MaxDpbSize is as specified in clause A.4.2, and setOfRefPics shall be the same for all slices of a picture.

As a note, number of active reference picture entries is set to 0 for I slices in the derivation of NumRefIdxActive.

…

else /* sh_slice_type  = =  I  | |  ( sh_slice_type  = =  P  &&  i  = =  1 ) */
		NumRefIdxActive[ i ] = 0
…

The problem occurs when reference picture list is signaled in a picture header and is set empty for IDR VCL NAL. It violates the constraint of setOfRefPics shall be the same for all slices of a picture.

It is proposed to empty reference picture list in “8.3.2 Decoding process for reference picture lists construction” when reference picture list is signaled in picture header, i.e pps_rpl_info_in_ph_flag is equal to 0 as follows.

If sps_idr_rpl_present_flag is equal to 0 and nal_unit_type is equal to IDR_W_RADL or IDR_N_LP, the following applies for each i equal to 0 or 1.

- The value of NumRefIdxActive[ i ] is inferred to be equal to 0.

- If pps_rpl_info_in_ph_flag is equal to 0, the following applies:

- The value of RplsIdx[ i ] is inferred to be equal to sps_num_ref_pic_lists[ i ].

- The value of num_ref_entries[ i ][ RplsIdx[ i ] ] is inferred to be equal to 0.

If reference picture list is signaled for IDR in a dependent layer (2.1.a in above), then the constraint is expresses as follows.

If sps_idr_rpl_present_flag is equal to 0, nal_unit_type is equal to IDR_W_RADL or IDR_N_LP and vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] equal to 1, the following applies for each i equal to 0 or 1.

- The value of NumRefIdxActive[ i ] is inferred to be equal to 0.

- If pps_rpl_info_in_ph_flag is equal to 0, the following applies:

- The value of RplsIdx[ i ] is inferred to be equal to sps_num_ref_pic_lists[ i ].

- The value of num_ref_entries[ i ][ RplsIdx[ i ] ] is inferred to be equal to 0.

8.1.2 Decoding process for a coded picture

…

The decoding process operates as follows for the current picture CurrPic:

- The decoding of NAL units is specified in clause 8.2.

- The processes in clause 8.3 specify the following decoding processes using syntax elements in the slice header layer and above:

–	Variables and functions relating to picture order count are derived as specified in clause 8.3.1. This needs to be invoked only for the first slice of a picture.

–	At the beginning of the decoding process for each slice of a picture, the decoding process for reference picture lists construction specified in clause 8.3.2 is invoked for derivation of reference picture list 0 (RefPicList[ 0 ]) and reference picture list 1 (RefPicList[ 1 ]).

–	The decoding process for reference picture marking in clause 8.3.3 is invoked, wherein reference pictures may be marked as "unused for reference" or "used for long-term reference". This needs to be invoked only for the first slice of a picture.

–	When the current picture is an IDR picture with sps_idr_rpl_present_flag equal to 1 or pps_rpl_info_in_ph_flag equal to 1, a CRA picture with NoOutputBeforeRecoveryFlag equal to 1 or GDR picture with NoOutputBeforeRecoveryFlag equal to 1, the decoding process for generating unavailable reference pictures specified in subclause 8.3.4 is invoked, which needs to be invoked only for the first slice of a picture.

### 8.3.2 Decoding process for reference picture lists construction

…

It is a requirement of bitstream conformance that the following constraints apply:

- For each i equal to 0 or 1, num_ref_entries[ i ][ RplsIdx[ i ] ] shall not be less than NumRefIdxActive[ i ].

- The picture referred to by each active entry in RefPicList[ 0 ] or RefPicList[ 1 ] shall be present in the DPB and shall have TemporalId less than or equal to that of the current picture.

- The picture referred to by each entry in RefPicList[ 0 ] or RefPicList[ 1 ] shall not be the current picture and shall have ph_non_reference_picture_flag equal to 0.

- An STRP entry in RefPicList[ 0 ] or RefPicList[ 1 ] of a slice of a picture and an LTRP entry in RefPicList[ 0 ] or RefPicList[ 1 ] of the same slice or a different slice of the same picture shall not refer to the same picture.

- There shall be no LTRP entry in RefPicList[ 0 ] or RefPicList[ 1 ] for which the difference between the PicOrderCntVal of the current picture and the PicOrderCntVal of the picture referred to by the entry is greater than or equal to 224.

- Let setOfRefPics be the set of unique pictures referred to by all entries in RefPicList[ 0 ] that have the same nuh_layer_id as the current picture and all entries in RefPicList[ 1 ] that have the same nuh_layer_id as the current picture. The number of pictures in setOfRefPics shall be less than or equal to MaxDpbSize − 1, inclusive, where MaxDpbSize is as specified in clause A.4.2, and setOfRefPics shall be the same for all slices of a picture.

- When the current slice has nal_unit_type equal to STSA_NUT, there shall be no active entry in RefPicList[ 0 ] or RefPicList[ 1 ] that has TemporalId equal to that of the current picture and nuh_layer_id equal to that of the current picture.

- When the current picture is a picture that follows, in decoding order, an STSA picture that has TemporalId equal to that of the current picture and nuh_layer_id equal to that of the current picture, there shall be no picture that precedes the STSA picture in decoding order, has TemporalId equal to that of the current picture, and has nuh_layer_id equal to that of the current picture included as an active entry in RefPicList[ 0 ] or RefPicList[ 1 ].

- When the current subpicture, with TemporalId equal to a particular value tId, nuh_layer_id equal to a particular value layerId, and subpicture index equal to a particular value subpicIdx, is a subpicture that follows, in decoding order, an STSA subpicture with TemporalId equal to tId, nuh_layer_id equal to layerId, and subpicture index equal to subpicIdx, there shall be no picture with TemporalId equal to tId and nuh_layer_id equal to layerId that precedes the picture containing the STSA subpicture in decoding order included as an active entry in RefPicList[ 0 ] or RefPicList[ 1 ].

- When the current picture, with nuh_layer_id equal to a particular value layerId, is an IRAP picture, there shall be no picture referred to by an entry in RefPicList[ 0 ] or RefPicList[ 1 ] that precedes, in output order or decoding order, any preceding IRAP picture with nuh_layer_id equal to layerId in decoding order (when present).

- When the current subpicture, with nuh_layer_id equal to a particular value layerId and subpicture index equal to a particular value subpicIdx, is an IRAP subpicture, there shall be no picture referred to by an entry in RefPicList[ 0 ] or RefPicList[ 1 ] that precedes, in output order or decoding order, any preceding picture, in decoding order (when present), containing an IRAP subpicture with nuh_layer_id equal to layerId and subpicture index equal to subpicIdx.

- When the current picture is not a RASL picture associated with a CRA picture with NoOutputBeforeRecoveryFlag equal to 1, there shall be no picture referred to by an active entry in RefPicList[ 0 ] or RefPicList[ 1 ] that was generated by the decoding process for generating unavailable reference pictures for the CRA picture associated with the current picture.

- When the current subpicture is not a RASL subpicture associated with a CRA subpicture in a CRA picture with NoOutputBeforeRecoveryFlag equal to 1, there shall be no picture referred to by an active entry in RefPicList[ 0 ] or RefPicList[ 1 ] that was generated by the decoding process for generating unavailable reference pictures for the CRA picture containing the CRA subpicture associated with the current subpicture.

- When the current picture, with nuh_layer_id equal to a particular value layerId, is not any of the following, there shall be no picture referred to by an entry in RefPicList[ 0 ] or RefPicList[ 1 ] that was generated by the decoding process for generating unavailable reference pictures for the IRAP or GDR picture associated with the current picture:

  - An IDR picture with sps_idr_rpl_present_flag equal to 1 or pps_rpl_info_in_ph_flag equal to 1

  - A CRA picture with NoOutputBeforeRecoveryFlag equal to 1

  - A picture, associated with a CRA picture with NoOutputBeforeRecoveryFlag equal to 1, that precedes, in decoding order, the leading pictures associated with the same CRA picture

  - A leading picture associated with a CRA picture with NoOutputBeforeRecoveryFlag equal to 1

  - A GDR picture with NoOutputBeforeRecoveryFlag equal to 1

  - A recovering picture of a GDR picture with NoOutputBeforeRecoveryFlag equal to 1 and nuh_layer_id equal to layerId

- When the current subpicture, with nuh_layer_id equal to a particular value layerId and subpicture index equal to a particular value subpicIdx, is not any of the following, there shall be no picture referred to by an entry in RefPicList[ 0 ] or RefPicList[ 1 ] that was generated by the decoding process for generating unavailable reference pictures for the IRAP or GDR picture containing the IRAP or GDR subpicture associated with the current subpicture:

  - An IDR subpicture in a IDR picture with sps_idr_rpl_present_flag equal to 1 or pps_rpl_info_in_ph_flag equal to 1

  - A CRA subpicture in a CRA picture with NoOutputBeforeRecoveryFlag equal to 1

  - A subpicture, associated with a CRA subpicture in a CRA picture with NoOutputBeforeRecoveryFlag equal to 1, that precedes, in decoding order, the leading pictures associated with the same CRA picture

  - A leading subpicture associated with a CRA subpicture in a CRA picture with NoOutputBeforeRecoveryFlag equal to 1

  - A GDR subpicture in a GDR picture with NoOutputBeforeRecoveryFlag equal to 1

  - A subpicure in a recovering picture of a GDR picture with NoOutputBeforeRecoveryFlag equal to 1 and nuh_layer_id equal to layerId

- When the current picture follows an IRAP picture having the same value of nuh_layer_id in both decoding order and output order, there shall be no picture referred to by an active entry in RefPicList[ 0 ] or RefPicList[ 1 ] that precedes that IRAP picture in output order or decoding order.

- When the current subpicture follows an IRAP subpicture having the same value of nuh_layer_id and the same value of subpicture index in both decoding and output order, there shall be no picture referred to by an active entry in RefPicList[ 0 ] or RefPicList[ 1 ] that precedes the picture containing that IRAP subpicture in output order or decoding order.

- When the current picture follows an IRAP picture having the same value of nuh_layer_id and the leading pictures, if any, associated with that IRAP picture in both decoding order and output order, there shall be no picture referred to by an entry in RefPicList[ 0 ] or RefPicList[ 1 ] that precedes that IRAP picture in output order or decoding order.

- When the current subpicture follows an IRAP subpicture having the same value of nuh_layer_id and the same value of subpicture index and the leading subpictures, if any, associated with that IRAP subpicture in both decoding and output order, there shall be no picture referred to by an entry in RefPicList[ 0 ] or RefPicList[ 1 ] that precedes the picture containing that IRAP subpicture in output order or decoding order.

- When the current picture is a RADL picture, there shall be no active entry in RefPicList[ 0 ] or RefPicList[ 1 ] that is any of the following:

  - A RASL picture

  - A picture that precedes the associated IRAP picture in decoding order

- When the current subpicture, with nuh_layer_id equal to a particular value layerId and subpicture index equal to a particular value subpicIdx, is a RADL subpicture, there shall be no active entry in RefPicList[ 0 ] or RefPicList[ 1 ] that is any of the following:

  - A picture with nuh_layer_id equal to layerId containing a RASL subpicture with subpicture index equal to subpicIdx

  - A picture that precedes the picture containing the associated IRAP subpicture in decoding order

- The picture referred to by each ILRP entry in RefPicList[ 0 ] or RefPicList[ 1 ] of a slice of the current picture shall be in the same AU as the current picture.

- The picture referred to by each ILRP entry in RefPicList[ 0 ] or RefPicList[ 1 ] of a slice of the current picture shall be present in the DPB, shall have nuh_layer_id refPicLayerId less than the nuh_layer_id of the current picture, and shall either be an IRAP picture or have TemporalId less than or equal to Max( 0, vps_max_tid_il_ref_pics_plus1[ currLayerIdx ][ refLayerIdx ] − 1 ), where currLayerIdx and refLayerIdx are equal to GeneralLayerIdx[ nuh_layer_id ] and GeneralLayerIdx[ refpicLayerId ], respectively.

- Each ILRP entry in RefPicList[ 0 ] or RefPicList[ 1 ] of a slice shall be an active entry.

- When vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 0, and sps_num_subpics_minus1 is greater than 0, either of the following two conditions (but not both) shall be true:

  - The picture referred to by each active entry in RefPicList[ 0 ] or RefPicList[ 1 ] has the same subpicture layout as the current picture (i.e., the SPSs referred to by that picture and the current picture have the same value of sps_num_subpics_minus1 and the same values of sps_subpic_ctu_top_left_x[ j ], sps_subpic_ctu_top_left_y[ j ], sps_subpic_width_minus1[ j ], and sps_subpic_height_minus1[ j ], respectively, for each value of j in the range of 0 to sps_num_subpics_minus1, inclusive).

  - The picture referred to by each active entry in RefPicList[ 0 ] or RefPicList[ 1 ] is an ILRP for which the value of sps_num_subpics_minus1 is equal to 0.

8.3.4.1	General decoding process for generating unavailable reference pictures

This process is invoked once per coded picture when the current picture is an IDR picture with sps_idr_rpl_present_flag equal to 1 or pps_rpl_info_in_ph_flag equal to 1, a CRA picture with NoOutputBeforeRecoveryFlag equal to 1 or a GDR picture with NoOutputBeforeRecoveryFlag equal to 1.

When this process is invoked, the following applies:

- For each 	RefPicList[ i ][ j ], with i in the range of 0 to 1, inclusive, and j in the range of 0 to num_ref_entries[ i ][ RplsIdx[ i ] ] − 1, inclusive, that is equal to "no reference picture", a picture is generated as specified in subclause 8.3.4.2 and the following applies:

- The value of nuh_layer_id for the generated picture is set equal to nuh_layer_id of the current picture.

- If st_ref_pic_flag[ i ][ RplsIdx[ i ] ][ j ] is equal to 1 and inter_layer_ref_pic_flag[ i ][ RplsIdx[ i ] ][ j ] is equal to 0, the value of PicOrderCntVal for the generated picture is set equal to RefPicPocList[ i ][ j ] and the generated picture is marked as "used for short-term reference".

- Otherwise, when st_ref_pic_flag[ i ][ RplsIdx[ i ] ][ j ] is equal to 0 and inter_layer_ref_pic_flag[ i ][ RplsIdx[ i ] ][ j ] is equal to 0, the value of PicOrderCntVal for the generated picture is set equal to RefPicLtPocList[ i ][ j ], the value of ph_pic_order_cnt_lsb for the generated picture is inferred to be equal to ( RefPicLtPocList[ i ][ j ] & ( MaxPicOrderCntLsb – 1 ) ), and the generated picture is marked as "used for long-term reference".

- The value of PicOutputFlag for the generated reference picture is set equal to 0.

- RefPicList[ i ][ j ] is set to be the generated reference picture.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).