| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0127 |
| --- | --- |

| Title: | AHG9: On general constraint info signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Muhammed Coban Vadim Seregin Yong He Yao-Jen Chang Marta Karczewicz | Tel: Email: | +1 858 658 3973 mcoban@qti.qualcomm.com vseregin@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to add a flag to signal the presence of general_constraint_info() syntax structure in profile level tier indication to enable skipping of signalling of this information when all constraint flags are equal to 0, and the values of max_bitdepth_constraint_idc and max_chroma_format_constraint_idc are equal to 2 and 1, respectively.

# Proposal

In current VVC draft [1], the general constraint info() is signalled in profile_tier_level() when profileTierPresentFlag is set to 1. When no additional contraints are applied to a particular profile, i.e. all constraints flags are equal 0, then skipping the signalling of general_constraint_info() syntax structure would avoid unnecesary signalling. Additionally, the default value of max_bitdepth_constraint_idc field is equal to 2 and the default value of 	max_chroma_format_constraint_idc is equal to 1 when general_constraint_info() is not signalled.

## Proposed text changes

The changed texts are based on the latest VVC text in JVET-R2001-vA. Parts that have been added or modified are highlighted in turquoise, and deleted parts are highlighted in yellow strikethrough in red fonts.

=========================VVC draft 9 changes BEGIN==============================

#### 7.3.3.1 General profile, tier, and level syntax

| profile_tier_level( profileTierPresentFlag, maxNumSubLayersMinus1 ) { | Descriptor |
| --- | --- |
| if( profileTierPresentFlag ) { |  |
| general_profile_idc | u(7) |
| general_tier_flag | u(1) |
| general_constraint_info_present_flag | u(1) |
| if( general_constraint_info_present_flag ) |  |
| general_constraint_info( ) |  |
| } |  |
| general_level_idc | u(8) |
| if( profileTierPresentFlag ) { |  |
| ptl_num_sub_profiles | u(8) |
| for( i = 0; i < ptl_num_sub_profiles; i++ ) |  |
| general_sub_profile_idc[ i ] | u(32) |
| } |  |
| for( i = 0; i < maxNumSubLayersMinus1; i++ ) |  |
| ptl_sublayer_level_present_flag[ i ] | u(1) |
| while( !byte_aligned( ) ) |  |
| ptl_alignment_zero_bit | f(1) |
| for( i = 0; i < maxNumSubLayersMinus1; i++ ) |  |
| if( ptl_sublayer_level_present_flag[ i ] ) |  |
| sublayer_level_idc[ i ] | u(8) |

#### 7.4.4.1 General profile, tier, and level semantics

A profile_tier_level( ) syntax structure provides level information and, optionally, profile, tier, sub-profile, and general constraints information.

When the profile_tier_level( ) syntax structure is included in a VPS, the OlsInScope is one or more OLSs specified by the VPS. When the profile_tier_level( ) syntax structure is included in an SPS, the OlsInScope is the OLS that includes only the layer that is the lowest layer among the layers that refer to the SPS, and this lowest layer is an independent layer.

general_profile_idc indicates a profile to which OlsInScope conforms as specified in Annex A. Bitstreams shall not contain values of general_profile_idc other than those specified in Annex A. Other values of general_profile_idc are reserved for future use by ITU-T | ISO/IEC.

general_tier_flag specifies the tier context for the interpretation of general_level_idc as specified in Annex A.

general_constraint_info_present_flag equal to 1 specifies that profile_level_tier() syntax structure contains general_constraint_info( ) syntax structure. general_constraint_info_present_flag equal to 0 specifies that contains general_constraint_info( ) syntax structure is not present in profile_level_tier() syntax structure.

#### 7.4.4.3 General constraint information semantics

When general_constraint_info_present_flag is equal to 0, all constraints fields in section 7.3.3.2 are inferred to be equal to 0, max_bitdepth_constraint_idc is inferred to be equal to 2, and max_chroma_format_constraint_idc is inferred to be equal to 1.

general_non_packed_constraint_flag equal to 1 specifies that there shall not be any frame packing arrangement SEI messages present in the bitstream of the OlsInScope. general_non_packed_constraint_flag equal to 0 does not impose such a constraint.

NOTE 1 – Decoders may ignore the value of general_non_packed_constraint_flag, as there are no decoding process requirements associated with the presence or interpretation of frame packing arrangement SEI messages.

general_frame_only_constraint_flag equal to 1 specifies that OlsInScope conveys pictures that represent frames. general_frame_only_constraint_flag equal to 0 specifies that OlsInScope conveys pictures that may or may not represent frames.

…

=========================VVC draft 9 changes END==============================

# References

- B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding (Draft 9)”, JVET-P2001, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, 18th Meeting: by teleconference, 15–24 April 2020.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).