| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0128 |
| --- | --- |

| Title: | AHG12: On entry point signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Muhammed Coban Vadim Seregin Yong He Yao-Jen Chang Marta Karczewicz | Tel: Email: | +1 858 658 3973 mcoban@qti.qualcomm.com vseregin@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

In this contribution the following are proposed:

- Move signalling of WPP enabling and presence of entry point offsets from SPS to PPS.

- Enabling signalling of tile entry points only when both tiles and WPP are enabled.

# Proposal

In current VVC draft [1], there is no mechanism to signal the tile entry points only when both tiles and WPP are enabled. In cases where encoder uses WPP for parallelization and enables tiles, the option to signal only tile entry points can be useful. It is proposed to enable this option by adding option to signal WPP or tile entry points as described in the proposed text changes.

## Proposed text changes

The changed texts are based on the latest VVC text in JVET-R2001-vA. Parts that have been added or modified are highlighted in turquoise, and deleted parts are highlighted in yellow strikethrough in red fonts. Complete specification changes to JVET-R2001-vA is attached.

=========================VVC draft 9 changes BEGIN==============================

#### 7.3.2.3 Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_seq_parameter_set_id | u(4) |
| sps_video_parameter_set_id | u(4) |
| ….. |  |
| sps_bit_depth_minus8 | ue(v) |
| sps_entropy_coding_sync_enabled_flag | u(1) |
| sps_entry_point_offsets_present_flag | u(1) |
| sps_log2_max_pic_order_cnt_lsb_minus4 | u(4) |
| … | u(1) |

#### 7.3.2.4 Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | u(6) |
| pps_seq_parameter_set_id | u(4) |
| …. |  |
| pps_no_pic_partition_flag | u(1) |
| pps_entropy_coding_sync_enabled_flag | u(1) |
| if( pps_entropy_coding_sync_enabled_flag ) |  |
| pps_entropy_coding_sync_entry_point_offsets_present_flag | u(1) |
| if( !pps_no_pic_partition_flag && !pps_entropy_coding_sync_entry_point_offsets_present_flag ) |  |
| pps_tile_entry_point_offsets_present_flag | u(1) |
| pps_subpic_id_mapping_present_flag | u(1) |
| …. |  |

#### 7.4.3.3 Sequence parameter set RBSP semantics

An SPS RBSP shall be available to the decoding process prior to it being referenced, included in at least one AU with TemporalId equal to 0 or provided through external means.

…

sps_entropy_coding_sync_enabled_flag equal to 1 specifies that a specific synchronization process for context variables is invoked before decoding the CTU that includes the first CTB of a row of CTBs in each tile in each picture referring to the SPS, and a specific storage process for context variables is invoked after decoding the CTU that includes the first CTB of a row of CTBs in each tile in each picture referring to the SPS. sps_entropy_coding_sync_enabled_flag equal to 0 specifies that no specific synchronization process for context variables is required to be invoked before decoding the CTU that includes the first CTB of a row of CTBs in each tile in each picture referring to the SPS, and no specific storage process for context variables is required to be invoked after decoding the CTU that includes the first CTB of a row of CTBs in each tile in each picture referring to the SPS.

NOTE 5 – When sps_entropy_coding_sync_enabled_flag is equal to 1, the so-called wavefront parallel processing (WPP) is enabled.

sps_entry_point_offsets_present_flag equal to 1 specifies that signalling for entry point offsets for tiles or tile-specific CTU rows may be present in the slice headers of pictures referring to the SPS. sps_entry_point_offsets_present_flag equal to 0 specifies that signalling for entry point offsets for tiles or tile-specific CTU rows are not present in the slice headers of pictures referring to the SPS.

sps_log2_max_pic_order_cnt_lsb_minus4 specifies the value of the variable MaxPicOrderCntLsb that is used in the decoding process for picture order count as follows:

#### …

#### 7.4.3.4 Picture parameter set RBSP semantics

#### ….

pps_no_pic_partition_flag equal to 1 specifies that no picture partitioning is applied to each picture referring to the PPS. pps_no_pic_partition_flag equal to 0 specifies that each picture referring to the PPS may be partitioned into more than one tile or slice.

It is a requirement of bitstream conformance that the value of pps_no_pic_partition_flag shall be the same for all PPSs that are referred to by coded pictures within a CLVS.

When sps_num_subpics_minus1 is greater than 0 or pps_mixed_nalu_types_in_pic_flag is equal to 1, the value of pps_no_pic_partition_flag shall be equal to 0.

pps_entropy_coding_sync_enabled_flag equal to 1 specifies that a specific synchronization process for context variables is invoked before decoding the CTU that includes the first CTB of a row of CTBs in each tile in each picture referring to the PPS, and a specific storage process for context variables is invoked after decoding the CTU that includes the first CTB of a row of CTBs in each tile in each picture referring to the PPS. pps_entropy_coding_sync_enabled_flag equal to 0 specifies that no specific synchronization process for context variables is required to be invoked before decoding the CTU that includes the first CTB of a row of CTBs in each tile in each picture referring to the PPS, and no specific storage process for context variables is required to be invoked after decoding the CTU that includes the first CTB of a row of CTBs in each tile in each picture referring to the PPS.

NOTE 5 – When pps_entropy_coding_sync_enabled_flag is equal to 1, the so-called wavefront parallel processing (WPP) is enabled.

pps_entropy_coding_sync_entry_point_offsets_present_flag equal to 1 specifies that signalling for entry point offsets for tile-specific CTU rows may be present in the slice headers of pictures referring to the PPS. pps_entropy_coding_sync_entry_point_offsets_present_flag equal to 0 specifies that signalling for entry point offsets for  tile-specific CTU rows are not present in the slice headers of pictures referring to the PPS.When not present, the value of pps_entropy_coding_sync_entry_point_offsets_present_flag is inferred to be equal to 0.

pps_tile_entry_point_offsets_present_flag equal to 1 specifies that signalling for entry point offsets for tiles may be present in the slice headers of pictures referring to the PPS. pps_entry_point_offsets_present_flag equal to 0 specifies that signalling for entry point offsets for tiles are not present in the slice headers of pictures referring to the PPS. When not present, the value of pps_tile_entry_point_offsets_present_flag shall be equal to pps_entropy_coding_sync_entry_point_offsets_present_flag.

#### 7.4.8.1 General slice header semantics

…

sh_slice_header_extension_data_byte[ i ] may have any value. Decoders conforming to this version of this Specification shall ignore the values of all the sh_slice_header_extension_data_byte[ i ] syntax elements. Its value does not affect decoder conformance to profiles specified in this version of specification.[Ed. (RSS): When profiles have been defined in Annex A, consider changing the last sentence to "Its value does not affect decoder conformance to profiles specified in Annex A".]

The variable NumEntryPoints, which specifies the number of entry points in the current slice, is derived as follows:

NumEntryPoints = 0
if( spspps_tile_entry_point_offsets_present_flag )
	for( i = 1; i < NumCtusInCurrSlice; i++ ) {
		ctbAddrX = CtbAddrInCurrSlice[ i ] % PicWidthInCtbsY
		ctbAddrY = CtbAddrInCurrSlice[ i ] / PicWidthInCtbsY	(145)		prevCtbAddrX = CtbAddrInCurrSlice[ i − 1 ] % PicWidthInCtbsY
		prevCtbAddrY = CtbAddrInCurrSlice[ i − 1 ] / PicWidthInCtbsY
		if( CtbToTileRowBd[ ctbAddrY ]  !=  CtbToTileRowBd[ prevCtbAddrY ]  | |
				CtbToTileColBd[ ctbAddrX ]  !=  CtbToTileColBd[ prevCtbAddrX ]  | |
				( ctbAddrY  !=  prevCtbAddrY  &&  sps_entropy_coding_sync_enabled_flagpps_entropy_coding_sync_entry_point_offsets_present_flag ) )
			NumEntryPoints++
	}

=========================VVC draft 9 changes END==============================

# References

- B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding (Draft 9)”, JVET-P2001, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, 18th Meeting: by teleconference, 15–24 April 2020.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).