| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0129-v2 |
| --- | --- |

| Title: | AHG9: cleanup on parameter sets and GCI |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ling Li, Xiang Li, Byeongdoo Choi, Stephan, Wenger, Shan Liu | Tel: Email: | {aurali, xlxiangli, bdchoi, swenger, shanl }@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

The contribution proposes following changes:

Aspect 1: add inference value for syntax elements in VPS

Aspect 2: introduce sps_one_picture_only_constraint_flag in SPS to skip inter, RPL and POC related syntax elements in both SPS and PH

Aspect 3: rearrange syntax elements in GCI like SPS and PH, and add inference rules

# Introduction

1) It is noticed that several inference values for syntax elements in VPS are missing, inference rules are specified in aspect 1.

2) With the decision of having Main 10 Still Picture and Main 4:4:4 10 Still Picture profile in VVC, general_one_picture_only_constraint_flag is introduced in general constraint information to conform bitstreams. Conformance of a bitstream to the Main 10 Still Picture profile is indicated by general_one_picture_only_constraint_flag being equal to 1 together with general_profile_idc being equal to 1.

When general_one_picture_only_constraint_flag equals to 1, SPS level flags of inter coding tool are set to 0, i.e. inter coding tools are turned off in SPS. It is asserted VVC has five times more HLS than that of HEVC, especially in SPS, PH and APS. For still picture profiles, bit budget on HLS may even more critical than Main 10 and Main 4:4:4 10 profiles. To reduce bits cost in HLS in still picture profiles, it is proposed to skip some syntax elements in SPS and PH.

3) Rearrange of GCI syntax elements in the fashion of SPS and PH is introduced in aspect 3. Additionally, missing inference rules are described.

# Proposed Spec Text

Changes compared to the latest VVC Draft 9 vA [1] are highlighted in yellow.

## Aspect 1: add inference value for syntax elements in VPS

Table 1. video parameter set RBSP syntax in VVC Draft 9

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| vps_video_parameter_set_id | u(4) |
| vps_max_layers_minus1 | u(6) |
| vps_max_sublayers_minus1 | u(3) |
| if( vps_max_layers_minus1 > 0 && vps_max_sublayers_minus1 > 0 ) |  |
| vps_all_layers_same_num_sublayers_flag | u(1) |
| if( vps_max_layers_minus1 > 0 ) |  |
| vps_all_independent_layers_flag | u(1) |
| for( i = 0; i <= vps_max_layers_minus1; i++ ) { |  |
| vps_layer_id[ i ] | u(6) |
| if( i > 0 && !vps_all_independent_layers_flag ) { |  |
| vps_independent_layer_flag[ i ] | u(1) |
| if( !vps_independent_layer_flag[ i ] ) { |  |
| vps_max_tid_ref_present_flag[ i ] | u(1) |
| for( j = 0; j < i; j++ ) { |  |
| vps_direct_ref_layer_flag[ i ][ j ] | u(1) |
| if( vps_max_tid_ref_present_flag[ i ] && vps_direct_ref_layer_flag[ i ][ j ] ) |  |
| vps_max_tid_il_ref_pics_plus1[ i ][ j ] | u(3) |
| } |  |
| } |  |
| } |  |
| } |  |
| if( vps_max_layers_minus1 > 0 ) { |  |
| if( vps_all_independent_layers_flag ) |  |
| vps_each_layer_is_an_ols_flag | u(1) |
| if( !vps_each_layer_is_an_ols_flag ) { |  |
| if( !vps_all_independent_layers_flag ) |  |
| vps_ols_mode_idc | u(2) |
| if( vps_ols_mode_idc = = 2 ) { |  |
| vps_num_output_layer_sets_minus1 | u(8) |
| for( i = 1; i <= vps_num_output_layer_sets_minus1; i ++) |  |
| for( j = 0; j <= vps_max_layers_minus1; j++ ) |  |
| vps_ols_output_layer_flag[ i ][ j ] | u(1) |
| } |  |
| } |  |
| } |  |
| … |  |
| } |  |

vps_max_tid_ref_present_flag[ i ] equal to 1 specifies that the syntax element vps_max_tid_il_ref_pics_plus1[ i ][ j ] is present. vps_max_tid_ref_present_flag[ i ] equal to 0 specifies that the syntax element vps_max_tid_il_ref_pics_plus1[ i ][ j ] is not present. When vps_max_tid_ref_present_flag[ i ] is not present, the value of vps_max_tid_ref_present_flag[ i ] is inferred to be 0.

vps_ols_mode_idc equal to 0 specifies that the total number of OLSs specified by the VPS is equal to vps_max_layers_minus1 + 1, the i-th OLS includes the layers with layer indices from 0 to i, inclusive, and for each OLS only the highest layer in the OLS is an output layer.

vps_ols_mode_idc equal to 1 specifies that the total number of OLSs specified by the VPS is equal to vps_max_layers_minus1 + 1, the i-th OLS includes the layers with layer indices from 0 to i, inclusive, and for each OLS all layers in the OLS are output layers.

vps_ols_mode_idc equal to 2 specifies that the total number of OLSs specified by the VPS is explicitly signalled and for each OLS the output layers are explicitly signalled and other layers are the layers that are direct or indirect reference layers of the output layers of the OLS.

The value of vps_ols_mode_idc shall be in the range of 0 to 2, inclusive. The value 3 of vps_ols_mode_idc is reserved for future use by ITU-T | ISO/IEC.

When vps_all_independent_layers_flag is equal to 1 and vps_each_layer_is_an_ols_flag is equal to 0, the value of vps_ols_mode_idc is inferred to be equal to 2. When vps_all_independent_layers_flag is equal to 1 and vps_each_layer_is_an_ols_flag is equal to 1, the value of vps_ols_mode_idc is inferred to be equal to 0

## Aspect 2: introduce sps_one_picture_only_constraint_flag in SPS to skip inter, RPL and POC related syntax elements in both SPS and PH

Table 2. proposed changes for SPS RBSP

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_seq_parameter_set_id | u(4) |
| sps_only_one_picture_present_flag | u(1) |
| if (!sps_only_one_picture_present_flag) { |  |
| sps_video_parameter_set_id | u(4) |
| sps_max_sublayers_minus1 | u(3) |
| } |  |
| sps_reserved_zero_4bits | u(4) |
| if (!sps_only_one_picture_present_flag) |  |
| sps_ptl_dpb_hrd_params_present_flag | u(1) |
| if( sps_ptl_dpb_hrd_params_present_flag ) |  |
| profile_tier_level( 1, sps_max_sublayers_minus1 ) |  |
| sps_gdr_enabled_flag | u(1) |
| sps_chroma_format_idc | u(2) |
| if( sps_chroma_format_idc = = 3 ) |  |
| sps_separate_colour_plane_flag | u(1) |
| if (!sps_only_one_picture_present_flag) |  |
| sps_ref_pic_resampling_enabled_flag | u(1) |
| if( sps_ref_pic_resampling_enabled_flag ) |  |
| sps_res_change_in_clvs_allowed_flag | u(1) |
| sps_pic_width_max_in_luma_samples | ue(v) |
| sps_pic_height_max_in_luma_samples | ue(v) |
| sps_conformance_window_flag | u(1) |
| if( sps_conformance_window_flag ) { |  |
| sps_conf_win_left_offset | ue(v) |
| sps_conf_win_right_offset | ue(v) |
| sps_conf_win_top_offset | ue(v) |
| sps_conf_win_bottom_offset | ue(v) |
| } |  |
| sps_log2_ctu_size_minus5 | u(2) |
| sps_subpic_info_present_flag | u(1) |
| if( sps_subpic_info_present_flag ) { |  |
| sps_num_subpics_minus1 | ue(v) |
| if( sps_num_subpics_minus1 > 0 ) |  |
| sps_independent_subpics_flag | u(1) |
| for( i = 0; sps_num_subpics_minus1 > 0 && i <= sps_num_subpics_minus1; i++ ) { |  |
| if( i > 0 && sps_pic_width_max_in_luma_samples > CtbSizeY ) |  |
| sps_subpic_ctu_top_left_x[ i ] | u(v) |
| if( i > 0 && sps_pic_height_max_in_luma_samples > CtbSizeY ) { |  |
| sps_subpic_ctu_top_left_y[ i ] | u(v) |
| if( i < sps_num_subpics_minus1 && sps_pic_width_max_in_luma_samples > CtbSizeY ) |  |
| sps_subpic_width_minus1[ i ] | u(v) |
| if( i < sps_num_subpics_minus1 && sps_pic_height_max_in_luma_samples > CtbSizeY ) |  |
| sps_subpic_height_minus1[ i ] | u(v) |
| if( !sps_independent_subpics_flag) { |  |
| sps_subpic_treated_as_pic_flag[ i ] | u(1) |
| sps_loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| } |  |
| } |  |
| sps_subpic_id_len_minus1 | ue(v) |
| sps_subpic_id_mapping_explicitly_signalled_flag | u(1) |
| if( sps_subpic_id_mapping_explicitly_signalled_flag ) { |  |
| sps_subpic_id_mapping_present_flag | u(1) |
| if( sps_subpic_id_mapping_present_flag ) |  |
| for( i = 0; i <= sps_num_subpics_minus1; i++ ) |  |
| sps_subpic_id[ i ] | u(v) |
| } |  |
| } |  |
| sps_bit_depth_minus8 | ue(v) |
| sps_entropy_coding_sync_enabled_flag | u(1) |
| sps_entry_point_offsets_present_flag | u(1) |
| if (!sps_only_one_picture_present_flag) { |  |
| sps_log2_max_pic_order_cnt_lsb_minus4 | u(4) |
| sps_poc_msb_cycle_flag | u(1) |
| } |  |
| if( sps_poc_msb_cycle_flag ) |  |
| sps_poc_msb_cycle_len_minus1 | ue(v) |
| sps_num_extra_ph_bits_bytes | u(2) |
| extra_ph_bits_struct( sps_num_extra_ph_bits_bytes ) |  |
| sps_num_extra_sh_bits_bytes | u(2) |
| extra_sh_bits_struct( sps_num_extra_sh_bits_bytes ) |  |
| if( sps_ptl_dpb_hrd_params_present_flag ) { |  |
| if( sps_max_sublayers_minus1 > 0 ) |  |
| sps_sublayer_dpb_params_flag | u(1) |
| dpb_parameters( sps_max_sublayers_minus1, sps_sublayer_dpb_params_flag ) |  |
| } |  |
| if( ChromaArrayType != 0 ) |  |
| sps_qtbtt_dual_tree_intra_flag | u(1) |
| sps_log2_min_luma_coding_block_size_minus2 | ue(v) |
| sps_partition_constraints_override_enabled_flag | u(1) |
| sps_log2_diff_min_qt_min_cb_intra_slice_luma | ue(v) |
| sps_max_mtt_hierarchy_depth_intra_slice_luma | ue(v) |
| if( sps_max_mtt_hierarchy_depth_intra_slice_luma != 0 ) { |  |
| sps_log2_diff_max_bt_min_qt_intra_slice_luma | ue(v) |
| sps_log2_diff_max_tt_min_qt_intra_slice_luma | ue(v) |
| } |  |
| if (!sps_only_one_picture_present_flag) { |  |
| sps_log2_diff_min_qt_min_cb_inter_slice | ue(v) |
| sps_max_mtt_hierarchy_depth_inter_slice | ue(v) |
| } |  |
| if( sps_max_mtt_hierarchy_depth_inter_slice != 0 ) { |  |
| sps_log2_diff_max_bt_min_qt_inter_slice | ue(v) |
| sps_log2_diff_max_tt_min_qt_inter_slice | ue(v) |
| } |  |
| if( sps_qtbtt_dual_tree_intra_flag ) { |  |
| sps_log2_diff_min_qt_min_cb_intra_slice_chroma | ue(v) |
| sps_max_mtt_hierarchy_depth_intra_slice_chroma | ue(v) |
| if( sps_max_mtt_hierarchy_depth_intra_slice_chroma != 0 ) { |  |
| sps_log2_diff_max_bt_min_qt_intra_slice_chroma | ue(v) |
| sps_log2_diff_max_tt_min_qt_intra_slice_chroma | ue(v) |
| } |  |
| } |  |
| if( CtbSizeY > 32 ) |  |
| sps_max_luma_transform_size_64_flag | u(1) |
| if( ChromaArrayType != 0 ) { |  |
| sps_joint_cbcr_enabled_flag | u(1) |
| sps_same_qp_table_for_chroma_flag | u(1) |
| numQpTables = sps_same_qp_table_for_chroma_flag ? 1 : ( sps_joint_cbcr_enabled_flag ? 3 : 2 ) |  |
| for( i = 0; i < numQpTables; i++ ) { |  |
| sps_qp_table_start_minus26[ i ] | se(v) |
| sps_num_points_in_qp_table_minus1[ i ] | ue(v) |
| for( j = 0; j <= sps_num_points_in_qp_table_minus1[ i ]; j++ ) { |  |
| sps_delta_qp_in_val_minus1[ i ][ j ] | ue(v) |
| sps_delta_qp_diff_val[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |
| sps_sao_enabled_flag | u(1) |
| sps_alf_enabled_flag | u(1) |
| if( sps_alf_enabled_flag && ChromaArrayType != 0 ) |  |
| sps_ccalf_enabled_flag | u(1) |
| sps_transform_skip_enabled_flag | u(1) |
| if( sps_transform_skip_enabled_flag ) { |  |
| sps_log2_transform_skip_max_size_minus2 | ue(v) |
| sps_bdpcm_enabled_flag | u(1) |
| } |  |
| if (!sps_only_one_picture_present_flag) { |  |
| sps_weighted_pred_flag | u(1) |
| sps_weighted_bipred_flag | u(1) |
| sps_long_term_ref_pics_flag | u(1) |
| if( sps_video_parameter_set_id > 0 ) |  |
| sps_inter_layer_ref_pics_present_flag | u(1) |
| sps_idr_rpl_present_flag | u(1) |
| sps_rpl1_same_as_rpl0_flag | u(1) |
| for( i = 0; i < sps_rpl1_same_as_rpl0_flag ? 1 : 2; i++ ) { |  |
| sps_num_ref_pic_lists[ i ] | ue(v) |
| for( j = 0; j < sps_num_ref_pic_lists[ i ]; j++) |  |
| ref_pic_list_struct( i, j ) |  |
| } |  |
| sps_ref_wraparound_enabled_flag | u(1) |
| sps_temporal_mvp_enabled_flag | u(1) |
| if( sps_temporal_mvp_enabled_flag ) |  |
| sps_sbtmvp_enabled_flag | u(1) |
| sps_amvr_enabled_flag | u(1) |
| sps_bdof_enabled_flag | u(1) |
| if( sps_bdof_enabled_flag ) |  |
| sps_bdof_control_present_in_ph_flag | u(1) |
| sps_smvd_enabled_flag | u(1) |
| sps_dmvr_enabled_flag | u(1) |
| if( sps_dmvr_enabled_flag) |  |
| sps_dmvr_control_present_in_ph_flag | u(1) |
| sps_mmvd_enabled_flag | u(1) |
| if( sps_mmvd_enabled_flag ) |  |
| sps_mmvd_fullpel_only_flag | u(1) |
| sps_six_minus_max_num_merge_cand | ue(v) |
| sps_sbt_enabled_flag | u(1) |
| sps_affine_enabled_flag | u(1) |
| if( sps_affine_enabled_flag ) { |  |
| sps_five_minus_max_num_subblock_merge_cand | ue(v) |
| sps_affine_type_flag | u(1) |
| if( sps_amvr_enabled_flag ) |  |
| sps_affine_amvr_enabled_flag | u(1) |
| sps_affine_prof_enabled_flag | u(1) |
| if( sps_affine_prof_enabled_flag ) |  |
| sps_prof_control_present_in_ph_flag | u(1) |
| } |  |
| sps_bcw_enabled_flag | u(1) |
| sps_ciip_enabled_flag | u(1) |
| if( MaxNumMergeCand >= 2 ) { |  |
| sps_gpm_enabled_flag | u(1) |
| if( sps_gpm_enabled_flag && MaxNumMergeCand >= 3 ) |  |
| sps_max_num_merge_cand_minus_max_num_gpm_cand | ue(v) |
| } |  |
| sps_log2_parallel_merge_level_minus2 | ue(v) |
| } |  |
| sps_isp_enabled_flag | u(1) |
| sps_mrl_enabled_flag | u(1) |
| sps_mip_enabled_flag | u(1) |
| if( ChromaArrayType != 0 ) |  |
| sps_cclm_enabled_flag | u(1) |
| if( sps_chroma_format_idc = = 1 ) { |  |
| sps_chroma_horizontal_collocated_flag | u(1) |
| sps_chroma_vertical_collocated_flag | u(1) |
| } |  |
| sps_mts_enabled_flag | u(1) |
| if( sps_mts_enabled_flag ) { |  |
| sps_explicit_mts_intra_enabled_flag | u(1) |
| if (!sps_only_one_picture_present_flag) |  |
| sps_explicit_mts_inter_enabled_flag | u(1) |
| } |  |
| … |  |
| } |  |

sps_only_one_picture_present_flag equals to 1 specifies that there is only one coded picture in the bitstream. sps_only_one_picture_present_flag equals to 0 specifies that there are more than one coded picture in the bitstream, and sh_slice_type shall be equal to 1. When general_one_picture_only_constraint_flag equals to 1, it is bitstream conformance that  sps_only_one_picture_present_flag equals to 1.

sps_video_parameter_set_id, when greater than 0, specifies the value of vps_video_parameter_set_id for the VPS referred to by the SPS. When sps_only_one_picture_present_flag equals to 1, sps_video_parameter_set_id is equal to 0.

When sps_video_parameter_set_id is equal to 0, the following applies:

–	The SPS does not refer to a VPS, and no VPS is referred to when decoding each CLVS referring to the SPS.

–	The value of vps_max_layers_minus1 is inferred to be equal to 0.

–	The value of vps_max_sublayers_minus1 is inferred to be equal to 6.

–	The CVS shall contain only one layer (i.e., all VCL NAL unit in the CVS shall have the same value of nuh_layer_id).

–	The value of GeneralLayerIdx[ nuh_layer_id ] is inferred to be equal to 0.

–	The value of vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is inferred to be equal to 1.

When vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 1, the SPS referred to by a CLVS with a particular nuh_layer_id value nuhLayerId shall have nuh_layer_id equal to nuhLayerId.

The value of sps_video_parameter_set_id shall be the same in all SPSs that are referred to by CLVSs in a CVS.

sps_max_sublayers_minus1 plus 1 specifies the maximum number of temporal sublayers that may be present in each CLVS referring to the SPS. The value of sps_max_sublayers_minus1 shall be in the range of 0 to vps_max_sublayers_minus1, inclusive. When sps_only_one_picture_present_flag equals to 1, sps_ max_sublayers_minus1 is equal to 0.

sps_ptl_dpb_hrd_params_present_flag equal to 1 specifies that a profile_tier_level( ) syntax structure and a dpb_parameters( ) syntax structure are present in the SPS, and a general_hrd_parameters( ) syntax structure and an ols_hrd_parameters( ) syntax structure may also be present in the SPS. sps_ptl_dpb_hrd_params_present_flag equal to 0 specifies that none of these four syntax structures is present in the SPS. When sps_only_one_picture_present_flag equals to 1, sps_ptl_dpb_hrd_params_present_flag is equal to 1.

When sps_video_parameter_set_id is greater than 0 and there is an OLS that contains only one layer with nuh_layer_id equal to the nuh_layer_id of the SPS, or when sps_video_parameter_set_id is equal to 0, the value of sps_ptl_dpb_hrd_params_present_flag shall be equal to 1.

sps_ref_pic_resampling_enabled_flag equal to 1 specifies that reference picture resampling is enabled and one or more slices of pictures in the CLVS may refer to a reference picture with a different spatial resolution in an active entry of a reference picture list. sps_ref_pic_resampling_enabled_flag equal to 0 specifies that reference picture resampling is disabled and no slice of pictures in the CLVS refers to a reference picture with a different spatial resolution in an active entry of a reference picture list. When not present, the value of sps_ref_pic_resampling_enabled_flag is inferred to be equal to 0.

NOTE 2 – When sps_ref_pic_resampling_enabled_flag is equal to 1, for a current picture the reference picture with a different spatial resolution may either belong to the same layer or a different layer than the layer containing the current picture.

sps_log2_max_pic_order_cnt_lsb_minus4 specifies the value of the variable MaxPicOrderCntLsb that is used in the decoding process for picture order count as follows:

MaxPicOrderCntLsb = 2( sps_log2_max_pic_order_cnt_lsb_minus4 + 4 )	(48)

The value of sps_log2_max_pic_order_cnt_lsb_minus4 shall be in the range of 0 to 12, inclusive. When sps_only_one_picture_present_flag equals to 1, sps_log2_max_pic_order_cnt_lsb_minus4 is equal to 0.

sps_poc_msb_cycle_flag equal to 1 specifies that the ph_poc_msb_cycle_present_flag syntax element is present in PHs referring to the SPS. sps_poc_msb_cycle_flag equal to 0 specifies that the ph_poc_msb_cycle_present_flag syntax element is not present in PHs referring to the SPS. When sps_only_one_picture_present_flag equals to 1, sps_poc_msb_cyle_flag is equal to 0.

sps_weighted_pred_flag equal to 1 specifies that weighted prediction may be applied to P slices referring to the SPS. sps_weighted_pred_flag equal to 0 specifies that weighted prediction is not applied to P slices referring to the SPS. When not present, the value of sps_weighted_pred_flag is inferred to be equal to 0.

sps_weighted_bipred_flag equal to 1 specifies that explicit weighted prediction may be applied to B slices referring to the SPS. sps_weighted_bipred_flag equal to 0 specifies that explicit weighted prediction is not applied to B slices referring to the SPS. When not present, the value of sps_weighted_bipred_flag is inferred to be equal to 0.

sps_long_term_ref_pics_flag equal to 0 specifies that no LTRP is used for inter prediction of any coded picture in the CLVS. sps_long_term_ref_pics_flag equal to 1 specifies that LTRPs may be used for inter prediction of one or more coded pictures in the CLVS. When not present, the value of sps_long_term_ref_pics_flag is inferred to be equal to 0.

sps_idr_rpl_present_flag equal to 1 specifies that reference picture list syntax elements are present in slice headers of IDR pictures. sps_idr_rpl_present_flag equal to 0 specifies that reference picture list syntax elements are not present in slice headers of IDR pictures. When not present, the value of sps_idr_rpl_present_flag is inferred to be equal to 0.

sps_rpl1_same_as_rpl0_flag equal to 1 specifies that the syntax element sps_num_ref_pic_lists[ 1 ] and the syntax structure ref_pic_list_struct( 1, rplsIdx ) are not present and the following applies:

–	The value of sps_num_ref_pic_lists[ 1 ] is inferred to be equal to the value of sps_num_ref_pic_lists[ 0 ].

–	The value of each of syntax elements in ref_pic_list_struct( 1, rplsIdx ) is inferred to be equal to the value of corresponding syntax element in ref_pic_list_struct( 0, rplsIdx ) for rplsIdx ranging from 0 to sps_num_ref_pic_lists[ 0 ] − 1.

When sps_rpl1_same_as_rpl0_flag is not present, the value of sps_rpl1_same_as_rpl0_flag is inferred to be equal to 0.

sps_num_ref_pic_lists[ i ] specifies the number of the ref_pic_list_struct( listIdx, rplsIdx ) syntax structures with listIdx equal to i included in the SPS. The value of sps_num_ref_pic_lists[ i ] shall be in the range of 0 to 64, inclusive. When not present, the value of sps_num_ref_pic_lists[ i ] is inferred to be equal to 0.

NOTE 6 – For each value of listIdx (equal to 0 or 1), a decoder should allocate memory for a total number of sps_num_ref_pic_lists[ i ] + 1 ref_pic_list_struct( listIdx, rplsIdx ) syntax structures since there may be one ref_pic_list_struct( listIdx, rplsIdx ) syntax structure directly signalled in the slice headers of a current picture.

sps_ref_wraparound_enabled_flag equal to 1 specifies that horizontal wrap-around motion compensation is enabled and may be applied in inter prediction when decoding pictures in the CLVS. sps_ref_wraparound_enabled_flag equal to 0 specifies that horizontal wrap-around motion compensation is disabled and not applied in inter prediction when decoding pictures in the CLVS. When not present, the value of sps_ref_wraparound_enabled_flag is inferred to be equal to 0

It is a requirement of bitstream conformance that, when there is one or more values of i in the range of 0 to sps_num_subpics_minus1, inclusive, for which sps_subpic_treated_as_pic_flag[ i ] is equal to 1 and sps_subpic_width_minus1[ i ] plus 1 is not equal to ( sps_pic_width_max_in_luma_samples + CtbSizeY− 1 )  >>  CtbLog2SizeY ), the value of sps_ref_wraparound_enabled_flag shall be equal to 0.

sps_temporal_mvp_enabled_flag equal to 1 specifies that temporal motion vector predictors are enabled and may be used in decoding of pictures in the CLVS. sps_temporal_mvp_enabled_flag equal to 0 specifies that temporal motion vector predictors are disabled and not used in decoding of pictures in the CLVS. When not present, the value of sps_temporal_mvp_enabled_flag is inferred to be equal to 0.

sps_amvr_enabled_flag equal to 1 specifies that adaptive motion vector difference resolution is enabled and may be used in motion vector coding in decoding of pictures in the CVLS. amvr_enabled_flag equal to 0 specifies that adaptive motion vector difference resolution is disabled and not used in motion vector coding in decoding of pictures in the CLVS. When not present, the value of sps_amvr_enabled_flag is inferred to be equal to 0.

sps_bdof_enabled_flag equal to 0 specifies that the bi-directional optical flow inter prediction is disabled and not used in decoding of pictures in the CLVS. sps_bdof_enabled_flag equal to 1 specifies that the bi-directional optical flow inter prediction is enabled and may be used in decoding of pictures in the CLVS. When not present, the value of sps_bdof_enabled_flag is inferred to be equal to 0.

sps_smvd_enabled_flag equal to 1 specifies that symmetric motion vector difference is enabled may be used in motion vector decoding in decoding of pictures in the CLVS. sps_smvd_enabled_flag equal to 0 specifies that symmetric motion vector difference is disabled and not used in motion vector coding in decoding of pictures in the CLVS. When not present, the value of sps_smvd_enabled_flag is inferred to be equal to 0.

sps_dmvr_enabled_flag equal to 1 specifies that decoder motion vector refinement based inter bi-prediction is enabled and may be used in decoding of pictures in the CLVS. sps_dmvr_enabled_flag equal to 0 specifies that decoder motion vector refinement based inter bi-prediction is disabled and not used in decoding of pictures in the CLVS. When not present, the value of sps_dmvr_enabled_flag is inferred to be equal to 0.

sps_mmvd_enabled_flag equal to 1 specifies that merge mode with motion vector difference is enabled and may be used in decoding of pictures in the CLVS. sps_mmvd_enabled_flag equal to 0 specifies that merge mode with motion vector difference is disabled and not used in in decoding of pictures in the CLVS. When not present, the value of sps_mmvd_enabled_flag is inferred to be equal to 0.

sps_six_minus_max_num_merge_cand specifies the maximum number of merging motion vector prediction (MVP) candidates supported in the SPS subtracted from 6. The value of sps_six_minus_max_num_merge_cand shall be in the range of 0 to 5, inclusive. When not present, the value of sps_six_minus_max_num_merge_cand is inferred to be equal to 6.

The maximum number of merging MVP candidates, MaxNumMergeCand, is derived as follows:

- MaxNumMergeCand = 6 − sps_six_minus_max_num_merge_cand	(64)

sps_sbt_enabled_flag equal to 0 specifies that subblock transform for inter-predicted CUs is disabled and not used in decoding of pictures in the CLVS. sps_sbt_enabled_flag equal to 1 specifies that subblock transform for inter-predicteds CU is enabled and may be used in decoding of pictures in the CLVS. When not present, the value of sps_sbt_enabled_flag is inferred to be equal to 0.

sps_affine_enabled_flag equal to 0 specifies that affine model based motion compensation is disabled and not used in decoding of pictures in the CLVS and inter_affine_flag and cu_affine_type_flag are not present in the coding unit syntax of the CLVS. sps_affine_enabled_flag equal to 1 specifies that affine model based motion compensation is enabled and may be used in decoding of pictures in the CLVS and inter_affine_flag and cu_affine_type_flag may be present in the coding unit syntax of the CLVS. When not present, the value of sps_affine_enabled_flag is inferred to be equal to 0.

sps_five_minus_max_num_subblock_merge_cand specifies the maximum number of subblock-based merging motion vector prediction candidates supported in the SPS subtracted from 5. The value of sps_five_minus_max_num_subblock_merge_cand shall be in the range of 0 to 5 − sps_sbtmvp_enabled_flag, inclusive. When not present, the value of sps_five_minus_max_num_subblock_merge_cand is inferred to be equal to 5 − sps_sbtmvp_enabled_flag.

sps_bcw_enabled_flag equal to 0 specifies that bi-prediction with CU weights is disabled and not used in decoding of picturs in the CLVS and bcw_idx is not present in the coding unit syntax of the CLVS. sps_bcw_enabled_flag equal to 1 specifies that  bi-prediction with CU weights is enabled and may be used in decoding of pictures in the CLVS and bcw_idx may be present in the coding unit syntax of the CLVS. When not present, the value of sps_bcw_enabled_flag is inferred to be equal to 0.

sps_ciip_enabled_flag specifies that ciip_flag may be present in the coding unit syntax for inter coding units. sps_ciip_enabled_flag equal to 0 specifies that ciip_flag is not present in the coding unit syntax for inter coding units. When not present, the value of sps_ciip_enabled_flag is inferred to be equal to 0.

sps_log2_parallel_merge_level_minus2 plus 2 specifies the value of the variable Log2ParMrgLevel, which is used in the derivation process for spatial merging candidates as specified in clause 8.5.2.3, the derivation process for motion vectors and reference indices in subblock merge mode as specified in clause 8.5.5.2, and to control the invocation of the updating process for the history-based motion vector predictor list in clause 8.5.2.1. When not present, the value of sps_log2_parallel_merge_level_minus2 is inferred to be equal to 0.The value of sps_log2_parallel_merge_level_minus2 shall be in the range of 0 to CtbLog2SizeY − 2, inclusive. The variable Log2ParMrgLevel is derived as follows:

Log2ParMrgLevel = sps_log2_parallel_merge_level_minus2 + 2

Table 3. proposed changes in picture header syntax

| picture_header_structure( ) { | Descriptor |
| --- | --- |
| ph_gdr_or_irap_pic_flag | u(1) |
| if( ph_gdr_or_irap_pic_flag ) |  |
| ph_gdr_pic_flag | u(1) |
| if (!sps_only_one_picture_present_flag) { |  |
| ph_inter_slice_allowed_flag | u(1) |
| if( ph_inter_slice_allowed_flag ) |  |
| ph_intra_slice_allowed_flag | u(1) |
| ph_non_ref_pic_flag | u(1) |
| } |  |
| ph_pic_parameter_set_id | ue(v) |
| if (!sps_only_one_picture_present_flag) { |  |
| ph_pic_order_cnt_lsb | u(v) |
| if( ph_gdr_or_irap_pic_flag ) |  |
| ph_no_output_of_prior_pics_flag | u(1) |
| if( ph_gdr_pic_flag ) |  |
| ph_recovery_poc_cnt | ue(v) |
| } |  |
| for( i = 0; i < NumExtraPhBits; i++ ) |  |
| ph_extra_bit[ i ] | u(1) |
| if( sps_poc_msb_cycle_flag ) { |  |
| ph_poc_msb_cycle_present_flag | u(1) |
| if( ph_poc_msb_cycle_present_flag ) |  |
| ph_poc_msb_cycle_val | u(v) |
| } |  |
| … |  |
| if( pps_output_flag_present_flag && !ph_non_ref_pic_flag &&!sps_only_one_picture_present_flag ) |  |
| ph_pic_output_flag | u(1) |
| if( pps_rpl_info_in_ph_flag &&!sps_only_one_picture_present_flag ) |  |
| ref_pic_lists( ) |  |
| … |  |
| } |  |

ph_inter_slice_allowed_flag equal to 0 specifies that all coded slices of the picture have sh_slice_type equal to 2. ph_inter_slice_allowed_flag equal to 1 specifies that there may or may not be one or more coded slices in the picture that have sh_slice_type equal to 0 or 1. When not present, the value of ph_inter_slice_allowed_flag is inferred to be equal to 0.

ph_non_ref_pic_flag equal to 1 specifies the picture associated with the PH is never used as a reference picture. ph_non_ref_pic_flag equal to 0 specifies the picture associated with the PH may or may not be used as a reference picture. When not present, the value of ph_non_ref_pic_flag is inferred to be equal to 1.

ph_pic_order_cnt_lsb specifies the picture order count modulo MaxPicOrderCntLsb for the current picture. The length of the ph_pic_order_cnt_lsb syntax element is sps_log2_max_pic_order_cnt_lsb_minus4 + 4 bits. The value of the ph_pic_order_cnt_lsb shall be in the range of 0 to MaxPicOrderCntLsb − 1, inclusive. When not present, the value of ph_pic_order_cnt_lsb is inferred to be equal to 0.

ph_no_output_of_prior_pics_flag affects the output of previously-decoded pictures in the DPB after the decoding of a picture in a CVSS AU that is not the first AU in the bitstream as specified in Annex C.

It is a requirement of bitstream conformance that, when present, the value of ph_no_output_of_prior_pics_flag shall be the same for all pictures in an AU.

When ph_no_output_of_prior_pics_flag is present in the PHs of the pictures in an AU, the ph_no_output_of_prior_pics_flag value of the AU is the ph_no_output_of_prior_pics_flag value of the pictures in the AU. When not present, the value of ph_no_output_of_prior_pics_flag is inferred to be equal to 0.

ph_recovery_poc_cnt specifies the recovery point of decoded pictures in output order.

When the current picture is a GDR picture, the variable recoveryPointPocVal is derived as follows:

- recoveryPointPocVal = PicOrderCntVal + ph_recovery_poc_cnt	(82)

If the current picture is a GDR picture, and there is a picture picA that follows the current GDR picture in decoding order in the CLVS that has PicOrderCntVal equal to recoveryPointPocVal, the picture picA is referred to as the recovery point picture. Otherwise, the first picture in output order that has PicOrderCntVal greater than recoveryPointPocVal in the CLVS is referred to as the recovery point picture. The recovery point picture shall not precede the current GDR picture in decoding order. The pictures that are associated with the current GDR picture and have PicOrderCntVal less than recoveryPointPocVal are referred to as the recovering pictures of the GDR picture. The value of ph_recovery_poc_cnt shall be in the range of 0 to MaxPicOrderCntLsb − 1, inclusive. When not present, the value of ph_recovery_poc_cnt is inferred to be equal to 0.

ph_poc_msb_cycle_present_flag equal to 1 specifies that the syntax element ph_poc_msb_cycle_val is present in the PH. ph_poc_msb_cycle_present_flag equal to 0 specifies that the syntax element ph_poc_msb_cycle_val is not present in the PH. When vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 0 and there is a picture in the current AU in a reference layer of the current layer, the value of ph_poc_msb_cycle_present_flag shall be equal to 0. When not present, the value of ph_poc_msb_cycle_present_flag is inferred to be equal to 0.

## Aspect 3: rearrange syntax elements in GCI like SPS and PH, and add inference rules

Table 4. proposed rearrangement of GCI syntax

| general_constraint_info( ) { | Descriptor |
| --- | --- |
| general_non_packed_constraint_flag | u(1) |
| general_frame_only_constraint_flag | u(1) |
| general_non_projected_constraint_flag | u(1) |
| general_one_picture_only_constraint_flag | u(1) |
| intra_only_constraint_flag | u(1) |
| max_bitdepth_constraint_idc | u(4) |
| max_chroma_format_constraint_idc | u(2) |
| single_layer_constraint_flag | u(1) |
| all_layers_independent_constraint_flag | u(1) |
| no_ref_pic_resampling_constraint_flag | u(1) |
| no_res_change_in_clvs_constraint_flag | u(1) |
| one_tile_per_pic_constraint_flag | u(1) |
| pic_header_in_slice_header_constraint_flag | u(1) |
| one_slice_per_pic_constraint_flag | u(1) |
| one_subpic_per_pic_constraint_flag | u(1) |
| no_partition_constraints_override_constraint_flag | u(1) |
| no_sao_constraint_flag | u(1) |
| no_alf_constraint_flag | u(1) |
| no_ccalf_constraint_flag | u(1) |
| no_joint_cbcr_constraint_flag | u(1) |
| no_mts_constraint_flag | u(1) |
| no_ladf_constraint_flag | u(1) |
| no_transform_skip_constraint_flag | u(1) |
| no_act_constraint_flag | u(1) |
| no_lmcs_constraint_flag | u(1) |
| no_cu_qp_delta_constraint_flag | u(1) |
| no_chroma_qp_offset_constraint_flag | u(1) |
| no_dep_quant_constraint_flag | u(1) |
| no_sign_data_hiding_constraint_flag | u(1) |
| no_tsrc_constraint_flag | u(1) |
| no_mixed_nalu_types_in_pic_constraint_flag | u(1) |
| no_trail_constraint_flag | u(1) |
| no_stsa_constraint_flag | u(1) |
| no_rasl_constraint_flag | u(1) |
| no_radl_constraint_flag | u(1) |
| no_idr_constraint_flag | u(1) |
| no_cra_constraint_flag | u(1) |
| no_gdr_constraint_flag | u(1) |
| no_aps_constraint_flag | u(1) |
| //put intra related syntax together |  |
| no_qtbtt_dual_tree_intra_constraint_flag | u(1) |
| no_mrl_constraint_flag | u(1) |
| no_isp_constraint_flag | u(1) |
| no_mip_constraint_flag | u(1) |
| no_cclm_constraint_flag | u(1) |
| no_lfnst_constraint_flag | u(1) |
| no_ibc_constraint_flag | u(1) |
| no_bdpcm_constraint_flag | u(1) |
| no_palette_constraint_flag | u(1) |
| //end of intra related syntax |  |
| // put inter related syntax together |  |
| no_ref_wraparound_constraint_flag | u(1) |
| no_temporal_mvp_constraint_flag | u(1) |
| no_sbtmvp_constraint_flag | u(1) |
| no_amvr_constraint_flag | u(1) |
| no_bdof_constraint_flag | u(1) |
| no_dmvr_constraint_flag | u(1) |
| no_sbt_constraint_flag | u(1) |
| no_affine_motion_constraint_flag | u(1) |
| no_mmvd_constraint_flag | u(1) |
| no_smvd_constraint_flag | u(1) |
| no_prof_constraint_flag | u(1) |
| no_bcw_constraint_flag | u(1) |
| no_ciip_constraint_flag | u(1) |
| no_gpm_constraint_flag | u(1) |
| //end of inter related syntax |  |
| while( !byte_aligned( ) ) |  |
| gci_alignment_zero_bit | f(1) |
| gci_num_reserved_bytes | u(8) |
| for( i = 0; i < gci_num_reserved_bytes; i++ ) |  |
| gci_reserved_byte[ i ] | u(8) |
| } |  |

no_sbt_constraint_flag equal to 1 specifies that sps_sbt_enabled_flag shall be equal to 0. no_sbt_constraint_flag equal to 0 does not impose such a constraint. When intra_only_constraint_flag is equal to 1, the value of no_sbt_constraint_flag shall be equal to 1.

# Conclusion

Several cleanups on parameter sets and GCI are proposed. It is recommended to adopt this proposal into next version of VVC.

# Reference

# Reference

- B. Bross, J. Chen, S. Liu, Y.K. Wang, “Versatile Video Coding (Draft 9)” Document of Joint Video Experts Team (JVET), JVET-R2001-vA, Teleconference, 2020.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).