| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0132 |
| --- | --- |

| Title: | AHG9: On syntax signaling order in SPS and PPS |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hong-Jheng Jhu, Yi-Wen Chen, Xiaoyu Xiu, Tsung-Chuan Ma, Xianglin Wang | Email: | jhuhong-jheng@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

This contribution proposes the following modifications to rearrange the syntax elements in the VVC draft specification version 9:

- Group the dual tree related syntax elements together in the sequence parameter set (SPS) to make the specification text cleaner.

- Rearrange the partition constraint related syntax elements in the SPS to be consistent with the ordering of the corresponding syntax elements in the picture header (PH). More specifically, it is proposed to also group such syntaxes in SPS based on whether they are intra or inter related.

- Rearrange the syntax elements related to inter slices in the picture parameter set (PPS) in a similar manner as the grouping of the inter-slice related syntax elements in the PH. More specifically, it is proposed to also group syntax elements in PPS based on whether they are intra or inter related.

# Proposal

In the current draft text of the VVC standard [1], the partition constraint syntax elements signaled in picture header are grouped based on slice type (i.e. intra or inter), which offers better clarity. It is noticed that similar syntax elements signaled in SPS are currently not grouped in a same way. Instead, they are signaled in an interleaved manner. It would be preferable to have them grouped in the similar manner.This contribution proposes the following modifications to rearrange the syntax elements:

- Group the dual tree related syntax elements together in the sequence parameter set (SPS) to make the specification text cleaner.

- Rearrange the partition constraint related syntax elements in the SPS to be consistent with the ordering of the corresponding syntax elements in the picture header (PH). More specifically, it is proposed to also group such syntaxes in SPS based on whether they are intra or inter related.

- Rearrange the syntax elements related to inter slices in the picture parameter set (PPS) in a similar manner as the grouping of the inter-slice related syntaxes in the PH. More specifically, it is proposed to also group syntaxes in PPS based on whether they are intra or inter related.

The changes to the VVC working draft are highlighted below. The added parts are highlighted in yellow while the deleted parts are highlighted in green.

## Group the dual tree related syntax elements together in SPS

7.3.2.3 Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if( ChromaArrayType != 0 ) |  |
| sps_qtbtt_dual_tree_intra_flag | u(1) |
| sps_log2_min_luma_coding_block_size_minus2 | ue(v) |
| if( ChromaArrayType != 0 ) |  |
| … |  |
| if( ChromaArrayType != 0 ) { |  |
| sps_qtbtt_dual_tree_intra_flag | u(1) |
| if( sps_qtbtt_dual_tree_intra_flag ) { |  |
| sps_log2_diff_min_qt_min_cb_intra_slice_chroma | ue(v) |
| sps_max_mtt_hierarchy_depth_intra_slice_chroma | ue(v) |
| if( sps_max_mtt_hierarchy_depth_intra_slice_chroma != 0 ) { |  |
| sps_log2_diff_max_bt_min_qt_intra_slice_chroma | ue(v) |
| sps_log2_diff_max_tt_min_qt_intra_slice_chroma | ue(v) |
| } |  |
| } |  |
| } |  |
| … |  |
| } |  |

## Rearrange the partition constraint related syntax elements in the SPS to be consistent with the ordering of the corresponding syntax elements in PH

7.3.2.3 Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_log2_min_luma_coding_block_size_minus2 | ue(v) |
| sps_partition_constraints_override_enabled_flag | u(1) |
| sps_log2_diff_min_qt_min_cb_intra_slice_luma | ue(v) |
| sps_max_mtt_hierarchy_depth_intra_slice_luma | ue(v) |
| if( sps_max_mtt_hierarchy_depth_intra_slice_luma != 0 ) { |  |
| sps_log2_diff_max_bt_min_qt_intra_slice_luma | ue(v) |
| sps_log2_diff_max_tt_min_qt_intra_slice_luma | ue(v) |
| } |  |
| if(sps_qtbtt_dual_tree_intra_flag ) { |  |
| sps_log2_diff_min_qt_min_cb_intra_slice_chroma | ue(v) |
| sps_max_mtt_hierarchy_depth_intra_slice_chroma | ue(v) |
| if( sps_max_mtt_hierarchy_depth_intra_slice_chroma != 0 ) { |  |
| sps_log2_diff_max_bt_min_qt_intra_slice_chroma | ue(v) |
| sps_log2_diff_max_tt_min_qt_intra_slice_chroma | ue(v) |
| } |  |
| } |  |
| sps_log2_diff_min_qt_min_cb_inter_slice | ue(v) |
| sps_max_mtt_hierarchy_depth_inter_slice | ue(v) |
| if( sps_max_mtt_hierarchy_depth_inter_slice != 0 ) { |  |
| sps_log2_diff_max_bt_min_qt_inter_slice | ue(v) |
| sps_log2_diff_max_tt_min_qt_inter_slice | ue(v) |
| } |  |
| if(sps_qtbtt_dual_tree_intra_flag ) { |  |
| sps_log2_diff_min_qt_min_cb_intra_slice_chroma | ue(v) |
| sps_max_mtt_hierarchy_depth_intra_slice_chroma | ue(v) |
| if( sps_max_mtt_hierarchy_depth_intra_slice_chroma != 0 ) { |  |
| sps_log2_diff_max_bt_min_qt_intra_slice_chroma | ue(v) |
| sps_log2_diff_max_tt_min_qt_intra_slice_chroma | ue(v) |
| } |  |
| } |  |
| … |  |
| } |  |

## Rearrange the syntax elements related to inter slices in the PPS in a similar manner as the grouping of the inter-slice related syntaxes in PH

The changes to the VVC working draft is highlighted below. The added parts are highlighted in yellow while the deleted parts are highlighted in green.

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | u(6) |
| pps_seq_parameter_set_id | u(4) |
| pps_mixed_nalu_types_in_pic_flag | u(1) |
| pps_pic_width_in_luma_samples | ue(v) |
| pps_pic_height_in_luma_samples | ue(v) |
| pps_conformance_window_flag | u(1) |
| if( pps_conformance_window_flag ) { |  |
| pps_conf_win_left_offset | ue(v) |
| pps_conf_win_right_offset | ue(v) |
| pps_conf_win_top_offset | ue(v) |
| pps_conf_win_bottom_offset | ue(v) |
| } |  |
| pps_scaling_window_explicit_signalling_flag | u(1) |
| if( pps_scaling_window_explicit_signalling_flag ) { |  |
| pps_scaling_win_left_offset | se(v) |
| pps_scaling_win_right_offset | se(v) |
| pps_scaling_win_top_offset | se(v) |
| pps_scaling_win_bottom_offset | se(v) |
| } |  |
| pps_output_flag_present_flag | u(1) |
| pps_no_pic_partition_flag | u(1) |
| pps_subpic_id_mapping_present_flag | u(1) |
| if( pps_subpic_id_mapping_present_flag ) { |  |
| if( !pps_no_pic_partition_flag ) |  |
| pps_num_subpics_minus1 | ue(v) |
| pps_subpic_id_len_minus1 | ue(v) |
| for( i = 0; i <= pps_num_subpic_minus1; i++ ) |  |
| pps_subpic_id[ i ] | u(v) |
| } |  |
| if( !pps_no_pic_partition_flag ) { |  |
| pps_log2_ctu_size_minus5 | u(2) |
| pps_num_exp_tile_columns_minus1 | ue(v) |
| pps_num_exp_tile_rows_minus1 | ue(v) |
| for( i = 0; i <= pps_num_exp_tile_columns_minus1; i++ ) |  |
| pps_tile_column_width_minus1[ i ] | ue(v) |
| for( i = 0; i <= pps_num_exp_tile_rows_minus1; i++ ) |  |
| pps_tile_row_height_minus1[ i ] | ue(v) |
| if( NumTilesInPic > 1 ) { |  |
| pps_loop_filter_across_tiles_enabled_flag | u(1) |
| pps_rect_slice_flag | u(1) |
| } |  |
| if( pps_rect_slice_flag ) |  |
| pps_single_slice_per_subpic_flag | u(1) |
| if( pps_rect_slice_flag && !pps_single_slice_per_subpic_flag ) { |  |
| pps_num_slices_in_pic_minus1 | ue(v) |
| if( pps_num_slices_in_pic_minus1 > 1 ) |  |
| pps_tile_idx_delta_present_flag | u(1) |
| for( i = 0; i < pps_num_slices_in_pic_minus1; i++ ) { |  |
| if( SliceTopLeftTileIdx[ i ] % NumTileColumns != NumTileColumns − 1 ) |  |
| pps_slice_width_in_tiles_minus1[ i ] | ue(v) |
| if( SliceTopLeftTileIdx[ i ] / NumTileColumns != NumTileRows − 1 && ( pps_tile_idx_delta_present_flag ¦ ¦ SliceTopLeftTileIdx[ i ] % NumTileColumns = = 0 ) ) |  |
| pps_slice_height_in_tiles_minus1[ i ] | ue(v) |
| if( pps_slice_width_in_tiles_minus1[ i ] = = 0 && pps_slice_height_in_tiles_minus1[ i ] = = 0 && RowHeight[ SliceTopLeftTileIdx[ i ] / NumTileColumns ] > 1 ) { |  |
| pps_num_exp_slices_in_tile[ i ] | ue(v) |
| for( j = 0; j < pps_num_exp_slices_in_tile[ i ]; j++ ) |  |
| pps_exp_slice_height_in_ctus_minus1[ i ][ j ] | ue(v) |
| i += NumSlicesInTile[ i ] − 1 |  |
| } |  |
| if( pps_tile_idx_delta_present_flag && i < pps_num_slices_in_pic_minus1 ) |  |
| pps_tile_idx_delta_val[ i ] | se(v) |
| } |  |
| } |  |
| if( !pps_rect_slice_flag ¦ ¦ pps_single_slice_per_subpic_flag ¦ ¦ pps_num_slices_in_pic_minus1 > 0 ) |  |
| pps_loop_filter_across_slices_enabled_flag | u(1) |
| } |  |
| pps_cabac_init_present_flag | u(1) |
| for( i = 0; i < 2; i++ ) |  |
| pps_num_ref_idx_default_active_minus1[ i ] | ue(v) |
| pps_rpl1_idx_present_flag | u(1) |
| pps_weighted_pred_flag | u(1) |
| pps_weighted_bipred_flag | u(1) |
| pps_ref_wraparound_enabled_flag | u(1) |
| if( pps_ref_wraparound_enabled_flag ) |  |
| pps_pic_width_minus_wraparound_offset | ue(v) |
| pps_init_qp_minus26 | se(v) |
| pps_cu_qp_delta_enabled_flag | u(1) |
| pps_chroma_tool_offsets_present_flag | u(1) |
| if( pps_chroma_tool_offsets_present_flag ) { |  |
| pps_cb_qp_offset | se(v) |
| pps_cr_qp_offset | se(v) |
| pps_joint_cbcr_qp_offset_present_flag | u(1) |
| if( pps_joint_cbcr_qp_offset_present_flag ) |  |
| pps_joint_cbcr_qp_offset_value | se(v) |
| pps_slice_chroma_qp_offsets_present_flag | u(1) |
| pps_cu_chroma_qp_offset_list_enabled_flag | u(1) |
| } |  |
| if( pps_cu_chroma_qp_offset_list_enabled_flag ) { |  |
| pps_chroma_qp_offset_list_len_minus1 | ue(v) |
| for( i = 0; i <= pps_chroma_qp_offset_list_len_minus1; i++ ) { |  |
| pps_cb_qp_offset_list[ i ] | se(v) |
| pps_cr_qp_offset_list[ i ] | se(v) |
| if( pps_joint_cbcr_qp_offset_present_flag ) |  |
| pps_joint_cbcr_qp_offset_list[ i ] | se(v) |
| } |  |
| } |  |
| pps_weighted_pred_flag | u(1) |
| pps_weighted_bipred_flag | u(1) |
| pps_deblocking_filter_control_present_flag | u(1) |
| if( pps_deblocking_filter_control_present_flag ) { |  |
| pps_deblocking_filter_override_enabled_flag | u(1) |
| pps_deblocking_filter_disabled_flag | u(1) |
| if( !pps_no_pic_partition_flag && pps_deblocking_filter_override_enabled_flag ) |  |
| pps_dbf_info_in_ph_flag | u(1) |
| if( !pps_deblocking_filter_disabled_flag ) { |  |
| pps_luma_beta_offset_div2 | se(v) |
| pps_luma_tc_offset_div2 | se(v) |
| if( pps_chroma_tool_offsets_present_flag ) { |  |
| pps_cb_beta_offset_div2 | se(v) |
| pps_cb_tc_offset_div2 | se(v) |
| pps_cr_beta_offset_div2 | se(v) |
| pps_cr_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| } |  |
| if( !pps_no_pic_partition_flag ) { |  |
| pps_rpl_info_in_ph_flag | u(1) |
| pps_sao_info_in_ph_flag | u(1) |
| pps_alf_info_in_ph_flag | u(1) |
| if( ( pps_weighted_pred_flag ¦ ¦ pps_weighted_bipred_flag ) && pps_rpl_info_in_ph_flag ) |  |
| pps_wp_info_in_ph_flag | u(1) |
| pps_qp_delta_info_in_ph_flag | u(1) |
| } |  |
| pps_ref_wraparound_enabled_flag | u(1) |
| if( pps_ref_wraparound_enabled_flag ) |  |
| pps_pic_width_minus_wraparound_offset | ue(v) |
| pps_picture_header_extension_present_flag | u(1) |
| pps_slice_header_extension_present_flag | u(1) |
| pps_extension_flag | u(1) |
| if( pps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| pps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

# Reference

B. Bross, J. Chen, S. Liu and Y.-K. Wang, “Versatile Video Coding (Draft 9)”, JVET-R2001-vA, the 18th JVET meeting, teleconference, 15–24 April 2020.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).