| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0145-v4 |
| --- | --- |

| Title: | AHG12: A summary of proposals on tiles, slices, and subpictures |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang 8910 University Center Lane San Diego, CA 92122, USA | Email: | yekui.wang@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution provides a summary of the 11 proposals on subpictures, slices, and tiles (the agenda item 3.2 in JVET-S0137 and JVET-S0237).

It is suggested that this summary, in terms of a list of proposed items, is used for the reviewing of these proposals, such that the discussions can be in a more structured and efficient manner.

# List of proposed items

On tiles and slices

- Allow the value of pps_no_pic_partition_flag to be different for different PPSs that are referred to by coded pictures within a CLVS. (S0049 aspect 3, S0095 aspect 4)

  - Alternatively, move pps_no_pic_partition_flag from the PPS to the SPS (and rename it to sps_no_pic_partition_flag). (S0095 aspect 4)

- Do either of the following to reduce the current two options for signalling of a tile containing exactly one rectangular slice to one option only: (S0095 aspect 1)

  - Replace pps_num_exp_slices_in_tile[ i ] with pps_num_exp_slices_in_tile_minus1[ i ].

  - Change the upper limit of pps_exp_slice_height_in_ctus_minus1[ i ][ j ] from RowHeight[ SliceTopLeftTileIdx[ i ] / NumTileColumns ] − 1 to RowHeight[ SliceTopLeftTileIdx[ i ] / NumTileColumns ] − 2.

- For a more precise value range, change the upper limit of pps_exp_slice_height_in_ctus_minus1[ i ][ j ] from RowHeight[ SliceTopLeftTileIdx[ i ] / NumTileColumns ] − 1 to RowHeight[ SliceTopLeftTileIdx[ i ] / NumTileColumns ] − pps_num_exp_slices_in_tile[ i ]. (S0095 aspect 2)

- Change the syntax condition for pps_rect_slice_flag from "if( NumTilesInPic > 1 )" to "if( NumTilesInPic > 3 )", because when the number of tiles in a picture is less than 4, the shape of each slice has to be rectangular. (S0095 aspect 3)

- Change the semantics of pps_rect_slice_flag to cover the case that rectangular slice is smaller than the tile. (S0095 aspect 5)

- The inference condition of pps_single_slice_per_subpic_flag is changed from "When not present" to "When the value of pps_no_pic_partition_flag is equal to 1" so that pps_single_slice_per_subpic_flag is inferred only when it is needed. (S0095 aspect 6)

- When pps_pic_width_in_luma_samples is less than or equal to CtbSizeY, pps_num_exp_tile_columns_minus1 and pps_tile_column_width_minus1[ i ] are skipped and inferred. (S0116 aspect 1)

  - Alternatively, when the condition is true, the values of pps_num_exp_tile_columns_minus1 and pps_tile_column_width_minus1[ i ] are both constrained to be equal to 0. (S0116 aspect 1)

- When pps_pic_height_in_luma_samples is less than or equal to CtbSizeY, pps_num_exp_tile_row_minus1 and pps_tile_row_height_minus1[ i ] are skipped and inferred. (S0116 aspect 1)

  - Alternatively, when the condition is true, the values pps_num_exp_tile_row_minus1 and pps_tile_row_height_minus1[ i ] are both constrained to be equal to 0. (S0116 aspect 1)

- When NumTilesInPic is equal to 1 and either pps_pic_width_in_luma_samples or pps_pic_height_in_luma_samples is less than or equal to CtbSizeY, pps_tile_idx_delta_present_flag is skipped and inferred to be equal to 0. (S0116 aspect 2)

On subpictures

- Add a shortcut for signalling of subpictures layout, as follows: (S0071)

  - Add sps_subpic_same_res_flag, equal to 1 specifies that all subpictures have the same width and height, respectively.

  - When sps_subpic_same_res_flag is equal to 1, sps_subpic_width_minus1[ i ] and sps_subpic_height_minus1[ i ] are only signalled when i is equal to 0, and sps_subpic_ctu_top_left_x[ i ] and sps_subpic_ctu_top_left_y[ i ] are skipped for all i values.

- When sps_pic_width_max_in_luma_samples and sps_pic_height_max_in_luma_samples are both less than or equal to CtbSizeY, sps_num_subpics_minus1 is skipped and inferred to be equal to 0. (S0115 aspect 1)

- Merge the two sets of constraints on the combination of subpictures and scalability, in the semantics of sps_subpic_treated_as_pic_flag[ i ] and in the end of clause 8.3.2, respectively, and the constraint that requires the same subpicture ID for subpicture inter prediction referencing within a CLVS, in the PPS semantics, to be places in the end of clause 8.3.2, as follows: (S0125 aspect 1) (removal: italic; addition: bold)

- When vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 0, and sps_num_subpics_minus1 is greater than 0 and the current subpicture with sub-picture index subPicIdx has sps_subpic_treated_as_pic_flag[ subPicIdx ] equal to 1, either of the following two conditions (but not both) shall be true:

  - The picture referred to by each active entry in RefPicList[ 0 ] or RefPicList[ 1 ] shall have the same value of pps_pic_width_in_luma_samples and the same value of pps_pic_height_in_luma_samples as the current picture, shall have has the same subpicture layout as the current picture (i.e., the SPSs referred to by that picture and the current picture have the same value of sps_num_subpics_minus1 and the same values of sps_subpic_ctu_top_left_x[ j ], sps_subpic_ctu_top_left_y[ j ], sps_subpic_width_minus1[ j ], and sps_subpic_height_minus1[ j ], respectively, for each value of j in the range of 0 to sps_num_subpics_minus1, inclusive), and shall have the same value of sub-picture IDs as the current picture (i.e. SubpicIdVal[ j ] for each value of j in the range of 0 to sps_num_subpics_minus1, inclusive).

  - The picture referred to by each active entry in RefPicList[ 0 ] or RefPicList[ 1 ] is an ILRP for which the value of sps_num_subpics_minus1 is equal to 0.

- On top of item 3), further require the same scaling window offsets, as follows: (S0125 aspect 2) (removal: italic; addition: bold)

- When sps_num_subpics_minus1 is greater than 0 and the current subpicture with sub-picture index subPicIdx has sps_subpic_treated_as_pic_flag[ subPicIdx ] equal to 1, either of the following two conditions (but not both) shall be true:

  - The picture referred to by each active entry in RefPicList[ 0 ] or RefPicList[ 1 ] shall have the same value of pps_pic_width_in_luma_samples and the same value of pps_pic_height_in_luma_samples as the current picture, shall have the same scaling windows offsets as the current picture (i.e., the SPSs referred to by that picture and the current picture have the same values of pps_scaling_win_left_offset, pps_scaling_win_right_offset, pps_scaling_win_top_offset, and pps_scaling_win_bottom_offset), shall have the same subpicture layout as the current picture (i.e., the SPSs referred to by that picture and the current picture have the same value of sps_num_subpics_minus1 and the same values of sps_subpic_ctu_top_left_x[ j ], sps_subpic_ctu_top_left_y[ j ], sps_subpic_width_minus1[ j ], and sps_subpic_height_minus1[ j ], respectively, for each value of j in the range of 0 to sps_num_subpics_minus1, inclusive), and shall have the same value of sub-picture IDs as the current picture (i.e. SubpicIdVal[ j ] for each value of j in the range of 0 to sps_num_subpics_minus1, inclusive).

  - The picture referred to by each active entry in RefPicList[ 0 ] or RefPicList[ 1 ] is an ILRP for which the value of sps_num_subpics_minus1 is equal to 0.

New submissions (for the June 10th submission deadline)

- Add the following constraint to the semantics of sps_subpic_treated_as_pic_flag[ ] (S0190 aspect 1):

When sps_num_subpics_minus1 is equal to 0, for each CLVS of a current layer referring to the SPS, let targetAuSet be all the AUs starting from the AU containing the first picture of the CLVS in decoding order, to the AU containing the last picture of the CLVS in decoding order, inclusive, it is a requirement of bitstream conformance that all of the following conditions are true for the targetLayerSet that consists of the current layer and all the layers that have the current layer as a reference layer:

–	All the SPSs referred to by the layers in targetLayerSet shall have the same value of sps_num_subpics_minus1 and shall have the same values of sps_subpic_ctu_top_left_x[ j ], sps_subpic_ctu_top_left_y[ j ], sps_subpic_width_minus1[ j ], sps_subpic_height_minus1[ j ], and sps_subpic_treated_as_pic_flag[ j ], respectively, for each value of j in the range of 0 to sps_num_subpics_minus1, inclusive.

–	For each AU in targetAuSet, all pictures of the layers in targetLayerSet shall have the same value of SubpicIdVal[ j ] for each value of j in the range of 0 to sps_num_subpics_minus1, inclusive.

- Add a constraint: When the value of sps_subpic_info_present_flag of the current layer is equal to 1, the value of sps_res_change_in_clvs_allowed_flag of the reference layers of the current layer shall be equal to 0. (S0190 aspect 2):

- Infer the value of pps_loop_filter_across_tiles_enabled_flag to be equal to 0 (instead of 1) when not present, for consistency with the inferences of sps_loop_filter_across_subpic_enabled_flag[ i ] and pps_loop_filter_across_slices_enabled_flag. (S0160 aspect 1)

- Add a subpicture merging software, which includes the following setup, into the VTM source code package: (S0162)

- Encoding several bitstreams, each with one subpicture per picture, using the VTM encoder.

- Merging selected encoded bitstreams into a bitstream with multiple subpictures, using the subpicture merging software.

- Decoding the bitstream having multiple subpictures, using the VTM decoder.

- Add a flag, sps_raster_scan_order_subpics_flag, to the SPS, the value equal to 1 specifies that subpictures are ordered in raster scan order in each picture in the CLVS, and when this flag is equal to 1, derive the top-left CTU positions of the subpictures instead of signalling them. (S0206)

- Add a constraint: When sps_num_subpics_minus1 is greater than 0 in the reference layer, the inter-layer reference picture (ILRP) shall have the same value of sps_log2_ctu_size_minus5 as that of the SPS referred to by the current picture. (S0236 aspect 1)

- Specify that the value of sps_num_subpics_minus1 shall be less than MaxSlicesPerPicture. (S0236 aspect 2)

- When pps_subpic_id_mapping_present_flag is equal to 1, skip pps_rect_slice_flag and infer it to be equal to 1. (S0236 aspect 3)

# List of proposals

JVET-S0049 AHG9/AHG8/AHG12: On parameter sets and picture header [Y.-K. Wang, L. Zhang, Z. Deng, K. Zhang (Bytedance)]

Aspect 3 of this contribution belongs to this category.

Allow the value of pps_no_pic_partition_flag to be different for different PPSs that are referred to by coded pictures within a CLVS.

JVET-S0071 AHG12: Cleanup of subpicture layout signalling [M. Katsumata, M. Hirabayashi, T. Suzuki (Sony)]

The followings are proposed for a shortcut method of signalling subpicture layout for bit-saving in SPS when all subpictures have same spatial resolution.

- To specify sps_subpic_same_res_flag in SPS whether all subpictures have same spatial resolution.

- When all subpictures have same spatial resolution,

  - to specify common width/height of subpicture indicated by width/height of the 0-th subpicture (sps_subpic_width_minus1[ 0 ] and sps_subpic_height_minus1[ 0 ]),

  - to omit width and height of other subpictures, and

  - to omit position of the i-th subpicture (sps_subpic_ctu_top_left_x[ i ] and sps_subpic_ctu_top_left_y[ i ]).

- To prohibit the use of the shortcut unless the picture is an exact multiple of common width/height of subpictures.

The asserted impact of this proposal on the software is only deriving the omitted value of subpicture layout.

JVET-S0095 AHG12: On tile and slice partitioning related syntax and semantics [J. Chen, Y. Ye, R.-L. Liao (Alibaba)]

This contribution includes six proposals related to tile and slice partitioning as follows

- Proposal 1: it is proposed to remove the signaling redundancy of rectangular slice within a tile. Two options are provided. Option 1 is to replace pps_num_exp_slices_in_tile[ i ] with pps_num_exp_slices_in_tile_minus1[ i ] and option 2 is to change the upper limit of pps_exp_slice_height_in_ctus_minus1[i][j] from RowHeight[ SliceTopLeftTileIdx[ i ] / NumTileColumns ] – 1 to  RowHeight[ SliceTopLeftTileIdx[ i ] / NumTileColumns ] – 2.

- Proposal 2: Considering the fact that the sum of explicitly signaled heights of rectangular slice within a tile should be equal to or less than the height of the tile, a more precise range of pps_exp_slice_height_in_ctus_minus1[ i ][ j ] which is from 0 to RowHeight[ SliceTopLeftTileIdx[ i ] / NumTileColumns ] − pps_num_exp_slices_in_tile[ i ] is proposed.

- Proposal 3: When the number of tiles in a picture is less than 4, the layout of raster-scan slice can be also indicated by rectangular slice mode. Thus, it is proposed to only signal of pps_rect_slice_flag when NumTilesInPic is greater than 3 to reduce unnecessary signaling.

- Proposal 4: Two options are proposed to change the constraint that the value of pps_no_pic_partition_flag shall be the same within a CLVS. Option 1 is to signal sps_no_pic_partition_flag in SPS instead of signaling pps_no_pic_partition_flag in PPS since the value of the flag is already restricted to be constant in a CLVS; option 2 is to remove this constraint, since even with this constraint the mixture of pictures with tile/slice partitioning and pictures without tile/slice partitioning within a CLVS is still allowed.

- Proposal 5: The semantics of pps_rect_slice_flag is fixed to cover the case that rectangular slice is smaller than the tile.

- Proposal 6: The inference condition of pps_single_slice_per_subpic_flag is changed from “not present” to “the value of pps_no_pic_partition_flag is equal to 1” so that pps_single_slice_per_subpic_flag is inferred only when it is needed.

JVET-S0115 AHG9: On SPS cleanups [Y. He, V. Seregin, M. Coban, M. Karczewicz (Qualcomm)]

Aspect 1 of this contribution belongs to this category.

When sps_pic_width_max_in_luma_samples and sps_pic_height_max_in_luma_samples are both less than or equal to CtbSizeY, sps_num_subpics_minus1 is skipped and inferred to be equal to 0.

JVET-S0116 AHG9: On PPS cleanups [Y. He, V. Seregin, M. Coban, M. Karczewicz (Qualcomm)]

This contribution proposes the following cleanup changes on PPS:

- Signaling condition on pps_num_exp_tile_columns_minus1 and pps_num_exp_tile_rows_minus1

- Signaling condition on pps_tile_idx_delta_present_flag

JVET-S0125 AHG9: On sub-picture constraints [V. Seregin, Y. He, Y.-J. Chang, M. Coban, M. Karczewicz (Qualcomm)]

There are multiple constraints applied for sub-pictures, some constraints are applied for sub-pictures treated as pictures, some other constraints are applied regardless of sps_subpic_treated_as_pic_flag value. In the latter case, the constraints are more restrictive.

- This contribution proposes to add a condition sps_subpic_treated_as_pic_flag[ i ] equal to 1 for subpicture constraints.

- It also proposes to add a constraint that when sps_num_subpics_minus1 is greater than 0 and sps_subpic_treated_as_pic_flag[ i ] is equal to 1, the scaling window offsets of the current layer and the scaling window offsets of the layers that have the current layer as a reference shall be the same.

JVET-S0190 AHG8/AHG9/AHG12: On reference picture resampling with scalability [B. Choi, S. Wenger, S. Liu (Tencent)]

This contribution proposes the following changes, related to the combination of reference picture resampling and subpicture with multiple layers:

- Add constraints to the semantics of sps_subpic_treated_as_pic_flag[ ], so that the pictures in the dependent layers have the same picture size and the same subpicture layout, when the reference layer is not partitioned into subpictures while the dependent layer referring the reference layer are partitioned into multiple subpictures.

- Add the following constraint that, when the value of sps_subpic_info_present_flag of the current layer is equal to 1, the value of sps_res_change_in_clvs_allowed_flag of the reference layers of the current layer shall be equal to 0.

JVET-S0160 AHG9/AHG12/AHG8: Miscellaneous HLS cleanups [Y.-K. Wang, Z. Deng, L. Zhang, K. Zhang (Bytedance)]

Aspect 1 of this contribution belongs to this category.

This contribution proposes the following miscellaneous HLS changes:

- Infer the value of pps_loop_filter_across_tiles_enabled_flag to be equal to 0 (instead of 1) when not present, for consistency with the inferences of sps_loop_filter_across_subpic_enabled_flag[ i ] and pps_loop_filter_across_slices_enabled_flag.

JVET-S0162 AHG3/AHG12: Subpicture merging software [A. Hallapuro, M. M. Hannuksela (Nokia)]

This contribution includes the source code for a subpicture merging software and proposes adding the software in the VTM source code package. The software enables the following setup:

- Encoding several bitstreams, each with one subpicture per picture, using the VTM encoder.

- Merging selected encoded bitstreams into a bitstream with multiple subpictures, using the subpicture merging software.

- Decoding the bitstream having multiple subpictures, using the VTM decoder.

JVET-S0206 AHG12: Raster scan order subpictures [M. Damghanian, D. Liu, R. Sjöberg, M. Pettersson, J. Enhorn, J. Ström, R. Yu (Ericsson)]

This contribution proposes the following:

- To add a flag, sps_raster_scan_order_subpics_flag, to the SPS, where sps_raster_scan_order_subpics_flag equal to 1 specifies that subpictures are ordered in raster scan order in each picture in the CLVS.

- To derive and not signal the top-left CTU position of the subpictures when sps_raster_scan_order_subpics_flag is equal to 1.

For use cases such as OMAF or 360 video, the proposed changes are claimed to reduce 50% of the bit cost for subpicture layout signalling.

Specification text on top of JVET-S0152-v5 and the software implementation are provided with the contribution.

JVET-S0236 AHG9/AHG12: High-level syntax cleanups on subpictures [S.-T. Hsiang, L. Chen, O. Chubach, Y.-W. Huang, S.-M. Lei (MediaTek)]

In this contribution, following modifications related to subpictures are proposed:

1	When sps_num_subpics_minus1 is greater than 0 in the reference layer, the inter-layer reference picture (ILRP) shall have the same value of sps_log2_ctu_size_minus5 as that of the SPS referred to by the current picture.

2	The value of sps_num_subpics_minus1 shall not be greater than MaxSlicesPerPicture − 1.

3	The presence of pps_rect_slice_flag is further conditioned on pps_subpic_id_mapping_present_flag.