| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0156-v2 |
| --- | --- |

| Title: | AHG9: On level definitions and bitstream conformance |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang1, Zhipin Deng2 (Bytedance) 18910 University Center Lane San Diego, CA 92122, USA 2Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China | Tel: Email: | yekui.wang@bytedance.com zhipin.deng@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution proposes the following changes related to the specifications of level definitions and bitstream conformance:

- In clause A.4.2 (Profile-specific level limits), change the definitions of the two level limits on the relationship between the CPB removal times for AU 0 and for AU n ( n > 0 ) and the number of slices i.e., items c and d, from being based on MaxSlicesPerPicture and the maximum picture size to be based on MaxSlicesPerPicture * ( the number of pictures in the AU ) and the maximum AU size.

- In clause A.4.2 (Profile-specific level limits), change the definitions of the two level limits on the relationship between the CPB removal times for AU 0 and for AU n ( n > 0 ) and the number of tiles i.e., items i and j, from being based on MaxTileCols * MaxTileRows and the maximum AU size to be based on MaxTileCols * MaxTileRows * ( the number of pictures in the AU ) and the maximum AU size.

- In clause C.4 (Bitstream conformance), the 6th constraint, change "CpbRemovalTime[ n ] less than CpbRemovalTime[ currPic ]" to "DpbOutputTime[ n ] greater than DpbOutputTime[ currPic ]". This is asserted to be a bug, because all decoded pictures in the DPB are always decoded earlier than decoding of the current picture, and thus CpbRemovalTime[ n ] in the context is always less than CpbRemovalTime[ currPic ]. This is also asserted to be a bug in the latest HEVC spec.

Version 2 of this contribution adds an alternative proposed text for items 1 and 2, after an offline discussion with Minhua Zhou, whom the authors really appreciate for the discussion.

# Introduction

The following issues exist in the specifications of level definitions and bitstream conformance:

- The definitions of the two level limits on the relationship between the CPB removal times for AU 0 and for AU n ( n > 0 ) and the number of slices, i.e., items c and d in clause A.4.2 (Profile-specific level limits), are based on MaxSlicesPerPicture and the maximum picture size. However, MaxSlicesPerPicture is defined as a picture-level limit, while the CPB removal times are AU-level parameters.

To match between them, either MaxSlicesPerPicture should be replaced with MaxSlicesPerAu (as suggested for further study by the editor's notes) or change these two items to be based on MaxSlicesPerPicture * ( the number of pictures in the AU ) and the maximum AU size.

This contribution proposes to take the latter approach, which seems easier.

- The definitions of the two level limits on the relationship between the CPB removal times for AU 0 and for AU n ( n > 0 ) and the number of tiles, i.e., items i and j in clause A.4.2 (Profile-specific level limits), are based on MaxTileCols * MaxTileRows and the maximum AU size. However, similarly as above, MaxTileCols and MaxTileRows are defined as a picture-level limits, while the CPB removal times are AU-level parameters.

To match between them, either redefine MaxTileCols and MaxTileRows to be AU-level limits (as suggested for further study by the editor's notes) or change these two items to be based on MaxTileCols * MaxTileRows * ( the number of pictures in the AU ) and the maximum AU size.

Again, this contribution proposes to take the latter approach, which seems easier.

- The 6th constraint in cause C.4 (Bitstream conformance) is as follows:

For each current picture, after invocation of the process for removal of pictures from the DPB as specified in clause C.3.2, the number of decoded pictures in the DPB, including all pictures n that are marked as "used for reference", or that have PictureOutputFlag equal to 1 and CpbRemovalTime[ n ] less than CpbRemovalTime[ currPic ], where currPic is the current picture, shall be less than or equal to max_dec_pic_buffering_minus1[ Htid ].

The part "CpbRemovalTime[ n ] less than CpbRemovalTime[ currPic ]" describes a condition that the decoding time of a decoded picture in the DPB is less than the decoding time of the current picture. However, all decoded pictures in the DPB are always decoded earlier than decoding of the current picture and thus CpbRemovalTime[ n ] in the context is always less than CpbRemovalTime[ currPic ].

The intent should be to specify a condition that a decoded picture in the DPB has output time greater than the decoding time of current picture, as when decoding of the current picture, only decoded pictures that are needed for reference or still need to be output stay in the DPB, other earlier decoded pictures would be removed from the DPB. Therefore, the constraint is changed to reflect such an intent.

It should be noted that this issue also exists in the latest HEVC specification.

# Proposal

The proposed text changes are as follows, where added parts are highlighted in turquoise, and deleted parts are highlighted in yellow strikethrough in red fonts:

#### A.4.2	Profile-specific level limits

The following is specified for expressing the constraints in this annex:

- Let the variable fR be set equal to 1  300.

The variable HbrFactor is defined as follows:

- If the bitstream is indicated to conform to the Main 10 profile or the Main 4:4:4 10 profile, HbrFactor is set equal to 1.

The variable BrVclFactor, which represents the VCL bit rate scale factor, is set equal to CpbVclFactor * HbrFactor.

The variable BrNalFactor, which represents the NAL bit rate scale factor, is set equal to CpbNalFactor * HbrFactor.

The variable MinCr is set equal to MinCrBase * MinCrScaleFactor ÷ HbrFactor.

When the specified level is not level 15.5, the value of max_dec_pic_buffering_minus1[ Htid ] + 1 shall be less than or equal to MaxDpbSize, which is derived as follows:

if( PicSizeMaxInSamplesY  <=  ( MaxLumaPs  >>  2 ) )
	MaxDpbSize = Min( 4 * maxDpbPicBuf, 16 )
else if( PicSizeMaxInSamplesY  <=  ( MaxLumaPs  >>  1 ) )
	MaxDpbSize = Min( 2 * maxDpbPicBuf, 16 )	(A.1)
else if( PicSizeMaxInSamplesY  <=  ( ( 3 * MaxLumaPs )  >>  2 ) )
	MaxDpbSize = Min( ( 4 * maxDpbPicBuf ) / 3, 16 )
else
	MaxDpbSize = maxDpbPicBuf

where MaxLumaPs is specified in Table A.1, maxDpbPicBuf is equal to 8, and max_dec_pic_buffering_minus1[ Htid ] is found in or derived from the applicable dpb_parameters( ) syntax structure.

Let numPics[ n ]numDecPics be the number of pictures in AU n. The variable AuSizeMaxInSamplesY[ n ] is set equal to PicSizeMaxInSamplesY * numPics[ n ]numDecPics.

Bitstreams conforming to the Main 10 or Main 4:4:4 10 profile at a specified tier and level shall obey the following constraints for each bitstream conformance test as specified in Annex C:

- The nominal removal time of AU n (with n greater than 0) from the CPB, as specified in clause C.2.3, shall satisfy the constraint that AuNominalRemovalTime[ n ] − AuCpbRemovalTime[ n − 1 ] is greater than or equal to Max( AuSizeMaxInSamplesY[ n − 1 ]  MaxLumaSr, fR ), where MaxLumaSr is the value specified in Table A.2 that applies to AU n − 1.

- The difference between consecutive output times of pictures of different AUs from the DPB, as specified in clause C.3.3, shall satisfy the constraint that DpbOutputInterval[ n ] is greater than or equal to Max( AuSizeMaxInSamplesY[ n ]  MaxLumaSr, fR ), where MaxLumaSr is the value specified in Table A.2 for AU n, provided that AU n has a picture that is output and AU n is not the last AU of the bitstream that has a picture that is output.

- The removal time of AU 0 shall satisfy the constraint that the number of slices in each picture in AU 0 is less than or equal to Min( Max( 1, MaxSlicesPerPicture * numPics[ 0 ] * MaxLumaSr / MaxLumaPs * ( AuCpbRemovalTime[ 0 ] − AuNominalRemovalTime[ 0 ] ) + MaxSlicesPerPicture * numPics[ 0 ] * AuSizeMaxInSamplesY[ 0 ] PicSizeMaxInSamplesY / MaxLumaPs ), MaxSlicesPerPicture * numPics[ 0 ] ), for the value of PicSizeMaxInSamplesY of picture 0, where MaxSlicesPerPicture, MaxLumaPs and MaxLumaSr are the values specified in Table A.1 and Table A.2, respectively, that apply to AU 0. [Ed (YK). Further study on whether MaxSlicesPerPicture used herein should be specified as MaxSlicesPerAu, as the other variables involved in the calculation are all at the AU level.]

- The difference between consecutive CPB removal times of AUs n and n − 1 (with n greater than 0) shall satisfy the constraint that the number of slices in each picture in AU n is less than or equal to Min( (Max( 1, MaxSlicesPerPicture * numPics[ n ] * MaxLumaSr / MaxLumaPs * ( AuCpbRemovalTime[ n ] − AuCpbRemovalTime[ n − 1 ] ) ), MaxSlicesPerPicture * numPics[ n ] ), where MaxSlicesPerPicture, MaxLumaPs and MaxLumaSr are the values specified in Table A.1 and Table A.2 that apply to AU n. [Ed (YK). Further study on whether MaxSlicesPerPicture used herein should be specified as MaxSlicesPerAu, as the other variables involved in the calculation are all at the AU level.]

- For the VCL HRD parameters, BitRate[Htid][ i ] shall be less than or equal to BrVclFactor * MaxBR for at least one value of i in the range of 0 to hrd_cpb_cnt_minus1, inclusive, where BitRate[Htid][ i ] is specified in clause 7.4.6.3 based on parameters selected as specified in clause C.1 and MaxBR is specified in Table A.2 in units of BrVclFactor bits/s.

- For the NAL HRD parameters, BitRate[Htid][ i ] shall be less than or equal to BrNalFactor * MaxBR for at least one value of i in the range of 0 to hrd_cpb_cnt_minus1, inclusive, where BitRate[Htid][ i ] is specified in clause 7.4.6.3 based on parameters selected as specified in clause C.1 and MaxBR is specified in Table A.2 in units of BrNalFactor bits/s.

- The sum of the NumBytesInNalUnit variables for AU 0 shall be less than or equal to FormatCapabilityFactor * ( Max( AuSizeMaxInSamplesY[ 0 ], fR * MaxLumaSr ) + MaxLumaSr * ( AuCpbRemovalTime[ 0 ] − AuNominalRemovalTime[ 0 ] ) ) ÷ MinCr, where MaxLumaSr and FormatCapabilityFactor are the values specified in Table A.2 and Table A.3, respectively, that apply to AU 0.

- The sum of the NumBytesInNalUnit variables for AU n (with n greater than 0) shall be less than or equal to FormatCapabilityFactor * MaxLumaSr * ( AuCpbRemovalTime[ n ] − AuCpbRemovalTime[ n − 1 ] ) ÷ MinCr, where MaxLumaSr and FormatCapabilityFactor are the values specified in Table A.2 and Table A.3 respectively, that apply to AU n.

- The removal time of AU 0 shall satisfy the constraint that the number of tiles in each picture in AU 0 is less than or equal to Min( Max( 1, MaxTileCols * MaxTileRows * numPics[ 0 ] * 120 * ( AuCpbRemovalTime[ 0 ] − AuNominalRemovalTime[ 0 ] ) + MaxTileCols * MaxTileRows * numPics[ 0 ] * AuSizeMaxInSamplesY[ 0 ] / MaxLumaPs ), MaxTileCols * MaxTileRows * numPics[ 0 ] ), where MaxTileCols and MaxTileRows are the values specified in Table A.1 that apply to AU 0. [Ed (YK). Further study on whether MaxTileCols and MaxTileRows used herein should be specified as for each AU instead of for each picture, as the other variables involved in the calculation are all at the AU level.]

- The difference between consecutive CPB removal times of AUs n and n − 1 (with n greater than 0) shall satisfy the constraint that the number of tiles in each picture in AU n is less than or equal to Min( Max( 1, MaxTileCols * MaxTileRows * numPics[ n ] * 120 * ( AuCpbRemovalTime[ n ] − AuCpbRemovalTime[ n − 1 ] ) ), MaxTileCols * MaxTileRows * numPics[ n ] ), where MaxTileCols and MaxTileRows are the values specified in Table A.1 that apply to AU n. [Ed (YK). Further study on whether MaxTileCols and MaxTileRows used herein should be specified as for each AU instead of for each picture, as the other variables involved in the calculation are all at the AU level.]

Table A.2 – Tier and level limits for the video profiles

| Level | Max luma sample rate MaxLumaSr (samples/sec) | Max bit rate MaxBR (BrVclFactor or BrNalFactor bits/s) |  | Min compression ratio MinCrBase |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Main tier | High tier | Main tier | High tier |
| 1 | 552 960 | 128 | - | 2 | 2 |
| 2 | 3 686 400 | 1 500 | - | 2 | 2 |
| 2.1 | 7 372 800 | 3 000 | - | 2 | 2 |
| 3 | 16 588 800 | 6 000 | - | 2 | 2 |
| 3.1 | 33 177 600 | 10 000 | - | 2 | 2 |
| 4 | 66 846 720 | 12 000 | 30 000 | 4 | 4 |
| 4.1 | 133 693 440 | 20 000 | 50 000 | 4 | 4 |
| 5 | 267 386 880 | 25 000 | 100 000 | 6 | 4 |
| 5.1 | 534 773 760 | 40 000 | 160 000 | 8 | 4 |
| 5.2 | 1 069 547 520 | 60 000 | 240 000 | 8 | 4 |
| 6 | 1 069 547 520 | 60 000 | 240 000 | 8 | 4 |
| 6.1 | 2 139 095 040 | 120 000 | 480 000 | 8 | 4 |
| 6.2 | 4 278 190 080 | 240 000 | 800 000 | 8 | 4 |

Table A.3 – Specification of CpbVclFactor, CpbNalFactor, FormatCapabilityFactor and MinCrScaleFactor

| Profile | CpbVclFactor | CpbNalFactor | FormatCapabilityFactor | MinCrScaleFactor |
| --- | --- | --- | --- | --- |
| Main 10 | 1 000 | 1 100 | 1.875 | 1.00 |
| Main 4:4:4 10 | 2 500 | 2 750 | 3.750 | 0.75 |
| Main 10 Still Picture | 1 000 | 1 100 | 1.875 | 1.00 |
| Main 4:4:4 10 Still Picture | 2 500 | 2 750 | 3.750 | 0.75 |

Informative clause A.4.3 shows the effect of these limits on picture rates for several example picture formats.

#### C.4	Bitstream conformance

...

- For each current picture, after invocation of the process for removal of pictures from the DPB as specified in clause C.3.2, the number of decoded pictures in the DPB, including all pictures n that are marked as "used for reference", or have PictureOutputFlag equal to 1 and DpbOutputTime[ n ] greater than CpbRemovalTime[ currPic ] that have PictureOutputFlag equal to 1 and CpbRemovalTime[ n ] less than CpbRemovalTime[ currPic ], where currPic is the current picture, shall be less than or equal to max_dec_pic_buffering_minus1[ Htid ].

...

# An alternative proposal for items 1 and 2

In below, for convenience, changes marks are kept only for the proposed changes in this alternative proposal.

#### 7.4.3.4	Picture parameter set RBSP semantics

...

pps_num_slices_in_pic_minus1 plus 1 specifies the number of rectangular slices in each picture referring to the PPS. The value of pps_num_slices_in_pic_minus1 shall be in the range of 0 to MaxSlicesPerAu − 1, inclusive, where MaxSlicesPerAu is specified in Annex A. When pps_no_pic_partition_flag is equal to 1, the value of pps_num_slices_in_pic_minus1 is inferred to be equal to 0. When pps_single_slice_per_subpic_flag is equal to 1, the value of pps_num_slices_in_pic_minus1 is inferred to be equal to sps_num_subpics_minus1.

...

#### A.4.1	General tier and level limits

For purposes of comparison of tier capabilities, the tier with general_tier_flag equal to 0 (i.e., the Main tier) is considered to be a lower tier than the tier with general_tier_flag equal to 1 (i.e., the High tier).

For purposes of comparison of level capabilities, a particular level of a specific tier is considered to be a lower level than some other level of the same tier when the value of the general_level_idc or sublayer_level_idc[ i ] of the particular level is less than that of the other level.

The following is specified for expressing the constraints in this annex:

- Let AU n be the n-th AU in decoding order, with the first AU being AU 0 (i.e., the 0-th AU).

- For an OLS with OLS index TargetOlsIdx, the variables PicWidthMaxInSamplesY, PicHeightMaxInSamplesY, and PicSizeMaxInSamplesY, and the applicable dpb_parameters( ) syntax structure are derived as follows:

  - If NumLayersInOls[ TargetOlsIdx ] is equal to 1, PicWidthMaxInSamplesY is set equal to sps_pic_width_max_in_luma_samples, PicHeightMaxInSamplesY is set equal to sps_pic_height_max_in_luma_samples, and PicSizeMaxInSamplesY is set equal to PicWidthMaxInSamplesY * PicHeightMaxInSamplesY, where  sps_pic_width_max_in_luma_samples and sps_pic_height_max_in_luma_samples are found in the SPS referred to by the layer in the OLS, and the applicable dpb_parameters( ) syntax structure is also found in that SPS.

  - Otherwise (NumLayersInOls[ TargetOlsIdx ] is greater than 1), PicWidthMaxInSamplesY is set equal to vps_ols_dpb_pic_width[ MultiLayerOlsIdx[ TargetOlsIdx ] ], PicHeightMaxInSamplesY is set equal to vps_ols_dpb_pic_height[ MultiLayerOlsIdx[ TargetOlsIdx ] ], PicSizeMaxInSamplesY is set equal to PicWidthMaxInSamplesY * PicHeightMaxInSamplesY, and the applicable dpb_parameters( ) syntax structure is identified by vps_ols_dpb_params_idx[ MultiLayerOlsIdx[ TargetOlsIdx ] ] found in the VPS.

When the specified level is not level 15.5, bitstreams conforming to a profile at a specified tier and level shall obey the following constraints for each bitstream conformance test as specified in Annex C:

- PicSizeMaxInSamplesY shall be less than or equal to MaxLumaPs, where MaxLumaPs is specified in Table A.1.

- The value of PicWidthMaxInSamplesY shall be less than or equal to Sqrt( MaxLumaPs * 8 ).

- The value of PicHeightMaxInSamplesY shall be less than or equal to Sqrt( MaxLumaPs * 8 ).

- For each referenced PPS, the value of NumTileColumns shall be less than or equal to MaxTileCols and the value of NumTilesInPic shall be less than or equal to MaxTilesPerAu, where MaxTileCols and MaxTilesPerAu are specified in Table A.1.

- For the VCL HRD parameters, CpbSize[Htid][ i ] shall be less than or equal to CpbVclFactor * MaxCPB for at least one value of i in the range of 0 to hrd_cpb_cnt_minus1, inclusive, where CpbSize[Htid][ i ] is specified in clause 7.4.6.3 based on parameters selected as specified in clause C.1, CpbVclFactor is specified in Table A.3 and MaxCPB is specified in Table A.1 in units of CpbVclFactor bits.

- For the NAL HRD parameters, CpbSize[Htid][ i ] shall be less than or equal to CpbNalFactor * MaxCPB for at least one value of i in the range of 0 to hrd_cpb_cnt_minus1, inclusive, where CpbSize[Htid][ i ] is specified in clause 7.4.6.3 based on parameters selected as specified in clause C.1, CpbNalFactor is specified in Table A.3, and MaxCPB is specified in Table A.1 in units of CpbNalFactor bits.

Table A.1 specifies the limits for each level of each tier for levels other than level 15.5.

A tier and level to which a bitstream conforms are indicated by the syntax elements general_tier_flag and general_level_idc, and a level to which a sublayer representation conforms are indicated by the syntax element sublayer_level_idc[ i ], as follows:

–	If the specified level is not level 15.5, general_tier_flag equal to 0 indicates conformance to the Main tier, general_tier_flag equal to 1 indicates conformance to the High tier, according to the tier constraints specified in Table A.1 and general_tier_flag shall be equal to 0 for levels below level 4 (corresponding to the entries in Table A.1 marked with "-"). Otherwise (the specified level is level 15.5), it is a requirement of bitstream conformance that general_tier_flag shall be equal to 1 and the value 0 for general_tier_flag is reserved for future use by ITU-T | ISO/IEC and decoders shall ignore the value of general_tier_flag.

–	general_level_idc and sublayer_level_idc[ i ] shall be set equal to a value of general_level_idc for the level number specified in Table A.1.

Table A.1 – General tier and level limits

| Level | general_level_idc value* | Max luma picture size MaxLumaPs (samples) | Max CPB size MaxCPB (CpbVclFactor or CpbNalFactor bits) |  | Max slices per AU MaxSlicesPerAu | Max # of tiles MaxTilesPerAu | Max # of tile columns MaxTileCols |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Main tier | High tier |  |  |  |
| 1.0 | 16 | 36 864 | 350 | - | 16 | 1 | 1 |
| 2.0 | 32 | 122 880 | 1 500 | - | 16 | 1 | 1 |
| 2.1 | 35 | 245 760 | 3 000 | - | 20 | 1 | 1 |
| 3.0 | 48 | 552 960 | 6 000 | - | 30 | 4 | 2 |
| 3.1 | 51 | 983 040 | 10 000 | - | 40 | 9 | 3 |
| 4.0 | 64 | 2 228 224 | 12 000 | 30 000 | 75 | 25 | 5 |
| 4.1 | 67 | 2 228 224 | 20 000 | 50 000 | 75 | 25 | 5 |
| 5.0 | 80 | 8 912 896 | 25 000 | 100 000 | 200 | 110 | 10 |
| 5.1 | 83 | 8 912 896 | 40 000 | 160 000 | 200 | 110 | 10 |
| 5.2 | 86 | 8 912 896 | 60 000 | 240 000 | 200 | 110 | 10 |
| 6.0 | 96 | 35 651 584 | 80 000 | 240 000 | 600 | 440 | 20 |
| 6.1 | 99 | 35 651 584 | 120 000 | 480 000 | 600 | 440 | 20 |
| 6.2 | 102 | 35 651 584 | 180 000 | 800 000 | 600 | 440 | 20 |
| *For the level number in the form of major.minor, the value of general_level_idc for each of the above-lisetd levels is equal to major * 16 + minor * 3. |  |  |  |  |  |  |  |

#### A.4.2	Profile-specific level limits

The following is specified for expressing the constraints in this annex:

- Let the variable fR be set equal to 1  300.

The variable HbrFactor is defined as follows:

- If the bitstream is indicated to conform to the Main 10 profile or the Main 4:4:4 10 profile, HbrFactor is set equal to 1.

The variable BrVclFactor, which represents the VCL bit rate scale factor, is set equal to CpbVclFactor * HbrFactor.

The variable BrNalFactor, which represents the NAL bit rate scale factor, is set equal to CpbNalFactor * HbrFactor.

The variable MinCr is set equal to MinCrBase * MinCrScaleFactor ÷ HbrFactor.

When the specified level is not level 15.5, the value of max_dec_pic_buffering_minus1[ Htid ] + 1 shall be less than or equal to MaxDpbSize, which is derived as follows:

if( PicSizeMaxInSamplesY  <=  ( MaxLumaPs  >>  2 ) )
	MaxDpbSize = Min( 4 * maxDpbPicBuf, 16 )
else if( PicSizeMaxInSamplesY  <=  ( MaxLumaPs  >>  1 ) )
	MaxDpbSize = Min( 2 * maxDpbPicBuf, 16 )	(A.1)
else if( PicSizeMaxInSamplesY  <=  ( ( 3 * MaxLumaPs )  >>  2 ) )
	MaxDpbSize = Min( ( 4 * maxDpbPicBuf ) / 3, 16 )
else
	MaxDpbSize = maxDpbPicBuf

where MaxLumaPs is specified in Table A.1, maxDpbPicBuf is equal to 8, and max_dec_pic_buffering_minus1[ Htid ] is found in or derived from the applicable dpb_parameters( ) syntax structure.

Let numDecPics be the number of pictures in AU n. The variable AuSizeMaxInSamplesY[ n ] is set equal to PicSizeMaxInSamplesY * numDecPics.

Bitstreams conforming to the Main 10 or Main 4:4:4 10 profile at a specified tier and level shall obey the following constraints for each bitstream conformance test as specified in Annex C:

- The nominal removal time of AU n (with n greater than 0) from the CPB, as specified in clause C.2.3, shall satisfy the constraint that AuNominalRemovalTime[ n ] − AuCpbRemovalTime[ n − 1 ] is greater than or equal to Max( AuSizeMaxInSamplesY[ n − 1 ]  MaxLumaSr, fR ), where MaxLumaSr is the value specified in Table A.2 that applies to AU n − 1.

- The difference between consecutive output times of pictures of different AUs from the DPB, as specified in clause C.3.3, shall satisfy the constraint that DpbOutputInterval[ n ] is greater than or equal to Max( AuSizeMaxInSamplesY[ n ]  MaxLumaSr, fR ), where MaxLumaSr is the value specified in Table A.2 for AU n, provided that AU n has a picture that is output and AU n is not the last AU of the bitstream that has a picture that is output.

- The removal time of AU 0 shall satisfy the constraint that the number of slices in AU 0 is less than or equal to Min( Max( 1, MaxSlicesPerAu * MaxLumaSr / MaxLumaPs * ( AuCpbRemovalTime[ 0 ] − AuNominalRemovalTime[ 0 ] ) + MaxSlicesPerAu * AuSizeMaxInSamplesY[ 0 ] / MaxLumaPs ), MaxSlicesPerAu ), for the value of PicSizeMaxInSamplesY of picture 0, where MaxSlicesPerAu, MaxLumaPs and MaxLumaSr are the values specified in Table A.1 and Table A.2, respectively, that apply to AU 0.

- The difference between consecutive CPB removal times of AUs n and n − 1 (with n greater than 0) shall satisfy the constraint that the number of slices in AU n is less than or equal to Min( (Max( 1, MaxSlicesPerAu * MaxLumaSr / MaxLumaPs * ( AuCpbRemovalTime[ n ] − AuCpbRemovalTime[ n − 1 ] ) ), MaxSlicesPerAu ), where MaxSlicesPerAu, MaxLumaPs and MaxLumaSr are the values specified in Table A.1 and Table A.2 that apply to AU n.

- For the VCL HRD parameters, BitRate[Htid][ i ] shall be less than or equal to BrVclFactor * MaxBR for at least one value of i in the range of 0 to hrd_cpb_cnt_minus1, inclusive, where BitRate[Htid][ i ] is specified in clause 7.4.6.3 based on parameters selected as specified in clause C.1 and MaxBR is specified in Table A.2 in units of BrVclFactor bits/s.

- For the NAL HRD parameters, BitRate[Htid][ i ] shall be less than or equal to BrNalFactor * MaxBR for at least one value of i in the range of 0 to hrd_cpb_cnt_minus1, inclusive, where BitRate[Htid][ i ] is specified in clause 7.4.6.3 based on parameters selected as specified in clause C.1 and MaxBR is specified in Table A.2 in units of BrNalFactor bits/s.

- The sum of the NumBytesInNalUnit variables for AU 0 shall be less than or equal to FormatCapabilityFactor * ( Max( AuSizeMaxInSamplesY[ 0 ], fR * MaxLumaSr ) + MaxLumaSr * ( AuCpbRemovalTime[ 0 ] − AuNominalRemovalTime[ 0 ] ) ) ÷ MinCr, where MaxLumaSr and FormatCapabilityFactor are the values specified in Table A.2 and Table A.3, respectively, that apply to AU 0.

- The sum of the NumBytesInNalUnit variables for AU n (with n greater than 0) shall be less than or equal to FormatCapabilityFactor * MaxLumaSr * ( AuCpbRemovalTime[ n ] − AuCpbRemovalTime[ n − 1 ] ) ÷ MinCr, where MaxLumaSr and FormatCapabilityFactor are the values specified in Table A.2 and Table A.3 respectively, that apply to AU n.

- The removal time of AU 0 shall satisfy the constraint that the number of tiles in AU 0 is less than or equal to Min( Max( 1, MaxTilesPerAu * 120 * ( AuCpbRemovalTime[ 0 ] − AuNominalRemovalTime[ 0 ] ) + MaxTilesPerAu * AuSizeMaxInSamplesY[ 0 ] / MaxLumaPs ), MaxTilesPerAu ), where MaxTilesPerAu is the value specified in Table A.1 that applies to AU 0.

- The difference between consecutive CPB removal times of AUs n and n − 1 (with n greater than 0) shall satisfy the constraint that the number of tiles in AU n is less than or equal to Min( Max( 1, MaxTilesPerAu * 120 * ( AuCpbRemovalTime[ n ] − AuCpbRemovalTime[ n − 1 ] ) ), MaxTilesPerAu ), where MaxTilesPerAu is the value specified in Table A.1 that applies to AU n.

Table A.2 – Tier and level limits for the video profiles

| Level | Max luma sample rate MaxLumaSr (samples/sec) | Max bit rate MaxBR (BrVclFactor or BrNalFactor bits/s) |  | Min compression ratio MinCrBase |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Main tier | High tier | Main tier | High tier |
| 1 | 552 960 | 128 | - | 2 | 2 |
| 2 | 3 686 400 | 1 500 | - | 2 | 2 |
| 2.1 | 7 372 800 | 3 000 | - | 2 | 2 |
| 3 | 16 588 800 | 6 000 | - | 2 | 2 |
| 3.1 | 33 177 600 | 10 000 | - | 2 | 2 |
| 4 | 66 846 720 | 12 000 | 30 000 | 4 | 4 |
| 4.1 | 133 693 440 | 20 000 | 50 000 | 4 | 4 |
| 5 | 267 386 880 | 25 000 | 100 000 | 6 | 4 |
| 5.1 | 534 773 760 | 40 000 | 160 000 | 8 | 4 |
| 5.2 | 1 069 547 520 | 60 000 | 240 000 | 8 | 4 |
| 6 | 1 069 547 520 | 60 000 | 240 000 | 8 | 4 |
| 6.1 | 2 139 095 040 | 120 000 | 480 000 | 8 | 4 |
| 6.2 | 4 278 190 080 | 240 000 | 800 000 | 8 | 4 |

Table A.3 – Specification of CpbVclFactor, CpbNalFactor, FormatCapabilityFactor and MinCrScaleFactor

| Profile | CpbVclFactor | CpbNalFactor | FormatCapabilityFactor | MinCrScaleFactor |
| --- | --- | --- | --- | --- |
| Main 10 | 1 000 | 1 100 | 1.875 | 1.00 |
| Main 4:4:4 10 | 2 500 | 2 750 | 3.750 | 0.75 |
| Main 10 Still Picture | 1 000 | 1 100 | 1.875 | 1.00 |
| Main 4:4:4 10 Still Picture | 2 500 | 2 750 | 3.750 | 0.75 |

Informative clause A.4.3 shows the effect of these limits on picture rates for several example picture formats.

#### D.7.2	Subpicture level information SEI message semantics

...

–	The value of SubpicWidthInTiles[ j ] shall be less than or equal to MaxTileCols and the value of SubpicHeightInTiles[ j ] shall be less than or equal to MaxTilesPerAu / MaxTilesCols, where MaxTileCols and MaxTilesPerAu are specified in Table A.1 for level sli_ref_level_idc[ i ].

–	The value of SubpicWidthInTiles[ j ] * SubpicHeightInTiles[ j ] shall be less than or equal to MaxTilesPerAu * LayerRefLevelFraction[ i ][ j ], where MaxTilesPerAu is specified in Table A.1 for level sli_ref_level_idc[ i ].

...

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).