| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0162 |
| --- | --- |

| Title: | AHG3/AHG12: Subpicture merging software |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Antti Hallapuro Miska M. Hannuksela Nokia Technologies, Tampere, Finland | Email: | firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution includes the source code for a subpicture merging software and proposes adding the software in the VTM source code package. The software enables the following setup:

- Encoding several bitstreams, each with one subpicture per picture, using the VTM encoder.

- Merging selected encoded bitstreams into a bitstream with multiple subpictures, using the subpicture merging software.

- Decoding the bitstream having multiple subpictures, using the VTM decoder.

# Introduction

JVET-R0148 contained a description of the subpicture merging software and reported performed experiments. There were requests of making the source code of the subpicture merging software available and maintained as a part of the VTM package. This contribution includes the source code for the subpicture merging software and proposes adding the software in the VTM source code package.

The subpicture merging software enables the use of the system setup illustrated in Figure 1 and described briefly below:

- Splitting to subpicture sequences
An original uncompressed video clip is spatially split into uncompressed subpicture sequences.

- VVC encoding
Each uncompressed subpicture sequence is encoded independently with an encoding configuration that generates only one subpicture per picture.

- Bitstream selection
Any subset of the encoded bitstreams is selected. For example, in viewport-dependent streaming, the bitstreams covering the viewport could be selected.

- Merging
The selected bitstreams are merged into one bitstream with multiple subpictures. Each input bitstream corresponds to a certain subpicture in the subpicture layout of the output bitstream. The subpicture merging software was developed for performing this step.

- VVC decoding
The bitstream with multiple subpictures is decoded. The VTM decoder was used in the performed experiments.

Figure 1. System for selective subpicture-based operation.

Our motivation and asserted benefits for the illustrated system setup include the following:

- It is envisioned that there could be VVC encoder implementations that do not provide support for multiple subpictures. An intent of the performed experiments was to verify that any encoders can be used to generate encoded content that can be merged as subpictures into a VVC bitstream.

- It is assumed that parallel encoding of subpictures, e.g. in separate processing cores, can be more easily achieved with the tested encoding setup than with an encoder that natively supports encoding multiple subpictures per picture. With the given approach, the encoder implementation does not need to include handling of separate processes or threads per each subpicture. The encoding approach used in the experiments could be applied e.g. in a cloud-based encoding setup.

- One of the goals of the subpicture design in the VVC standard is to enable straightforward merging of subpictures. The merger software intends to serve as evidence that this design goal has been successfully met.

- The merger software can be regarded as an independent implementation that generates bitstreams with multiple subpictures. However, it is remarked that merger software shares some VTM source code modules for high-level syntax parsing and generation.

- The system setup can also be used to verify that the VTM decoder is capable of decoding bitstreams that have multiple selectable subpictures and originate from another source than directly from the VTM encoder.

- The merger software implements operations that are needed in players that select a subset of available subpictures to be received and/or decoded. For example, players that support viewport-dependent 360° streaming are expected to include subpicture merging. The merger software can be considered to be a building block in such players.

# Merger software

The merger tool takes multiple subpicture bitstreams, along with their dimensions (width and height) and target on-screen locations, as an input and generates a merged bitstream. The tool integrates with VTM encoder and decoder libraries so that useful parsing and coding functions from VTM can be used easily.

The tool parses all parameter sets, picture header NAL units (if they exist) and slice headers of all subpictures for the current time instance. It is assumed that parameter sets and picture headers of subpicture bitstreams have the same features enabled so that common parameter sets can be shared in the merged stream. It is also assumed that APS NAL units having the same ID and type have identical content.

The merger tool makes the following modifications to the NAL units:

- All parameter set and picture header NAL units of the input bitstreams are discarded except for the those of the first subpicture that are used as the basis for the merged stream.

- Subpicture information is written to the SPS(s) of the merged stream.

- Picture partitioning information is written to the PPS(s) of the merged stream. It is possible to merge input bitstreams that contain tiles and slices.

- Picture headers are changed to be coded as PH NAL units, if they were originally coded within slice header.

- Subpicture IDs are written to the slice headers, if the slice headers did not originally contain them.

- All SEI NAL units are discarded in the current implementation.

- All other non-VCL NAL units are taken from the bitstream of the first subpicture only.

Modified NAL units, slice headers, remaining non-VCL NAL units and VCL NAL slice data sections from the subpictures are then written to the target bitstream in the correct decoding order.

The tool also includes a simple YUV merging option for verification purposes. Assuming that YUV files from individually decoded subpicture bitstreams are available then these files can be merged and the results could be compared (using diff/comp etc.) with YUV file from merged bitstream decode.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).