| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0163 |
| --- | --- |

| Title: | AHG9: On target OLS and sublayers for decoding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Robert Skupin Fraunhofer HHI Hendry LG Electronics Byeongdoo Choi, Stephan Wenger Tencent | Email: | miska.hannuksela@nokia.com firstname.lastname@hhi.fraunhofer.de dr.hendry@lge.com bdchoi@tencent.com |
| Source: | Nokia, Fraunhofer HHI, LG Electronics, Tencent |  |  |

_____________________________

# Abstract

This contribution presents the results of offline work on JVET-S0087 and JVET-S0103 aspect 1.

JVET-S0087 proposed indicating applicable values of TargetOlsIdx and Htid within AU delimiter NAL unit for IRAP and GDR AUs. The JVET HLS AHG on May 27 – 28 recommended offline work on an approach with a distinct NAL unit and text such that the information needs to be available – either in the bitstream or by external means. The offline work was also suggested to consider JVET-S0103 aspect 1, which proposed inferring TargetOlsIdx and Htid, when they are not provided through external means.

This contribution proposes:

- Operation point information (OPI) NAL unit for carrying syntax elements indicating TargetOlsIdx and Htid.

- Including an extension mechanism in OPI NAL unit similar to that in DCI and all parameter sets.

- Allowing OPI NAL unit in all AUs with IRAP or GDR slices (and thus may become IRAP or GDR AUs as a result of layer and/or subpicture extraction), and requiring the content of the OPI NAL unit to be the same within a CVS.

- TargetOlsIdx and Htid remain valid until for one or more CVSs, until they are updated the next time by an OPI NAL unit or through external means.

- When both AUD and OPI NAL units are present in an AU, the OPI NAL unit shall be the next NAL unit after the AUD NAL unit. When no AUD NAL unit is present in an AU and the OPI NAL unit is present in the AU, the OPI NAL unit shall be the first NAL unit in the AU. No more than one instance of the OPI NAL unit shall be present in an AU.

- Allowing to have neither external means nor the OPI NAL unit present for the first AU of the bitstream.

- When no target OLS index is provided in the OPI NAL unit or through external means for the first AU of the bitstream, TargetOlsIdx is inferred equal to the lowest OLS index that contains the highest number of layers and the highest number of output layers among all OLSs.

- When no highest temporal ID is provided in the OPI NAL unit or through external means for the first AU of the bitstream, Htid is inferred equal to vps_ptl_max_tid[ vps_ols_ptl_idx[ TargetOlsIdx ] ].

- Assigning nal_unit_type equal to 12 (which was earlier used for RSV_IRAP_12) for the OPI NAL unit.

- nuh_temporal_id_plus1 of OPI NAL unit shall be equal to 1 and TemporalId of the containing AU shall also be 0.

- Allowing any nuh_layer_id value for the OPI NAL unit. Consequently, in the sub-bitstream extraction process, the OPI NAL unit is added among the NAL units that are not removed based on nuh_layer_id values.

If item 8 above is adopted, the following items are also proposed:

- In the current VVC draft, if no VPS is present, then vps_ptl_max_tid[ vps_ols_ptl_idx[ TargetOlsIdx ] ] is not inferred. This document proposes to infer it equal to sps_max_sublayers_minus1 when no VPS is present.

- It is proposed to consider changing the syntax element name of vps_ptl_max_tid[ i ] and change its semantics to be more generic.

The proposed semantics are (the bolded italics text at the end of the sentence added): vps_ptl_max_tid[ i ] specifies the TemporalId of the highest sublayer representation for which the level information is present in the i-th profile_tier_level( ) syntax structure in the VPS and present in the OLSs with index olsIdx such that vps_ols_ptl_idx[ olsIdx ] is equal to i.

# Background: JVET-S0087 and JVET-S0103 aspect 1 in the JVET HLS AHG meeting, 27- 28 May.

JVET-S0087 can be summarized as follows: When aud_irap_or_gdr_au_flag is equal to 1, in the AUD syntax, optionally signal the OLS index of the OLS represented by the CVS (aud_cvs_ols_idx) and the highest temporal sublayer present in the CVS (aud_cvs_htid_plus1), each conditioned by its own presence flag, with the following additional details:

- aud_cvs_htid_plus1 equal to 0 specifies that all the pictures in the CVS starting with the AUD NAL unit are IRAP pictures or GDR pictures with ph_recovery_poc_cnt equal to 0. aud_cvs_htid_plus1 greater than 0 specifies that all the pictures in the CVS starting with the AUD NAL unit have TemporalId less than aud_cvs_htid_plus1.

- aud_cvs_ols_idx specifies that the CVS starting with the AUD NAL unit does not contain any other layers than those included in the OLS with OLS index equal to aud_cvs_ols_idx.

- When aud_cvs_htid_plus1 is present, it is used to derive the value of Htid. Otherwise, Htid is set by external means (like in VVC Draft 9).

- When aud_cvs_ols_idx is present, it is used to derive the value of TargetOlsIdx. Otherwise, TargetOlsIdx is set by external means (like in VVC Draft 9).

- The sub-bitstream extraction process is modified so that the highest temporal sublayer and the target OLS given as inputs to the process are used in modifying the AUD NAL units in the output sub-bitstream.

The following notes were taken on JVET-S0087 in the JVET HLS ad-hoc group meeting on May 27 – 28 (copied from JVET-S0137):

It was noted that this requires AUD syntax rewriting for extraction.

A participant suggested to consider using the AUD content only if such information is not provided by external means. Others said that any external means and the AUD content (if present) should be required to match each other. Another participant said this could have a similar concept as for parameter sets – it needs to be available – either in the bitstream or by external means.

Another participant commented that dropping of some NAL units may cause mismatch, although we would not need to specify a decoder response to accidental dropping that produces non-conforming bitstreams.

The spirit is to allow the information to be signalled “in band”. If not present, the current scheme would apply such that external means is needed.

Something similar had been proposed at the January 2020 (Brussels) meeting in JVET-Q0256, using a different NAL unit rather than the AUD. This proposal would make the AUD have variable length.

It was commented that having a different NAL unit might help deal with the rewriting issue.

This was deferred for further study until the main meeting to propose an approach with a distinct NAL unit and text such that the information needs to be available – either in the bitstream or by external means.

JVET-S0103 aspect 1 can be summarized as follows: Add a rule to select among OLSs matching BitstreamToDecode: When TargetOlsIdx is not provided via external means, it is set equal to the OLS with the lowest index that contains all layers of AU 0 of BitstreamToDecode and the highest number of output layers.

The following notes were taken on JVET-S0103 aspect 1 in the JVET HLS ad-hoc group meeting on May 27 – 28 (copied from JVET-S0137):

Currently, there is no default, and TargetOlsIdx needs to be provided.

This was deferred for further study until the main meeting in conjunction with the previous item above.

# Introduction

This contribution proposes changes based on JVET-S0087 and JVET-S0103 aspect 1. The proposal is asserted to be aligned with the direction agreed in the JVET HLS AHG meeting on May 27 – 28.

The following list contains the design questions that were considered when authoring this contribution, the choices that were taken for each design question, and the rationale why the choices were taken.

- Editorial question: What is the new NAL unit called?

Answer: Operation point information NAL unit.

Rationale: The target output layer set index and the highest temporal ID determine an operation point (OP).

- Should the new NAL unit have an extension mechanism similar to that in DCI and all parameter sets?

Answer: Yes.

Rationale: Extension capability have been found useful in HEVC extensions.

- In which AUs can the new NAL unit be present?

Answer: In all AUs that may become IRAP or GDR AUs as a result of layer and/or subpicture extraction. The content of the new NAL unit must be the same within a CVS.

Rationale: Leave freedom for the encoder to insert the new NAL unit in all AUs that may become CVSS AUs as a result of layer and/or subpicture extraction.

- What is the "validity scope" of the new NAL unit?

Answer: One or more CVSs.

Rationale: The parameters may change during the bitstream (e.g. splicing) and thus be included and may change in content in each CVSS AU. If the parameters do not change in the bitstream, they have to be included or inferred only at the start of the bitstream. When streamed over DASH, the parameters could be present in the Initialization Segment and hence made available regardless of which random access point is used to start decoding.

- What are the ordering constraints of the new NAL unit in an AU?

Answer: When both AUD and the new NAL unit are present in an AU, the new NAL unit shall be the next NAL unit after the AUD NAL unit. When no AUD NAL unit is present in an AU and the new NAL unit is present in the AU, the new NAL unit shall be the first NAL unit in the AU. No more than one instance of the new NAL unit shall be present in an AU.

Rationale: The information in the new NAL unit may be used in selecting valid syntax elements in parameter sets and HRD SEI messages, so its presence before those is reasonable. An early location in an AU also serves the purpose of initializing the decoding based on the values carried in the NAL unit.

- Is it allowed to have neither external means nor the new NAL unit present (and consequently infer Htid and TargetOlsIdx)?

Answer: Yes.

Rationale: It seems reasonable to have a default fallback behaviour. This would be nice e.g. when the bitstream contains only a single layer and only a single sublayer.

- If yes to above, which value TargetOlsIdx is inferred to?

Answer: TargetOlsIdx is inferred equal to the lowest OLS index that contains the highest number of layers and the highest number of output layers among all OLSs.

Rationale: It could be assumed that the entire encoded bitstream is initially intended for decoding – thus, selecting the highest number of layers among all OLSs is reasonable. Compared to JVET-S0103 aspect 1, a solution that does not require parsing the first AU was desired in order to initiate the decoding process and to set up the DPB when the first slice of the first AU of the bitstream is decoded.

- If yes to above, which value Htid is inferred to?

Answer: Htid is inferred equal to vps_ptl_max_tid[ vps_ols_ptl_idx[ TargetOlsIdx ] ].

Rationale: When the number of sublayers depends on OLS, vps_ptl_max_tid[ vps_ols_ptl_idx[ TargetOlsIdx ] ] is more precise than vps_max_sublayers_minus1. The following related changes are also proposed:

- In the current VVC draft, if no VPS is present, then vps_ptl_max_tid[ vps_ols_ptl_idx[ TargetOlsIdx ] ] is not inferred. This document proposes to infer it equal to sps_max_sublayers_minus1 when no VPS is present.

- It is proposed to consider changing the syntax element name of vps_ptl_max_tid[ i ] and change its semantics to be more generic.

The current semantics are: vps_ptl_max_tid[ i ] specifies the TemporalId of the highest sublayer representation for which the level information is present in the i-th profile_tier_level( ) syntax structure in the VPS.

The proposed semantics are: vps_ptl_max_tid[ i ] specifies the TemporalId of the highest sublayer representation for which the level information is present in the i-th profile_tier_level( ) syntax structure in the VPS and present in the OLSs with index olsIdx such that vps_ols_ptl_idx[ olsIdx ] is equal to i.

Note also that the current VVC draft specifies: When vps_ptl_max_tid[ i ] is not present, it is inferred to be equal to vps_max_sublayers_minus1. When vps_max_sublayers_minus1 is not present, it is inferred to be equal to sps_max_sublayers_minus1.

- Which NAL unit type value does the new NAL unit take?

Answer: nal_unit_type equal to 12, which was earlier used for RSV_IRAP_12.

Rationale: There is only one such reserved non-VCL NAL unit left that can start a PU (RSV_NVCL_26). It would be good to keep this reserved non-VCL NAL unit for future extension capability. In the current VVC draft, there are two reserved IRAP NAL unit types left, so taking one of them into other use still keeps one reserved IRAP NAL unit type left.

- Which nuh_temporal_id_plus1 value does the new NAL unit take?

Answer: Its nuh_temporal_id_plus1 shall be equal to 1 and TemporalId of the containing AU shall also be 0.

Rationale: The new NAL unit can only be present in AUs that contain IRAP or GDR slices and hence have TemporalId equal to 0.

- Which nuh_layer_id value does the new NAL unit take?

Answer: It can take any nuh_layer_id value. Consequently, in the sub-bitstream extraction process, it is added among the NAL units that are not removed based on nuh_layer_id values.

Rationale: The new NAL unit is similar to DCI, VPS, AUD, and EOB NAL units, which can also take any nuh_layer_id values.

- When both external means or the new NAL unit are applied, does the external means or the new NAL unit have precedence?

Answer: The bitstream shall comply to both, i.e. both shall be correct. Thus, it is ok to remain silent on which takes precedence and leave it up to implementations.

# Proposed specification text

## Syntax

| operation_point_information_rbsp( ) { | Descriptor |
| --- | --- |
| opi_ols_info_present_flag | u(1) |
| opi_htid_info_present_flag | u(1) |
| if( opi_ols_info_present_flag ) |  |
| opi_ols_idx | ue(v) |
| if( opi_htid_info_present_flag ) |  |
| opi_htid_plus1 | u(3) |
| opi_extension_flag | u(1) |
| if( opi_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| opi_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

## Semantics

All OPI NAL units in a CVS shall have the same content. An OPI NAL unit shall not be present in an AU that does not contain any VCL NAL unit with nal_unit_type is in the range of IDR_W_RADL to RSV_IRAP_11, inclusive.

opi_ols_info_present_flag equal to 0 specifies that opi_ols_idx is not present in the OPI NAL unit. opi_ols_info_present_flag equal to 1 specifies that opi_ols_idx is present in the OPI NAL unit.

opi_htid_info_present_flag equal to 0 specifies that opi_htid_plus1 is not present in the OPI NAL unit. opi_htid_info_present_flag equal to 1 specifies that opi_htid_plus1 is present in the OPI NAL unit.

opi_ols_idx specifies that the current CVS and the next CVSs in decoding order up to and not including the next CVS for which opi_ols_idx is provided in an OPI NAL unit or through external means do not contain any other layers than those included in the OLS with OLS index equal to opi_ols_idx.

opi_htid_plus1 equal to 0 specifies that all the pictures in the current CVS and the next CVSs in decoding order up to and not including the next CVS for which opi_htid_plus1 is provided in an OPI NAL unit or through external means are IRAP pictures or GDR pictures with ph_recovery_poc_cnt equal to 0. opi_htid_plus1 greater than 0 specifies that all the pictures in the current CVS and the next CVSs in decoding order up to and not including the next CVS for which opi_htid_plus1 is provided in an OPI NAL unit or through external means have TemporalId less than opi_htid_plus1.

opi_extension_flag equal to 0 specifies that no opi_extension_data_flag syntax elements are present in the OPI RBSP syntax structure. opi_extension_flag equal to 1 specifies that there may be opi_extension_data_flag syntax elements present in the OPI RBSP syntax structure.

One or more of opi_htid_info_present_flag, opi_ols_info_present_flag, and opi_extension_flag shall be equal to 1.

opi_extension_data_flag may have any value. Its presence and value do not affect decoder conformance to profiles specified in Annex A. Decoders conforming to this version of this Specification shall ignore all opi_extension_data_flag syntax elements.

## General NAL unit semantics

In the semantics of nuh_layer_id, add OPI_NUT among the NAL units for which nuh_layer_id is not constrained:

NOTE 1 – The value of nuh_layer_id for OPI, DCI, VPS, AUD, and EOB NAL units is not constrained.

In Table 5, remove RSV_IRAP_12 and add OPI_NUT:

Table 5 – NAL unit type codes and NAL unit type classes

| nal_unit_type | Name of nal_unit_type | Content of NAL unit and RBSP syntax structure | NAL unit type class |
| --- | --- | --- | --- |
| 11 12 | RSV_IRAP_11 RSV_IRAP_12 | Reserved IRAP VCL NAL unit types | VCL |
| 12 | OPI_NUT | Operation point information operation_point_information_rbsp( ) | non-VCL |

In the semantics of nuh_temporal_id_plus1, replace RSV_IRAP_12 with RSV_IRAP_11:

When nal_unit_type is in the range of IDR_W_RADL to RSV_IRAP_1211, inclusive, TemporalId shall be equal to 0.

In the semantics of nuh_temporal_id_plus1, add OPI_NUT among the NAL units for which TemporalId shall be equal to 0 and the TemporalId of the containing AU shall also be 0:

The value of TemporalId for non-VCL NAL units is constrained as follows:

–	If nal_unit_type is equal to OPI_NUT, DCI_NUT, VPS_NUT, or SPS_NUT, TemporalId shall be equal to 0 and the TemporalId of the AU containing the NAL unit shall be equal to 0.

## Order of NAL units in the bitstream

There can be at most one OPI NAL unit in an AU. When an OPI NAL unit is present in an AU, it shall be the first NAL unit following the AUD NAL unit, if any, and otherwise shall be the first NAL unit of the AU.

…

Let firstVclNalUnitInAu be the first VCL NAL unit of an AU. The first of any of the following NAL units preceding firstVclNalUnitInAu and succeeding the last VCL NAL unit preceding firstVclNalUnitInAu, if any, specifies the start of a new AU:

–	AUD NAL unit (when present),

–	OPI NAL unit (when present),

…

The order of the non-VCL NAL units (other than the AUD, OPI, and EOB NAL units) within a PU shall obey the following constraints:

…

## Decoding process

Replace the following paragraph in clause 8.1.1:

The variables TargetOlsIdx, which identifies the OLS index of the target OLS to be decoded, and the variable Htid, which identifies the highest temporal sublayer to be decoded, are set by some external means not specified in this Specification. The bitstream BitstreamToDecode does not contain any other layers than those included in the target OLS and does not include any NAL unit with TemporalId greater than Htid.

with the following:

The variable TargetOlsIdx, which identifies the OLS index of the target OLS to be decoded, is derived as follows:

–	The following applies in the first AU of the bitstream:

–	If opi_ols_idx is present in an OPI NAL unit in the first AU of the bitstream or provided through external means for the first AU of the bitstream, the variable TargetOlsIdx is set equal to opi_ols_idx.

–	Otherwise (no opi_ols_idx is present or provided through external means in the first AU of the bitstream), the variable TargetOlsIdx is set equal to the lowest OLS index that contains the highest number of layers and the highest number of output layers among all OLSs.

–	The following applies in the first AU of each CVS that is not the first AU of the bitstream:

–	When opi_ols_idx is present in an OPI NAL unit in the first AU of the CVS or provided through external means for the first AU of the CVS, the variable TargetOlsIdx is set equal to opi_ols_idx.

The variable Htid, which identifies the highest temporal sublayer to be decoded, is derived as follows:

–	The following applies in the first AU of the bitstream:

–	If opi_htid_plus1 is present in an OPI NAL unit in the first AU of the bitstream or provided through external means for the first AU of the bitstream, Htid is set equal to ( ( opi_htid_plus1 > 0 ) ? opi_htid_plus1 – 1 : 0 ).

–	Otherwise, if vps_video_parameter_set_id is greater than 0, Htid is set equal to vps_ptl_max_tid[ vps_ols_ptl_idx[ TargetOlsIdx ] ].

–	Otherwise, Htid is set equal to sps_max_sublayers_minus1.

–	The following applies in the first AU of each CVS that is not the first AU of the bitstream:

–	When opi_htid_plus1 is present in an OPI NAL unit in the first AU of the CVS or provided through external means for the first AU of the CVS, Htid is set equal to ( ( opi_htid_plus1 > 0 ) ? opi_htid_plus1 – 1 : 0 ).

NOTE – The CVS does not contain any other layers than those included in the target OLS and does not include any NAL unit with TemporalId greater than Htid.

## Sub-bitstream extraction

Add OPI NAL unit to the list of NAL units that are not removed from the bitstream

The output sub-bitstream OutBitstream is derived as follows:

…

–	Remove from outBitstream all NAL units with nal_unit_type not equal to any of OPI_NUT, VPS_NUT, DCI_NUT, and EOB_NUT and with nuh_layer_id not included in the list LayerIdInOls[ targetOlsIdx ].

–	When all VCL NAL units of an AU are removed by any derivation step above and an OPI NAL unit is present in the AU, remove the OPI NAL unit from outBitstream.

Add the following into the sub-bitstream extraction process of clause C.6:

–	For each OPI NAL unit, the following applies:

–	Set opi_htid_info_present_flag equal to 1 in the OPI NAL unit.

–	Set opi_ols_info_present_flag is set equal to 1 in the OPI NAL unit.

–	Set opi_htid_plus1 is set equal to tIdTarget + 1 in the OPI NAL unit.

–	Set opi_ols_idx is set equal to targetOlsIdx in the OPI NAL unit.

## Abbreviations

Add OPI (Operation Point Information) into the list of abbreviations in clause 4.

# Patent rights declaration(s)

Nokia, Fraunhofer HHI, LG Electronics, and Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).