| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0164-v1 |
| --- | --- |

| Title: | AHG9: On signalling six PPS flags for indicating the presence of syntax in PH/SH |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shih-Ta Hsiang, Lulin Chen, Olena Chubach, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {shih-ta.hsiang, lulin.chen, olena.chubach, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In this contribution it is proposed that the presence of six PPS flags pps_alf_info_in_ph_flag, pps_rpl_info_in_ph_flag, pps_dbf_info_in_ph_flag, pps_sao_info_in_ph_flag, pps_wp_info_in_ph_flag, and pps_qp_delta_info_in_ph_flag is further conditioned on pps_rect_slice_flag, pps_single_slice_per_subpic_flag, and pps_num_slices_in_pic_minus1. When pps_rect_slice_flag is equal to 1, pps_single_slice_per_subpic_flag is equal to 0 and pps_num_slices_in_pic_minus1 is equal to 0, the six PPS flags are all inferred to be equal to 0.

# Introduction

In aspect 5 of JVET-S0049 [1], it is proposed that when pps_rect_slice_flag is equal to 1 and pps_num_slices_in_pic_minus1 is equal to 0, the values of the six PPS flags pps_alf_info_in_ph_flag, pps_rpl_info_in_ph_flag, pps_dbf_info_in_ph_flag, pps_sao_info_in_ph_flag, pps_wp_info_in_ph_flag, and pps_qp_delta_info_in_ph_flag shall all be equal to 0.

This proposal was discussed during the previous JVET-S HLS AHG pre-meeting, and it was commented that this proposal in its suggested design may require SH rewriting when a subpicture is extracted from an original bitstream as one picture with one slice.

# Proposal

In this contribution it is proposed that the presence of six PPS flags pps_alf_info_in_ph_flag, pps_rpl_info_in_ph_flag, pps_dbf_info_in_ph_flag, pps_sao_info_in_ph_flag, pps_wp_info_in_ph_flag, and pps_qp_delta_info_in_ph_flag is further conditioned on pps_rect_slice_flag, pps_single_slice_per_subpic_flag, and pps_num_slices_in_pic_minus1. When pps_rect_slice_flag is equal to 1, pps_single_slice_per_subpic_flag is equal to 0 and pps_num_slices_in_pic_minus1 is equal to 0, the six PPS flags are all inferred to be equal to 0.

By setting pps_no_pic_partition_flag equal to 0 and pps_single_slice_per_subpic_flag equal to 1, the six PPS flags are present in the PPS and the need for SH rewriting for BEAM applications can be avoided. When pps_single_slice_per_subpic_flag is equal to 1, the parsing process is not dependent on the value of pps_num_slices_in_pic_minus1 and bitstream parsing dependency on SPS does not exist.

The proposed modifications to the VVC Draft 9 with integration of the May 2020 HLS AHG meeting recommendation [2] are marked in red as follows:

#### 7.3.2.4 Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | u(6) |
| pps_seq_parameter_set_id | u(4) |
| … |  |
| if( !pps_no_pic_partition_flag ) { |  |
| pps_log2_ctu_size_minus5 | u(2) |
| pps_num_exp_tile_columns_minus1 | ue(v) |
| pps_num_exp_tile_rows_minus1 | ue(v) |
| for( i = 0; i <= pps_num_exp_tile_columns_minus1; i++ ) |  |
| pps_tile_column_width_minus1[ i ] | ue(v) |
| for( i = 0; i <= pps_num_exp_tile_rows_minus1; i++ ) |  |
| pps_tile_row_height_minus1[ i ] | ue(v) |
| if( NumTilesInPic > 1 ) { |  |
| pps_loop_filter_across_tiles_enabled_flag | u(1) |
| pps_rect_slice_flag | u(1) |
| } |  |
| if( pps_rect_slice_flag ) |  |
| pps_single_slice_per_subpic_flag | u(1) |
| if( pps_rect_slice_flag && !pps_single_slice_per_subpic_flag ) { |  |
| pps_num_slices_in_pic_minus1 | ue(v) |
| if( pps_num_slices_in_pic_minus1 > 1 ) |  |
| pps_tile_idx_delta_present_flag | u(1) |
| for( i = 0; i < pps_num_slices_in_pic_minus1; i++ ) { |  |
| if( SliceTopLeftTileIdx[ i ] % NumTileColumns != NumTileColumns − 1 ) |  |
| pps_slice_width_in_tiles_minus1[ i ] | ue(v) |
| if( SliceTopLeftTileIdx[ i ] / NumTileColumns != NumTileRows − 1 && ( pps_tile_idx_delta_present_flag ¦ ¦ SliceTopLeftTileIdx[ i ] % NumTileColumns = = 0 ) ) |  |
| pps_slice_height_in_tiles_minus1[ i ] | ue(v) |
| if( pps_slice_width_in_tiles_minus1[ i ] = = 0 && pps_slice_height_in_tiles_minus1[ i ] = = 0 && RowHeight[ SliceTopLeftTileIdx[ i ] / NumTileColumns ] > 1 ) { |  |
| pps_num_exp_slices_in_tile[ i ] | ue(v) |
| for( j = 0; j < pps_num_exp_slices_in_tile[ i ]; j++ ) |  |
| pps_exp_slice_height_in_ctus_minus1[ i ][ j ] | ue(v) |
| i += NumSlicesInTile[ i ] − 1 |  |
| } |  |
| if( pps_tile_idx_delta_present_flag && i < pps_num_slices_in_pic_minus1 ) |  |
| pps_tile_idx_delta_val[ i ] | se(v) |
| } |  |
| } |  |
| if( !pps_rect_slice_flag ¦ ¦ pps_single_slice_per_subpic_flag ¦ ¦ pps_num_slices_in_pic_minus1 > 0 ) |  |
| pps_loop_filter_across_slices_enabled_flag | u(1) |
| } |  |
| … |  |
| pps_deblocking_filter_control_present_flag | u(1) |
| if( pps_deblocking_filter_control_present_flag ) { |  |
| pps_deblocking_filter_override_enabled_flag | u(1) |
| pps_deblocking_filter_disabled_flag | u(1) |
| if( !pps_no_pic_partition_flag && ( !pps_rect_slice_flag ¦ ¦ pps_single_slice_per_subpic_flag ¦ ¦ pps_num_slices_in_pic_minus1 > 0 ) && pps_deblocking_filter_override_enabled_flag ) |  |
| pps_dbf_info_in_ph_flag | u(1) |
| if( !pps_deblocking_filter_disabled_flag ) { |  |
| pps_luma_beta_offset_div2 | se(v) |
| pps_luma_tc_offset_div2 | se(v) |
| if( pps_chroma_tool_offsets_present_flag ) { |  |
| pps_cb_beta_offset_div2 | se(v) |
| pps_cb_tc_offset_div2 | se(v) |
| pps_cr_beta_offset_div2 | se(v) |
| pps_cr_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| } |  |
| if( !pps_no_pic_partition_flag && ( !pps_rect_slice_flag ¦ ¦ pps_single_slice_per_subpic_flag ¦ ¦ pps_num_slices_in_pic_minus1 > 0 ) ) { |  |
| pps_rpl_info_in_ph_flag | u(1) |
| pps_sao_info_in_ph_flag | u(1) |
| pps_alf_info_in_ph_flag | u(1) |
| if( ( pps_weighted_pred_flag ¦ ¦ pps_weighted_bipred_flag ) && pps_rpl_info_in_ph_flag ) |  |
| pps_wp_info_in_ph_flag | u(1) |
| pps_qp_delta_info_in_ph_flag | u(1) |
| } |  |
| … |  |
| } |  |

# References

- Y.-K. Wang, L. Zhang, Z. Deng, and K. Zhang, “AHG9/AHG8/AHG12: On parameter sets and picture header,” Document of Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-S0049, 19th Meeting: by teleconference, 22 June – 1 July 2020.

- Y.-K. Wang, “AHG2: Editorial input of a text integration for the May 2020 HLS AHG meeting outcome,” Document of Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-S0152, 19th Meeting: by teleconference, 22 June – 1 July 2020.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).