| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0166-v1 |
| --- | --- |

| Title: | AHG9: Inferring information of the SPS from the reference layer |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shih-Ta Hsiang, Lulin Chen, Olena Chubach, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {shih-ta.hsiang, lulin.chen, olena.chubach, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

- Abstract

In HEVC, the syntax elements of scaling list data structure of the current SPS can be inferred to be equal to those of the SPS referred to by the selected direct reference layer. Based on similar mechanism, this contribution proposes for VVC to allow inferring information of the SPS from the corresponding syntax elements of the SPS referred to by the selected direct reference layer as follows:

- Propose to allow inferring the RPLs of the current SPS from the SPS referred to by the direct reference layer.

- Propose to allow inferring subpicture information of the current SPS from the SPS referred to by the direct reference layer.

Proposals

Proposal 1: Inferring RPLs from the reference layer

The proposed modifications to the VVC Draft 9 with integration of the May 2020 HLS AHG meeting outcome [1] are marked in red as follows:

#### 7.3.2.3 Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_seq_parameter_set_id | u(4) |
| sps_video_parameter_set_id | u(4) |
| … |  |
| if( sps_video_parameter_set_id > 0 ) |  |
| sps_inter_layer_ref_pics_present_flag | u(1) |
| sps_idr_rpl_present_flag | u(1) |
| if( sps_inter_layer_ref_pics_present_flag ) |  |
| sps_inferred_rpl_flag | u(1) |
| if( sps_inferred_rpl_flag ) |  |
| sps_inferred_rpl_ref_layer_idx | ue(v) |
| else { |  |
| sps_rpl1_same_as_rpl0_flag | u(1) |
| for( i = 0; i < sps_rpl1_same_as_rpl0_flag ? 1 : 2; i++ ) { |  |
| sps_num_ref_pic_lists[ i ] | ue(v) |
| for( j = 0; j < sps_num_ref_pic_lists[ i ]; j++) |  |
| ref_pic_list_struct( i, j ) |  |
| } |  |
| } |  |
| … |  |
| } |  |

#### 7.4.3.3 Sequence parameter set RBSP semantics

…

sps_inferred_rpl_flag equal to 1 specifies that the sps_inferred_rpl_ref_layer_idx syntax element is present in the SPS. sps_inferred_rpl_flag equal to 0 specifies that the sps_inferred_rpl_ref_layer_idx syntax element is not present in the SPS. When not present, sps_inferred_rpl_flag is inferred to be equal to 0.

sps_inferred_rpl_ref_layer_idx specifies the index to the list of the direct reference layers. The value of sps_inferred_rpl_ref_layer_idx shall be in the range of 0 to NumDirectRefLayers[ GeneralLayerIdx[ nuh_layer_id ] ] − 1, inclusive.

When sps_inferred_rpl_flag is equal to 1 and nuh_layer_id is equal to a particular value layerId, the following applies:

–	The values of sps_num_ref_pic_lists[ 0 ] and sps_num_ref_pic_lists[ 1 ] are inferred to be equal to the values of corresponding syntax elements in the SPS that is referred to by the direct reference layer with nuh_layer_id equal to vps_layer_id[ DirectRefLayerIdx[ GeneralLayerIdx[ layerId ] ][ sps_inferred_rpl_ref_layer_idx ] ].

–	The value of each of the following syntax elements in ref_pic_list_struct( i, rplsIdx ) is inferred to be equal to the value of corresponding syntax element in ref_pic_list_struct( i, rplsIdx ) of the SPS that is referred to by the direct reference layer with nuh_layer_id equal to vps_layer_id[ DirectRefLayerIdx[ GeneralLayerIdx[ layerId ] ][ sps_inferred_rpl_ref_layer_idx ] ] for i in the range of 0 to 1 and rplsIdx in the range of 0 to sps_num_ref_pic_lists[ i ] – 1, inclusive.

Proposal 2: Inferring subpicture information from the reference layer

The proposed modifications to the VVC Draft 9 with integration of the May 2020 HLS AHG meeting outcome [1] are marked in red as follows:

#### 7.3.2.3 Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_seq_parameter_set_id | u(4) |
| sps_video_parameter_set_id | u(4) |
| … |  |
| if( sps_video_parameter_set_id > 0 ) |  |
| sps_inter_layer_ref_pics_present_flag | u(1) |
| sps_log2_ctu_size_minus5 | u(2) |
| sps_subpic_info_present_flag | u(1) |
| if( sps_subpic_info_present_flag ) { |  |
| if( sps_inter_layer_ref_pics_present_flag ) |  |
| sps_inferred_subpic_info_flag | u(1) |
| if( sps_inferred_subpic_info_flag ) |  |
| sps_inferred_subpic_info_ref_layer_idx | ue(v) |
| else { |  |
| sps_num_subpics_minus1 | ue(v) |
| if( sps_num_subpics_minus1 > 0 ) |  |
| sps_independent_subpics_flag | u(1) |
| for( i = 0; sps_num_subpics_minus1 > 0 && i <= sps_num_subpics_minus1; i++ ) { |  |
| if( i > 0 && sps_pic_width_max_in_luma_samples > CtbSizeY ) |  |
| sps_subpic_ctu_top_left_x[ i ] | u(v) |
| if( i > 0 && sps_pic_height_max_in_luma_samples > CtbSizeY ) { |  |
| sps_subpic_ctu_top_left_y[ i ] | u(v) |
| if( i < sps_num_subpics_minus1 && sps_pic_width_max_in_luma_samples > CtbSizeY ) |  |
| sps_subpic_width_minus1[ i ] | u(v) |
| if( i < sps_num_subpics_minus1 && sps_pic_height_max_in_luma_samples > CtbSizeY ) |  |
| sps_subpic_height_minus1[ i ] | u(v) |
| if( !sps_independent_subpics_flag) { |  |
| sps_subpic_treated_as_pic_flag[ i ] | u(1) |
| sps_loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| } |  |
| } |  |
| sps_subpic_id_len_minus1 | ue(v) |
| sps_subpic_id_mapping_explicitly_signalled_flag | u(1) |
| if( sps_subpic_id_mapping_explicitly_signalled_flag ) { |  |
| sps_subpic_id_mapping_present_flag | u(1) |
| if( sps_subpic_id_mapping_present_flag ) |  |
| for( i = 0; i <= sps_num_subpics_minus1; i++ ) |  |
| sps_subpic_id[ i ] | u(v) |
| } |  |
| } |  |
| } |  |
| … |  |
| if( sps_video_parameter_set_id > 0 ) |  |
| sps_inter_layer_ref_pics_present_flag | u(1) |
| sps_idr_rpl_present_flag | u(1) |
| sps_rpl1_same_as_rpl0_flag | u(1) |
| … |  |
| } |  |

#### 7.4.3.3 Sequence parameter set RBSP semantics

sps_inferred_ subpic_info_flag equal to 1 specifies that the sps_inferred_subpic_info_ref_layer_idx syntax element is present in the SPS. sps_inferred_subpic_info_flag equal to 0 specifies that the sps_inferred_subpic_info_ref_layer_idx syntax element is not present in the SPS. When not present, sps_inferred_subpic_info_flag is inferred to be equal to 0.

sps_inferred_subpic_info_ref_layer_idx specifies the index to the list of the direct reference layers. The value of sps_inferred_subpic_info_ref_layer_idx shall be in the range of 0 to NumDirectRefLayers[ GeneralLayerIdx[ nuh_layer_id ] ] − 1, inclusive.

When sps_inferred_subic_info_flag is equal to 1 and nuh_layer_id equal to a particular value layerId, the following applies:

–	The value of each of the following syntax elements sps_num_subpics_minus1, sps_independent_subpics_flag, sps_subpic_id_len_minus1, sps_subpic_id_mapping_explicitly_signalled_flag, and sps_subpic_id_mapping_present_flag is inferred to be equal to the value of each corresponding syntax element in the SPS that is referred to by the direct reference layer with nuh_layer_id equal to vps_layer_id[ DirectRefLayerIdx[ GeneralLayerIdx[ layerId ] ][ sps_inferred_subpic_info_ref_layer_idx ] ].

–	The value of each of the following syntax elements sps_subpic_ctu_top_left_x[ i ], sps_subpic_ctu_top_left_y[ i ], sps_subpic_width_minus1[ i ], sps_subpic_height_minus1[ i ], sps_subpic_treated_as_pic_flag[ i ], sps_loop_filter_across_subpic_enabled_flag[ i ], and sps_subpic_id[ i ] is inferred to be equal to the value of each corresponding syntax element in the SPS that is referred to by the direct reference layer with nuh_layer_id equal to vps_layer_id[ DirectRefLayerIdx[ GeneralLayerIdx[ layerId ] ][ sps_inferred_subpic_info_ref_layer_idx ] ] for i in the range of 0 to sps_num_subpics_minus1, inclusive.

–	It is a requirement of bitstream conformance that the value of each of the following syntax elements sps_subpic_info_present_flag, sps_log2_ctu_size_minus5,  sps_pic_width_max_in_luma_samples, and sps_pic_height_max_in_luma_samples is equal to the value of each corresponding syntax element in the SPS that is referred to by the direct reference layer with nuh_layer_id equal to vps_layer_id[ DirectRefLayerIdx[ GeneralLayerIdx[ layerId ] ][ sps_inferred_subpic_info_ref_layer_idx ] ].

References

- Y.-K. Wang “AHG2: Editorial input of a text integration for the May 2020 HLS AHG meeting outcome,” Document of Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-S0152, 19th Meeting: by teleconference, 22 June – 1 July 2020.

Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).