| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0169-v1 |
| --- | --- |

| Title: | AHG9: Miscellaneous Cleanups for HLS |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Biao Wang, Semih Esenlik, Han Gao, Elena Alshina | Tel: Email: | biao.wang@huawei.com |
| Source: | Huawei Technologies Co., Ltd |  |  |

_____________________________

# Abstract

This contribution addresses two HLS problems identified in scalability functionality.

# Introduction

- The current semantics of sps_sublayer_dpb_params_flag and vps_sublayer_dpb_params_present_flag is inaccurate and in some corner cases would results in semantic bugs. Specifically, when there is only one sublayer in a coding layer, these two flags do not control the presence of syntax elements of decoded picture buffer, which is contradicted to their current definition. This contribution proposes corrected semantics for them.

- The derivation process of intermediate variables dependencyFlag[ i ][ j ] ,NumDirectRefLayers[ i ], DirectRefLayerIdx[i][d], NumRefLayers[i], RefLayerIdx[i][r], and LayerUsedAsRefLayerFlag[j] does not express the design intent precisely. The loop defined in equation 37 can be simplified by considering the fact that a reference layer can only be a lower layer.

# Proposed cleanups/bug fixes

For the latest working draft of VVC [1], the following items are proposed to further improve the readability and clarity of the specification.

## Fix semantics of sps_sublayer_dpb_params_flag and vps_sublayer_dpb_params_present_flag

The semantic of sps_sublayer_dpb_params_flag is defined as follows:

sps_sublayer_dpb_params_flag is used to control the presence of max_dec_pic_buffering_minus1[ i ], max_num_reorder_pics[ i ], and max_latency_increase_plus1[ i ] syntax elements in the dpb_parameters( ) syntax strucure in the SPS. When not present, the value of sps_sublayer_dpb_params_flag is inferred to be equal to 0.*

*there is a typo in current spec, where the last sentence is “When not present, the value of sps_sub_dpb_params_info_present_flag is inferred to be equal to 0”, however, the highlighted flag is never defined

sps_sublayer_dpb_params_flag is passed as the second paremeter to dpb_parameter in SPS:

| … |  |
| --- | --- |
| dpb_parameters( sps_max_sublayers_minus1, sps_sublayer_dpb_params_flag ) |  |
| … |  |

and dpb_parameters syntax structure is defined as follows:

| dpb_parameters( maxSubLayersMinus1, subLayerInfoFlag ) { | Descriptor |
| --- | --- |
| for( i = ( subLayerInfoFlag ? 0 : maxSubLayersMinus1 ); i <= maxSubLayersMinus1; i++ ) { |  |
| max_dec_pic_buffering_minus1[ i ] | ue(v) |
| max_num_reorder_pics[ i ] | ue(v) |
| max_latency_increase_plus1[ i ] | ue(v) |
| } |  |
| } |  |

The dpb_parameters syntax structure either signals the highest one sublayer’s decoded picture buffer information or each sublayer’s decoded picture information, controlled by the value of subLayerInfoFlag (i.e. corresponding to sps_sublayer_dpb_params_flag). When a value of subLayerInfoFlag is 0, the decoded picture buffer information of the highest sublayer among the available temporal sublayers is signaled (i=maxSubLayersMinus1). When a value of subLayerInfoFlag is 1, the decoded picture buffer information of each sublayer among the available temporal sublayers is signaled (a value of i ranges from 0 to maxSubLayersMinus1, inclusive).

It is asserted that when i in dpb_parameter structure equals to sps_max_sublayers_minus1, the semantics of sps_sublayer_dpb_params_flag is incorrect. No matter sps_sublayer_dpb_params_flag is equal to 0 or 1, the syntax elements max_dec_pic_buffering_minus1[i], max_num_reorder_pics[i], and max_latency_increase_plus1[i] (for i equal to sps_max_sublayers_minus1) are always signaled. sps_sublayer_dpb_params_flag does NOT control the presence of max_dec_pic_buffering_minus1[i], max_num_reorder_pics[i], and max_latency_increase_plus1[i] in this case, which is contradict to the current definition.

In fact, when there is only one sublayer, max_dec_pic_buffering_minus1[i], max_num_reorder_pics[i], and max_latency_increase_plus1[i] are always signaled regardless of the value of sps_sublayer_dpb_params_flag. What sps_sublayer_dpb_params_flag really controls is the presence of max_dec_pic_buffering_minus1[i], max_num_reorder_pics[i], and max_latency_increase_plus1[i] for i in the range of 0 to sps_max_sublayers_minus1-1, inclusive, when sps_max_sublayers_minus1 is larger than 0.

Similarly, the semantic of vps_sublayer_dpb_params_present_flag is defined as follows:

vps_sublayer_dpb_params_present_flag is used to control the presence of max_dec_pic_buffering_minus1[ ], max_num_reorder_pics[ ], and max_latency_increase_plus1[ ] syntax elements in the dpb_parameters( ) syntax strucures in the VPS. When not present, vps_sub_dpb_params_info_present_flag is inferred to be equal to 0.

And passed as the second parameter of dpb_parameter in VPS

| … |  |
| --- | --- |
| dpb_parameters( vps_dpb_max_tid[ i ], vps_sublayer_dpb_params_present_flag ) |  |
| … |  |

When there is only one sublayer (i.e. vps_dpb_max_tid[ i ] equal to 0), max_dec_pic_buffering_minus1[], max_num_reorder_pics[], and max_latency_increase_plus1[] are always signaled regardless of the value of vps_sublayer_dpb_params_present_flag, which is contradict to the current semantics.

To fix the above problems, it is proposed to change the semantics of sps_sublayer_dpb_params_flag and vps_sublayer_dpb_params_present_flag as follows, with addition highlighted, and deletion strike through:

sps_sublayer_dpb_params_flag is used to control the presence of max_dec_pic_buffering_minus1[ i ], max_num_reorder_pics[ i ], and max_latency_increase_plus1[ i ] syntax elements in the dpb_parameters( ) syntax strucure in the SPS for i in range from 0 to sps_max_sublayers_minus1 - 1, inclusive, when sps_max_sublayers_minus1 is larger than 0. When not present, the value of sps_sublayer_dpb_params_flag is inferred to be equal to 0.

vps_sublayer_dpb_params_present_flag is used to control the presence of max_dec_pic_buffering_minus1[ j ], max_num_reorder_pics[ j ], and max_latency_increase_plus1[ j ] syntax elements in the dpb_parameters( ) syntax strucure in the VPS for j in range from 0 to vps_dpb_max_tid[ i ] - 1, inclusive, when vps_dpb_max_tid[ i ] in VPS is larger than 0. When not present, vps_sub_dpb_params_info_present_flag is inferred to be equal to 0. It is noted that j instead of i is used for specifying the range of syntax element array max_dec_pic_buffering_minus1[  ], max_num_reorder_pics[  ], and max_latency_increase_plus1[  ] in order to avoid confusion of the i in vps_dpb_max_tid[ i ]

The underlined text is optional for corrected semantics of vps_sublayer_dpb_params_present_flag. It is suggested as editor’s choice.

## Derivation of intermediate variables for inter-layer information.

The variables dependencyFlag[ i ][ j ], NumDirectRefLayers[ i ], DirectRefLayerIdx[ i ][ d ], NumRefLayers[ i ], RefLayerIdx[ i ][ r ], and LayerUsedAsRefLayerFlag[ j ] are derived as follows:

for( i = 0; i  <=  vps_max_layers_minus1; i++ ) {
	for( j = 0; j  <=  vps_max_layers_minus1; j++ ) {
		dependencyFlag[ i ][ j ] = vps_direct_ref_layer_flag[ i ][ j ]
		for( k = 0; k < i; k++ )
			if( vps_direct_ref_layer_flag[ i ][ k ]  &&  dependencyFlag[ k ][ j ] )
				dependencyFlag[ i ][ j ] = 1
	}
	LayerUsedAsRefLayerFlag[ i ] = 0
}
for( i = 0; i  <=  vps_max_layers_minus1; i++ ) {
	for( j = 0, d = 0, r = 0; j  <=  vps_max_layers_minus1; j++ ) {	(37)
		if( vps_direct_ref_layer_flag[ i ][ j ] ) {
			DirectRefLayerIdx[ i ][ d++ ] = j
			LayerUsedAsRefLayerFlag[ j ] = 1
		}
		if( dependencyFlag[ i ][ j ] )
			RefLayerIdx[ i ][ r++ ] = j
	}
	NumDirectRefLayers[ i ] = d
	NumRefLayers[ i ] = r
}

For a specific layer with index i, its reference layer can only be a lower layer with an index smaller than i. considering this, the above derivation process for these variables can be modified as follows:

for( i = 0; i  <=  vps_max_layers_minus1; i++ ) {
	for( j = 0; j  < i <=  vps_max_layers_minus1; j++ ) {
		dependencyFlag[ i ][ j ] = vps_direct_ref_layer_flag[ i ][ j ]
		for( k = j+10; k < i; k++ )
			if( vps_direct_ref_layer_flag[ i ][ k ]  &&  dependencyFlag[ k ][ j ] )
				dependencyFlag[ i ][ j ] = 1
	}
	LayerUsedAsRefLayerFlag[ i ] = 0
}
for( i = 0; i  <=  vps_max_layers_minus1; i++ ) {
	for( j = 0, d = 0, r = 0; j  < i <=  vps_max_layers_minus1; j++ ) {	 (37)
		if( vps_direct_ref_layer_flag[ i ][ j ] ) {
			DirectRefLayerIdx[ i ][ d++ ] = j
			LayerUsedAsRefLayerFlag[ j ] = 1
		}
		if( dependencyFlag[ i ][ j ] )
			RefLayerIdx[ i ][ r++ ] = j
	}
	NumDirectRefLayers[ i ] = d
	NumRefLayers[ i ] = r
}

For derivation of indirect reference layers between the i-th and j-th layer, only a layer with index k between j and i would be possible to make (vps_direct_ref_layer_flag[ i ][ k ]  &&  dependencyFlag[ k ][ j ]) to be true, therefore, the starting index of k is modified to j+1

It is asserted that the specification benefits from this modification in several aspects. First, it simplifies the derivation logic. Second, the derivation design of the for loop “for( j = 0; j  < i; j++ )” is aligned with the signaling of vps_direct_ref_layer_flag[ i ][ j ] in VPS, as shown below. Last and most importantly, it expresses the design intent (only a lower layer can be a reference layer) more preciously.

| … |  |
| --- | --- |
| for( j = 0; j < i; j++ ) { |  |
| vps_direct_ref_layer_flag[ i ][ j ] | u(1) |
| … |  |

# Conclusions

This contribution proposes two bug fixes/cleanups in high level syntax. It is asserted that the proposed aspects improve the clarity and readability of current specification, by either fixing semantic bugs or expressing the design intent more explicitly. It is suggested to adopt the proposed modifications to the next version of VVC specification.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- “AHG2: Editorial input of a text integration for the May 2020 HLS AHG meeting outcome”, JVET-S0152-v5, B. Bross, J. Chen, S. Liu, Y.-K. Wang