| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0178-v2 |
| --- | --- |

| Title: | AHG9: General SEI semantics and constraints |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang1, Zhipin Deng2 18910 University Center Lane San Diego, CA 92122, USA 2Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China | Tel: Email: | yekui.wang@bytedance.com zhipin.deng@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution proposes the following changes to the general SEI semantics and constraints specified in clause D.2.2:

- Fix a missing integration of an agreement (from JVET-Q0394) that specifies that non-scalable-nested HRD-related SEI messages apply to the OLSs that include the same set of layers as the entire bitstream (instead of only to the 0th OLS).

- Consequently, add a constraint such that when there is no OLS that includes the set of layers same as the entire bitstream, there shall be no non-scalable-nested SEI message with payloadType equal to 0 (BP), 1 (PT), 130 (DUI), or 203 (SLI).

- And remove the constraint requiring the value of nuh_layer_id for an SEI NAL unit containing non-scalable-nested HRD-related SEI messages to be equal to vps_layer_id[ 0 ]

- , such that the value of nuh_layer_id for such SEI NAL units is unconstrained, same as for the nuh_layer_id for DCI, VPS, AUD, and EOS NAL units.

- Remove the payloadType value 203 (SLI) from the list VclAssociatedSeiList.

- Add the clarification (that was recently added to the HEVC specification as an corrigendum item) on the value ranges of tone_map_id, frame_packing_arrangement_id, knee_function_id, and colour_remap_id, including potential “collisions” of the interpretation for values of these syntax elements.

- Add a constraint such that when general_same_pic_timing_in_all_ols_flag is equal to 1, there shall be no SEI NAL units that contain a scalable-nested SEI message with payloadType equal to 1 (PT).

- Modify an existing constraint, to add the SLI SEI message, such that when an SEI NAL unit contains a non-scalable-nested SEI message with payloadType equal to 0 (BP), 1 (PT), 130 (DUI), or 203 (SLI), the SEI NAL unit shall not contain any other SEI message with payloadType not equal to 0, 1, 130, or 203.

- Modify an existing constraint, to add the SLI SEI message, that when an SEI NAL unit contains a scalable-nested SEI message with payloadType equal to 0 (BP), 1 (PT), 130 (DUI), or 203 (SLI), the SEI NAL unit shall not contain any other SEI message with payloadType not equal to 0, 1, 130, 203, or 133 (scalable nesting).

- Add a constraint such that when an SEI NAL unit contains an SEI message with payloadType equal to 3 (filler payload), the SEI NAL unit shall not contain any other SEI message with payloadType not equal to 3

- Add a constraint such that when an SLI SEI message and a BP SEI message that apply to a particular OLS are present within an AU, the SLI SEI messages shall precede the BP SEI message in decoding order.

# Introduction

The existing general SEI payload semantics, including the general SEI constraints, in the latest VVC draft text has the following issues:

- There is a missing integration of an agreement (from JVET-Q0394) that specifies that non-scalable-nested HRD-related SEI messages apply to the OLSs that include the same set of layers as the entire bitstream.

- When this is done, then it needs to be disallowed to have non-scalable-nested HRD-related SEI message when there is no OLS that includes the set of layers same as the entire bitstream.

- When this is done, also t

he following constraint needs to be removed: The value of nuh_layer_id for an SEI NAL unit containing non-scalable-nested HRD-related SEI messages shall be equal to vps_layer_id[ 0 ].

- Removal of this constraint means that the value of nuh_layer_id for SEI NAL units containing non-scalable-nested HRD-related SEI messages is unconstrained, same as for the nuh_layer_id for DCI, VPS, AUD, and EOS NAL units.

- The list variable VclAssociatedSeiList currently consists of the SEI payloadType values for non-HRD-related SEI messages. However, the SLI SEI message is also an HRD-related SEI message. Therefore, the payloadType value 203 (SLI) should be removed from the list.

- The clarification on the value ranges of tone_map_id, frame_packing_arrangement_id, knee_function_id, and colour_remap_id, etc., including potential “collisions” of the interpretation for values of these syntax elements, recently added to the HEVC specification as an corrigendum item, should be added.

- When general_same_pic_timing_in_all_ols_flag is equal to 1, it does not make sense to have PT SEI messages contained in scalable nesting SEI messages. Therefore, it should constrained such that, when general_same_pic_timing_in_all_ols_flag is equal to 1, there shall be no SEI NAL units that contain a scalable-nested SEI message with payloadType equal to 1 (PT).

- There lacks a constraint such that when an SEI NAL unit contains a non-scalable-nested SEI message with payloadType equal to 0 (BP), 1 (PT), 130 (DUI), or 203 (SLI), the SEI NAL unit shall not contain any other SEI message with payloadType not equal to 0, 1, 130, or 203. Only with this, removal of these four HRD-related SEI messages from the output bitstream during a sub-bitstream extraction process can be performed by simply removing SEI NAL units containing one or more of these SEI messages.

- There lacks a constraint such that when an SEI NAL unit contains a scalable-nested SEI message with payloadType equal to 0 (BP), 1 (PT), 130 (DUI), or 203 (SLI), the SEI NAL unit shall not contain any other SEI message with payloadType not equal to 0, 1, 130, 203, or 133 (scalable nesting). Only with this, it is possible to set the value of sn_ols_flag of scalable nesting SEI messages for scalable-nested HRD-related and non-HRD-related SEI messages without a problem.

- There lacks a constraint such that when an SEI NAL unit contains an SEI message with payloadType equal to 3 (filler payload), the SEI NAL unit shall not contain any other SEI message with payloadType not equal to 3. Only with this, removal of filler payload SEI messages from the output bitstream during a sub-bitstream extraction process can be performed by simply removing SEI NAL units containing one or more filler payload SEI messages.

- There lacks a constraint such that when an SLI SEI message and a BP SEI message that apply to a particular OLS are present within an AU, the SLI SEI messages shall precede the BP SEI message in decoding order. This is needed because an SLI SEI message provides sequence level information like VPSs and SPSs, which also precede BP SEI messages, when present within the same AU as the BP SEI messages.

# Proposal

The proposal for addressing the issues above is summarized as above in the abstract. The proposed text changes are as follows, where added parts are highlighted in turquoise, and deleted parts are highlighted in yellow strikethrough in red fonts:

#### D.2.2	General SEI payload semantics

...

The list VclAssociatedSeiList is set to consist of the payloadType values 3, 19, 45, 129, 132, 137, 144, 145, 147 to 150, inclusive, 153 to 156, inclusive, 168, 203, and 204.

The list PicUnitRepConSeiList is set to consist of the payloadType values 0, 1, 19, 45, 129, 132, 133, 137, 147 to 150, inclusive, 153 to 156, inclusive, 168, 203, and 204.

NOTE 4 – VclAssociatedSeiList consists of the payloadType values of the SEI messages that, when non-scalable-nested and contained in an SEI NAL unit, infer constraints on the NAL unit header of the SEI NAL unit on the basis of the NAL unit header of the associated VCL NAL unit. PicUnitRepConSeiList consists of the payloadType values of the SEI messages that are subject to the restriction on 4 repetitions per PU.

It is a requirement of bitstream conformance that the following restrictions apply on containing of SEI messages in SEI NAL units:

–	When general_same_pic_timing_in_all_ols_flag is equal to 1, there shall be no SEI NAL unit that contain a scalable-nested SEI message with payloadType equal to 1 (PT).

–	When an SEI NAL unit contains a non-scalable-nested SEI message with payloadType equal to 0 (BP), 1 (PT), or 130 (DUI), or 203 (SLI), the SEI NAL unit shall not contain any other SEI message with payloadType not equal to 0, 1, or 130, or 203.

–	When an SEI NAL unit contains a scalable-nested SEI message with payloadType equal to 0 (BP), 1 (PT), or 130 (DUI), or 203 (SLI), the SEI NAL unit shall not contain any other SEI message with payloadType not equal to 0, 1, 130, 203, or 133 (scalable nesting).

–	When an SEI NAL unit contains an SEI message with payloadType equal to 3 (filler payload), the SEI NAL unit shall not contain any other SEI message with payloadType not equal to 3.

The following applies on the applicable OLSs or layers of non-scalable-nested SEI messages:

–	For a non-scalable-nested SEI message, when payloadType is equal to 0 (BP), 1 (PT), or 130 (DUI), or 203 (SLI), the non-scalable-nested SEI message applies to the OLSs, when present, that include the same set of layers as in the entire bitstream only to the 0-th OLS. When there is no OLS that includes the same set of layers as the entire bitstream, there shall be no non-scalable-nested SEI message with payloadType equal to 0 (BP), 1 (PT), 130 (DUI), or 203 (SLI).

–	For a non-scalable-nested SEI message, when payloadType is equal to any value among VclAssociatedSeiList, the non-scalable-nested SEI message applies only to the layer for which the VCL NAL units have nuh_layer_id equal to the nuh_layer_id of the SEI NAL unit containing the SEI message.

It is a requirement of bitstream conformance that the following restrictions apply on the value of nuh_layer_id of SEI NAL units:

–	When a non-scalable-nested SEI message has payloadType equal to 0 (BP), 1 (PT), or 130 (DUI), the SEI NAL unit containing the non-scalable-nested SEI message shall have nuh_layer_id equal to vps_layer_id[ 0 ].

–	When a non-scalable-nested SEI message has payloadType equal to any value among VclAssociatedSeiList, the SEI NAL unit containing the non-scalable-nested SEI message shall have nuh_layer_id equal to the value of nuh_layer_id of the VCL NAL unit associated with the SEI NAL unit.

–	An SEI NAL unit containing a scalable nesting SEI message shall have nuh_layer_id equal to the lowest value of nuh_layer_id of all layers to which the scalable-nested SEI messages apply (when sn_ols_flag of the scalable nesting SEI message is equal to 0) or the lowest value of nuh_layer_id of all layers in the OLSs to which the scalable-nested SEI message apply (when sn_ols_flag of the scalable nesting SEI message is equal to 1).

NOTE 4 – Same as for DCI, VPS, AUD, and EOB NAL units, the value of nuh_layer_id for SEI NAL units that contain non-scalable-nested SEI messages with payloadType equal to 0 (BP), 1 (PT), or 130 (DUI), or 203 (SLI) is not constrained.

It is a requirement of bitstream conformance that the following restrictions apply on repetition of SEI messages:

–	For each of the payloadType values included in PicUnitRepConSeiList, there shall be less than or equal to 4 identical sei_payload( ) syntax structures within a PU.

–	There shall be less than or equal to 4 identical sei_payload( ) syntax structures with payloadType equal to 130 within a DU.

The following applies on the order of SLI, BP, PT, and DUI SEI messages:

–	When an SLI SEI message and a BP SEI message that apply to a particular OLS are present within an AU, the SLI SEI messages shall precede the BP SEI message in decoding order.

–	When a BP SEI message and a PT SEI message that apply to a particular OPOLS are present within an AU, the BP SEI messages shall precede the PT SEI message in decoding order.

–	When a BP SEI message and a DUI SEI message that apply to a particular OPOLS are present within an AU, the BP SEI messages shall precede the DUI SEI message in decoding order.

–	When a PT SEI message and a DUI SEI message that apply to a particular OPOLS are present within an AU, the PT SEI messages shall precede the DUI SEI message in decoding order.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).