| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0180_r1 |
| --- | --- |

| Title: | Addition of a GOP hierarchy of 32 for random access configuration for VTM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kenneth Andersson, Jack Enhorn, Rickard Sjöberg, Jacob Ström, Lukasz Litwic | Tel: Email: | +46730371390 kenneth.r.andersson@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

It is proposed to include a GOP hierarchy of 32 for random access configuration to VTM and it is also suggested to use it for CTC. This will increase the number of pictures in the DPB by one compared to a GOP hierarchy of 16 such that the DPB requirements are 8, e.g. at the maxDpbPicBuf, instead of 7. It will also increase the end-to-end latency by additional 2x16/60 seconds for 60 Hz video which is expected to be acceptable for a wide range of random access applications. Enabling a GOP hierarchy of 32 also for sequences which not have an intra period which is a multiple of 32 requires adjustment of the intra period for those sequences such that the intra period becomes a multiple of 32, e.g. frame rate 50/60Hz will have an intra period of 64 and frame rate 30Hz or below will have an intra period of 32.

The BD rate effect of this change is as follows:

CTC SDR RA -3.0%, -6.3%, -6.4% (luma,Cb/Cr)

CTC HDR RA -7.8%, -5.2, -4.4%, -4.2% (deltaE, psnrL, wpsnrY, psnrY)

It is also asserted that the visual quality is improved.

# Introduction

In JVET-B0039 the benefits of a GOP hierarchy of 32 compared to GOP hierarchy of 16 was shown. An informal viewing was also held at the meeting and reported in JVET-B0075. The results indicated visual benefits of using a GOP hierarchy of 32 compared to a GOP hierarchy of 8. The doubling of the memory for the encoder compared to a GOP hierarchy of 16 was at the time (year 2016) regarded as an issue and a GOP hierarchy of 16 was instead selected for CTC. However, we argue that these memory constraints are more relaxed now, more than four years later. At a recent meeting the maxDpbPicBuf was also increased for VVC to 8 pictures which gives additional capability that can be used by a larger GOP hierarchy for random access.

# Proposal

This proposal follows that of JVET-B0039 which here has been aligned to VTM-9.0.

GOPSize                       : 32      # GOP Size (number of B slice = GOPSize-1)

IntraQPOffset                 : -3

LambdaFromQpEnable            : 1       # see JCTVC-X0038 for suitable parameters for IntraQPOffset, QPoffset, QPOffsetModelOff, QPOffsetModelScale when enabled

#        Type POC QPoffset QPOffsetModelOff QPOffsetModelScale CbQPoffset CrQPoffset QPfactor tcOffsetDiv2 betaOffsetDiv2 CbTcOffsetDiv2 CbBetaOffsetDiv2 CrTcOffsetDiv2 CrBetaOffsetDiv2 temporal_id #ref_pics_active_L0 #ref_pics_L0   reference_pictures_L0 #ref_pics_active_L1 #ref_pics_L1   reference_pictures_L1

Frame1: B    32  -1          0.0        0.0        0    0    1.0      0  0      0   0      0     0    0   2      3      32 48 64    2    2     32 48

Frame2: B    16   0        -4.9309  0.2265   0    0    1.0      0  0      0   0      0     0    1   2      2      16 32         2    2     -16 16

Frame3: B      8   0        -4.5000  0.2353   0    0    1.0      0  0      0   0      0     0    2   2      2      8 24           2    2     -8 -24

Frame4: B      4   3        -5.4095  0.2571   0    0    1.0      0  0      0   0      0     0    3   2      2      4 20           2    3     -4 -12 -28

Frame5: B      2   5        -4.4895  0.1947   0    0    1.0      0  0      0   0      0     0    4   2      2      2 18           2    4     -2 -6 -14 -30

Frame6: B      1   6        -5.4429  0.2429   0    0    1.0      0  0      0   0      0     0    5   2      2      1 -1            2    5     -1 -3 -7 -15 -31

Frame7: B      3   6        -5.4429  0.2429   0    0    1.0      0  0      0   0      0     0    5   2      2      1 3             2    4     -1 -5 -13 -29

Frame8: B      6   5        -4.4895  0.1947   0    0    1.0      0  0      0   0      0     0    4   2      2      2 6             2    3     -2 -10 -26

Frame9: B      5   6        -5.4429  0.2429   0    0    1.0      0  0      0   0      0     0    5   2      2      1 5             2    4     -1 -3 -11 -27

Frame10: B    7   6        -5.4429  0.2429   0    0    1.0      0  0      0   0      0     0    5   2      3      1 3 7           2    3     -1 -9 -25

Frame11: B   12   3       -5.4095  0.2571   0    0    1.0      0  0      0   0      0     0    3   2      2      4 12            2    2     -4 -20

Frame12: B   10   5       -4.4895  0.1947   0    0    1.0      0  0      0   0      0     0    4   2      2      2 10            2    3     -2 -6 -22

Frame13: B    9    6       -5.4429  0.2429   0    0    1.0      0  0      0   0      0     0    5   2      2      1 9              2    4     -1 -3 -7 -23

Frame14: B   11   6       -5.4429  0.2429   0    0    1.0      0  0      0   0      0     0    5   2      3      1 3 11          2    3     -1 -5 -21

Frame15: B   14   5       -4.4895  0.1947   0    0    1.0      0  0      0   0      0     0    4   2      3      2 6 14          2    2     -2 -18

Frame16: B   13   6       -5.4429  0.2429   0    0    1.0      0  0      0   0      0     0    5   2      3      1 5 13          2    3     -1 -3 -19

Frame17: B   15   6       -5.4429  0.2429   0    0    1.0      0  0      0   0      0     0    5   2      4      1 3 7 15       2    2     -1 -17

Frame18: B   24   0       -4.5000  0.2353   0    0    1.0      0  0      0   0      0     0    2   2      2      8 24             2    2     -8 8

Frame19: B   20   3       -5.4095  0.2571   0    0    1.0      0  0      0   0      0     0    3   2      2      4 20             2    2     -4 -12

Frame20: B   18   5       -4.4895  0.1947   0    0    1.0      0  0      0   0      0     0    4   2      2      2 18             2    3     -2 -6 -14

Frame21: B   17   6       -5.4429  0.2429   0    0    1.0      0  0      0   0      0     0    5   2      2      1 17             2    4     -1 -3 -7 -15

Frame22: B   19   6       -5.4429  0.2429   0    0    1.0      0  0      0   0      0     0    5   2      3      1 3 19           2    3     -1 -5 -13

Frame23: B   22   5       -4.4895  0.1947   0    0    1.0      0  0      0   0      0     0    4   2      3      2 6 22           2    2     -2 -10

Frame24: B   21   6       -5.4429  0.2429   0    0    1.0      0  0      0   0      0     0    5   2      3      1 5 21           2    3     -1 -3 -11

Frame25: B   23   6       -5.4429  0.2429   0    0    1.0      0  0      0   0      0     0    5   2      4      1 3 7 23         2    2     -1 -9

Frame26: B   28   3       -5.4095  0.2571   0    0    1.0      0  0      0   0      0     0    3   2      3      4 12 28          2    2     -4 4

Frame27: B   26   5       -4.4895  0.1947   0    0    1.0      0  0      0   0      0     0    4   2      3      2 10 26          2    2     -2 -6

Frame28: B   25   6       -5.4429  0.2429   0    0    1.0      0  0      0   0      0     0    5   2      3      1 9 25            2    3     -1 -3 -7

Frame29: B   27   6       -5.4429  0.2429   0    0    1.0      0  0      0   0      0     0    5   2      4      1 3 11 27       2    2     -1 -5

Frame30: B   30   5       -4.4895  0.1947   0    0    1.0      0  0      0   0      0     0    4   2      3      2 14 30          2    2     -2 2

Frame31: B   29   6       -5.4429  0.2429   0    0    1.0      0  0      0   0      0     0    5   2      3      1 13 29          2    2     -1 -3

Frame32: B   31   6       -5.4429  0.2429   0    0    1.0      0  0      0   0      0     0    5   2      4      1 3 15 31       2    2     -1 1

The impact of extending the length of the GOP hierarchy from 16 to 32 are:

- The number of temporal layers increases from 5 to 6,

- The DPB size increases from 7 to 8

- An increase of the maximum reorder latency from 16 to 32 pictures which corresponds to an end-to-end (encoder + decoder) latency of 2x32/60 instead of 2x16/60 for a frame rate of 60 Hz.

Since most current and future content is expected to be at least 60 Hz, the added latency may be found acceptable for several broadcast, streaming and offline video delivery applications.

# Results

## Subjective performance

The proposed GOP hierarchy of 32 pictures makes predictions more stable and also give less intra flickering.

## Objective performance

The objective performance (BDR) for the proposal is shown below in comparison to VTM-9.0 [2] using CTC [3] with modification of intra period to be a multiple of 32. The objective results show a nice benefit in coding efficiency. Encoding and decoding time is measured on simulations run on a cluster with machines with similar capability.

Additionally, we also provide results for same setup for HM-16.20 [4], where a similar benefit can be obtained but that would require a higher level than 5.1.

### VTM-9.0

Table 1: BDR for a GOP hierarchy of 32 for SDR CTC VTM-9.0 random access configuration

|  | Over VTM-9.0 (Intra period 64 for 50Hz and 32 for 20Hz) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1,94% | -4,82% | -5,04% | 100% | 94% |
| Class A2 | -3,10% | -6,84% | -6,06% | 100% | 97% |
| Class B | -3,31% | -7,22% | -7,59% | 99% | 96% |
| Class C | -3,37% | -5,95% | -6,13% | 99% | 92% |
| Class E |  |  |  |  |  |
| Overall | -3,01% | -6,33% | -6,38% | 99% | 95% |
| Class D | -3,27% | -5,94% | -6,21% | 100% | 94% |
| Class F | -6,45% | -7,37% | -7,24% | 96% | 93d% |

Table 2: BDR for a GOP hierarchy of 32 for HDR CTC VTM-9.0 random access configuration

|  | Random Access |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-9.0 (50Hz with intra period 64) |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 |  | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -7,84% |  | -5,18% | -4,41% | -8,72% | -8,91% | -4,24% | -8,04% | -8,37% | 100% | 100% |
| Class H2 |  |  |  |  |  |  | -4,00% | -6,35% | -5,98% | 99% | 103% |
| Overall | -7,84% | -5,18% |  | -4,41% | -8,72% | -8,91% | -4,15% | -7,43% | -7,50% | 99% | 101% |

Table 3: BDR for a GOP hierarchy of 32 for some additional SDR sequences VTM-9.0 random access configuration (QP 30, 34, 38, 42)

|  | PSNR_Y | PSNR_Cb | PSNR_Cr |
| --- | --- | --- | --- |
| Marathon_3840x2160_30fps_8bit | -5,46% | -10,05% | -9,42% |
| Netflix_DrivingPOV_4096x2160_60 | -4,76% | -12,31% | -8,75% |
| MountainBay2_3840x2160_30fps_42 | -8,45% | -18,17% | -17,65% |
| NeptuneFountain3_3840x2160_60fp | -1,13% | -7,16% | -7,88% |
| TallBuildings_3840x2160_30fps_8 | -9,26% | -11,55% | -8,05% |
| Runners_3840x2160_30fps_8bit_42 | -7,90% | -11,19% | -10,45% |

### HM-16.20

Table 4: BDR for a GOP hierarchy of 32 for SDR HM-16.20 CTC random access configuration

|  | Over VTM-9.0 (Intra period 64 for 50Hz and 32 for 20Hz) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1,11% | -4,56% | -4,63% | 105% | 94% |
| Class A2 | -1,67% | -4,62% | -4,82% | 101% | 87% |
| Class B | -2,57% | -7,88% | -7,37% | 97% | 82% |
| Class C | -3,31% | -5,65% | -5,52% | 101% | 87% |
| Class E |  |  |  |  |  |
| Overall | -2,30% | -5,97% | -5,82% | 100% | 86% |
| Class D | -3,17% | -5,78% | -5,63% | 103% | 87% |
| Class F | -6,06% | -6,86% | -6,69% | 97% | 82% |

Table 5: BDR for a GOP hierarchy of 32 for HDR HM-16.20 CTC random access configuration

|  |  | Random Access |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-9.0 (50Hz with intra period 64) |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 |  |  | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 |  |  |  |  |  |  |  | -3,77% | -9,70% | -13,77% | 104% | 100% |
| Class H2 |  |  |  |  |  |  |  | -5,31% | -6,18% | -6,31% | 105% | 108% |
| Overall |  |  |  |  |  |  |  | -4,33% | -8,42% | -11,06% | 104% | 103% |

# Conclusions

We propose to at least add an additional configuration file for random access enabling a GOP hierarchy of 32 for VTM but preferably also update the CTC to also use that by default.

# References

[1] K. Andersson,  P. Wennersten, R. Sjöberg, J. Samuelsson, J. Ström, P. Hermansson, M. Pettersson, ”Non-normative JEM encoder improvements”, JVET-B0039, San Diego, USA, February 2016

[2] VTM-9.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-9.0

[3] F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations”, JVET-N1010

[4] HM-16.20, available at https://vcgit.hhi.fraunhofer.de/jct-vc/HM/-/tags/HM-16.20

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).