| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0183-v1 |
| --- | --- |

| Title: | AHG9: On VPS Information Signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sachin Deshpande Jonatan Samuelsson Andrew Segall Philip Cowan | Tel: Email: | +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

Modifications are proposed related to VPS information signalling.

Following is proposed:

- Proposal 1: It is proposed to signal the syntax element vps_num_output_layer_sets_minus1 instead as vps_num_output_layer_sets_minus2. It is asserted that vps_num_output_layer_sets_minus1 is signalled when vps_max_layers_minus1 is greater than 0 (and when vps_ols_mode_idc is equal to 2) and in this case since the 0-th OLS has the lowest layer, there are at least two output layer sets.

- Proposal 2: It is proposed to signal a flag for each multi-layer OLS (except the first one) to indicate if its OLS DPB picture width, height, chroma format and bit depth values are same as the values for the previous OLS. It is asserted that this can save bits in common cases since many of these syntax elements are coded as ue(v).

# Introduction

VVC WD [1] includes video parameter set which provides among other things, a syntax element which signals total number of OLSs (vps_num_output_layer_sets_minus1). Additionally there is information about output layer sets’ DPB picture width, height, chroma format and bit-depth.

Following is proposed:

- Proposal 1: It is proposed to signal the syntax element vps_num_output_layer_sets_minus1 instead as vps_num_output_layer_sets_minus2. It is asserted that vps_num_output_layer_sets_minus1 is signalled when vps_max_layers_minus1 is greater than 0 (and when vps_ols_mode_idc is equal to 2) and in this case since the 0-th OLS has the lowest layer, there are at least two output layer sets.

- Proposal 2: It is proposed to signal a flag for each multi-layer OLS (except the first one) to indicate if its OLS DPB picture width, height, chroma format and bit depth values are same as the values for the previous OLS. It is asserted that this can save bits in common cases since many of these syntax elements are coded as ue(v).

# Proposal

## Proposal 1

It is proposed to signal the syntax element vps_num_output_layer_sets_minus1 instead as vps_num_output_layer_sets_minus2. It is asserted that vps_num_output_layer_sets_minus1 is signalled when vps_max_layers_minus1 is greater than 0 (and when vps_ols_mode_idc is equal to 2) and in this case since the 0-th OLS has the lowest layer, there are at least two output layer sets. Thus it is asserted that the number of output layer sets can be signalled with a minus 2 coding. Since currently the syntax element is already coded with an offset (of minus 1), it is proposed to actually code it with a correct offset of minus 2, since the value of 0 and 1 both are not allowed for this syntax element.

This allows signalling one more output layer sets than currently supported in VVC Draft text.

The proposed changes are shown highlighted below compared to VVC WD (JVET-S0152).

#### 7.3.2.2 Video parameter set RBSP syntax

| video_parameter_set_rbsp( ) { | Descriptor |  |
| --- | --- | --- |
| vps_video_parameter_set_id | u(4) |  |
| vps_max_layers_minus1 | u(6) |  |
| vps_max_sublayers_minus1 | u(3) |  |
| if( vps_max_layers_minus1 > 0 && vps_max_sublayers_minus1 > 0 ) |  |  |
| vps_default_ptl_dpb_hrd_max_tid_flag | u(1) |  |
| if( vps_max_layers_minus1 > 0 ) |  |  |
| vps_all_independent_layers_flag | u(1) |  |
| for( i = 0; i <= vps_max_layers_minus1; i++ ) { |  |  |
| vps_layer_id[ i ] | u(6) |  |
| if( i > 0 && !vps_all_independent_layers_flag ) { |  |  |
| vps_independent_layer_flag[ i ] | u(1) |  |
| if( !vps_independent_layer_flag[ i ] ) { |  |  |
| vps_max_tid_ref_present_flag[ i ] | u(1) |  |
| for( j = 0; j < i; j++ ) { |  |  |
| vps_direct_ref_layer_flag[ i ][ j ] | u(1) |  |
| if( vps_max_tid_ref_present_flag[ i ] && vps_direct_ref_layer_flag[ i ][ j ] ) |  |  |
| vps_max_tid_il_ref_pics_plus1[ i ][ j ] | u(3) |  |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| if( vps_max_layers_minus1 > 0 ) { |  |  |
| if( vps_all_independent_layers_flag ) |  |  |
| vps_each_layer_is_an_ols_flag | u(1) |  |
| if( !vps_each_layer_is_an_ols_flag ) { |  |  |
| if( !vps_all_independent_layers_flag ) |  |  |
| vps_ols_mode_idc | u(2) |  |
| if( vps_ols_mode_idc = = 2 ) { |  |  |
| vps_num_output_layer_sets_minus12 | u(8) |  |
| for( i = 1; i <= vps_num_output_layer_sets_minus12 + 1; i ++) |  |  |
| for( j = 0; j <= vps_max_layers_minus1; j++ ) |  |  |
| vps_ols_output_layer_flag[ i ][ j ] | u(1) |  |
| } |  |  |
| } |  |  |
| vps_num_ptls_minus1 | u(8) |
| } |  |  |
| for( i = 0; i <= vps_num_ptls_minus1; i++ ) { |  |  |
| if( i > 0 ) |  |  |
| vps_pt_present_flag[ i ] | u(1) |  |
| if( !vps_default_ptl_dpb_hrd_max_tid_flag ) |  |  |
| vps_ptl_max_tid[ i ] | u(3) |  |
| } |  |  |
| while( !byte_aligned( ) ) |  |  |
| vps_ptl_alignment_zero_bit /* equal to 0 */ | f(1) |  |
| … |  |  |
| } |  |  |

vps_num_output_layer_sets_minus12 plus 12 specifies the total number of OLSs specified by the VPS when vps_ols_mode_idc is equal to 2.

The variable olsModeIdc derived as follows:

if( !vps_each_layer_is_an_ols_flag )
	olsModeIdc = vps_ols_mode_idc		(39)
else
	olsModeIdc = 4

The variable TotalNumOlss, specifying the total number of OLSs specified by the VPS, is derived as follows:

if( vps_max_layers_minus1  = =  0 )
	TotalNumOlss = 1
else if( olsModeIdc  = =  4  | |  olsModeIdc  = =  0  | |  olsModeIdc  = =  1 )
	TotalNumOlss = vps_max_layers_minus1 + 1	(40)
else if( vps_ols_mode_idc  = =  2 )
	TotalNumOlss = vps_num_output_layer_sets_minus12 + 12

Alternatively a  conformance constraint is proposed as follows:

It is a requirement of bitstream conformance that vps_num_output_layer_sets_minus1 shall be greater than 0.

## Proposal 2

It is asserted that in some typical cases (e.g. multi-view) values of OLS DPB picture width, height, chroma format and bit depth may be same for multiple OLSs. It is proposed to signal a flag for each multi-layer OLS (except the first multilayer OLS) to indicate if OLS DPB picture width, height, chroma format and bit depth values are the same as the corresponding values for the previous OLS.  It is asserted that this can save bits in common cases.

| video_parameter_set_rbsp( ) { | Descriptor |  |
| --- | --- | --- |
| vps_video_parameter_set_id | u(4) |  |
| vps_max_layers_minus1 | u(6) |  |
| vps_max_sublayers_minus1 | u(3) |  |
| if( vps_max_layers_minus1 > 0 && vps_max_sublayers_minus1 > 0 ) |  |  |
| vps_default_ptl_dpb_hrd_max_tid_flag | u(1) |  |
| if( vps_max_layers_minus1 > 0 ) |  |  |
| vps_all_independent_layers_flag | u(1) |  |
| for( i = 0; i <= vps_max_layers_minus1; i++ ) { |  |  |
| vps_layer_id[ i ] | u(6) |  |
| if( i > 0 && !vps_all_independent_layers_flag ) { |  |  |
| vps_independent_layer_flag[ i ] | u(1) |  |
| if( !vps_independent_layer_flag[ i ] ) { |  |  |
| vps_max_tid_ref_present_flag[ i ] | u(1) |  |
| for( j = 0; j < i; j++ ) { |  |  |
| vps_direct_ref_layer_flag[ i ][ j ] | u(1) |  |
| if( vps_max_tid_ref_present_flag[ i ] && vps_direct_ref_layer_flag[ i ][ j ] ) |  |  |
| vps_max_tid_il_ref_pics_plus1[ i ][ j ] | u(3) |  |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| if( vps_max_layers_minus1 > 0 ) { |  |  |
| if( vps_all_independent_layers_flag ) |  |  |
| vps_each_layer_is_an_ols_flag | u(1) |  |
| if( !vps_each_layer_is_an_ols_flag ) { |  |  |
| if( !vps_all_independent_layers_flag ) |  |  |
| vps_ols_mode_idc | u(2) |  |
| if( vps_ols_mode_idc = = 2 ) { |  |  |
| vps_num_output_layer_sets_minus1 | u(8) |  |
| for( i = 1; i <= vps_num_output_layer_sets_minus1; i ++) |  |  |
| for( j = 0; j <= vps_max_layers_minus1; j++ ) |  |  |
| vps_ols_output_layer_flag[ i ][ j ] | u(1) |  |
| } |  |  |
| } |  |  |
| vps_num_ptls_minus1 | u(8) |
| } |  |  |
| for( i = 0; i <= vps_num_ptls_minus1; i++ ) { |  |  |
| if( i > 0 ) |  |  |
| vps_pt_present_flag[ i ] | u(1) |  |
| if( !vps_default_ptl_dpb_hrd_max_tid_flag ) |  |  |
| vps_ptl_max_tid[ i ] | u(3) |  |
| } |  |  |
| while( !byte_aligned( ) ) |  |  |
| vps_ptl_alignment_zero_bit /* equal to 0 */ | f(1) |  |
| for( i = 0; i <= vps_num_ptls_minus1; i++ ) |  |  |
| profile_tier_level( vps_pt_present_flag[ i ], vps_ptl_max_tid[ i ] ) |  |
| for( i = 0; i < TotalNumOlss; i++ ) |  |  |
| if( vps_num_ptls_minus1 > 0 && vps_num_ptls_minus1 + 1 != TotalNumOlss ) |  |  |
| vps_ols_ptl_idx[ i ] | u(8) |  |
| if( !vps_each_layer_is_an_ols_flag ) { |  |  |
| vps_num_dpb_params_minus1 | ue(v) |
| if( vps_max_sublayers_minus1 > 0 ) |  |  |
| vps_sublayer_dpb_params_present_flag | u(1) |  |
| for( i = 0; i < VpsNumDpbParams; i++ ) { |  |  |
| if( !vps_default_ptl_dpb_hrd_max_tid_flag ) |  |  |
| vps_dpb_max_tid[ i ] | u(3) |  |
| dpb_parameters( vps_dpb_max_tid[ i ], vps_sublayer_dpb_params_present_flag ) |  |
| } |  |
| for( i = 0; i < NumMultiLayerOlss; i++ ) { |  |  |
| if( i > 0 ) |  |  |
| vps_ols_representation_format_info_signalled_flag[ i ] | u(1) |  |
| if(vps_ols_representation_format_info_signalled_flag[ i ]) { |  |  |
| vps_ols_dpb_pic_width[ i ] | ue(v) |  |
| vps_ols_dpb_pic_height[ i ] | ue(v) |  |
| vps_ols_dpb_chroma_format[ i ] | u(2) |  |
| vps_ols_dpb_bitdepth_minus8[ i ] | ue(v) |  |
| } |  |  |
| if( VpsNumDpbParams > 1 && VpsNumDpbParams != NumMultiLayerOlss ) |  |  |
| vps_ols_dpb_params_idx[ i ] | ue(v) |  |
| } |  |  |
| vps_general_hrd_params_present_flag | u(1) |  |
| if( vps_general_hrd_params_present_flag ) { |  |  |
| general_hrd_parameters( ) |  |  |
| if( vps_max_sublayers_minus1 > 0 ) |  |  |
| vps_sublayer_cpb_params_present_flag | u(1) |  |
| vps_num_ols_hrd_params_minus1 | ue(v) |  |
| for( i = 0; i <= vps_num_ols_hrd_params_minus1; i++ ) { |  |  |
| if( !vps_default_ptl_dpb_hrd_max_tid_flag ) |  |  |
| vps_hrd_max_tid[ i ] | u(3) |  |
| firstSubLayer = vps_sublayer_cpb_params_present_flag ? 0 : vps_hrd_max_tid[ i ] |  |  |
| ols_hrd_parameters( firstSubLayer, vps_hrd_max_tid[ i ] ) |  |  |
| } |  |  |
| if( vps_num_ols_hrd_params_minus1 > 0 && vps_num_ols_hrd_params_minus1 + 1 != NumMultiLayerOlss ) |  |  |
| for( i = 0; i < NumMultiLayerOlss; i++ ) |  |  |
| vps_ols_hrd_idx[ i ] | ue(v) |  |
| } |  |  |
| } |  |  |
| vps_extension_flag | u(1) |  |
| if( vps_extension_flag ) |  |  |
| while( more_rbsp_data( ) ) |  |  |
| vps_extension_data_flag | u(1) |  |
| rbsp_trailing_bits( ) |  |  |
| } |  |  |

vps_ols_representation_format_info_signalled_flag[ i ] equal to 1 specifies that syntax elements vps_ols_dpb_pic_width[ i ], vps_ols_dpb_pic_height[ i ], vps_ols_dpb_chroma_format[ i ], and vps_ols_dpb_bitdepth_minus8[ i ] are signalled for the i-th OLS in the VPS. vps_ols_representation_format_info_signalled_flag[ i ] equal to 0 specifies that syntax elements vps_ols_dpb_pic_width[ i ], vps_ols_dpb_pic_height[ i ], vps_ols_dpb_chroma_format[ i ], and vps_ols_dpb_bitdepth_minus8[ i ] are not signalled for the i-th OLS in the VPS. The value of vps_ols_representation_format_info_signalled_flag[ 0 ] is inferred to be equal to 1.

vps_ols_dpb_pic_width[ i ] specifies the width, in units of luma samples, of each picture storage buffer for the i-th multi-layer OLS. When not present vps_ols_dpb_pic_width[ i ] is inferred to be equal to vps_ols_dpb_pic_width[ i −1 ].

vps_ols_dpb_pic_height[ i ] specifies the height, in units of luma samples, of each picture storage buffer for the i-th multi-layer OLS. When not present vps_ols_dpb_pic_height[ i ] is inferred to be equal to vps_ols_dpb_pic_height[ i − 1 ].

vps_ols_dpb_chroma_format[ i ] specifies the greatest allowed value of sps_chroma_format_idc for all SPSs that are referred to by CLVSs in the CVS for the i-th multi-layer OLS. When not present vps_ols_dpb_chroma_format[ i ] is inferred to be equal to vps_ols_dpb_chroma_format[ i − 1 ].

vps_ols_dpb_bitdepth_minus8[ i ] specifies the greatest allowed value of sps_bit_depth_minus8 for all SPSs that are referred to by CLVSs in the CVS for the i-th multi-layer OLS. When not present vps_ols_dpb_bitdepth_minus8[ i ] is inferred to be equal to vps_ols_dpb_bitdepth_minus8[ i − 1 ].

NOTE 2 – For decoding the i-th multi-layer OLS, the deoder can safely allocate memory for the DPB according to the values of the syntax elements vps_ols_dpb_pic_width[ i ], vps_ols_dpb_pic_height[ i ], vps_ols_dpb_chroma_format[ i ], and vps_ols_dpb_bitdepth_minus8[ i ].

# Conclusion

This document proposes modifications to VPS information signalling in VVC. It is proposed to adopt the proposas in the next version of VVC Draft text.

# Patent rights declaration(s)

Sharp Labs of America, Inc., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] JVET-S0152, AHG2: Editorial Input of a text integration for the May 2020 HLS AHG meeting outcome, May 2020