| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0188 |
| --- | --- |

| Title: | AHG9: On gradual decoding refresh |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Stephan Wenger Shan Liu | Email: | bdchoi@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This contribution proposes the following modifications related to GDR picture.

- Add a constraint that the recovering picture shall precede the associated recovery point picture in decoding order.

- Add a constraint that a picture with pps_mixed_nalu_types_in_pic_flag is equal to 1 shall not be the recovering or recovery picture associated with a GDR picture.

- Add a constraint that the value of recoveryPointPocVal of a layer k shall be equal to or greater than recoveryPointPocVal of the reference layer of the layer k.

- Move the syntax element ph_recovery_poc_cnt to be signaled after ph_poc_msb_cycle_val.

# Proposal-1

It is asserted that the recovering picture shall precede the associated recovery point picture in both decoding and output order. However, in the semantics of ph_recovery_poc_cnt, only the following sentence is described as the definition of the recovering picture.

“The pictures that are associated with the current GDR picture and have PicOrderCntVal less than recoveryPointPocVal are referred to as the recovering pictures of the GDR picture.”

Since it is not clearly described that the recovering picture shall precede the recovery point picture in decoding, we propose to add the constraint.

Proposed Spec Text

If the current picture is a GDR picture, and there is a picture picA that follows the current GDR picture in decoding order in the CLVS that has PicOrderCntVal equal to recoveryPointPocVal, the picture picA is referred to as the recovery point picture. Otherwise, the first picture in output order that has PicOrderCntVal greater than recoveryPointPocVal in the CLVS is referred to as the recovery point picture. The recovery point picture shall not precede the current GDR picture in decoding order. The pictures that are associated with the current GDR picture and have PicOrderCntVal less than recoveryPointPocVal are referred to as the recovering pictures of the GDR picture. The recovering picture shall precede the associated recovery point picture in decoding order. The value of ph_recovery_poc_cnt shall be in the range of 0 to MaxPicOrderCntLsb − 1, inclusive.

# Proposal-2

In the current VVC spec, it is disallowed that GDR subpicture is mixed with other subpictures when pps_mixed_nalu_types_in_pic_flag is equal to 1. When two or more subpictures are mixed, the GDR picture cannot correctly work due to the confusion of the recovery point. To make sure this constraint, it is proposed to impose an additional constraint that the picture with pps_mixed_nalu_types_in_pic_flag is equal to 1 shall not be the recovering or recovery picture associated with a GDR picture. When the recovering or recovery subpictures are mixed with other subpictures, the recovery point of the associated GDR picture is confused and conflict with the original value of ph_recovery_poc_cnt. It is clear and desired to disallow that a GDR subpicture and its associated recoverying and recovery pictures are mixed with other subpictures.

Proposed Spec Text

Otherwise (pps_mixed_nalu_types_in_pic_flag is equal to 1), the following applies:

–	The picture shall have at least two subpictures.

–	VCL NAL units of the picture shall have two or more different nal_unit_type values.

–	There shall be no VCL NAL unit of the picture that has nal_unit_type_equal to GDR_NUT.

–	The picture shall not be the recovering or recovery picture associated with a GDR picture.

–	When the VCL NAL units of at least one subpicture of the picture have a particular value of nal_unit_type equal to IDR_W_RADL, IDR_N_LP, or CRA_NUT, the VCL NAL units of other subpictures in the picture shall all have nal_unit_type equal to TRAIL_NUT.

# Proposal-3

When GDR pictures are used with multiple layers, it is proposed to add a constraint that the value of recoveryPointPocVal of a layer k shall be equal to or greater than recoveryPointPocVal of the reference layer of the layer k. If a picture in the reference layer has “dirty area”, the picture that refers to the interlayer reference picture should have “dirty area” as well. So, the recovery point of the current layer cannot precede the recovery point of the reference layer. In terms of use cases, the recovery point of a dependent layer shall be at least the same as that of the independent layer that is referred to as a reference, or the recovery point of a dependent layer shall be present after the recovery point of the referenced independent layer.

Proposed Spec Text

The recoveryPointPocVal of a layer shall be equal to or greater than recoveryPointPocVal of the reference layer of the layer.

# Proposal-4

In the semantics of ph_recovery_poc_cnt, when the current picture is a GDR picture, the variable recoveryPointPocVal is derived as follows:

recoveryPointPocVal = PicOrderCntVal + ph_recovery_poc_cnt

So, it is proposed to move the syntax element ph_recovery_poc_cnt to be signaled after ph_poc_msb_cycle_val, so that the value of PicOrderCntVal is available when parsing ph_recovery_poc_cnt. In the current syntax structure, there is no parsing dependency. However, signaling ph_recovery_poc_cnt after POC-related syntax elements is logical due to the semantics dependency.

Proposed Spec Text

| picture_header_structure( ) { | Descriptor |
| --- | --- |
| ph_gdr_or_irap_pic_flag | u(1) |
| ph_non_ref_pic_flag | u(1) |
| if( ph_gdr_or_irap_pic_flag ) |  |
| ph_gdr_pic_flag | u(1) |
| … |  |
| ph_pic_order_cnt_lsb | u(v) |
| if( ph_gdr_or_irap_pic_flag ) |  |
| ph_no_output_of_prior_pics_flag | u(1) |
| if( ph_gdr_pic_flag ) |  |
| ph_recovery_poc_cnt | ue(v) |
| for( i = 0; i < NumExtraPhBits; i++ ) |  |
| ph_extra_bit[ i ] | u(1) |
| if( sps_poc_msb_cycle_flag ) { |  |
| ph_poc_msb_cycle_present_flag | u(1) |
| if( ph_poc_msb_cycle_present_flag ) |  |
| ph_poc_msb_cycle_val | u(v) |
| } |  |
| if( ph_gdr_pic_flag ) |  |
| ph_recovery_poc_cnt | ue(v) |
| … |  |
| } |  |

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).