| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0193 |
| --- | --- |

| Title: | AHG9: Bugfix and Cleanup on ph_no_output_of_prior_pics_flag and ph_gdr_or_irap_pic_flag |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi, Stephan Wenger Shan Liu (Tencent) 2747 Park Blvd, Palo Alto, CA 94306, USA Lulin Chen, Chih-Wei Hsu, Shih-Ta Hsiang, Olena Chubach, Yu-Wen Huang, Shaw-Min Lei (MediaTek) No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan |  | {bdchoi, swenger, shanl}@tencent.com {lulin.chen, cw.hsu, shih-ta.hsiang, olena.chubach, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | Tencent, MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution proposes the harmonized solutions of JVET-S0055 proposal-1 and JVET-S0070 proposal-1 for bug fix of ph_no_output_of_prior_pics_flag and cleanups on ph_gdr_or_irap_pic_flag. The following Proposal 1 Option 3 and Proposal 2 are recommended.

Proposal-1: Four options are proposed to resolve the issue that the value of ph_no_output_of_prior_pics_flag is used without inferrence rule, when ph_no_output_of_prior_pics_flag is not present.

- Option-1: Change the semantics of ph_gdr_or_irap_pic_flag to be a two-way

- Option-2: Replace the flag ph_gdr_or_irap_pic_flag with two new flags: ph_irap_pic_flag and ph_gdr_pic_flag with two-way semantics.

- Option-3: Add the following inference: when not present, the value of ph_no_output_of_prior_pics_flag is inferred to be equal to 1.

- Option-4: Move no_output_of_prior_pics_flag to slice header

Proposal-2: When Option-3 or 4 is adopted, it is proposed to replace ph_gdr_or_irap_pic_flag with ph_irap_pic_flag for syntax cleanup.

# Proposals

In JVET HLS interim AhG meeting, JVET-S0055 proposal-1 and JVET-S0070 proposal-1 were discussed and deferred to the JVET main meeting. In order to resolve the issue that the value of ph_no_output_of_prior_pics_flag is used without inferrence rule, when ph_no_output_of_prior_pics_flag is not present. The following four options are proposed as Proposal 1.

- Option-1: Change the semantics of ph_gdr_or_irap_pic_flag to be a two-way

- Option-2: Replace the flag ph_gdr_or_irap_pic_flag with two new flags: ph_irap_pic_flag and ph_gdr_pic_flag with two-way semantics.

- Option-3: Add the following inference: when not present, the value of ph_no_output_of_prior_pics_flag is inferred to be equal to 1.

- Option-4: Move no_output_of_prior_pics_flag to slice header

Four options can address the issue, but option-3 is recommended, because it was noted that one-way semantics was to avoid PH rewriting for BEAM usage. Also, option-4 has an issue with mixed NAL unit types. When pps_mixed_nalu_types_in_pic_flag is equal to 1, a subpicture may have no_output_of_prior_pics_flag, while the other subpicture may not have the flag. The case is conflict with the existing constraint that the value of no_output_of_prior_pics_flag shall be the same in an AU. Therefore, we recommend Option-3 that is the simplest modification.

In addition, as Proposal 2, we propose to replace ph_gdr_or_irap_pic_flag with ph_irap_pic_flag for syntax cleanup. The current semantics of ph_gdr_or_irap_pic_flag is somewhat confused because one-way and two-way constraints are mixed.

Proposed Text (proposal-1 option-1)

ph_gdr_or_irap_pic_flag equal to 1 specifies that the current picture is a GDR or IRAP picture. ph_gdr_or_irap_pic_flag equal to 0 specifies that the current picture is not a GDR or IRAP picture. and may or may not be an IRAP picture.

Proposed Text (proposal-1 option-2)

| picture_header_structure( ) { | Descriptor |
| --- | --- |
| ph_gdr_or_irap_pic_flag ph_irap_pic_flag | u(1) |
| if( ph_gdr_or_irap_pic_flag !ph_irap_pic_flag ) |  |
| ph_gdr_pic_flag | u(1) |
| … |  |
| if( ph_gdr_or_irap_pic_flag ph_irap_pic_flag ¦ ¦ ph_gdr_pic_flag ) |  |
| ph_no_output_of_prior_pics_flag | u(1) |
| … |  |
| } |  |

ph_irap_pic_flag equal to 1 specifies that the current picture is an IRAP picture. ph_irap_pic_flag equal to 0 specifies that the current picture is not an IRAP picture.

Proposed Text (proposal-1 option-3)

ph_no_output_of_prior_pics_flag affects the output of previously-decoded pictures in the DPB after the decoding of a picture in a CVSS AU that is not the first AU in the bitstream as specified in Annex C. When not present, the value of ph_no_output_of_prior_pics_flag is inferred to be equal 1.

Proposed Text (proposal-1 option-4)

| picture_header_structure( ) { | Descriptor |
| --- | --- |
| … |  |
| if( ph_gdr_or_irap_pic_flag ) |  |
| ph_no_output_of_prior_pics_flag | u(1) |
| … |  |
| } |  |

| slice_header( ) { | Descriptor |
| --- | --- |
| sh_picture_header_in_slice_header_flag | u(1) |
| if( sh_picture_header_in_slice_header_flag ) |  |
| picture_header_structure( ) |  |
| … |  |
| if( nal_unit_type == IDR_W_RADL ¦¦ nal_unit_type == IDR_N_LP ¦¦ nal_unit_type == CRA_NUT ¦¦ nal_unit_type == GDR_NUT) |  |
| sh_no_output_of_prior_pics_flag | u(1) |
| … |  |
| } |  |

sh_no_output_of_prior_pics_flag affects the output of previously-decoded pictures in the DPB after the decoding of a picture in a CVSS AU that is not the first AU in the bitstream as specified in Annex C.

It is a requirement of bitstream conformance that, when present, the value of sh_no_output_of_prior_pics_flag shall be the same for all pictures in an AU.

When sh_no_output_of_prior_pics_flag is present in the SHs of the pictures in an AU, the sh_no_output_of_prior_pics_flag value of the AU is the sh_no_output_of_prior_pics_flag value of the pictures in the AU.

When not present, the value of sh_no_output_of_prior_pics_flag is inferred to be equal to 1.

Proposed Text (proposal-2)

| picture_header_structure( ) { | Descriptor |
| --- | --- |
| ph_gdr_or_irap_pic_flag ph_irap_pic_flag | u(1) |
| if( ph_gdr_or_irap_pic_flag !ph_irap_pic_flag ) |  |
| ph_gdr_pic_flag | u(1) |
| … |  |
| if( ph_gdr_or_irap_pic_flagph_irap_pic_flag ) |  |
| ph_no_output_of_prior_pics_flag | u(1) |
| } |  |

ph_irap_pic_flag equal to 1 specifies that the current picture is an IRAP picture. ph_irap_pic_flag equal to 0 specifies that the current picture may or may not be an IRAP picture.

The proponents recommend Proposal-1 Option 3 and Proposal 2.

# Patent rights declaration(s)

Tencent and MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).