| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0200 |
| --- | --- |

| Title: | AHG9: On ph_inter_slice_allowed_flag in GDR picture |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hong-Jheng Jhu, Xiaoyu Xiu, Yi-Wen Chen, Tsung-Chuan Ma, Che-Wei Kuo, Xianglin Wang | Email: | jhuhong-jheng@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

In this contribution, to allow inter-related tools in GDR picture two methods are proposed to prevent signaling one picture as one inter slice allowed picture in picture header when ph_gdr_pic_flag in PH is equal to one: 1) adding one constraint for ph_inter_slice_allowed_flag to be 1 when ph_gdr_pic_flag is equal to 1; 2) conditioning the presence of ph_inter_slice_allowed_flag on ph_gdr_pic_flag being equal to 0. It is claimed that this modification makes the specification text cleaner as it includes the spirit of the intent of the GDR design.

# Introduction

In VVC draft 9[1], ph_inter_slice_allowed_flag equal to 0 specifies that all coded slices of the picture have sh_slice_type equal to 2. ph_inter_slice_allowed_flag equal to 1 specifies that there may or may not be one or more coded slices in the picture that have sh_slice_type equal to 0 or 1. During the last JVET meeting, it was commented that the spirit of the intent of the GDR design is to allow inter-related tools in GDR picture.

On the other hand, in VVC draft 9[1], two flags, namely ph_gdr_or_irap_pic_flag and ph_gdr_pic_flag, are signaled at the beginning of picture header to indicate whether one current picture is an IRAP/GDR picture. When ph_gdr_or_irap_pic_flag is equal to 1 and ph_gdr_pic_flag is equal to 0, the current picture is an IRAP picture. When ph_gdr_or_irap_pic_flag is equal to 1 and ph_gdr_pic_flag is equal to 1, the current picture is a GDR picture. In the current specification, there is a constraint between ph_gdr_pic_flag and ph_inter_slice_allowed_flag when ph_gdr_or_irap_pic_flag is equal to 1 and ph_gdr_pic_flag is equal to 0. However, when ph_gdr_or_irap_pic_flag is equal to 1 and ph_gdr_pic_flag is equal to 1, there is no constraint between ph_gdr_pic_flag and ph_inter_slice_allowed_flag.

# Proposal

To resolve the above problem, two solutions are proposed in this contribution: 1) adding one constraint for ph_inter_slice_allowed_flag to be 1 when ph_gdr_pic_flag is equal to 1; 2) conditioning the presence of ph_inter_slice_allowed_flag on ph_gdr_pic_flag being equal to 0. The changes to the VVC working draft are highlighted in yellow below.

## Solution One

In the first solution, it is proposed to add one constraint for ph_inter_slice_allowed_flag to be 1 when ph_gdr_pic_flag is equal to 1.

ph_inter_slice_allowed_flag equal to 0 specifies that all coded slices of the picture have sh_slice_type equal to 2. ph_inter_slice_allowed_flag equal to 1 specifies that there may or may not be one or more coded slices in the picture that have sh_slice_type equal to 0 or 1

When ph_gdr_or_irap_pic_flag is equal to 1 and ph_gdr_pic_flag is equal to 0 (i.e., the picture is an IRAP picture), and vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 1, the value of ph_inter_slice_allowed_flag shall be equal to 0.

When ph_gdr_pic_flag is equal to 1, the value of ph_inter_slice_allowed_flag shall be equal to 1

## Solution Two

In the second solution, it is proposed to make the presence of ph_inter_slice_allowed_flag conditioned on the value of ph_gdr_pic_flag being equal to 0, and add the inference that ph_inter_slice_allowed_flag is equal to 1 when not present.

| picture_header_structure( ) { | Descriptor |
| --- | --- |
| if(!ph_gdr_pic_flag ) |  |
| ph_inter_slice_allowed_flag | u(1) |

ph_inter_slice_allowed_flag equal to 0 specifies that all coded slices of the picture have sh_slice_type equal to 2. ph_inter_slice_allowed_flag equal to 1 specifies that there may or may not be one or more coded slices in the picture that have sh_slice_type equal to 0 or 1. When ph_inter_slice_allowed_flag is not present, it is inferred to be 1.

When ph_gdr_or_irap_pic_flag is equal to 1 and ph_gdr_pic_flag is equal to 0 (i.e., the picture is an IRAP picture), and vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 1, the value of ph_inter_slice_allowed_flag shall be equal to 0.

# Reference

B. Bross, J. Chen, S. Liu and Y.-K. Wang, “Versatile Video Coding (Draft 9)”, JVET-R2001-vA, the 18th JVET meeting, teleconference, 15–24 April 2020.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).