| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0201-v1 |
| --- | --- |

| Title: | AHG9: On signaling the JCCR sign information in PH |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Che-Wei Kuo, Xiaoyu Xiu, Hong-Jheng Jhu, Yi-Wen Chen, Tsung-Chuan Ma, Xianglin Wang | Email: | guozhewei@kuaishou.com xiaoyuxiu@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

In common test conditions, JCCR positive sign is used in only 3 over 26 sequences. This contribution proposes to signal an SPS flag to indicate whether ph_joint_cbcr_sign_flag is present in PH. When ph_joint_cbcr_sign_flag is not present, the value of ph_joint_cbcr_ sign_flag is inferred to be equal to 1.

# Proposal

The proposed changes to the VVC working draft are highlighted in yellow.

7.3.2.3 Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if( ChromaArrayType != 0 ) { |  |
| sps_joint_cbcr_enabled_flag | u(1) |
| if( sps_joint_cbcr_enabled_flag ) |  |
| sps_joint_cbcr_info_present_in_ph_flag | u(1) |
| … |  |
| sps_bdof_enabled_flag | u(1) |
| if( sps_bdof_enabled_flag ) |  |
| sps_bdof_control_present_in_ph_flag | u(1) |
| sps_smvd_enabled_flag | u(1) |
| sps_dmvr_enabled_flag | u(1) |
| if( sps_dmvr_enabled_flag) |  |
| sps_dmvr_control_present_in_ph_flag | u(1) |
| sps_mmvd_enabled_flag | u(1) |
| six_minus_max_num_merge_cand | ue(v) |
| sps_sbt_enabled_flag | u(1) |
| sps_affine_enabled_flag | u(1) |
| if( sps_affine_enabled_flag ) { |  |
| five_minus_max_num_subblock_merge_cand | ue(v) |
| sps_affine_type_flag | u(1) |
| if( sps_amvr_enabled_flag ) |  |
| sps_affine_amvr_enabled_flag | u(1) |
| sps_affine_prof_enabled_flag | u(1) |
| if( sps_affine_prof_enabled_flag ) |  |
| sps_prof_control_present_in_ph_flag | u(1) |
| } |  |
| … |  |
| } |  |

7.3.2.7 Picture header structure syntax

| picture_header_structure( ) { | Descriptor |
| --- | --- |
| … |  |
| if( !rpl_info_in_ph_flag ¦ ¦ num_ref_entries[ 1 ][ RplsIdx[ 1 ] ] > 0 ) { |  |
| mvd_l1_zero_flag | u(1) |
| if( sps_bdof_control_present_in_ph_flag ) |  |
| ph_disable_bdof_flag | u(1) |
| if( sps_dmvr_control_present_in_ph_flag ) |  |
| ph_disable_dmvr_flag | u(1) |
| } |  |
| if( sps_prof_control_present_in_ph_flag ) |  |
| ph_disable_prof_flag | u(1) |
| if( ( pps_weighted_pred_flag ¦ ¦ pps_weighted_bipred_flag ) && wp_info_in_ph_flag ) |  |
| pred_weight_table( ) |  |
| } |  |
| if( qp_delta_info_in_ph_flag ) |  |
| ph_qp_delta | se(v) |
| if( sps_joint_cbcr_enabled_flag sps_joint_cbcr_info_present_in_ph_flag) |  |
| ph_joint_cbcr_sign_flag | u(1) |
| if( sps_sao_enabled_flag && sao_info_in_ph_flag ) { |  |
| ph_sao_luma_enabled_flag | u(1) |
| if( ChromaArrayType != 0 ) |  |
| ph_sao_chroma_enabled_flag | u(1) |
| } |  |
| if( sps_dep_quant_enabled_flag ) |  |
| ph_dep_quant_enabled_flag | u(1) |
| if( sps_sign_data_hiding_enabled_flag && !ph_dep_quant_enabled_flag ) |  |
| ph_sign_data_hiding_enabled_flag | u(1) |
| … |  |
| } |  |

7.4.3.3 Sequence parameter set RBSP semantics

sps_joint_cbcr_enabled_flag equal to 0 specifies that the joint coding of chroma residuals is disabled. sps_joint_cbcr_enabled_flag equal to 1 specifies that the joint coding of chroma residuals is enabled. When not present, the value of sps_joint_cbcr_enabled_flag is inferred to be equal to 0.

sps_joint_cbcr_info_present_in_ph_flag equal to 1 specifies that ph_joint_cbcr_sign_flag is present in PHs referring to the SPS. sps_joint_cbcr_info_present_in_ph_flag equal to 0 specifies that ph_joint_cbcr_sign_flag is not present in PHs referring to the SPS. When sps_joint_cbcr_info_present_in_ph_flag is not present, the value of sps_joint_cbcr_info_present_in_ph_flag is inferred to be equal to 0.

7.4.3.7 Picture header structure semantics

ph_joint_cbcr_sign_flag specifies whether, in transform units with tu_joint_cbcr_residual_flag[ x0 ][ y0 ] equal to 1, the collocated residual samples of both chroma components have inverted signs. When tu_joint_cbcr_residual_flag[ x0 ][ y0 ] equal to 1 for a transform unit, ph_joint_cbcr_sign_flag equal to 0 specifies that the sign of each residual sample of the Cr (or Cb) component is identical to the sign of the collocated Cb (or Cr) residual sample and ph_joint_cbcr_sign_flag equal to 1 specifies that the sign of each residual sample of the Cr (or Cb) component is given by the inverted sign of the collocated Cb (or Cr) residual sample. When ph_joint_cbcr_sign_flag is not present, the value of ph_joint_cbcr_sign_flag is inferred to be equal to 1.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).