| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET- S0213 |
| --- | --- |

| Title: | AHG8/AHG9: Refinement of proposed positioning information SEI message of output independent layers with example bitstreams |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Emmanuel Thomas Anna van Buerenplein 1 2595 DA Den Haag The Netherlands | Email: | emmanuel.thomas@tno.nl |
| Source: | TNO |  |  |

_____________________________

# Abstract

This contribution is following up on JVET-R0307, JVET-R0308 and JVET-S0108. It is asserted that the contribution aims at supporting use cases where a multi-layer bitstream with independent layers can be used. Multi-party video conferencing is considered to be a scenario where multiple participant views can be encoded as independent layers of a single multi-layer bitstream.

This contribution proposed the definition of a SEI message to signal the positioning information of each output layer for a given output layer set into a final output picture. Based on the discussion related to JVET-R0307 at JVET #18, the following changes are introduced in this contribution:

- All the layers in an OLS are connected to the same 4-connected graph, i.e. no orphan layer.

- The process creating the final output picture clarifies that cropped decoded pictures are used and not decoded pictures.

- An informative algorithm describes how to calculate the position of each cropped decoded picture in the final output picture

JVET- S0214 provides the updated software implementation in VTM based on JVET-R0308 corresponding to the presented syntax and operations in the present contribution.

Lastly, example bitstreams with different arrangements are provided in the present contribution.

# Introduction

This contribution is following up on JVET-R0307 [3], JVET-R0308 [4] and JVET-S0108 [2]. The purpose of the contribution is to enable the support of multi-layer bitstream use cases with independent layers. For example, multi-party video conferencing is a suitable candidate scenario where multiple participant views can be encoded as part as independent layers of a single multi-layer bitstream.

To this end, this contribution proposed the definition of a new SEI message to signal the positioning information of each output layer for a given output layer set in a final output picture.

Based on the past discussions, the following changes were made:

- All the layers in an OLS are connected to the same 4-connected graph, i.e. no orphan layer.

- The process creating the final output picture clarifies that cropped decoded pictures are used and not decoded pictures.

- An informative algorithm describes how to calculate the position of each cropped decoded picture in the final output picture.

As a reminder, JVET-R0308 [4] provided the first software implementation in VTM corresponding to the presented syntax in JVET-R0307 [3].

- Implementation of a multi-layer decoding support

- Arrangement into a single picture of independent output layers of the same output layer set

JVET- S0214 [1] provides details updates on this software implementation.

Lastly, example bitstreams with different arrangements are provided in the present contribution.

# Proposed syntax

Table D.1 – Persistence scope of SEI messages (informative)

| SEI message | Persistence scope |
| --- | --- |
| Buffering period | The remainder of the bitstream |
| Picture timing | The AU containing the SEI message |
| DU information | The AU containing the SEI message |
| Scalable nesting | Depending on the scalable-nested SEI messages. Each scalable-nested SEI message has the same persistence scope as if the SEI message was not scalable-nested |
| Subpicture level information | The CLVS containing the SEI message |
| Independent layer info | The CLVS containing the SEI message |

### D.x Independent layer info SEI message syntax

### D.x.1 Independent layer info SEI message syntax

| independent_layer_info( payloadSize ) { | Descriptor |
| --- | --- |
| boundary_identifier_north_present_flag | u(1) |
| if(boundary_identifier_north_present_flag) |  |
| boundary_identifier_north | u(16) |
| boundary_identifier_east_present_flag | u(1) |
| if(boundary_identifier_east_present_flag) |  |
| boundary_identifier_east | u(16) |
| boundary_identifier_south_present_flag | u(1) |
| if(boundary_identifier_south_present_flag) |  |
| boundary_identifier_south | u(16) |
| boundary_identifier_west_present_flag | u(1) |
| if(boundary_identifier_west_present_flag) |  |
| boundary_identifier_west | u(16) |
| } |  |

boundary_identifier_north_present_flag, boundary_identifier_east_present_flag, boundary_identifier_south_present_flag and boundary_identifier_west_present_flag equal to 1 specify that the SEI message contains a boundary identifier, respectively, for the north, east, south and west boundary.

boundary_identifier_north, boundary_identifier_east, boundary_identifier_south and boundary_identifier_west specifiy the boundary identifier, respecitvely, at the north, east, south and west boundary of the decoded picture of the associated layer; the associated layer being the layer whose nuh_layer_id is equal to the nuh_layer_id of the SEI message. If not present, the boundary identifiers respecitvely at the north, east, south and west boundary of the decoded picture of the associated layer are not defined.

Figure 1 - Representation of where the bououndaries are on the layers

For two layers, the i-th and j-th layers, when the pair of the boundary_identifier_north value of the i-th layer and the boundary_identifier_south value of the j-th layer are equal then the decoded picture of the i-th layer and the decoded picture of the j-th layer are adjacent in the composed output picture and they share a common boundary at the boundary north/south. For i-th and j-th layers, when the pair of the boundary_identifier_east value of the i-th layer and the boundary_identifier_west value of the j-th layer are equal then the decoded picture of the i-th layer and the decoded picture of the j-th layer are adjacent in the composed output picture and they share a common boundary at the boundary east/west.

Two decoded pictures adjacent by the north/south boundary are aligned on their west boundary in the final output picture. Two decoded pictures adjacent by the east/west boundary are aligned on their north boundary in the final output picture.

All the Independent layer info SEI message present in layers of an OLS shall collectively describe a 4-connected graph and each layer of the OLS shall be connected to the graph.

### D.x.2 Process for generating the aggregated output picture

The process for generating the final output picture is informative. The following section provides the expected operations performed for generating the final output picture based on the decoded pictures of each layer from a selected OLS:

- For each Access Unit:

  - If VPS present, parse VPS and store the list of layers in the bitstream.

  - For each present PPS, determine the size in luma samples of the corresponding layer.

  - For each present Independent layer info SEI, parse the payload and store the boundary identifiers for the corresponding layer.

  - If any of VPS, PPS or Independent layer info SEI is present in the current Access Unit, execute calculate_layer_layout_in_final_output_picture(), that returns the horizontal, XPos, and vertical, YPos, position of the top-left corner of each cropped decoded picture per layer in the final output picture.

  - Initialize a picture buffer of size FinalWidth of width and FinalHeight of height for the final output picture.

  - For each Picture Unit:

    - Decode the coded picture.

  - If pictures are ready for output:

    - For each layer in selected OLS.

      - Apply conformance window cropping on the decoded picture of the current layer.

      - Retrieve XPos and YPos, the positions of the current layer in the final output picture in luma sample.

      - Copy cropped decoded picture in final output picture buffer at position (XPos, YPos) corresponding to the top-left corner of the cropped decoded picture.

### D.x.2.1 Process for calculating the layer layout in the final output picture

Input to this process are:

- the list of layer identifiers included in the VPS, LayerIdList

- the list of cropped picture size of each layer, LayerPicSizeList

- the boundary identifiers for each layer.

Outputs of this process are:

- the horizontal and vertical position in luma sample of the cropped decoded picture in each layer in the final output picture, [{layer_id, XPos, YPos}][NbLayer]

- the size in luma samples of the final output picture, {FinalWidth, FinalHeight}

NOTE: There may be several ways to implement the function that returns the correct output given the input above. The following is an exemplary algorithm.

The steps of the process are:

- Select a layer as starting point, whose layer identifier is layer0. The top-left corner of this layer will be the origin for calculating the coordinates top-left corner of each layer.

- Set curLayerId to layer0.

- Until all layers are traversed, do:

  - Set XPos and Ypos for the current layer using LayerPicSizeList[curLayerId].

NOTE: For layer0, XPos are Ypos set to ‘0’. Otherwise, the rule defined in the semantic defines hwo to derive XPos and YPos. That is, for two vertical neighbors (North or South), the top-left corner of both layer have the same horizontal coordinate. Similarly, for two horizonatal neighbors (East or West), the top-left corner of both layer have the same vertical coordinate. The second component of the top-left corner position, vertical or horizontal, is determined by using the width or height of the cropped decoded picture of the layers.

  - Find a neaighbor of the current layer, and set curLayerId to this layer id.

- At this point XPos and YPos are relative to the top-left corner layer0. Apply a change of origin such that all the XPos and YPos values are positive or null.

- Set FinalWidth to the maximum value calculated by summing each row of grid of horizontal neighbor layers (layers having the same YPos)

- Set FinalHeight to the maximum value calculated by summing each height of vertical neighbor layers (layers having the same XPos)

# Example of an output picture construction

The image below has an example of how layer alignment would look line. Note that at this stage, the figure shows the layer in a normalized fashion since the picture size is orthogonal to the signaling of connections.

Figure 2 - How layers relate to each other spatially based with examples of their identifiers

When the values of two different layers’ boundary_identifier_north/boundary_identifier_south or boundary_identifier_east/boundary_identifier_west are equal, then the decoded picture of the first and second layer are adjacent in the output picture and they share a common boundary respectively at the north/south or east/west boundary. In addition, the semantic defines that two adjacent layers by the east/west boundary are aligned on their north boundary in the output final picture. Two adjacent layers by the north/south boundary are aligned on their west boundary in the output final picture.

For each layer, the following sizes are used to arrange the final output picture:

conf_win_width=pps_pic_width_in_luma_samples - SubWidthC ×pps_conf_win_left_offset + pps_conf_win_right_offset

conf_win_height=pps_pic_height_in_luma_samples - SubWidthC ×pps_conf_win_top_offset + pps_conf_win_bottom_offset

Also, in the example of Figure 4, the following coordinates will be calculated based on the process described previously:

- Select layer identifier layer1 as starting point.

- For Layer 1, XPos = 0, YPos = 0.

- Layer 1 has two neighbors, decide to go to Layer 2.

- For Layer 2, XPos = 0 (because vertical neighbor), YPos =  Layer2Height.

- Layer 2 has no further neighbor. Going back to Layer 1, and decide to go to Layer 3.

- For Layer 3, XPos =  Layer3Width, YPos = 0 (because horizontal neighbor).

- We have :

  - Layer 1 { XPos = 0, YPos = 0}

  - Layer 2 { XPos = 0, YPos =  Layer2Height }

  - Layer 3 { XPos =  Layer3Width, YPos = 0}

- We reset the origin so that all coordinates are positive or null.

- We have :

  - Layer 1 { XPos = Layer3Width, YPos = Layer2Height }

  - Layer 2 { XPos = Layer3Width, YPos = 0}

  - Layer 3 { XPos = 0, YPos = Layer2Height }

- We set FinalWidth and FinalHeight

  - FinalWidth = max(Layer2Width + Layer3Width, Layer1Width + Layer3Width)

  - FinalHeight = max(Layer2Height + Layer3Height, Layer1Height + Layer2Height)

# Example bitstreams

The example provided with the contribution are listed in Table 2.

Table 2 - List of examples

| # | Composed picture | GOP structure | FPS | Bit depth | Number of frames |
| --- | --- | --- | --- | --- | --- |
| 1 |  | intra | same in each layer | same in each layer | same in each layer |
| 2 |  | lowdelay_P | same in each layer | same in each layer | same in each layer |
| 3 |  | randomaccess | same in each layer | same in each layer | same in each layer |
| 4 |  | intra | same in each layer | same in each layer | same in each layer |
| 5 |  | intra | same in each layer | same in each layer | same in each layer |
| 6 |  | intra | same in each layer | same in each layer | same in each layer |
| 7 |  | lowdelay_P | different | same in each layer | different |

## Example 1

File name: BasketballPass_BlowingBubbles_left_right_intra.vvc

Encoded layers:

| Layer | Sequence name | Resolution | Encoding configuration | Boundary identifiers (N, E, S, W) |
| --- | --- | --- | --- | --- |
| 0 | BasketballPass | 416 x 240 | encoder_intra_vtm.cfg | 0 1 0 0 |
| 1 | BlowingBubbles | 416 x 240 | encoder_intra_vtm.cfg | 0 0 0 1 |

Signaled connected map:

| Layer 0 1 | Layer 1 1 |
| --- | --- |

Decoding output:

Resolution

832 x 240

Screenshot

## Example 2

File name: BasketballPass_BlowingBubbles_left_right_lowdelay_P.vvc

Encoded layers:

| Layer | Sequence name | Resolution | Encoding configuration | Boundary identifiers (N, E, S, W) |
| --- | --- | --- | --- | --- |
| 0 | BasketballPass | 416 x 240 | encoder_lowdelay_P_vtm.cfg | 0 1 0 0 |
| 1 | BlowingBubbles | 416 x 240 | encoder_lowdelay_P_vtm.cfg | 0 0 0 1 |

Signaled connected map:

| Layer 0 1 | Layer 1 1 |
| --- | --- |

Decoding output:

Resolution

832 x 240

Screenshot

## Example 3

File name: BasketballPass_BlowingBubbles_left_right_randomaccess.vvc

Encoded layers:

| Layer | Sequence name | Resolution | Encoding configuration | Boundary identifiers (N, E, S, W) |
| --- | --- | --- | --- | --- |
| 0 | BasketballPass | 416 x 240 | encoder_randomaccess_vtm.cfg | 0 1 0 0 |
| 1 | BlowingBubbles | 416 x 240 | encoder_randomaccess_vtm.cfg | 0 0 0 1 |

Signaled connected map:

| Layer 0 1 | Layer 1 1 |
| --- | --- |

Decoding output:

Resolution

832 x 240

Screenshot

## Example 4

File name: BasketballPass_BlowingBubbles_BasketballDrill_left_right_bottom_intra.vvc

Encoded layers:

| Layer | Sequence name | Resolution | Encoding configuration | Boundary identifiers (N, E, S, W) |
| --- | --- | --- | --- | --- |
| 0 | BasketballPass | 416 x 240 | encoder_intra_vtm.cfg | 0 1 2 0 |
| 1 | BlowingBubbles | 416 x 240 | encoder_intra_vtm.cfg | 0 0 0 1 |
| 2 | BasketballDrill | 832x480 | encoder_intra_vtm.cfg | 2 0 0 0 |

Signaled connected map:

| Layer 0 1 2 |  |  | Layer 1 1 |
| --- | --- | --- | --- |
| 2 Layer 2 |  |

Decoding output:

Resolution

832 x 720

Screenshot

## Example 5

File name: BasketballPass_BlowingBubbles_BasketballDrill_left_right_bottom_intra.vvc

Encoded layers:

| Layer | Sequence name | Resolution | Encoding configuration | Boundary identifiers (N, E, S, W) |
| --- | --- | --- | --- | --- |
| 0 | BasketballPass | 416 x 240 | encoder_intra_vtm.cfg | 0 1 0 0 |
| 1 | BlowingBubbles | 416 x 240 | encoder_intra_vtm.cfg | 0 0 2 1 |
| 2 | BasketballPass | 416 x 240 | encoder_intra_vtm.cfg | 2 0 0 0 |

Signaled connected map:

| Layer 0 1 | Layer 1 1 2 |
| --- | --- |
|  | 2 Layer 2 |

Decoding output:

Resolution

832 x 480

Screenshot

## Example 6

File name: BasketballPass_BlowingBubbles_BasketballDrill_left_right_bottom_intra.vvc

Encoded layers:

| Layer | Sequence name | Resolution | Encoding configuration | Boundary identifiers (N, E, S, W) |
| --- | --- | --- | --- | --- |
| 0 | BasketballPass | 416 x 240 | encoder_intra_vtm.cfg | 0 1 2 0 |
| 1 | BlowingBubbles | 416 x 240 | encoder_intra_vtm.cfg | 2 0 0 0 |
| 2 | BasketballDrill | 832 x 480 | encoder_intra_vtm.cfg | 0 0 0 1 |

Signaled connected map:

| Layer 0 1 2 | Layer 2 1 |
| --- | --- |
| 2 Layer 1 |  |

Decoding output:

Resolution

1248 x 480

Screenshot

## Example 7

File name: BQSquare_RaceHorses_top_bottom_lowdelay_P.vvc

Encoded layers:

| Layer | Sequence name | Resolution | Nb frames | Encoding configuration | Boundary identifiers (N, E, S, W) |
| --- | --- | --- | --- | --- | --- |
| 0 | BQSquare | 416 x 240 | 600 | encoder_lowdelay_P_vtm.cfg | 0 0 1 0 |
| 1 | RaceHorses | 416 x 240 | 300 | encoder_lowdelay_P_vtm.cfg | 1 0 0 0 |

Signaled connected map:

Layer 0 1

1 Layer 1

Decoding output:

Resolution

416 x 480

Screenshot

# Conclusion

It is proposed to adopt the proposed SEI message for signaling the aggregation of cropped decoded pictures of layers of an OLS into a final output picture.

# References

- JVET-S0214, “AHG8/AHG9: Updates on the implementation of multi-layer decoding and output independent layer arrangement in VTM”, Emmanuel Thomas, Karim El Assal, Alexandre Gabriel (TNO)

- JVET-S0108, “AHG8/AHG9: Refinement of proposed positioning information SEI message of output independent layers”, Emmanuel Thomas (TNO)

- JVET-R0307, “AHG8/AHG9: Positioning information SEI message of output independent layers”, Emmanuel Thomas (TNO)

- JVET-R0308, “AHG8: Implementation of multi-layer decoding and output independent layer composition in VTM”, Emmanuel Thomas, Karim El Assal, Alexandre Gabriel (TNO)

# Patent rights declaration(s)

TNO may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).