| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0218 |
| --- | --- |

| Title: | 4K HLG test sequences for HDR verification test |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | S. Iwamura S. Nemoto A. Ichigaya | Tel: Email: | iwamura.s-gc@nhk.or.jp |
| Source: | NHK |  |  |

_____________________________

- Abstract

This contribution provides new 4K test sequences with Hybrid Log-Gamma (HLG) transfer characteristics. A total of 12 sequences are provided as candidates for VVC HDR verification test. All the sequences are captured in 8K cameras without pre-compression and down converted into 4K resolution using HDRTools. Note that the chroma location type of those provided sequences is type-2. In the 18th JVET meeting, it was pointed out that the chroma location type of existing 7 HLG test sequences (H2_HLG1 to H2_HLG7) was type-0 and the preference of recreation of those sequences as type-2 sequences was expressed. As per the request, the recreated existing sequences as type-2 are also provided in this contribution. Furthermore, during intermediate AHG meeting between 18th and 19th JVET meeting, 5 candidate sequences and suggested QP range were selected. The coding experiments for those sequences with the selected QP range are also provide.

Test sequences

The provided sequences are all 4K resolution while the original captured sequences are 8K resolution. For the down conversion, HDRTools was used using the attached config file (HDRConvertTIFF2YUV_ClassH3.cfg) with the following command line option:

HDRConvert.exe -f HDRConvertTIFF2YUV_ClassH3.cfg -p OutputWidth=3840 -p OutputHeight=2160 -p OutputChromaLocationTop=2 -p OutputChromaLocationBottom=2 -p StartFrame=0 -p OutputFile="H3_AMSxx_3840x2160_10bit_420_HLG.yuv"

All the provided sequences are produced by NHK and all intellectual property rights remain with NHK. The copyright and disclaimer are attached in individual zip files, which will be made available later. Upon duplication of the data, all the frames including the copyright and disclaimer should be copied.

The sequence has been generated by Japan Broadcasting Corporation (NHK), and all intellectual property rights remain with NHK. The material shall only be used for developing future video coding standards, for evaluating performance of test models in JVET and the subsequent standardization project, and for the relational parent body activities. The sequence should be processed in the original 4K resolution. It is not allowed to modify the characteristics of test materials including transfer characteristics and colorimetric characteristics.

This copyright and permission notice shall be duplicated whenever the data are copied or distributed. The material cannot be distributed with charge. NHK makes no warranties with respect to the material and expressly disclaims any warranties regarding its fitness for any purpose. Unless the above conditions are agreed to by the recipient, no permission is granted for any use, copying, and distribution of the data. By using the sequence, the user agrees to the conditions of this copyright and disclaimer.

New test sequences

The specifications of 8K camera used for shooting are listed Table 1.

Table 1. Specifications of 8K camera

| Model | SONY UHC-8300 |
| --- | --- |
| Censor | 1.25-inch 3-CMOS |
| Color space | ITU-R Rec. BT.2020 |
| Frame rate | 59.94/60 Hz (progressive) |
| Resolution | 7680x4320 |
| Chroma format | YCbCr 4:2:2 |
| Bit depth | 10bit |
| Transfer characteristics | ITU-R Rec. BT-2100-HLG |

Table 2. Thumbnails

| Title | Thumbnail |
| --- | --- |
| H3_AMS01 |  |
| H3_AMS02 |  |
| H3_AMS03 |  |
| H3_AMS04 |  |
| H3_AMS05 |  |
| H3_AMS06 |  |
| H3_AMS07 |  |
| H3_AMS08 |  |
| H3_AMS09 |  |
| H3_AMS10 |  |
| H3_AMS11 |  |
| H3_AMS12 |  |

The coding experiments using HM-16.20 using QP raging from 22 to 50 (every 2 QP) is summarized in Figure 1.

Figure 1. RD curve of H3_AMS sequences

Recreated existing sequences as type-2

The existing sequences are initially provided in JVET-B0086. The specifications of 8K camera used for capturing those sequences are listed in Table 3.

Table 3. Specification of 8K camera

| Spec. | 8K dual green camera |
| --- | --- |
| Model/Manufacturer | SK-UHD8060/Hitachi-Kokusai |
| Pickup device | 1-chip Super 35mm type CMOS |
| Frame rate | 59.94fps, progressive |
| Bit depth | 12bit |
| Video range | Limited |
| Signal format | RGB RAW |
| Post-processing | AVC-Intra (Approx. 1.5Gbps) |

Table 4. Thumbnail

| H3_FlyingBirds3 |  |
| --- | --- |
| H3_DriftIce |  |
| H3_RiverByBoat |  |
| H3_WaterFallWide |  |
| H3_WaterFallClose |  |
| H3_GreenLeaves |  |
| H3_SunsetBeach3 |  |

Coding experiments using suggested QP range

During intermediate AHG meeting between 18th and 19th JVET meeting, 5 candidate sequences and suggested QP range were selected as follows:

Selected sequences:

H3_RiverByBoat (previously H2_HLG3)

H3_WaterFallWide (previously H2_HLG4)

H3_AMS01

H3_AMS03

H3_AMS09

QP range:

HM: {24, 30, 34, 40, 44}

VTM: {27, 32, 37, 42, 46}

Patent rights declaration(s)

NHK does not have any current or pending patent rights relating to the technology described in this contribution.