| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0234 |
| --- | --- |

| Title: | Mismatch between text specification and reference software on chroma residual scaling when ACT is enabled |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu, Yi-Wen Chen, Hong-Jheng Jhu, Tsung-Chuan Ma, Che-Wei Kuo, Xianglin Wang | Email: | xiaoyuxiu@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

- Abstract

This contribution reports one mismatch between VVC draft 9 and test model software VTM-9.0 on chroma residual scaling when the ACT is applied. In VVC draft 9, when the ACT is applied to one CU, it residual samples are firstly converted from YCgCo space to RGB space before the chroma residual scaling is applied. However, in VTM-9.0, the chroma residual samples are directly scaled in the YCgCo space which are then converted back to the RGB space. Because the reconstructed luma samples used for determining chroma residual scaling factors are made in the RGB domain, the chroma residual scaling should be applied in the same domain as well. Therefore, in this contribution, software patches are provided to align the software with the specification. Simulation results reportedly show that the proposed changes lead to average {G, B, R} BD-rate impacts of {-0.41%, 0.05%, -0.34%}, {-0.49%, -0.02%, -0.51%} and {xx%, xx%, xx%} for AI, RA and LDB configurations, respectively, for RGB camera-captured contents.

Problem statement and proposal

In VVC draft 9 [1], chroma residual scaling is applied to compensate the quantization difference between luma and chroma signals when in-loop luma mapping is applied to luma signal. When the chroma residual scaling is applied, the corresponding scaling factor of one coding unit (CU) is derived based on the average of reconstructed luma samples above and left to one 64×64 region that the CU belongs to. Meanwhile, adaptive color transform (ACT) is included VVC draft 9 [1] to enhance the efficiency of RGB coding. When the ACT is applied, the conversion between RGB and YCgCo spaces are carried out in residual domain.

In this contribution, one mismatch was identified between VVC draft 9 [1] and test model software VTM-9.0 [2] on the interaction between the chroma residual scaling and the ACT. Specifically, in the current specification, chroma residual scaling is always applied in the original color space. If the ACT is applied to one CU, inverse ACT (IACT) needs to be firstly applied for converting all its residual samples from YCgCo space to RGB space before the corresponding chroma residual samples are further scaled. In other words, the IACT is applied prior to the chroma residual scaling process during decoding process. However, in VTM-9.0, when the ACT is applied, the chroma residual samples of the current CU are directly scaled in YCgCo space, which are then converted back to RGB space for reconstruction.

Given that the reconstructed luma samples used for determining chroma residual scaling factors are in the RGB domain (i.e. G component), the chroma residual scaling shall be applied to scale the chroma residual samples in the same domain as well (i.e., B and R components). Therefore, this contribution proposes to change the reference software to be aligned with the current specification text. Additionally, after moving the chroma residual scaling process after the IACT, the existing chroma CBF flags, i.e., tu_cb_coded_flag and tu_cr_coded_flag, which are signaled for the chroma residuals in YCgCo domain cannot be used to indicate whether there are non-zero residual samples for the B and R components in the RGB domain. Therefore, the existing conditions for the invocation of the chroma residual scaling process in section 8.7.5.3 of VVC draft 9 should be slightly modified by considering the value of cu_act_enabled_flag, i.e.,

- 8.7.5.3 Picture reconstruction with luma dependent chroma residual scaling process for chroma samples

……

- If one or more of the following conditions are true, recSamples[ xCurr + i ][ yCurr + j ] is set equal to Clip1( predSamples[ i ][ j ] + resSamples[ i ][ j ] ):

- ph_chroma_residual_scale_flag is equal to 0.

- sh_lmcs_used_flag is equal to 0.

- nCurrSw * nCurrSh is less than or euqal to 4.

- tu_cb_coded_flag [ xCurr ][ yCurr ] is equal to 0 and tu_cr_coded_flag [ xCurr ][ yCurr ] is equal to 0 and cu_act_enabled_flag[xCurr * SubWidthC][ yCurr * SubHeightC] is equal to 0.

- Otherwise, the following applies:

……

Simulation results

The proposed changes were implemented on top of VTM-9.0 reference software and tested by following the common test conditions for non-420 color formats as specified in [3].

Table 1 BD-rate performance of the proposed changes on RGB camera-captured sequences

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | VTM9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Traffic | -0.89% | 0.01% | -0.90% | 119% | 117% |
| Kimono | -0.25% | 0.00% | -0.62% | 98% | 101% |
| EBULupoCandlelight | 0.00% | 0.00% | 0.00% | 104% | 97% |
| EBURainFruits | 0.00% | 0.00% | 0.00% | 70% | 67% |
| VenueVu | 0.00% | 0.00% | 0.00% | 96% | 95% |
| DucksAndLegs | -0.48% | 0.13% | -0.68% | 98% | 102% |
| OldTownCross | -1.45% | 0.22% | -0.40% | 105% | 100% |
| ParkScene | -0.20% | 0.04% | -0.14% | 98% | 98% |
| Overall | -0.41% | 0.05% | -0.34% | 99% | 100% |

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | VTM9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Traffic | -2.08% | -0.23% | -1.86% | 104% | 106% |
| Kimono | -0.83% | -0.19% | -0.87% | 103% | 104% |
| EBULupoCandlelight | 0.00% | 0.00% | 0.00% | 99% | 103% |
| EBURainFruits | 0.00% | 0.00% | 0.00% | 99% | 97% |
| VenueVu | 0.00% | 0.00% | 0.00% | 99% | 101% |
| DucksAndLegs | -1.35% | -0.12% | -0.77% | 98% | 100% |
| OldTownCross | 0.33% | 0.31% | -0.56% | 103% | 99% |
| ParkScene | 0.01% | 0.06% | -0.06% | 98% | 102% |
| Overall | -0.49% | -0.02% | -0.51% | 99% | 99% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | VTM9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Traffic | -0.45% | 0.09% | -0.61% | 101% | 107% |
| Kimono | -0.44% | 0.02% | -0.45% | 103% | 122% |
| EBULupoCandlelight | 0.00% | 0.00% | 0.00% | 97% | 98% |
| EBURainFruits | 0.00% | 0.00% | 0.00% | 111% | 108% |
| VenueVu | 0.00% | 0.00% | 0.00% | 95% | 92% |
| DucksAndLegs | 0.23% | 0.07% | 0.28% | 64% | 67% |
| OldTownCross | -1.53% | 1.30% | -1.39% | 94% | 92% |
| ParkScene | -0.58% | 0.02% | -0.19% | 98% | 100% |
| Overall | -0.34% | 0.19% | -0.30% | 101% | 99% |

Table 2 BD-rate performance of the proposed changes on SCC camera-captured sequences

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | RGB 4:4:4 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.00% | 0.00% | 0.00% | 100% | 100% |
| TGM 720p | 0.00% | 0.00% | 0.00% | 96% | 94% |
| Animation | -0.31% | 0.09% | -0.16% | 99% | 98% |
| Mixed content | 0.00% | 0.00% | 0.00% | 101% | 98% |
| Overall | -0.07% | 0.02% | -0.03% | 99% | 98% |

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | RGB 4:4:4 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.00% | 0.00% | 0.00% | 101% | 101% |
| TGM 720p | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Animation | -0.16% | -0.05% | -0.27% | 101% | 101% |
| Mixed content | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Overall | -0.03% | -0.01% | -0.06% | 100% | 100% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | RGB 4:4:4 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.00% | 0.00% | 0.00% | 102% | 107% |
| TGM 720p | 0.00% | 0.00% | 0.00% | 97% | 96% |
| Animation | 0.13% | -0.03% | -0.09% | 100% | 96% |
| Mixed content | 0.00% | 0.00% | 0.00% | 98% | 98% |
| Overall | 0.03% | -0.01% | -0.02% | 99% | 99% |

Reference

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 9)”, JVET-R2001, April 2020, Teleconference

- VTM-9.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-9.0.

- Y.-H. Chao, Y.-C. Sun, J. Xu, X. Xu, “JVET common test conditions and software reference configurations for non-4:2:0 colour formats”, JVET-R2013, April 2020, Teleconference

Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).