| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0238 |
| --- | --- |

| Title: | AHG9: GCI for RRC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Karam Naser Fabrice Le Leannec Tangi Poirier Philippe de Lagrange | Tel: Email: | karam.naser@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

In VVC draft 9, a new constraint flag “no_tsrc_constraint_flag” was introduced for defining a profile/sub-profiles where only regular residual coding (RRC) is used, i.e., transform skip residual coding (TSRC) is disabled. It is asserted that that such a profile requires both dependent quantization and sign data hiding to be disabled, which leads to significant coding loss. This contribution proposes instead to modify this flag to restrict RRC usage for transform skip coded blocks. The proposed modification to the specification is just renaming “no_tsrc_constraint_flag” to “no_ts_rrc_constraint_flag” since the current semantics of no_tsrc_constraint_flag already support this proposal.

# Introduction

no_tsrc_constraint_flag was proposed in JVET-R0179 to support profile/sub-profiles that do not switch between RRC and TSRC, where only single method for residual coding of transform-skip blocks is defined. It was proposed to have the GCI flag “no_tsrc_constraint_flag” to disallow the usage of TSRC (RRC only). However, due to the adoption of JVET-R0483, such a profile/sub-profile would require disabling dependent quantization and sign data hiding. This leads to high coding loss that limits the interest if such a profile.

With the same motivation of having no_tsrc_constraint_flag, this contribution proposes having a GCI flag that allows TSRC as a single method of residual coding for transform-skip blocks. The usage of such a flag is for typical lossy scenarios, such as CTC, where dependent quantization or sign data hiding is used, and only TSRC is used for transform-skip blocks.

# Proposed Text Change

According to #1071, the current specification contains a bug regarding no_tsrc_constraint_flag. It is currently written as:

no_tsrc_constraint_flag equal to 1 specifies that sh_ts_residual_coding_disabled_flag shall be equal to 0. no_tsrc_constraint_flag equal to 0 does not impose such a constraint.

The highlighted text, sh_ts_residual_coding_disabled_flag equal to 0, imposes that RRC is disabled for transform skip blocks. This is the inverse of the intended usage of this flag to disable TSRC. For this reason, the change to the specification to define no_ts_rrc_constraint_flag is just renaming this flag. Namely, the proposed changes are:

no_ts_rrc_constraint_flag equal to 1 specifies that sh_ts_residual_coding_disabled_flag shall be equal to 0. no_ts_r rc_constraint_flag equal to 0 does not impose such a constraint.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).