| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0249_v1 |
| --- | --- |

| Title: | AHG3: Bugfix to LMCS with multiple slices in VTM-9.0 encoder |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jongseok Lee Yongjo Ahn | Tel: Email: | +82-2-941-6470 jslee@digitalinsights.co.kr |
| Source: | Digital Insights Inc. |  |  |

_____________________________

- Abstract

In the current VTM, the forward LMCS function applies to an original picture at the encoder-side. If an original picture is divided into multiple slices, tiles, and sub-pictures, it is required to applying the forward LMCS function for each slice, respectively. However, VTM-9.0 encoder repeatedly applies the function to the whole original picture for the coding loop of slices. In this contribution, it is proposed that one-line source code change in VTM-9.0 software to fix this bug.

Problem Statement

In the current VTM, the forward LMCS function applies to an original picture when sh_lmcs_used_flag of a slice is equal to 1 at the encoder-side. If an original picture is divided into multiple slices, tiles, and sub-pictures, it is required to applying the forward LMCS function for each slice, respectively. However, the VTM-9.0 encoder repeatedly applies the forward LMCS function to the whole original picture for the coding loop of slices. Figure 1 and Figure 2 are the original picture and the reconstructed picture of “ParkScene” sequence with the partitioning configuration for 8 slices, 12 tiles, and 2 subpictures. As shown in Fig. 2, The first slice of the picture is correctly coded but not the following slices.

In this contribution, it is proposed that a one-line source code change to solve the problem in VTM-9.0 software.

Figure 1. The original picture for “ParkScene” sequence at POC 0.

Figure 2. The reconstructed picture for the “ParkScene” sequence with the partitioning configuration for 8 slices, 12 tiles, and 2 subpictures at POC 0.

Software change

<EncGOP.cpp, line 2708.>

Patent rights declaration(s)

Digital Insights Inc. does not have any current or pending patent rights relating to the technology described in this contribution.