| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0250 |
| --- | --- |

| Title: | Crosscheck of JVET-S0206 (AHG12: Raster scan order subpictures) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Seethal Paluri Hendry | Email: | seethal.paluri@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This contribution reports crosscheck results of JVET-S0206-v3. It is reported that the software matches the description in JVET-S0206. A signalling bit count savings of 50% is observed for the OMAF example that was verified with both text and software. Additional examples were checked using the text provided.

# Introduction

JVET-S0206 proposes to add a new flag, sps_raster_scan_order_subpics_flag in the SPS, which when equal to 1 is used to specify that subpictures are ordered in raster scan order in each picture in the CLVS. Further to this, it is used to derive the top-left CTU position of the subpictures when sps_raster_scan_order_subpicus_flag is equal to 1. The spec changes are detailed below.

6.5.1.CTB raster scanning, tile scanning, and subpicture scanning processes

…

When sps_raster_scan_order_subpics_flag is equal to 1, the lists SubpicCtuTopLeftX[ i ] and SubpicCtuTopLeftY[ i ], for i ranging from 0 to sps_num_subpics_minus1, inclusive, specifying the horizontal and vertical position of top left CTU of i-th subpicture in unit of CtbSizeY are derived as follows:

for( i = 0; i < PicSizeInCtbsY; i++ ) AssignedCtu[ i ] = 0
for( i = 0; i <= sps_num_subpics_minus1; i++ ) {
	j = 0
	while( AssignedCtu[ j ] ) j++
	SubpicCtuTopLeftX[ i ] = j % PicWidthInCtbsY
	SubpicCtuTopLeftY[ i ] = j / PicWidthInCtbsY
	for( s = SubpicCtuTopLeftX[ i ]; s <= SubpicCtuTopLeftX[ i ] + sps_subpic_width_minus1[ i ]; s++ ) 
		for( t = SubpicCtuTopLeftY[ i ]; t <= SubpicCtuTopLeftY[ i ] + sps_subpic_height_minus1[ i ]; t++ ) 
			AssignedCtu[ s + t * PicWidthInCtbsY ] = 1
}

…

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| subpic_info_present_flag | u(1) |
| if( subpic_info_present_flag ) { |  |
| sps_num_subpics_minus1 | ue(v) |
| if( sps_num_subpics_minus1 > 0 ) { |  |
| sps_raster_scan_order_subpics_flag | u(1) |
| sps_independent_subpics_flag | u(1) |
| } |  |
| for( i = 0; sps_num_subpics_minus1 > 0 && i <= sps_num_subpics_minus1; i++ ) { |  |
| if( i > 0 && !sps_raster_scan_order_subpics_flag && pic_width_max_in_luma_samples > CtbSizeY ) |  |
| subpic_ctu_top_left_x[ i ] | u(v) |
| if( i > 0 && !sps_raster_scan_order_subpics_flag && pic_height_max_in_luma_samples > CtbSizeY ) { |  |
| subpic_ctu_top_left_y[ i ] | u(v) |
| if( i < sps_num_subpics_minus1 && pic_width_max_in_luma_samples > CtbSizeY ) |  |
| subpic_width_minus1[ i ] | u(v) |
| if( i < sps_num_subpics_minus1 && pic_height_max_in_luma_samples > CtbSizeY ) |  |
| subpic_height_minus1[ i ] | u(v) |
| if( !sps_independent_subpics_flag) { |  |
| subpic_treated_as_pic_flag[ i ] | u(1) |
| loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| } |  |
| } |  |
| … |  |
| } |  |
| … |  |
| } |  |

sps_raster_scan_order_subpics_flag equal to 1 specifies that subpictures are ordered in raster scan order in each picture in the CLVS such that for any two integer values i and j in the range of 0 to sps_num_subpics_minus1, inclusive, with j less than i, the position of the top left CTU of the i-th subpicture follows the position of the top left CTU of the j-th subpicture in raster scan order. sps_raster_scan_order_subpics_flag equal to 0 does not impose such a constraint. When not present, the value of sps_raster_scan_order_subpics_flag is inferred to be equal to 0.

sps_subpic_ctu_top_left_x[ i ] specifies horizontal position of top left CTU of i-th subpicture in unit of CtbSizeY. The length of the syntax element is Ceil( Log2( ( sps_pic_width_max_in_luma_samples + CtbSizeY  − 1 )  >>  CtbLog2SizeY ) ) bits. When not present, the value of sps_subpic_ctu_top_left_x[ i ] is inferred to be equal to 0.  (sps_raster_scan_order_subpics_flag ? SubpicCtuTopLeftX[ i ] : 0).

sps_subpic_ctu_top_left_y[ i ] specifies vertical position of top left CTU of i-th subpicture in unit of CtbSizeY. The length of the syntax element is Ceil( Log2( ( sps_pic_height_max_in_luma_samples + CtbSizeY − 1 )  >>  CtbLog2SizeY ) ) bits. When not present, the value of sps_subpic_ctu_top_left_y[ i ] is inferred to be equal to 0.  (sps_raster_scan_order_subpics_flag ? SubpicCtuTopLeftY[ i ] : 0).

# Simulation results

We ran the code provided by the proponent to verify the signalling bit count of Figure 1 presented in JVET-S0206. Figure 1 illustrates a subpicture layout in one OMAF example with picture size 1920 × 4608 and 24 subpictures. In this layout, the size of the larger subpictures is 768 × 768 and the size of the smaller subpictures is 384 × 384 and the CTU size is 128. Our simulation and analysis matches the results provided in the in JVET-S0206-v3. All other results in the proposal presented were checked using the text provided.

Figure 1 OMAF example subpicture layout with 24 subpictures (from Figure D.10 in OMAF 2nd edition)